3GPP TSG-RAN5 Meeting #105	R5-248005r1

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3109 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of Reference sensitivity power level for eRedCap UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The suffix I should align with RAN4 dependency CR to capture eRedCap in the definition, and the Refsens TC for eRedCap should be updated. Also, the n13 is currently missing in eRedCap refsensTC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated 7.1I and 7.3I as the parallel CR of RAN4 Removed the editior’s note of TC 7.3I.3 Added n13 into the Uplink configuration Table 7.3I.3.4.1-3. Updated the message contents. Updated the test requirement tables: Added n13 for it has been introduced into 7.3.2 and removed n18 for it has not been introduced into single carrier TC yet |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The eRedCap Rx TC will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1I 7.3I.3 F.1.3 7.2 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency R4-2420459 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an outcome of R5-247211 R1: Updated clause 7.2 for (e)RedCap definition to align with core spec Updated 7.3.2 for (e)RedCap defnition to align with core spec Updated coversheet of clause affected. |  |  |  |  |  |  |  |  |

<Start of changes>

7.1I	General

For a (e)Redcap UE the requirements in Section 7 shall be verified with the channel bandwidth up to 20MHz and REFSENS specified in clause 7.3I.

<Skipped>

7.2	Diversity characteristics

The UE is required to be equipped with a minimum of two Rx antenna ports in all operating bands except for the bands n7, n38, n41, n77, n78, n79 where the UE is required to be equipped with a minimum of four Rx antenna ports. An exception is allowed for two Rx vehicular UE to be equipped with a minimum of two Rx antenna ports in bands n7, n38, n41, n77, n78, n79. This requirement applies when the band is used as a standalone band or as part of a band combination.

For the single carrier REFSENS requirements in clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports.

For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four Rx antenna ports, otherwise, the UE shall be verified with two Rx antenna ports.

The above rules apply for all subclasses with the exception of clause 7.9.

A (e)RedCap UE is required to be equipped with a minimum of single Rx antenna port and maximum of two Rx antenna ports. Clause 7 requirements for four Rx antenna ports do not apply to a (e)RedCap UE.

<Skipped>

.3.2	Reference sensitivity power level

7.3.2.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3.2.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that don’t support RedCap.

7.3.2.3	Minimum conformance requirements

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.2.3.2, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2.3-1a, 7.3.2.3-1b, Table 7.3.2.3-1c, Table 7.3.2.3-1d and Table 7.3.2.3-2.

Table 7.3.2.3-1a: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

|  | Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band |  | SCS (kHz) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n1 |  | 15 |  | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 | -91.9 |  | -90.6 | -90.1 | -89.6 |
|  |  | 30 |  |  | -97.1 | -95.1 | -94.0 | -92.8 | -92.0 |  | -90.7 | -90.2 | -89.7 |
|  |  | 60 |  |  | -97.5 | -95.4 | -94.2 | -93.0 | -92.1 |  | -90.9 | -90.3 | -89.7 |
| n2 |  | 15 |  | -98 | -94.8 | -93 | -91.8 | -90.7 | -84.1 |  | -81.5 |  |  |
|  |  | 30 |  |  | -95.1 | -93.1 | -92 | -90.8 | -84.2 |  | -81.6 |  |  |
|  |  | 60 |  |  | -95.5 | -93.4 | -92.2 | -90.9 | -84.3 |  | -81.7 |  |  |
| n3 |  | 15 |  | -97.0 | -93.8 | -92.0 | -90.8 | -89.7 | -88.9 | -86.2 | -82.3 | -81.3 | -79.7 |
|  |  | 30 |  |  | -94.1 | -92.1 | -91.0 | -89.8 | -89.0 | -86.3 | -82.4 | -81.4 | -79.8 |
|  |  | 60 |  |  | -94.5 | -92.4 | -91.2 | -90.0 | -89.1 | -86.4 | -82.6 | -81.5 | -79.9 |
| n5 |  | 15 |  | -98.0 | -94.8 | -93.0 | -86.8 | -84.8 |  |  |  |  |  |
|  |  | 30 |  |  | -95.1 | -93.1 | -88.6 | -84.9 |  |  |  |  |  |
| n71 |  | 15 |  | -98.0 | -94.8 | -93.0 | -91.8 | -90.7 | -89.9 |  | -88.6 |  | -81.5 |
|  |  | 30 |  |  | -95.1 | -93.1 | -92.0 | -90.8 | -90.0 |  | -88.7 |  | -81.5 |
|  |  | 60 |  |  | -95.5 | -93.4 | -92.2 | -91.0 | -90.1 |  | -88.9 |  | -81.5 |
| n8 |  | 15 |  | -97.0 | -93.8 | -91.4 | -85.8 |  |  | -78.4 |  |  |  |
|  |  | 30 |  |  | -94.1 | -91.7 | -87.2 |  |  | -78.5 |  |  |  |
| n12 |  | 15 |  | -97.0 | -93.8 | -84.0 |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 | -84.1 |  |  |  |  |  |  |  |
| n13 |  | 15 |  | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 |  |  |  |  |  |  |  |  |
| n14 |  | 15 |  | -97.0 | -93.8 |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 |  |  |  |  |  |  |  |  |
| n20 |  | 15 |  | -97.0 | -93.8 | -91.0 | -89.8 |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.1 | -91.1 | -90.0 |  |  |  |  |  |  |
| n24 |  | 15 |  | -100.0 | -96.8 |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -97.1 |  |  |  |  |  |  |  |  |
|  |  | 60 |  |  | -97.5 |  |  |  |  |  |  |  |  |
| n25 |  | 15 |  | -96.5 | -93.3 | -91.5 | -90.3 | -89.3 | -82.2 | -81.7 | -79.5 | -77.6 |  |
|  |  | 30 |  |  | -93.6 | -91.6 | -90.5 | -89.4 | -82.3 | -81.8 | -79.6 | -77.7 |  |
|  |  | 60 |  |  | -94.0 | -91.9 | -90.7 | -89.6 | -82.4 | -81.9 | -79.7 | -77.8 |  |
| n26 |  | 15 | -99.7 | -97.56 | -94.56 | -92.76 | -87.6 |  |  |  |  |  |  |
|  |  | 30 |  |  | -94.86 | -92.76 | -87.7 |  |  |  |  |  |  |
| n28 |  | 15 | -100.2 | -98.5 | -95.5 | -93.5 | -90.8 |  | -78.5 |  |  |  |  |
|  |  | 30 |  |  | -95.6 | -93.6 | -91.0 |  | -78.6 |  |  |  |  |
| n30 |  | 15 |  | -99.0 | -95.8 |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  | -96.1 |  |  |  |  |  |  |  |  |
| n31 |  | 15 | -95.7 | -93.5 |  |  |  |  |  |  |  |  |  |
| n65 |  | 15 |  | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  | -89.2 |
|  |  | 30 |  |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  | -89.3 |
|  |  | 60 |  |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  | -89.4 |
| n66 |  | 15 |  | -99.5 | -96.3 | -94.5 | -93.3 | -92.2 | -91.4 |  | -90.1 | -89.6 |  |
|  |  | 30 |  |  | -96.6 | -94.6 | -93.5 | -92.3 | -91.5 |  | -90.2 | -89.7 |  |
|  |  | 60 |  |  | -97.0 | -94.9 | -93.7 | -92.5 | -91.6 |  | -90.4 | -89.8 |  |
|  |  | 15 |  | -100.0 | -96.8 | -95.0 | -93.8 | -92.7 |  |  |  |  |  |
| n70 |  | 30 |  |  | -97.1 | -95.1 | -94.0 | -92.8 |  |  |  |  |  |
|  |  | 60 |  |  | -97.5 | -95.4 | -94.2 | -93.0 |  |  |  |  |  |
| n71 |  | 15 |  | -97.2 | -94.0 | -91.6 | -86.0 | -84.18 -74.89 | -82.58 -67.19 | -80.78 -64.09 |  |  |  |
|  |  | 30 |  |  | -94.3 | -91.9 | -87.4 | -84.28 -74.99 | -82.68 -67.29 | -80.88 -64.19 |  |  |  |
| n72 |  | 15 | -95.7 | -93.5 |  |  |  |  |  |  |  |  |  |
| n74 |  | 15 |  | -99.53 | -96.33 | -94.53 | -89.33 |  |  |  |  |  |  |
|  |  | 30 |  |  | -96.63 | -94.63 | -89.53 |  |  |  |  |  |  |
|  |  | 60 |  |  | -97.03 | -94.93 | -89.63 |  |  |  |  |  |  |
| n85 |  | 15 | -99.2 |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  |  |  |  |  |  |  |  |
| n100 |  | 15 | -102.2 | -100 |  |  |  |  |  |  |  |  |  |
| n106 |  | 15 | -99.2 |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for (e)RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9 - 1510.9 MHz. NOTE 4: Void NOTE 5: Void NOTE 6: Values are modified by -0.5dB when carrier channel BW is between 865MHz and 894MHz. NOTE 7: Void. NOTE 8: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 9: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2.3-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS (kHz) | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15 | -97.5 + 10log10(NRB/11) |  |
| n381 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |  |
| n39 | 15 | 5, 10, 15, 20, 25, 30, 40 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40 | -97.5 + 10log10(NRB/11) |  |
| n40 | 15 | 5, 10, 15, 20, 25, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | -97.5 + 10log10(NRB/11) |  |
| n411 | 15 | 10, 15, 20, 30, 40, 50 | -94.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 | -95.5 + 10log10(NRB/11) |  |
| n481 | 15 | 5, 10, 15, 20, 30, 40, 505 | -99 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 505, 605, 705, 805, 905, 1005 | -96.5 + 10log10(NRB/11) |  |
| n50 | 15 | 5, 10, 15, 20, 30, 40, 50 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 30, 40, 50, 60, 80 | -97.5 + 10log10(NRB/11) |  |
| n51 | 15 | 5 | -100 | TDD |
| n53 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 |  |
|  | 60 | 10 | -97.5 |  |
| n54 | 15 | 5 | -100 | TDD |
| n757 | 15 | 5,10,15,20 | -100 + 10log10(NRB/25) | SDL |
|  | 30 | 10,15,20 | -97.1 + 10log10(NRB/24) |  |
|  | 60 | 10,15,20 | -97.5 + 10log10(NRB/11) |  |
| n767 | 15 | 5 | -100 | SDL |
| n771,4 | 15 | 10, 15, 20, 40, 50 | -95.3 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -95.6 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | -96.0 + 10log10(NRB/11) |  |
| n781 | 15 | 10, 15, 20, 25, 30, 40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 15, 20, 40, 50, 60, 70, 80, 90, 100 | -96.5 + 10log10(NRB/11) |  |
| n791 | 15 | 10,20,40, 50 | -95.8 + 10log10(NRB/52) | TDD |
|  | 30 | 10, 20, 40, 50, 60, 80, 100 | -96.1 + 10log10(NRB/24) |  |
|  | 60 | 10, 20, 40, 50, 60, 80, 100 | -96.5 + 10log10(NRB/11) |  |
| n91 | 15 | 5 | -100 | FDD |
| n92 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24) |  |
| n93 | 15 | 5 | -100 | FDD |
| n94 | 15 | 5, 10, 15, 20 | -100 + 10log10(NRB/25) | FDD |
|  | 30 | 10, 15, 20 | -97.1 + 10log10(NRB/24) |  |
| n101 | 15 | 5, 10 | -100 + 10log10(NRB/25) | TDD |
|  | 30 | 10 | -97.1 + 10log10(NRB/24) |  |
| NOTE 1: Four Rx antenna ports shall be the baseline for this operating band except for two Rx vehicular UE. Four Rx antenna ports for (e)RedCap UE is not supported for this operating band. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: For these bandwidths, the minimum requirements are restricted to operation when carrier is configured as a downlink carrier part of CA configuration. NOTE 6: Void NOTE 7: For SDL bands, the reference sensitivity requirements shall be verified by inter-band CA combinations with SDL band, which are supported by UE. NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. |  |  |  |  |

<Skipped>

7.3I	Reference sensitivity for (e)RedCap

7.3I.1	General

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

7.3I.2	Reference sensitivity power level for RedCap

<Skipped>

7.3I.3	Reference sensitivity power level for eRedCap

7.3I.3.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under conditions of low signal level, ideal propagation and no added noise.

7.3I.3.2	Test applicability

This test case applies to all types of NR UE release 18 and forward that support NR eRedCap.

7.3I.3.3	Minimum conformance requirements

For UE supporting IE supportOfERedCap-r18 and IE eRedCapNotReducedBB-BW-r18, the REFSENS requirements for RedCap UE in clause 7.3I.2 apply.

For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18, the reference sensitivity requirements for 5 MHz channel bandwidth with 15 kHz SCS defined in clause 7.3I.2 apply. These reference sensitivity requirements for 5 MHz channel bandwidth apply also for 10, 15 and 20 MHz channel bandwidths with 15 kHz SCS. In case the reference sensitivity requirements for 5 MHz channel bandwidth are not defined, the reference sensitivity requirements for 10 MHz channel bandwidth apply with the reference sensitivity level reduced by 3.2 dB, and the UL configuration shall be less than or equal to minimum between RB number specified in Table 7.3I.2-4  and 25 RBs for HD-FDD operation, and minimum between RB number specified in Table 7.3.2-3 and 25RBs otherwise. These reference sensitivity requirements for 10 MHz channel bandwidth apply also for 15 and 20 MHz channel bandwidths. Both Tx RBs in UL configuration and Rx RBs in FRC, when applicable, shall be allocated within the range from RBlow = ceil(NRB/2 - NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 -1, where RBlow and RBhigh are the lowest and highest available RB position and NRB_PR3 is 25 RBs for 15 kHz SCS.

For UE supporting IE supportOfERedCap-r18 but not supporting eRedCapNotReducedBB-BW-r18, for 30 kHz SCS, the reference sensitivity requirements defined for 10 MHz channel bandwidth in clause 7.3I.2 apply with reference sensitivity level reduced by 3.0 dB and the UL configuration shall be less than or equal to minimum between RB number specified in Table 7.3I.2-4 and 12 RBs for HD-FDD operation, and minimum between RB number specified in Table 7.3.2-3 and 12RBs otherwise. These reference sensitivity requirements for 10 MHz channel bandwidth apply also for 15 and 20 MHz channel bandwidths with 30 kHz SCS. Both Tx RBs in UL configuration and Rx RBs in FRC, when applicable, shall be allocated within the range from RBlow = ceil(NRB/2 - NRB_PR3/2) to RBhigh = RBlow + NRB_PR3 -1, where RBlow and RBhigh are the lowest and highest available RB position and NRB_PR3 is 12 RBs for 30 kHz SCS.

NOTE:	It is not necessary to repeat verification when same requirement applies for multiple channel bandwidths.

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3I.3.

7.3I.3.4	Test description

7.3I.3.4.1	Initial conditions

For eRedCap UE supporting IE eRedCapNotReducedBB-BW-r18, same initial conditions as in 7.3I.2.4.1 with following exception:

For eRedCap UE not supporting eRedCapNotReducedBB-BW-r18, same initial conditions as in 7.3I.2.4.1 with following exception:

-	The test channel bandwidth are specified in TS 38.508-1 [5] subclause 4.3.1 for RedCap.

-	The downlink RB allocation in test configuration table refers to Table 7.3I.3.4.1-1

-	The uplink RB allocation in test configuration table refers to Table 7.3I.3.4.1-2 for FDD and TDD eRedCap UE, and Table 7.3I.3.4.1-3 for HD-FDD eRedCap UE respectively.

Table 7.3I.3.4.1-1: Downlink Configuration of each RB allocation of eRedCap UE not supporting eRedCapNotReducedBB-BW-r18

| Channel Bandwidth | SCS(kHz) | LCRBmax | Outer RB allocation / Normal RB allocation |
| --- | --- | --- | --- |
| 5MHz | 15 | 25 | 25@0 |
|  | 30 | 11 | 11@0 |
| 10MHz | 15 | 25 | 25@14 |
|  | 30 | 12 | 12@6 |
| 15MHz | 15 | 25 | 25@27 |
|  | 30 | 12 | 12@13 |
| 20MHz | 15 | 25 | 25@41 |
|  | 30 | 12 | 12@20 |

Table 7.3I.3.4.1-2: Uplink configuration for reference sensitivity of FDD or TDD eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18, LCRB @ RBstart format

| Operating Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- |
| n1 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n2 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 | 10@11 | 12@6 | 12@13 | 12@20 |  |
| n3 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n5 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 10@81 | 10@151 | 10@221 |  |
| n7 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n8 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 10@81 | 10@151 | 10@221 |  |
| n12 | 15 | 20@51 | 20@191 | 20@321 |  | FDD |
|  | 30 |  | 10@81 | 10@151 |  |  |
| n13 | 15 | 20@01 | 20@141 |  |  | FDD |
|  | 30 |  | 10@61 |  |  |  |
| n14 | 15 | 20@01 | 20@141 |  |  | FDD |
|  | 30 |  | 10@61 |  |  |  |
| n20 | 15 | 25@0 | 20@141 | 20@27 | 20@41 | FDD |
|  | 30 |  | 10@61 | 10@132 | 10@202 |  |
| n24 | 15 | 25@0 | 25@14 |  |  | FDD |
|  | 30 |  | 12@6 |  |  |  |
| n25 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n26 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n28 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 10@81 | 10@151 | 10@221 |  |
| n30 | 15 | 20@51 | 20@191 |  |  | FDD |
|  | 30 |  | 10@81 |  |  |  |
| n34 | 15 | 25@0 | 25@14 | 25@27 |  | TDD |
|  | 30 |  | 12@6 | 12@13 |  |  |
| n38 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n39 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n40 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n41 | 15 |  | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n48 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n50 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n51 | 15 | 25@0 |  |  |  | TDD |
|  | 30 |  |  |  |  |  |
| n53 | 15 | 25@0 | 25@14 |  |  | TDD |
|  | 30 |  | 12@6 |  |  |  |
| n65 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n66 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n70 | 15 | 25@0 | 25@14 | 25@27 | NOTE 3 | FDD |
|  | 30 |  | 12@6 | 12@13 | NOTE 3 |  |
| n71 | 15 | 25@0 | 25@14 | 20@321 | 20@411 | FDD |
|  | 30 |  | 12@6 | 10@151 | 10@201 |  |
| n74 | 15 | 25@0 | 25@14 | 25@27 | 25@41 | FDD |
|  | 30 |  | 10@81 | 10@151 | 10@221 |  |
| n77 | 15 |  | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n78 | 15 |  | 25@14 | 25@27 | 25@41 | TDD |
|  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n79 | 15 |  | 25@14 |  | 25@41 | TDD |
|  | 30 |  | 12@6 |  | 12@20 |  |
| n85 | 15 | 20@51 | 20@191 | 20@321 |  | FDD |
|  | 30 |  | 10@81 | 10@151 |  |  |
| n91 | 15 | 25@04 | 20@191, 4 |  |  | FDD |
|  | 30 |  |  |  |  |  |
| n92 | 15 | 20@51 | 20@191 | 20@321 | 20@411 | FDD |
|  | 30 |  | 10@81 | 10@151 | 10@201 |  |
| n93 | 15 | 25@04 | 25@144 |  |  | FDD |
|  | 30 |  |  |  |  |  |
| n94 | 15 | 25@0 | 25@14 | 20@321 | 20@411 | FDD |
|  | 30 |  | 12@6 | 10@151 | 10@201 |  |
| n100 | 15 | 25@0 |  |  |  | FDD |
|  | 30 |  |  |  |  |  |
| n101 | 15 | 25@0 | 25@14 |  |  | TDD |
|  | 30 |  | 12@6 |  |  |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). NOTE 2: For band n20; for 15kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =16; for 30kHz SCS, in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart =6 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart =8. NOTE 3: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest duplex distance shall be used. NOTE 4: For band n91 and n93, largest supported UL bandwidth configuration shall be used. |  |  |  |  |  |  |

Table 7.3I.3.4.1-3: Uplink configuration for reference sensitivity of HD-FDD eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18, LCRB @ RBstart format

| Operating Band |  | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n2 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n3 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n5 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n7 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n8 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n12 |  | 15 | 25@0 | 25@14 | 25@27 |  | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 |  |  |
| n13 |  | 15 | 20@0 | 25@14 |  |  | HD-FDD |
|  |  | 30 |  | 12@6 |  |  |  |
| n14 |  | 15 | 25@0 | 25@14 |  |  | HD-FDD |
|  |  | 30 |  | 12@6 |  |  |  |
| n20 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n24 |  | 15 | 25@0 | 25@14 |  |  | HD-FDD |
|  |  | 30 |  | 12@6 |  |  |  |
| n25 |  | 15 | 25@0 | 50@2 | 75@4 | 25@41 | HD-FDD |
|  |  | 30 |  | 24@0 | 36@2 | 12@20 |  |
| n26 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n28 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n30 |  | 15 | 25@0 | 25@14 |  |  | HD-FDD |
|  |  | 30 |  | 12@6 |  |  |  |
| n65 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n66 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n70 |  | 15 | 25@0 | 25@14 | 25@27 | NOTE 1 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | NOTE 1 |  |
| n71 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n74 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n85 | 15 | 25@0 | 25@14 | 25@27 |  | HD-FDD |
|  | 30 |  | 12@6 | 12@13 |  |  |
| n91 |  | 15 | 25@0 |  |  |  | HD-FDD |
| n92 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| n93 |  | 15 | 25@0 |  |  |  | HD-FDD |
| n94 |  | 15 | 25@0 | 25@14 | 25@27 | 25@41 | HD-FDD |
|  |  | 30 |  | 12@6 | 12@13 | 12@20 |  |
| NOTE 1: For DL channel bandwidths that do not have symmetric UL channel bandwidth, highest valid UL configuration with lowest TX-RX separation (Table 5.4.4-1) shall be used unless otherwise specified. |  |  |  |  |  |  |  |

7.3I.3.4.2	Test procedure

For eRedCap UE supporting IE eRedCapNotReducedBB-BW-r18, same test procedure as steps 1~4 of clause 7.3I.2.4.2.

For eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18, same test procedure as steps 1~4 of clause 7.3.2.4.2 with the following exceptions on step 3.

-	Set the Downlink signal level to the appropriate REFSENS value defined in Tables 7.3I.3.5-1, 7.3I.3.5-2 and 7.3I.3.5-5 if 2Rx antennas connected or Tables 7.3I.3.5-3, 7.3I.3.5-4 and 7.3I.3.5-6 if 1Rx antennas connected.

7.3I.3.4.3	Message contents

Message contents are according to TS 38.508-1 [5] clause 4.6 ensuring Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED for NR band

7.3I.3.5	Test requirement

For eRedCap UE supporting IE eRedCapNotReducedBB-BW-r18, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Table 7.3I.2.5-1 and Table 7.3I.2.5-2 for eRedCap UE with 2 Rx antenna port, Tables 7.3I.2.5-3 and Table 7.3I.2.5-4 for eRedCap UE with single antenna port, Table 7.3I.2.5-5 for HD-FDD eRedCap UE with 2 Rx antenna port, Table 7.3I.2.5-6 for HD-FDD eRedCap UE with single antenna port, and parameters specified in Table 7.3.2.4.1-1, Table 7.3.2.4.1-2 and Table 7.3I.2.4.1-1.

For eRedCap UE not supporting IE eRedCapNotReducedBB-BW-r18, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A3.2 with reference receive power level specified in Table 7.3I.3.5-1 and Table 7.3I.3.5-2 for eRedCap UE with 2 Rx antenna port, Tables 7.3I.3.5-3 and Table 7.3I.3.5-4 for eRedCap UE with single antenna port, Table 7.3I.3.5-5 for HD-FDD eRedCap UE with 2 Rx antenna port, Table 7.3I.3.5-6 for HD-FDD eRedCap UE with single antenna port, and parameters specified in Table 7.3I.3.4.1-1, Table 7.3I.3.4.1-2 and Table 7.3I.3.4.1-3.

Table 7.3I.3.5-1: Two antenna port Reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth / REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| n1 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | FDD |
|  | 30 |  | -97.1-3 +TT | -97.1-3 +TT | -97.1-3 +TT |  |
| n2 | 15 | -98.0 +TT | -98.0 +TT | -98.0 +TT | -98.0 +TT | FDD |
|  | 30 |  | -95.1 -3+TT | -95.1 -3+TT | -95.1 -3+TT |  |
| n3 | 15 | -97.0 +TT | -97.0 +TT | -97.0 +TT | -97.0 +TT | FDD |
|  | 30 |  | -94.1 -3+TT | -94.1 -3+TT | -94.1 -3+TT |  |
| n5 | 15 | -98.0 +TT | -98.0 +TT | -98.0 +TT | -98.0 +TT | FDD |
|  | 30 |  | -95.1 -3+TT | -95.1 -3+TT | -95.1 -3+TT |  |
| n7 | 15 | -98.0 +TT | -98.0 +TT | -98.0 +TT | -98.0 +TT | FDD |
|  | 30 |  | -95.1 -3+TT | -95.1 -3+TT | -95.1 -3+TT |  |
| n8 | 15 | -97.0 +TT | -97.0 +TT | -97.0 +TT | -97.0 +TT | FDD |
|  | 30 |  | -94.1 -3+TT | -94.1 -3+TT | -94.1 -3+TT |  |
| n12 | 15 | -97.0 +TT | -97.0 +TT | -97.0 +TT |  | FDD |
|  | 30 |  | -94.1 -3+TT | -94.1 -3+TT |  |  |
| n13 | 15 | -97.0 +TT | -97.0 +TT |  |  | FDD |
|  | 30 |  | -94.1 +TT |  |  |  |
| n14 | 15 | -97.0 +TT | -97.0 +TT |  |  | FDD |
|  | 30 |  | -94.1 -3+TT |  |  |  |
| n20 | 15 | -97.0 +TT | -97.0 +TT | -97.0 +TT | -97.0 +TT | FDD |
|  | 30 |  | -94.1 -3+TT | -94.1 -3+TT | -94.1 -3+TT |  |
| n24 | 15 | -100.0 +TT | -100.0 +TT |  |  | FDD |
|  | 30 |  | -97.1 -3+TT |  |  |  |
| n25 | 15 | -96.5 +TT | -96.5 +TT | -96.5 +TT | -96.5 +TT | FDD |
|  | 30 |  | -93.6 -3+TT | -93.6 -3+TT | -93.6 -3+TT |  |
| n26 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | FDD |
|  | 30 |  | -94.8 -3+TT | -94.8 -3+TT | -94.8 -3+TT |  |
| n28 | 15 | -98.5 +TT | -98.5 +TT | -98.5 +TT | -98.5 +TT | FDD |
|  | 30 |  | -95.6 -3 +TT | -95.6 -3 +TT | -95.6 -3 +TT |  |
| n30 | 15 | -99.0 +TT | -99.0 +TT |  |  | FDD |
|  | 30 |  | -96.1 -3+TT |  |  |  |
| n65 | 15 | -99.5+TT | -99.5+TT | -99.5+TT | -99.5+TT | FDD |
|  | 30 |  | -96.6 -3+TT | -96.6 -3+TT | -96.6 -3+TT |  |
| n66 | 15 | -99.5 +TT | -99.5 +TT | -99.5 +TT | -99.5 +TT | FDD |
|  | 30 |  | -96.6 -3+TT | -96.6 -3+TT | -96.6 -3+TT |  |
| n70 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | FDD |
|  | 30 |  | -97.1 -3+TT | -97.1 -3+TT | -97.1 -3+TT |  |
| n71 | 15 | -97.2 +TT | -97.2 +TT | -97.2 +TT | -97.2 +TT | FDD |
|  | 30 |  | -94.3 -3+TT | -94.3 -3+TT | -94.3 -3+TT |  |
| n74 | 15 | -99.53 +TT | -99.53 +TT | -99.53 +TT | -99.53 +TT | FDD |
|  | 30 |  | -96.63 -3+TT | -96.63 -3+TT | -96.63 -3+TT |  |
| n85 | 15 | -97.0 +TT | -97.0 +TT | -97.0 +TT |  | FDD |
|  | 30 |  | -94.1 -3+TT | -94.1 -3+TT |  |  |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: The requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. |  |  |  |  |  |  |

Table 7.3I.3.5-2: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS / Duplex Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 +TT | TDD |
|  | 30 | 10, 15 | -97.1 -3 +TT |  |
| n381 | 15 | 5, 10, 15, 20 | -100 +TT | TDD |
|  | 30 | 10, 15, 20, | -97.1 -3 +TT |  |
| n39 | 15 | 5, 10, 15, 20 | -100 +TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 -3+TT |  |
| n40 | 15 | 5, 10, 15, 20 | -100 +TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 -3+TT |  |
| n411 | 15 | 5, 10, 15, 20 | -94.8+TT | TDD |
|  | 30 | 10, 15, 20 | -95.1 -3+TT |  |
| n481 | 15 | 5, 10, 15, 20, | -99 +TT | TDD |
|  | 30 | 10, 15, 20, | -96.1 -3+TT |  |
| n50 | 15 | 5, 10, 15, 20 | -100 +TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 -3+TT |  |
| n51 | 15 | 5 | -100+TT | TDD |
| n53 | 15 | 5, 10 | -100 +TT | TDD |
|  | 30 | 10 | -97.1-3+TT |  |
| n771,4 | 15 | 10, 15, 20 | -95.3 -3.2+TT | TDD |
|  | 30 | 10, 15, 20 | -95.6 -3 +TT |  |
| n781 | 15 | 10, 15, 20 | -95.8 -3.2+TT | TDD |
|  | 30 | 10, 15, 20 | -96.-3 +TT |  |
| n791 | 15 | 10, 20, | -95.8 -3.2+TT | TDD |
|  | 30 | 10, 20, | -96.1 -3 +TT |  |
| n91 | 15 | 5 | -100+TT | FDD |
| n92 | 15 | 5,10,15,20 | -100 +TT | FDD |
|  | 30 | 10,15,20 | -97.1 -3+TT |  |
| n93 | 15 | 5 | -100+TT | FDD |
| n94 | 15 | 5,10,15,20 | -100 +TT | FDD |
|  | 30 | 10,15,20 | -97.1 -3+TT |  |
| n101 | 15 | 5, 10 | -100 +TT | TDD |
|  | 30 | 10 | -97.1 -3+TT |  |
| NOTE 1: Void. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. |  |  |  |  |

Table 7.3I.3.5-3: Single antenna port Reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| n1 | 15 | -100.0+2.5 +TT | -100.0+2.5 +TT | -100.0+2.5 +TT | -100.0+2.5 +TT | FDD |
|  | 30 |  | -97.1 +TT | -97.1 +TT | -97.1 +TT |  |
| n2 | 15 | -98.0 +2.5+TT | -98.0 +2.5+TT | -98.0 +2.5+TT | -98.0 +2.5+TT | FDD |
|  | 30 |  | -95.1 +TT | -95.1 +TT | -95.1 +TT |  |
| n3 | 15 | -97.0+2.5 +TT | -97.0+2.5 +TT | -97.0+2.5 +TT | -97.0+2.5 +TT | FDD |
|  | 30 |  | -94.1 +TT | -94.1 +TT | -94.1 +TT |  |
| n5 | 15 | -98.0+2.5 +TT | -98.0+2.5 +TT | -98.0+2.5 +TT | -98.0+2.5 +TT | FDD |
|  | 30 |  | -95.1 +TT | -95.1 +TT | -95.1 +TT |  |
| n7 | 15 | -98.0+2.5 +TT | -98.0+2.5 +TT | -98.0+2.5 +TT | -98.0+2.5 +TT | FDD |
|  | 30 |  | -95.1 +TT | -95.1 +TT | -95.1 +TT |  |
| n8 | 15 | -97.0+2.5 +TT | -97.0+2.5 +TT | -97.0+2.5 +TT | -97.0+2.5 +TT | FDD |
|  | 30 |  | -94.1 +TT | -94.1 +TT | -94.1 +TT |  |
| n12 | 15 | -97.0 +2.5+TT | -97.0 +2.5+TT | -97.0 +2.5+TT |  | FDD |
|  | 30 |  | -94.1 +TT | -94.1 +TT |  |  |
| n13 | 15 | -97.0+2.5 +TT | -97.0+2.5 +TT |  |  | FDD |
|  | 30 |  | -94.1 +TT |  |  |  |
| n14 | 15 | -97.0+2.5 +TT | -97.0+2.5 +TT |  |  | FDD |
|  | 30 |  | -94.1 +TT |  |  |  |
| n20 | 15 | -97.0 +2.5+TT | -97.0 +2.5+TT | -97.0 +2.5+TT | -97.0 +2.5+TT | FDD |
|  | 30 |  | -94.1 +TT | -94.1 +TT | -94.1 +TT |  |
| n24 | 15 | -100.0 +2.5+TT | -100.0 +2.5+TT |  |  | FDD |
|  | 30 |  | -97.1 +TT |  |  |  |
| n25 | 15 | -96.5 +2.5+TT | -96.5 +2.5+TT | -96.5 +2.5+TT | -96.5 +2.5+TT | FDD |
|  | 30 |  | -93.6 +TT | -93.6 +TT | -93.6 +TT |  |
| n26 | 15 | -97.5+2.5 +TT | -97.5+2.5 +TT | -97.5+2.5 +TT | -97.5+2.5 +TT | FDD |
|  | 30 |  | -94.8 +TT | -94.8 +TT | -94.8 +TT |  |
| n28 | 15 | -98.5 +2.5+TT | -98.5 +2.5+TT | -98.5 +2.5+TT | -98.5 +2.5+TT | FDD |
|  | 30 |  | -95.6 +TT | -95.6 +TT | -95.6 +TT |  |
| n30 | 15 | -99.0 +2.5+TT | -99.0 +2.5+TT |  |  | FDD |
|  | 30 |  | -96.1 +TT |  |  |  |
| n65 | 15 | -99.5 +2.5+TT | -99.5 +2.5+TT | -99.5 +2.5+TT | -99.5 +2.5+TT | FDD |
|  | 30 |  | -96.6 +TT | -96.6 +TT | -96.6 +TT |  |
| n66 | 15 | -99.5 +2.5+TT | -99.5 +2.5+TT | -99.5 +2.5+TT | -99.5 +2.5+TT | FDD |
|  | 30 |  | -96.6 +TT | -96.6 +TT | -96.6 +TT |  |
| n70 | 15 | -100.0 +2.5 +TT | -100.0 +2.5 +TT | -100.0 +2.5 +TT | -100.0 +2.5 +TT | FDD |
|  | 30 |  | -97.1 +TT | -97.1 +TT | -97.1 +TT |  |
| n71 | 15 | -97.2+2.5 +TT | -97.2+2.5 +TT | -97.2+2.5 +TT | -97.2+2.5 +TT | FDD |
|  | 30 |  | -94.3 +TT | -94.3 +TT | -94.3 +TT |  |
| n74 | 15 | -99.53 +2.5 +TT | -99.53 +2.5 +TT | -99.53 +2.5 +TT | -99.53 +2.5 +TT | FDD |
|  | 30 |  | -96.63 +TT | -96.63 +TT | -96.63 +TT |  |
| n85 |  | 15 | -97+2.5 +TT | -97+2.5 +TT | -97+2.5 +TT |  | FDD |
|  |  | 30 |  | -94.1 +TT | -94.1 +TT |  |  |
| NOTE 1: Void NOTE 2: The transmitter shall be set to PUMAX as defined in subclause 6.2.4 NOTE 3: That the requirement is modified by -0.5 dB when the assigned NR channel bandwidth is confined within 1475.9-1510.9 MHz. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. |  |  |  |  |  |  |

Table 7.3I.3.5-4: Single antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS/Duplex Mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n34 | 15 | 5, 10, 15 | -100 +2.5+TT | TDD |
|  | 30 | 10, 15 | -97.1 +2.5 -3+TT |  |
| n381 | 15 | 5, 10, 15, 20 | -100 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 +2.5 -3+TT |  |
| n39 | 15 | 5, 10, 15, 20 | -100 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 +2.5 -3+TT |  |
| n40 | 15 | 5, 10, 15, 20 | -100 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 +2.5 -3+TT |  |
| n411 | 15 | 10, 15, 20 | -94.8 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -95.1 +2.5 -3+TT |  |
| n481 | 15 | 5, 10, 15, 20 | -99 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -96.1+2.5 -3+TT |  |
| n50 | 15 | 5, 10, 15, 20 | -100 +2.5+TT | TDD |
|  | 30 | 10, 15, 20 | -97.1 +2.5 -3+TT |  |
| n51 | 15 | 5 | -100 +2.5+TT | TDD |
| n53 | 15 | 5, 10 | -100 +2.5+TT | TDD |
|  | 30 | 10 | -97.1 +2.5 -3+TT |  |
| n771,4 | 15 | 10, 15, 20 | -95.3 +2.5 -3.2 +TT | TDD |
|  | 30 | 10, 15, 20 | -95.6 +2.5 -3 +TT |  |
| n781 | 15 | 10, 15, 20 | -95.8 +2.5 -3.2+TT | TDD |
|  | 30 | 10, 15, 20 | -96.1 +2.5 -3 +TT |  |
| n791 | 15 | 10, 20, | -95.8 +2.5+TT | TDD |
|  | 30 | 10, 20, | -96.1 +2.5 -3+TT |  |
| n91 | 15 | 5 | -100+2.5+TT | FDD |
| n92 | 15 | 5,10,15,20 | -100 +2.5+TT | FDD |
|  | 30 | 10,15,20 | -97.1 +2.5 -3+TT |  |
| n93 | 15 | 5 | -100 +2.5+TT | FDD |
| n94 | 15 | 5,10,15,20 | -100 +2.5+TT | FDD |
|  | 30 | 10,15,20 | -97.1 +2.5 -3+TT |  |
| n101 | 15 | 5, 10 | -100 +2.5+TT | TDD |
|  | 30 | 10 | -97.1 +2.5 -3+TT |  |
| NOTE 1: Void. NOTE 2: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 3: Void NOTE 4: The requirement is modified by -0.5 dB when the assigned UE channel bandwidth is confined within 3300 - 3800 MHz. NOTE 5: Void NOTE 6: Void NOTE 7: Void NOTE 8: The REFSENS value is rounded to the nearest number down to one decimal point. “NRB” in REFSENS formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 9: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. |  |  |  |  |

Table 7.3I.3.5-5: Two antenna port reference sensitivity QPSK PREFSENS for HD-FDD operation

| Operating band / SCS / Channel bandwidth/ REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| n1 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n2 | 15 | -98.8 +TT | -98.8 +TT | -98.8 +TT | -98.8 +TT | HD-FDD |
|  | 30 |  | -96.0 -3+TT | -96.0 -3+TT | -96.0 -3+TT |  |
| n3 | 15 | -97.8 +TT | -97.8 +TT | -97.8 +TT | -97.8 +TT | HD-FDD |
|  | 30 |  | -95.0 -3+TT | -95.0 -3+TT | -95.0 -3+TT |  |
| n5 | 15 | -98.8 +TT | -98.8 +TT | -98.8 +TT | -98.8 +TT | HD-FDD |
|  | 30 |  | -96.0 -3+TT | -96.0 -3+TT | -96.0 -3+TT |  |
| n7 | 15 | -98.8 +TT | -98.8 +TT | -98.8 +TT | -98.8 +TT | HD-FDD |
|  | 30 |  | -96.0 -3+TT | -96.0 -3+TT | -96.0 -3+TT |  |
| n8 | 15 | -97.8 +TT | -97.8 +TT | -97.8 +TT | -97.8 +TT | HD-FDD |
|  | 30 |  | -95.0 -3+TT | -95.0 -3+TT | -95.0 -3+TT |  |
| n12 | 15 | -97.8 +TT | -97.8 +TT | -97.8 +TT |  | HD-FDD |
|  | 30 |  | -95.0 -3+TT | -95.0 -3+TT |  |  |
| n13 | 15 | -97.8 +TT | -97.8 +TT |  |  | HD-FDD |
|  | 30 |  | -95.0 -3+TT |  |  |  |
| n14 | 15 | -97.8 +TT | -97.8 +TT |  |  | HD-FDD |
|  | 30 |  | -95.0 -3+TT |  |  |  |
| n20 | 15 | -97.8 +TT | -97.8 +TT | -97.8 +TT | -97.8 +TT | HD-FDD |
|  | 30 |  | -95.0 -3+TT | -95.0 -3+TT | -95.0 -3+TT |  |
| n24 | 15 | -100.0 +TT | -100.0 +TT |  |  | HD-FDD |
|  | 30 |  | -97.2 -3+TT |  |  |  |
| n25 | 15 | -97.3 +TT | -97.3 +TT | -97.3 +TT | -97.3 +TT | HD-FDD |
|  | 30 |  | -94.5 -3+TT | -94.5 -3+TT | -94.5 -3+TT |  |
| n26 | 15 | -98.3 +TT | -98.3 +TT | -98.3 +TT | -98.3 +TT | HD-FDD |
|  | 30 |  | -95.5 -3+TT | -95.5 -3+TT | -95.5 -3+TT |  |
| n28 | 15 | -99.3 +TT | -99.3 +TT | -99.3 +TT | -99.3 +TT | HD-FDD |
|  | 30 |  | -96.5 -3+TT | -96.5 -3+TT | -96.5 -3+TT |  |
| n30 | 15 | -99.5 +TT | -99.5 +TT |  |  | HD-FDD |
|  | 30 |  | -96.7 -3+TT |  |  |  |
| n65 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n66 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n70 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n71 | 15 | -98.0 +TT | -98.0 +TT | -98.0 +TT | -98.0 +TT | HD-FDD |
|  | 30 |  | -95.2 -3+TT | -95.2 -3+TT | -95.2 -3+TT |  |
| n74 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n85 | 15 | -97.8 +TT | -97.8 +TT | -97.8 +TT |  | HD-FDD |
|  | 30 |  | -95.0 -3+TT | -95.0 -3+TT |  |  |
| n91 | 15 | -100.0 +TT |  |  |  | HD-FDD |
| n92 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| n93 | 15 | -100.0 +TT |  |  |  | HD-FDD |
| n94 | 15 | -100.0 +TT | -100.0 +TT | -100.0 +TT | -100.0 +TT | HD-FDD |
|  | 30 |  | -97.2 -3+TT | -97.2 -3+TT | -97.2 -3+TT |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. |  |  |  |  |  |  |

Table 7.3I.3.5-6: Single antenna port Reference sensitivity QPSK PREFSENS for HD-FDD operation

| Operating band / SCS / Channel bandwidth/ REFSENS / Duplex Mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| n1 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  | -94.7 -3+TT | -94.7 -3+TT | -94.7 -3+TT |  |
| n2 | 15 | -96.3 +TT | -96.3 +TT | -96.3 +TT | -96.3 +TT | HD-FDD |
|  | 30 |  | -93.5 -3+TT | -93.5 -3+TT | -93.5 -3+TT |  |
| n3 | 15 | -95.3 +TT | -95.3 +TT | -95.3 +TT | -95.3 +TT | HD-FDD |
|  | 30 |  | -92.5 -3+TT | -92.5 -3+TT | -92.5 -3+TT |  |
| n5 | 15 | -96.3 +TT | -96.3 +TT | -96.3 +TT | -96.3 +TT | HD-FDD |
|  | 30 |  | -93.5 -3+TT | -93.5 -3+TT | -93.5 -3+TT |  |
| n7 | 15 | -96.3 +TT | -96.3 +TT | -96.3 +TT | -96.3 +TT | HD-FDD |
|  | 30 |  | -93.5 -3+TT | -93.5 -3+TT | -93.5 -3+TT |  |
| n8 | 15 | -95.3 +TT | -95.3 +TT | -95.3 +TT | -95.3 +TT | HD-FDD |
|  | 30 |  | -92.5 -3+TT | -92.5 -3+TT | -92.5 -3+TT |  |
| n12 | 15 | -95.3 +TT | -95.3 +TT | -95.3 +TT |  | HD-FDD |
|  | 30 |  | -92.5 -3+TT | -92.5 -3+TT |  |  |
| n13 | 15 | -95.3 +TT | -95.3 +TT |  |  | HD-FDD |
|  | 30 |  | -92.5 -3+TT |  |  |  |
| n14 | 15 | -95.3 +TT | -95.3 +TT |  |  | HD-FDD |
|  | 30 |  | -92.5 -3+TT |  |  |  |
| n20 | 15 | -95.3 +TT | -95.3 +TT | -95.3 +TT | -95.3 +TT | HD-FDD |
|  | 30 |  | -92.5 -3+TT | -92.5 -3+TT | -92.5 -3+TT |  |
| n24 | 15 | -97.5 +TT | -97.5 +TT |  |  | HD-FDD |
|  | 30 |  | -94.7 -3+TT |  |  |  |
| n25 | 15 | -94.8 +TT | -94.8 +TT | -94.8 +TT | -94.8 +TT | HD-FDD |
|  | 30 |  | -92.0 -3+TT | -92.0 -3+TT | -92.0 -3+TT |  |
| n26 | 15 | -95.8 +TT | -95.8 +TT | -95.8 +TT | -95.8 +TT | HD-FDD |
|  | 30 |  | -93.0 -3+TT | -93.0 -3+TT | -93.0 -3+TT |  |
| n28 | 15 | -96.8 +TT | -96.8 +TT | -96.8 +TT | -96.8 +TT | HD-FDD |
|  | 30 |  | -94.0 -3+TT | -94.0 -3+TT | -94.0 -3+TT |  |
| n30 | 15 | -97.0 +TT | -97.0 +TT |  |  | HD-FDD |
|  | 30 |  | -94.2 -3+TT |  |  |  |
| n65 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  | -94.7 -3+TT | -94.7 -3+TT | -94.7 -3+TT |  |
| n66 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  | -94.7 -3+TT | -94.7 -3+TT | -94.7 -3+TT |  |
| n70 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  | -94.7 -3+TT | -94.7 -3+TT | -94.7 -3+TT |  |
| n71 | 15 | -95.5 +TT | -95.5 +TT | -95.5 +TT | -95.5 +TT | HD-FDD |
|  | 30 |  | -92.7 -3+TT | -92.7 -3+TT | -92.7 -3+TT |  |
| n74 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  | -94.7 -3+TT | -94.7 -3+TT | -94.7 -3+TT |  |
| n85 | 15 | -95.3 +TT | -95.3 +TT | -95.3 +TT |  | HD-FDD |
|  | 30 |  | -92.5 -3+TT | -92.5 -3+TT |  |  |
| n91 | 15 | -97.5 +TT |  |  |  | HD-FDD |
| n92 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  | -94.7 -3+TT | -94.7 -3+TT | -94.7 -3+TT |  |
| n93 | 15 | -97.5 +TT |  |  |  | HD-FDD |
| n94 | 15 | -97.5 +TT | -97.5 +TT | -97.5 +TT | -97.5 +TT | HD-FDD |
|  | 30 |  | -94.7 -3+TT | -94.7 -3+TT | -94.7 -3+TT |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 7.3I.3.5-7. |  |  |  |  |  |  |

Table 7.3I.3.5-7: Test Tolerance (TT) for RX sensitivity level for eRedCap UE

| f ≤ 3.0GHz | 3.0GHz < f ≤ 6.0 GHz |
| --- | --- |
| 0.7 dB | 1.0 dB |

<Skipped>

F.1.3	Measurement of receiver

- MU and TT for >6GHz (band n96) are working assumption based on analysis of single TE vendor. Values will be revisited once analysis from other TE vendors is available.

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 7.3.2 Reference sensitivity power level | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz |  |
| 7.3A Reference sensitivity for CA (Same MU apply to all subsections including 7.3A.1, 7.3A.1_1, 7.3A.2, 7.3A.3, 7.3A.4, etc.) | Same as 7.3.2 for each component carrier |  |
| 7.3C.2 Reference sensitivity power level | Same as 7.3.2 |  |
| 7.3D Reference sensitivity for MIMO | Same as 7.3.2 |  |
| 7.3D.2_1 Reference sensitivity power level for SUL with UL MIMO | Same as 7.3D |  |
| 7.3F.2 Reference sensitivity power level | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 7.125GHz |  |
| 7.3I.2 Reference sensitivity power level for RedCap | Same as 7.3.2 |  |
| 7.3I.3 Reference sensitivity power level for eRedCap | Same as 7.3.2 |  |
| 7.4 Maximum input level | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Uplink power measurement same as 6.2.1 |  |

<End of changes>