3GPP TSG-RAN5 Meeting #105	                       													   R5-247502

Orlando, USA, 18th Nov 2024 – 22nd Nov 2024

Agenda item:	5.4.20

Source:   Apple Benelux B.V.

Title:       Discussion on AI ML testing

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

The topic of AI/ML has been undergoing wide-ranging discussions across core working groups RAN1, RAN2 and RAN4. At RAN#105, there were agreements on the scope and handling of the testability and interoperability for this topic.

The immediate need identified is to evaluate the feasibility of RAN5 making contributions on this topic to the ongoing testability discussions ongoing in RAN4. We propose some initial assessment towards this objective.

# 2	Discussion

### 2.1	Assessment of the feasibility of RAN5 making technical contributions to the AI/ML WID.

Below is a snapshot of some of the key guidance from RAN5 leadership around which some early discussions are encouraged in RAN5 [1]

- Current RAN5 responsibility - RAN5 aspects related to testability and interoperability to be addressed on a request basis.

- RAN5 to make an assessment whether RAN5 is in a position now to make technical contributions to RAN4 specs covering any of the objectives in the WID before being added under secondary responsibility in the WID.

- OTA Test setup and methodology for the beam management is being studied under this the core AI/ML WI (updated in RP#105) and a potential area of assessment for RAN5 to contribute.

Observation 1: The feasibility of RAN5 contributions to the test setup and methodology section of beam management work in RAN4 is pending assessment

There is currently no RAN4 TR to capture testability outcomes as the discussions are in early stages and hence there is no framework for RAN5 to submit any contributions even if a feasibility assessment was made.

Furthermore, the RAN4 WID will have to be updated after the RAN5 assessment, to include RAN5 as secondary working group and clearly outline the scope of such contributions, if agreed by RAN4. Therefore, a clear understanding of the dependencies should first be agreed in RAN5.

Proposal 1: A clear framework for potential RAN5 contributions on the AI/ML BM OTA discussion is critical –

- Evaluate whether there are topics relevant to AI/ML BM OTA which RAN5 can contribute to RAN4 discussions. If so, list such areas and the RAN5 agreement status for each.

- If there is agreement to i), then RAN5 needs to be added as secondary WG to the RAN4 WID which also may have to be updated with clear scope of RAN5 work.

- If there is no agreement to i), then re-visit the assessment at a later (TBD) meeting.

With the above framework in mind, it would help to look at some UE feature and test aspects that could be of potential interest to RAN5.

### 2.2	UE Capability Signalling for AI/ML

For the Beam Management use case, the identified scenarios and configurations can be initially understood as those reported by UE through capability signalling as part of functionality identification.

Observation 2: RAN2 has been discussing and reached some key agreements in [2] wherein it was determined that –

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling).

- RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

With this understanding, RAN5 can wait till RAN2 signalling is defined and then determine how to use the UE capability information during the initial call setup to identify the specific AI/ML features/IEs that the UE supports. Subsequently, it can be discussed on the method to be used within the conformance test (UE declaration, PIXIT etc). Please note that there is already precedence for this in the FR2 Beam Correspondence tests defined during Release 16.

Proposal 2: The UE capability information, once RAN2 signalling aspects and optional/mandatory nature of the AI/ML specific features on the UE are concluded, can be used for functionality identification and incorporated appropriately within the conformance test.

### 2.3	AI/ML BM OTA Testability – Lessons learnt from Rel-15 FR2 Testability Challenges

With reference to the ongoing test methodology discussion in RAN5, there are multiple aspects and metrics being discussed in RAN4. The key metrics under discussion in RAN4 can be broadly classified under, including but not limited to -

- RSRP Accuracy

- Beam Prediction accuracy/delay requirements

- CSI Reporting Accuracy

It should be noted that while RSRP accuracy and CSI reporting have legacy requirements, beam management as such does not have an explicit “legacy” 5G FR2 conformance test.

From a test methods standpoint some active discussions, including but not limited to the below, are –

- Channel Model for Beam Prediction measurements

- Number of AoAs/AoDs for testing

- Test Setup requirements (Number of probes etc)

With the expertise within RAN5 to define conformance test procedure for FR2 OTA, and analysing the ongoing RAN4 discussions for AI/ML BM OTA, the following questions need to be eventually answered in RAN5 to evaluate the feasibility of the FR2 OTA-based test procedure:

- How can we generate multiple beams from the Set-B and Set-A Tx beams given the limitation of two AoAs?

- What assumption is made regarding Rx beam sweeping? Does the UE utilize a fixed Rx beam, or does it sweep to find the optimal Rx beam?

- How can we simultaneously emulate different AoAs (Rx beam) and AoD (Tx beams)?

- How can we achieve dynamic variation in the AoD domain (Tx beam sweeping) for BM case 2 prediction?

- What is the expected dynamic range of the setup considering there are scenarios such as RSRP accuracy which (based on legacy tests) operate at very low power levels but other scenarios like Beam Prediction or CSI reporting might be associated with much

Observation 3: There are significant testability issues ongoing discussion in RAN4 which will eventually impact RAN5 test procedures, MU element determination and MU value computation.

We have performed an extensive assessment of some lessons learnt from the Rel-15 FR2 testability issue discussions detailed and previously discussed in [3] through [9] and how a more optimized approach can significantly help in Rel-19 AI/ML BM testing with RAN5 more proactively involved in the preliminary MU assessment. This will help ensure early identification of test infeasibility and potential resolution.

Rel-15 FR2 OTA testability issue discussion timeline

Observation 4: In the case of Rel-15 FR2 OTA, it took more than 1 year from identification of Testability issues in RAN4 up to the point when RAN5 could trigger discussions

Observation 5: TE vendors, who lead both RAN4 preliminary MU and RAN5 MU discussions, had reported potential testability issues in RAN4, but preference/focus was to complete performance requirements and wait till Perf part is complete and RAN5 can then assess MU. This took almost 1.5 years after Perf part completion

Following similar approach risks waiting too late to confirm whether the preferred/finalized test method is even usable from testability/MU standpoint. Therefore, we propose a more proactive approach for this aspect.

Proposal 3: Facilitate proactive involvement of RAN5 in preliminary MU/testability assessment in RAN4 for AI/ML related BM OTA testing, starting with the following broad steps:

- Request RAN5 be added immediately as secondary WG with specific scope of collaborating/contributing to preliminary and final MU assessment at appropriate trigger from RAN4.

- At a timepoint deemed suitable by RAN4 (for e.g close to finish of test methodology associated with any of the BM-related requirements), RAN4 triggers a request to RAN5 for collaboration and contributions on preliminary MU assessment.

# 3	Summary

Observation 1: The feasibility of RAN5 contributions to the test setup and methodology section of beam management work in RAN4 is pending assessment

Proposal 1: A clear framework for potential RAN5 contributions on the AI/ML BM OTA discussion is critical –

- Evaluate whether there are topics relevant to AI/ML BM OTA which RAN5 can contribute to RAN4 discussions. If so, list such areas and the RAN5 agreement status for each.

- If there is agreement to i), then RAN5 needs to be added as secondary WG to the RAN4 WID which also may have to be updated with clear scope of RAN5 work.

- If there is no agreement to i), then re-visit the assessment at a later (TBD) meeting.

Observation 2: RAN2 has been discussing and reached some key agreements in [2] wherein it was determined that –

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling).

- RAN2 discuss further on the need of defining available functionalities separately from applicable functionalities.

Proposal 2: The UE capability information, once RAN2 signalling aspects and optional/mandatory nature of the AI/ML specific features on the UE are concluded, can be used for functionality identification and incorporated appropriately within the conformance test.

Observation 3: There are significant testability issues ongoing discussion in RAN4 which will eventually impact RAN5 test procedures, MU element determination and MU value computation.

Observation 4: In the case of Rel-15 FR2 OTA, it took more than 1 year from identification of Testability issues in RAN4 up to the point when RAN5 could trigger discussions

Observation 5: TE vendors, who lead both RAN4 preliminary MU and RAN5 MU discussions had reported potential testability issues in RAN4, but preference/focus was to complete performance requirements and wait till Perf part is complete and RAN5 can then assess MU. This took almost 1.5 years after Perf part completion

Proposal 3: Facilitate proactive involvement of RAN5 in preliminary MU/testability assessment in RAN4 for AI/ML related BM OTA testing, starting with the following broad steps:

- Request RAN5 be added immediately as secondary WG with specific scope of collaborating/contributing to preliminary and final MU assessment at appropriate trigger from RAN4.

- At a timepoint deemed suitable by RAN4 (for e.g close to finish of test methodology associated with any of the BM-related requirements), RAN4 triggers a request to RAN5 for collaboration and contributions on preliminary MU assessment.

# 4	References

RP-242399: AI/ML for NR Air Interface WID

R2-2407165: [AI/ML PHY] LCM– Phase 1

RP-191603: 5G-NR FR2 Transmitter & Receiver Testability Issues, RAN#84, June 2019

R5-187273: Consideration on MU and TT for FR2 Tx test cases with low SNR, Nov 2018

R5-185805: Testability issue of maximum input level and ACS (case 2) in FR2 (2), RAN5 AH#3 Oct 2018

R4-1800325: Open testability issues for mmWave UE RF test cases, RAN4-AH-1801, Jan 2018

R4-1800325: Open testability issues for mmWave UE RF test cases, RAN4-AH-1801, Jan 2018

R4-1712546, “Guideline of Tx/Rx UE spurious emission test at FR2”, RAN4 #85, Nov 2017

R4-1707230, “Estimation of general spurious emission test time for mmWave UE”, Anritsu, RAN4 #84, Aug 2017

- [10] TR 38.843: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface