3GPP TSG-RAN5 Meeting #105	R5-246722

# Orlando, US, Nov.18th – 22nd 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3511 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Annex F for FR2 RRM test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - To update Annex F for following FR2 Rel-15 RRM test cases and RedCap RRM test cases: - Re-establishment without serving cell timing: 7.3.2.1.3, 17.3.2.1.3; - RLM scheduling restriction: 5.5.1.9, 7.5.1.9, 17.5.1.9. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Test tolerance for test cases mentioned above are added to Annex F |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 CR 3807, 3509, 3510, 3520, 3521 TS 38.903 CR 0865, 0866 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

Table F.1.1.2-5: Maximum test system uncertainty for RRM requirements for EN-DC FR2 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |
| <Unchanged contents are omitted> |  |  |
| 5.5.1.8 EN-DC FR2 radio link monitoring in-sync test for PSCell configured with CSI-RS-based RLM RS in DRX mode | Same as 5.5.1.5 | Same as 5.5.1.5 |
| 5.5.1.9 EN-DC FR2 radio link monitoring UE scheduling restrictions | Average Noc ±5.65 dB, f<=40.8 GHz Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB | Ês1 / Noc is the SNR of SSB #0 Ês2 / Noc is the SNR of SSB #1 |
| 5.5.1.10 EN-DC FR2 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion | Same as 5.5.1.1 | Same as 5.5.1.1 |
| <Unchanged contents are omitted> |  |  |
| 5.7.9.2 EN-DC inter frequency FR2 CSI-SINR measurement accuracy | Same as 5.7.8.2 | Same as 5.7.8.2 |

Table F.1.1.2-6: Maximum test system uncertainty for RRM requirements for SA FR2 test cases

| Subclause |  | Maximum Test System Uncertainty1 |  |  | Derivation of Test System Uncertainty |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7.1.1.1 NR SA FR2 cell re-selection |  | Noc, averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es1/Noc averaged over BWConfig ±0.3 dB Es1/Noc averaged over Measurement PRB ±0.3 dB Es2/Noc averaged over BWConfig ±0.3 dB Es2/Noc averaged over Measurement PRB ±0.3 dB |  |  | Noc averaged over BWConfig is the AWGN absolute power of RF channel 1 averaged over BWConfig Noc averaged over Measurement PRB is the AWGN absolute power of RF channel 1 averaged over Measurement PRB Es1/Noc averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |  |  |
| <Unchanged contents are omitted> |  |  |  |  |  |  |  |
| 7.3.2.1.2 NR SA FR2 - FR2 RRC re-establishment |  |  | Average Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc1 ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB |  |  | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |  |
| 7.3.2.1.3 NR SA FR2 RRC re-establishment without serving cell timing |  |  | Same as 7.3.2.1.1 |  |  | Same as 7.3.2.1.1 |  |  |
| 7.3.2.2.1 NR SA FR2 contention based random access |  |  | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink |  |  | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |  |
| <Unchanged contents are omitted> |  |  |  |  |  |  |  |  |
| 7.5.1.4 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode |  | Same as 5.5.1.1 |  |  | Same as 5.5.1.1 |  |  |
| 7.5.1.9 EN-DC FR2 radio link monitoring UE scheduling restrictions |  | Same as 5.5.1.1 |  |  | Same as 5.5.1.1 |  |  |
| 7.5.3.4 NR SA FR2 direct SCell activation at SCell addition of known SCell | Average Noc1 ±5.65 dB Average Noc2 ±5.65 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB |  |  | Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |  |
| <Unchanged contents are omitted> |  |  |  |  |  |  |
| 7.7.9.2 NR SA inter frequency FR2 CSI-SINR measurement accuracy |  | Same as 5.7.9.2 |  |  | Same as 5.7.9.2 |  |  |

<End of Change 1>

<Start of Change 2>

Table F.1.1.2-10: Maximum test system uncertainty for RRM requirements for SA FR2 test cases for RedCap

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 17.1.1.1 NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx | Same as 7.1.1.1 | Same as 7.1.1.1 |
| 17.1.1.2 NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR case | Same as 7.1.1.1 | Same as 7.1.1.1 |
| <Unchanged contents are omitted> |  |  |
| 17.1.1.4 NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR case for UE fulfilling stationary mobility relaxed measurement criterion for 2 Rx UE | Same as 7.1.1.1 | Same as 7.1.1.1 |
| 17.3.1.1 NR SA FR2 Intra-frequency handover from FR2 to FR2; unknown target cell for 2 Rx | Same as 7.3.1.2 | Same as 7.3.1.2 |
| 17.3.1.2 NR SA FR2-FR2 Inter-frequency handover from FR2 to FR2; unknown target cell for 2 Rx | Same as 7.3.1.3 | Same as 7.3.1.3 |
| 17.3.2.1.1 NR SA FR2 Intra-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.1 | Same as 7.3.2.1.1 |
| 17.3.2.1.2 NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.2 | Same as 7.3.2.1.2 |
| 17.3.2.1.3 NR SA FR2 RRC re-establishment without serving cell timing | Same as 7.3.2.1.3 | Same as 7.3.2.1.3 |
| 17.3.2.3.1 NR SA FR2-FR2 Redirection from NR in FR2 to NR in FR2 | Same as 7.3.2.3 | Same as 7.3.2.3 |
| 17.4.1.1 NR SA FR2 NR UE Transmit Timing Test for FR2 | Same as 7.4.1.1 | Same as 7.4.1.1 |
| <Unchanged contents are omitted> |  |  |
| 17.5.1.7 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.8 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.1.9 NR SA FR2 radio link monitoring UE scheduling restrictions | Same as 5.5.1.1 | Same as 5.5.1.1 |
| 17.5.2.1 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode | Same as 7.5.5.1 | Same as 7.5.5.1 |
| 17.5.2.2 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode | Same as 7.5.5.2 | Same as 7.5.5.2 |
| <Unchanged contents are omitted> |  |  |
| 17.7.4.1 Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.3.1 | Same as 7.7.3.1 |
| 17.7.4.2 Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.3.2 | Same as 7.7.3.2 |

<End of Change 2>

<Start of Change 3>

Table F.1.3.2-3: Derivation of test requirements for EN-DC FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 5.3.2.2.1 EN-DC FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| <Unchanged contents are omitted> |  |  |  |
| 5.5.1.10 EN-DC FR2 Radio Link Monitoring Out-of-sync Test for PSCell configured with SSB-based RLM RS for UE fulfilling relaxed measurement criterion | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T3: Noc: -104.7dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB | During T1: 0 dB 1.25 dB 1.25 dB During T2: 0 dB 1.25 dB 1.25 dB During T3: 0 dB -0.5 dB -0.5 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.25 dB Ês2 / Noc: +3.25 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.25 dB Ês2 / Noc: +3.25 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.5 dB Ês2 / Noc: -15.5 dB |
| 5.5.1.9 EN-DC FR2 radio link monitoring UE scheduling restrictions | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB | During T1: -2.7 dB 2.1 dB 2.1 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB |
| 5.5.2.3 EN-DC FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC | During T1: Noc1: -111.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc: +12 dB Ês2 / Noc: +5 dB | During T1: 0 dB 0 dB 0 dB 0 dB | During T1: Noc1: -111.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc: +12 dB Ês2 / Noc: +5 dB |
| <Unchanged contents are omitted> |  |  |  |
| 5.7.9.2 EN-DC inter frequency FR2 CSI-SINR measurement accuracy | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Reported absolute SINR values: ±3dB Reported relative SINR values: ±3.5dB Test 2: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Reported absolute SINR values: ±3dB Reported relative RSRQ values: ±3.5dB Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute SINR values: ±3.5dB Reported relative SINR values: ±4dB | Test 1: 0dB 0dB 0dB 0dB Via Mapping Via Mapping Test 2: -0.1dB -0.1dB 0dB 0dB Via Mapping Via Mapping Test 3: 0dB 0dB 0.9dB 0.9dB Via Mapping Via Mapping | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Absolute accuracy: Cell 2: n257, n258, n261: SINR_37 to SINR_52 n260: SINR_36 to SINR_52 Cell 3: n257, n258, n261: SINR_37 to SINR_52 n260: SINR_36 to SINR_52 Cell 3: SINR_x-8 to SINR_x+8 Test 2: Noc1: -105.1dBm/15kHz Noc2: -105.1dBm/15kHz Ês1 / Noc1: 11dB Ês2 / Noc2: 11dB Cell 2: n257, n258, n261: SINR_54 to SINR_81 n260: SINR_54 to SINR_80 Cell 3: n257, n258, n261: SINR_54 to SINR_81 n260: SINR_54 to SINR_80 Cell 3: SINR_x-17 to SINR_x+17 Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB Cell 2: n257, n258, n261: SINR_33 to SINR_50 n260: SINR_32 to SINR_50 Cell 3: n257, n258, n261: SINR_33 to SINR_50 n260: SINR_32 to SINR_50 Cell 3: SINR_x-9 to SINR_x+9 |

Table F.1.3.2-4: Derivation of test requirements for NR SA FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- | --- |
| 7.1.1.1 NR SA FR2 cell re-selection | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -3dB Es2/Noc = 1.5dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 1.5dB Es2/Noc = -3dB In signalling: Qrxlevmin = -138dBm/SCS for config. 1 Qrxlevmin = -135dBm/SCS for config. 2 |  | During T1: 0dB 0dB N/A During T2: 0dB 0.1dB 0.55dB During T3: 0dB 0.55dB 0.1dB In signalling: 18 dB 18 dB | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -2.90dB Es2/Noc = 2.05dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 2.05dB Es2/Noc = -2.90dB In signalling: Qrxlevmin = -120dBm/SCS for config. 1 Qrxlevmin = -117dBm/SCS for config. 2 |
| <Unchanged contents are omitted> |  |  |  |  |
| 7.3.2.1.1 NR SA FR2 RRC re-establishment | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 4 dB Ês2 / Noc = 2 dB During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 4 dB Ês2 / Noc = 2 dB During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB |
| 7.3.2.1.2 NR SA FR2 - FR2 RRC re-establishment | During T1: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = 0 dB Ês2 / Noc 2= - Infinity During T2: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = - Infinity During T3: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = 0 dB |  | During T1: -0.2dB -0.2dB 0dB 0dB During T2: -0.2dB -0.2dB 0dB 0dB During T3: -0.2dB -0.2dB 0dB 0dB | During T1: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = 0 dB Ês2 / Noc 2= - Infinity During T2: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = - Infinity During T3: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = 0 dB |
| 7.3.2.1.3 NR SA FR2 RRC re-establishment without serving cell timing | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 5 dB Ês2 / Noc = - infinity During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = - infinity During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 5 dB |  | During T1: 0dB 0dB N/A During T2: 0dB N/A N/A During T3: 0dB N/A 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 5 dB Ês2 / Noc = - infinity During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = - infinity During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 5 dB |
| <Unchanged contents are omitted> |  |  |  |  |
| 7.5.1.4 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -14 dB |  | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB During T4: 0 dB -0.4 dB -0.4 dB During T5: 0 dB 1.3 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.9 dB Ês2 / Noc: -15.4 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: 3.3 dB Ês2 / Noc: -15.4 dB |
| 7.5.1.9 NR SA FR2 radio link monitoring UE scheduling restrictions | Same as 5.5.1.9 |  | Same as 5.5.1.9 | Same as 5.5.1.9 |
| 7.5.3.4 NR SA FR2 direct SCell activation at SCell addition of known SCell |  | During T1: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB During T2: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB During T3: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB In signalling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB In signalling: 0dB | During T1: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB During T2: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB During T3: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB In signalling: A3-offset = -15dB |
| <Unchanged contents are omitted> |  |  |  |  |
| 7.7.9.2 NR SA inter frequency FR2 CSI-SINR measurement accuracy | Same as 5.7.9.2 |  | Same as 5.7.9.2 | Same as 5.7.9.2 |

<End of Change 3>

<Start of Change 4>

Table F.1.3.2-8: Derivation of test requirements for NR SA FR2 RRM tests for RedCap

| Test | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) |  | Test requirement in TS 38.533 |
| --- | --- | --- | --- | --- | --- |
| 17.1.1.1 NR SA FR2 Cell reselection to FR2 intra-frequency NR case for 2 Rx | Same as 7.1.1.1 | Same as 7.1.1.1 |  | Same as 7.1.1.1 |  |
| 17.1.1.2 NR SA FR2-FR2 Cell reselection to FR2 inter-frequency NR case | Same as 7.1.1.2 | Same as 7.1.1.2 |  | Same as 7.1.1.2 |  |
| <Unchanged contents are omitted> |  |  |  |  |  |
| 17.3.2.1.2 NR SA FR2-FR2 Inter-frequency RRC Re-establishment in FR2 | Same as 7.3.2.1.2 |  | Same as 7.3.2.1.2 |  | Same as 7.3.2.1.2 |
| 17.3.2.1.3 NR SA FR2 RRC re-establishment without serving cell timing | Same as 7.3.2.1.3 |  | Same as 7.3.2.1.3 |  | Same as 7.3.2.1.3 |
| 17.3.2.3.1 NR SA FR2-FR2 Redirection from NR in FR2 to NR in FR2 | Same as 7.3.2.3 |  | Same as 7.3.2.3 |  | Same as 7.3.2.3 |
| <Unchanged contents are omitted> |  |  |  |  |  |
| 17.5.1.7 NR SA FR2 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode | Same as 7.5.1.7 |  | Same as 7.5.1.7 |  | Same as 7.5.1.7 |
| 17.5.1.8 NR SA FR2 Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode | Same as 7.5.1.8 |  | Same as 7.5.1.8 |  | Same as 7.5.1.8 |
| 17.5.1.9 NR SA FR2 radio link monitoring UE scheduling restrictions | Same as 7.5.1.9 |  | Same as 7.5.1.9 |  | Same as 7.5.1.9 |
| 17.5.2.1 NR SA FR2 Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode | Same as 7.5.5.1 |  | Same as 7.5.5.1 |  | Same as 7.5.5.1 |
| <Unchanged contents are omitted> |  |  |  |  |  |
| 17.7.4.2 Inter-frequency measurement accuracy with FR2 serving cell and FR2 target cell | Same as 7.7.3.2 |  | Same as 7.7.3.2 |  | Same as 7.7.3.2 |

<End of Change 4>