3GPP TSG-RAN5 Meeting #106	R5-250070

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0760 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to PICS of Further NR mobility enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For WI NR_Mob_enh2-Core, below new UE capability has been added into TS 38.306 and 38.331: ltm-interFreqL1-OnlyInBC-r18 Below UE capability descriptions have been updated: pdcch-RACH-PrepTimeList-r18 pdcch-RACH-SwitchingTimeList-r18 interFreqL1-MeasConfig-r18 Below UE capability has been re-named: supportedMaxIntraInterFreqLayersWithGaps-r18 Therefore, the corresponding PICS items need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PICS items were updated in Table A.4.3.2-1 and A.4.3.6-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of corresponding PICS for RRM test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.4.3.2 and A.4.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support PDSCH reception based on semi-persistent scheduling | 38.306, 4.2.7.10 | Rel-15 | pc_downlinkSPS | No |  |  |
| … |  |  |  |  |  |  |  |
| 184 | Support PDCCH-ordered RACH transmission for the corresponding band pair indicating the RF/BB preparation time for PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells | 38.306 4.2.7.5 | Rel-18 | pc_pdcch_RACH_SwitchingTimeList_r18 | No |  | A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18 (pc_rach_EarlyTA_Measurement_r18). |
| 185 | Support PDCCH-ordered RACH transmission for the corresponding band pair indicating the interruption length (Y ms) due to RF re-tuning for PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells | 38.306 4.2.7.5 | Rel-18 | pc_pdcch_RACH_PrepTimeList_r18 | No |  | A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18 (pc_rach_EarlyTA_Measurement_r18). |
| 186 | Support simultaneous transmission to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s) | 38.306 4.2.7.7 | Rel-18 | pc_rach_EarlyTA_BandList_r18 | No |  | A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18 (pc_rach_EarlyTA_Measurement_r18). |
| 187 | Indicates whether the UE supports cell DTX operation by RRC configuration | 38.306 4.2.7.2 | Rel-18 | pc_nes_CellDTX_r18 | No |  | nes-CellDTX-DRX-r18 is set to the value 'cellDTXonly' or 'both' and UE shall also indicate support of longDRX-Cycle |
| 188 | Indicates whether the UE supports cell DRX operation by RRC configuration | 38.306 4.2.7.2 | Rel-18 | pc_nes_CellDRX_r18 | No |  | nes-CellDTX-DRX-r18 is set to the value 'cellDRXonly' or 'both' |
| 189 | Indicates whether the UE supports cell DTX/DRX configuration activation and deactivation via DCI 2_9 | 38.306 4.2.7.2 | Rel-18 | pc_nes_CellDTX_DRX_DCI2_9_r18 | No |  | A UE supporting this feature shall also indicate support of nes-CellDTX-DRX-r18 |
| 190 | Indicates whether the UE supports R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression, when co-scheduled UE(s)' modulation order is explicitly signalled by DCI. | 38.306 4.2.7.10 | Rel-18 | pc_advReceiver_MU_MIMO_r18 | No |  |  |
| 191 | Support intra-frequency LTM for MCG with RACH without NR-DC configured | 38.306 4.2.7.2 | Rel-18 | pc_ltm_MCG_IntraFreq_r18 | No |  | A UE supporting this feature shall also indicate support for ltm-BeamIndicationJointTCI-r18 (pc_ltm_BeamIndicationJointTCI_r18) or ltm-BeamIndicationSeparateTCI-r18 (pc_ltm_BeamIndicationSeparateTCI_r18). |
| 192 | Support intra-frequency LTM for SCG with RACH | 38.306 4.2.7.2 | Rel-18 | pc_ltm_SCG_IntraFreq_r18 | No |  | A UE supporting this feature shall also indicate support for ltm-BeamIndicationJointTCI-r18 (pc_ltm_BeamIndicationJointTCI_r18) or ltm-BeamIndicationSeparateTCI-r18 (pc_ltm_BeamIndicationSeparateTCI_r18). |
| 193 | Support of always including the current SpCell in the L1 measurement report | 38.306 4.2.7.5 | Rel-18 | pc_currentSpCellInclL1_Report_r18 | No |  | A UE supporting this feature shall also indicate support of intraFreqL1-MeasConfig-r18 (pc_intraFreqL1_MeasConfig_r18). |
| 194 | Support of inter-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s), regardless whether the candidate cell(s) are inside or outside of the BC (unless the UE also indicates support of ltm-interFreqL1-OnlyInBC-r18) | 38.306 4.2.7.5 | Rel-18 | pc_interFreqL1_MeasConfig_r18 | No |  | A UE supporting this feature shall also indicate support of intraFreqL1-MeasConfig-r18 (pc_intraFreqL1_MeasConfig_r18). |
| 195 | Support of SSB based inter-frequency L1-RSRP measurements on SSBs within active DL BWP without measurement gaps (without interruption on serving cell(s)) for LTM | 38.306 4.2.7.5 | Rel-18 | pc_interFreqSSB_L1_MeasWithoutGaps_r18 | No |  | A UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18 (pc_interFreqL1_MeasConfig_r18). |
| 196 | Support of intra-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) | 38.306 4.2.7.5 | Rel-18 | pc_intraFreqL1_MeasConfig_r18 | No |  | A UE supporting this feature shall also indicate support of periodicBeamReport or aperiodicBeamReport or sp-BeamReportPUCCH or sp-BeamReportPUSCH. |
| 197 | Support of reporting the maximum number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement | 38.306 4.2.7.5 | Rel-18 | pc_supportedMaxIntraInterFreqLayersWithoutGaps_r18 | No |  | A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 (pc_intraFreqL1_MeasConfig_r18) and/or interFreqSSB-L1-MeasWithoutGaps-r18 (pc_interFreqSSB_L1_MeasWithoutGaps_r18). |
| 198 | Support of reporting the maximum number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps | 38.306 4.2.7.5 | Rel-18 | pc_supportedMaxInterFreqLayersWithGaps_r18 | No |  | A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18 (pc_ltm_InterFreqMeasGap_r18). |
| … |  |  |  |  |  |  |  |

[Skip unchanged text]

A.4.3.6	Measurement Capabilities

Table A.4.3.6-1: UE Measurement Capabilities

| Item | UE Measurement Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support NR measurements and events A triggered reporting | 38.306, 4.2.9 | Rel-15 | pc_eventA_MeasAndReport | Yes |  |  |
| … |  |  |  |  |  |  |  |
| 100 | Support inter-frequency MCG LTM on all the bands where the UE indicates support of ltm-MCG-IntraFreq-r18 or inter-frequency SCG LTM on all the bands where the UE indicates support of ltm-SCG-IntraFreq-r18 respectively | 38.306, 4.2.9 | Rel-18 | pc_ltm_InterFreq_r18 | No |  | A UE supporting this feature shall also indicate support of ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18) or ltm-SCG-IntraFreq-r18 (pc_ltm_SCG_IntraFreq_r18). |
| 100A | Support of inter-frequency L1-RSRP measurement and reporting based on SSB(s) of LTM candidate cell(s) that are inside the BC only in which the UE indicates support of interFreqL1-MeasConfig-r18 | 38.306, 4.2.9 | Rel-18 | pc_ltm_interFreqL1_OnlyInBC_r18 | No |  | A UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18. (pc_interFreqL1_MeasConfig_r18). |
| 101 | Support RACH-less LTM with configured grant for MCG LTM if the UE indicates support of ltm-MCG-IntraFreq-r18 or for SCG LTM if the UE indicates support of ltm-SCG-IntraFreq-r18 respectively | 38.306, 4.2.9 | Rel-18 | pc_ltm_RACH_LessCG_r18 | No |  | A UE indicating support for this feature shall also indicate support of either ltm-BeamIndicationJointTCI-r18 (pc_ltm_BeamIndicationJointTCI_r18) or ltm-BeamIndicationSeparateTCI-r18 (pc_ltm_BeamIndicationSeparateTCI_r18) for at least one band and either ta-IndicationCellSwitch-r18 (pc_ta_IndicationCellSwitch_r18) or ue-TA-Measurement-r18 (pc_ue_TA_Measurement_r18). |
| … |  |  |  |  |  |  |  |

[End of change]