3GPP TSG-RAN5 Meeting #106	R5-250076

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5398 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to NB-IoT NTN enh. TC 22.2.17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. As there is no paging procedure in this case, possiblity of paging sent on same subframe is out of consideration, so no need to modify the default Tcell as defined in TS 36.523-3 Table 7A.9.2-1. (Detail is mentioned in R5-194246 for TC 22.2.5). 2. The satellite ephemeris of Ncell 2 needs to be defined as the 2nd cell in TS 36.508-1 Cl. 6.3.5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Tcell offset is removed in pre-test condition. 2. SIB31-NB of Ncell 2 was updated. 3. Editor`s Note was removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC will still be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.2.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

22.2.17	NB-IoT / NTN / cell reselection / Intra E-UTRAN / distance based measurement

22.2.17.1	Test Purpose (TP)

(1)

with { UE in RRC_IDLE state supports location-based measurement initiation and has obtained its location information }

ensure that {

when { the distance between the UE and the serving cell reference location is larger than or equal to distanceThresh }

then { UE performs intra-frequency measurements and reselects the new cell after UE detects the cell re-selection criteria are met }

}

22.2.17.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 36.304, clause 5.2.4.1, 5.2.4.2a and 5.2.4.6. Unless otherwise stated these are Rel-18 requirements.

[TS 36.304, clause 5.2.4.1]

The UE shall not consider any exclude-listed cells as candidate for cell reselection.

The UE shall only perform cell reselection evaluation for E-UTRAN frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

[TS 36.304, clause 5.2.4.2a]

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the serving cell fulfils Srxlev > SIntraSearchP:

-	If distanceThresh and referenceLocation are broadcast in SystemInformationBlock31-NB, and if the UE has obtained its location:

-	If referenceLocation is set to fixedReferenceLocation and the UE supports location-based measurement initiation for (quasi-)Earth fixed cell, the referenceLocation is used as serving cell reference location.

-	If the distance between UE and serving cell reference location is shorter than distanceThresh, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

-	If referenceLocation is set to movingReferenceLocation and the UE supports location-based measurement initiation for Earth moving cell the UE derives the serving cell reference location based on ephemeris, epochTime and referenceLocation.

-	If the distance between UE and serving cell reference location is shorter than distanceThresh, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

[TS 36.304, clause 5.2.4.6]

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Except for NB-IoT, equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. For NB-IoT equals to QoffsetDedicatedfrequency for any frequency other than the frequency of the dedicated frequency offset, if QoffsetDedicatedfrequency is valid, otherwise this equals to Qoffsetfrequency (if QoffsetDedicatedfrequency is valid Qoffsetfrequency is not used). |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 36.331 [3] |
| QoffsetSCPTM | Offset temporarily applied to an SC-PTM frequency as specified below. The offset is applied to all cells on the SC-PTM frequency. If QoffsetSCPTM is valid, Qoffset for inter-frequency neighbour cells is not used. |

If the NB-IoT UE or UE in enhanced coverage is capable of SC-PTM reception and is receiving or interested to receive an MBMS service and can only receive this MBMS service while camping on a frequency on which it is provided (SC-PTM frequency), the UE considers QoffsetSCPTM to be valid during the MBMS session TS 36.300 [2] as long as the following condition is fulfilled:

Either:

-	SIB15 (or SIB15-NB) of the serving cell indicates for that frequency one or more MBMS SAIs included in the MBMS User Service Description (USD) TS 26.346 [22] of this service; or

-	SIB15 (or SIB15-NB) is not broadcast in the serving cell and that frequency is included in the USD of this service.

NOTE:	UE should search for a higher ranked cell on another frequency for cell reselection as soon as possible after the UE stops using QoffsetSCPTM.

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2 (5.2.3.2a for NB-IoT), but may exclude all CSG cells that are known by the UE not to be CSG member cells.

The cells shall be ranked according to the R criteria specified above, deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If a cell is ranked as the best cell the UE shall perform cell reselection to that cell. If this cell is found to be not-suitable, the UE shall behave according to clause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better ranked than the serving cell during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

When the UE uses infinite dBs for QoffsetSCPTM, the UE shall use QoffsetSCPTM zero and rank the cells on the SC-PTM frequency(ies) only first. If the UE cannot find a suitable cell on an SC-PTM frequency, the UE shall rank the cells on all frequencies.

22.2.17.3	Test description

22.2.17.3.1	Pre-test conditions

System Simulator:

-	Ncells 1, 2 in different tracking areas according to TS 36.508 [18] clause 8.1.4.2.

-	System information combination 9 as defined in TS 36.508 [18] clause 8.1.4.3.1 is used.

UE:

-	The pre-configured UE location is defined in TS 36.508 [18] Clause 4.13.

Preamble:

-	The UE is in State 3-NB Registered, Idle Mode on Ncell 1 according to TS 36.508 [18].

22.2.17.3.2	Test procedure sequence

Table 22.2.17.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration T0 indicates the initial conditions for preamble. Configurations marked "T1" are applied at the points indicated in the Main behaviour description in Table 22.2.17.3.2-2.

Table 22.2.17.3.2-1: Time instances of cell power level and parameter change

|  | Parameter | Unit | Ncell 1 | Ncell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | Cell-specific NRS EPRE | dBm/15kHz dB | -91 | “Off” | - The power level values are assigned to ensure the UE registered on NR Cell 1 and Srxlev of Ncell 1 is greater than SintrasearchP. - The distance between the UE and the Ncell 1 reference location is larger than or equal to distanceThresh. |
|  | SintrasearchP | dB | 12 | - |  |
| T1 | Cell-specific NRS EPRE | dBm/15kHz dB | -91 | -79 | - Ncell 2 becomes the highest ranked cell. - The distance between the UE and the Ncell 2 reference location is shorter than distanceThresh. |
|  | SintrasearchP | dB | 12 | 12 |  |

Table 22.2.17.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS re-adjusts the cell-specific reference signal levels according to row "T1" in Table 22.2.17.3.2-1. | - | - | - | - |
| 2 | Wait for 34 seconds to allow UE to recognise the change. | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 36.508 [18] subclause 8.1.5A.5 indicate that the UE is camped on Ncell 2? NOTE: The UE performs a TAU procedure and the RRC connection is released. | - | - | 1 | - |

22.2.17.3.3	Specific message contents

Table 22.2.17.3.3-1: SystemInformationBlockType3-NB for Ncells 1 and 2 (Preamble and step 1, Table 22.2.17.3.2-2)

| Derivation path: TS 36.508 [18] Table 8.1.4.3.3-2 |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| SystemInformationBlockType3-NB-r13 ::= SEQUENCE { |  |  |
| intraFreqCellReselectionInfo-r13 SEQUENCE { |  |  |
| s-IntraSearchP-r13 | 6 | 12 dB |
| } |  |  |
| } |  |  |

Table 22.2.17.3.3-2: SystemInformationBlockType31-NB for Ncell 1 (Preamble and step 1, Table 22.2.17.3.2-2)

Derivation Path: TS 36.508 [18] Table 8.1.4.3.3-10 with condition NTN enh.

Table 22.2.17.3.3-3: SystemInformationBlockType31-NB for Ncell 2 (step 1, Table 22.2.17.3.2-2)

| Derivation Path: TS 36.508 [18] Table 8.1.4.3.3-10 with condition NTN enh. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |
| servingSatelliteInfo-r17 | Refer to the 2nd cell in TS 36.508 Table 6.3.5.1-1 |  | GSO |  |
| servingSatelliteInfo-r17 | Refer to the 2nd cell in TS 36.508 Table 6.3.5.2-1 |  | NGSO |  |
| servingSatelliteInfo-v1820 SEQUENCE { |  |  |  |  |
| distanceThresh-r18 | 5000 |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

[End of change]