3GPP TSG-RAN5 Meeting #106	R5-250091

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5400 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MDT TC 8.6.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Tejet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2025-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In T4 of power level Table 8.6.2.3.3.2-1, UE camps on Cell 1 and TP3 shall checks against Cell 1. The specfic RRC message needs to be updated based on TS 36.331 and 36.508. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Cell 2 was updated as Cell 1 in T4 of Table 8.6.2.3.3.2-1. Specific message contents were updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Error will exist in this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

8.6.2.3	Logged MDT / Logging and reporting / Limiting area scope

8.6.2.3.1	Test Purpose (TP)

(1)

with { UE received LoggedMeasurementConfiguration message with a cellGlobalIdList on a LTE cell }

ensure that {

when { UE camps on a LTE cell in the cellGlobalIdList }

then { UE performs logged measurements }

}

(2)

with { UE received LoggedMeasurementConfiguration message with a cellGlobalIdList on a LTE cell }

ensure that {
  when { UE camps on a LTE cell not in the cellGlobalIdList }

then { UE does not perform logged measurements }

}

(3)

with { UE received LoggedMeasurementConfiguration message with a trackingAreaCodeList on a LTE cell }

ensure that {
  when { UE camps on a LTE cell in the trackingAreaCodeList }

then { UE performs logged measurements }

}

(4)

with { UE received LoggedMeasurementConfiguration message with a trackingAreaCodeList on a LTE cell }

ensure that {
  when { UE camps on a LTE cell not in the trackingAreaCodeList }

then { UE does not perform logged measurements }

}

8.6.2.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.304, clause 8; TS 36.331 clauses 5.6.6.3, 5.6.8.2, 6.3.4 and 6.3.6.

[TS 36.304, clause 8 (TP1,TP2,TP3,TP4)]

The UE may be configured to perform logging of measurement results in RRC_IDLE mode with the LoggedMeasurementsConfiguration message as specified in TS 36.331 [3]. This configuration is valid while the logging duration timer is running.

If the configuration of logged measurements is valid, the UE shall perform logging of measurement results if all of the following conditions are met:

-	The UE is in camped normally state in RRC_IDLE mode;

-	RPLMN of the UE is the same as the RPLMN at the point of time of LoggedMeasurementConfiguration message reception;

-	The UE is camped on a cell belonging to the areaConfiguration (see TS 36.331 [3]), if configured;

-	The UE is camped on the RAT where the logged measurement configuration was received.

Otherwise, the logging of measurement results shall be suspended.

NOTE 1:	Even if logging of measurement results is suspended, the logging duration timer and time stamp will continue, and the logged measurement configuration and corresponding log are kept.

[TS 36.331, clause 5.6.6.3 (TP1,TP2,TP3,TP4)]

Upon receiving the LoggedMeasurementConfiguration message the UE shall:

1>	discard the logged measurement configuration as well as the logged measurement information as specified in 5.6.7;

1>	store the received loggingDuration, loggingInterval and areaConfiguration, if included, in VarLogMeasConfig;

1>	store the RPLMN as plmn-Identity in VarLogMeasReport;

1>	store the received absoluteTimeInfo, traceReference, traceRecordingSessionRef and tce-Id in VarLogMeasReport;

1>	start timer T330 with the timer value set to the loggingDuration;

[TS 36.331, clause 5.6.8.2 (TP1,TP2,TP3,TP4)]

While T330 is running, the UE shall:

1>	perform the logging in accordance with the following:

2>	if the UE is camping normally on an E-UTRA cell and the RPLMN of the UE is the same as the plmn-Identity stored in VarLogMeasReport and, if the cell is part of the area indicated by areaConfiguration if configured in VarLogMeasConfig:

3>	perform the logging at regular time intervals, as defined by the loggingInterval in VarLogMeasConfig;

2>	when adding a logged measurement entry in VarLogMeasReport, include the fields in accordance with the following:

3>	set the relativeTimeStamp to indicate the elapsed time since the moment at which the logged measurement configuration was received;

3>	if detailed location information became available during the last logging interval, set the content of the locationInfo as follows:

4>	include the locationCoordinates;

3>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

3>	set the measResultServCell to include the quantities of the cell the UE is camping on;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements that became available during the last logging interval for at most the following number of neighbouring cells; 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies (GERAN) per RAT;

NOTE 2:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 36.133 [16].

2>	when the memory reserved for the logged measurement information becomes full, stop timer T330 and perform the same actions as performed upon expiry of T330, as specified in 5.6.6.4;

[TS 36.331, clause 6.3.6 (TP1, TP2, TP3, TP4)]

The AreaConfiguration indicates area for which UE is requested to perform measurement logging. If not configured, measurement logging applies in the entire RPLMN of the UE at the point of receiving the configuration

AreaConfiguration information element

-- ASN1START

AreaConfiguration-r10 ::=	CHOICE {

cellGlobalIdList-r10			CellGlobalIdList-r10,

trackingAreaCodeList-r10		TrackingAreaCodeList-r10

}

CellGlobalIdList-r10 ::=				SEQUENCE (SIZE (1..32)) OF CellGlobalIdEUTRA

TrackingAreaCodeList-r10 ::=			SEQUENCE (SIZE (1..8)) OF TrackingAreaCode

-- ASN1STOP

[TS 36.331, clause 6.3.4 (TP1,TP2)]

The IE CellGlobalIdEUTRA specifies the Evolved Cell Global Identifier (ECGI), the globally unique identity of a cell in E-UTRA.

CellGlobalIdEUTRA information element

-- ASN1START

CellGlobalIdEUTRA ::=					SEQUENCE {

plmn-Identity							PLMN-Identity,

cellIdentity							CellIdentity

}

-- ASN1STOP

[TS 36.331, clause 6.3.4 (TP3,TP4)]

The IE TrackingAreaCode is used to identify a tracking area within the scope of a PLMN, see TS 24.301 [35].

TrackingAreaCode information element

-- ASN1START

TrackingAreaCode ::=				BIT STRING (SIZE (16))

-- ASN1STOP

8.6.2.3.3	Test description

8.6.2.3.3.1	Pre-test conditions

System Simulator:

-	Cell 1, Cell 2 and Cell 11.

-	System information combination 2 as defined in TS 36.508 [18] clause 4.4.3.1 is used in E-UTRA cells.

UE:

None

Preamble:

-	The UE is in state Generic RB Established (state 3) on Cell 1 according to [18].

8.6.2.3.3.2	Test procedure sequence

Table 8.6.2.3.3.2-1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1”, “T2”, “T3”and “T4” are applied at the points indicated in the Main behaviour description in Table 8.6.2.3.3.2-2.

Table 8.6.2.3.3.2-1: Cell configuration changes over time

|  | Parameter | Unit | Cell 1 | Cell 2 | Cell 11 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | RS EPRE | dBm/15kHz | -85 | -91 | -91 | Configure logged measurement with limiting area scope using cellGlobalIdList |
| T1 | RS EPRE | dBm/15kHz | -91 | -91 | -85 | Verify that UE is perform logging of Cell 11 (TP1, Cell 11 is configured in cellGlobalIdList) |
| T2 | RS EPRE | dBm/15kHz | -91 | -85 | -91 | Verify that UE is not logging Cell 2 (TP2, Cell 2 is not configured in cellGlobalIdList) and then re-configure logged measurement with limiting area scope using trackingAreaCodeList |
| T3 | RS EPRE | dBm/15kHz | -91 | -91 | -85 | Verify that UE is not perform logging of Cell 11 (TP4, Cell 11 is not in configured trackingAreaCodeList) |
| T4 | RS EPRE | dBm/15kHz | -85 | -91 | -91 | Verify that UE is logging Cell 1 (TP3, Cell 1 is in configured trackingAreaCodeList) |

Table 8.6.2.3.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a LoggedMeasurementConfiguration message with a cellGlobalIdList on Cell 1. | <-- | LoggedMeasurementConfiguration | - | - |
| 2 | SS transmits a RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 3 | The SS changes Cell 1, Cell 2 and Cell 11 level according to the row "T1" in Table 8.6.2.3.3.2-1. | - | - | - | - |
| 4-9 | Steps 1 to 6 of generic test procedure in TS 36.508 subclause 6.4.2.7. And the UE move to idle mode on Cell 11. | - | - | - | - |
| 10 | Wait 30 seconds for UE logging interval timer to expire at least once. | - | - | - | - |
| 11-18 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 11. | - | - | - | - |
| 19 | The SS send a UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest |  |  |
| 20 | Check: Does the UE send an UEInformationResponse message including at least one logMeasReport with serving cell measurements for Cell 11? | --> | UEInformationResponse | 1 | P |
| 21 | SS transmits a RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 22 | The SS changes Cell 1, Cell 2 and Cell 11 level according to the row "T2" in Table 8.6.2.3.3.2-1. | - | - | - | - |
| 23-28 | Steps 1 to 6 of generic test procedure in TS 36.508 subclause 6.4.2.7. And the UE move to idle mode on Cell 2. | - | - | - | - |
| 29 | Wait 30 seconds for UE logging interval timer to expire at least once. | - | - | - | - |
| 30 | The SS transmits a Paging message. | <-- | Paging | - | - |
| 31 | The UE transmits an RRCConnectionRequest message. | --> | RRCConnectionRequest | - | - |
| 32 | SS transmit an RRCConnectionSetup message. | <-- | RRCConnectionSetup | - | - |
| - | EXCEPTION: In case the UE had performed a logging before the cell re-selection to Cell 2 then the steps 33a1-33a7 are executed. |  |  |  |  |
| 33a1 | The UE transmit an RRCConnectionSetupComplete message including logMeasAvailable IE set it to true. | --> | RRCConnectionSetupComplete | - | - |
| 33a2-33a5 | Steps 6 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 2. | - | - | - | - |
| 33a6 | The SS send an UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest | - | - |
| 33a7 | Check: Does the UE send an UEInformationResponse message with logged serving cell measurements of Cell 2? | --> | UEInformationResponse | 2 | F |
| - | EXCEPTION: In case the UE had not performed any logging before the cell re-selection to Cell 2 then the step 33b1 is executed. | - | - | - | - |
| 33b1 | Check: Does UE transmit an RRCConnectionSetupComplete message with logMeasAvailable IE not present? | --> | RRCConnectionSetupComplete | 2 | P |
| 34 | The SS transmits a LoggedMeasurementConfiguration message with a trackingAreaCodeList on Cell 2. | <-- | LoggedMeasurementConfiguration | - | - |
| 35 | The SS transmits a RRCConnectionRelease message to release RRC connection and move to RRC_IDLE on Cell 2. | <-- | RRCConnectionRelease | - | - |
| 36 | The SS changes Cell 1, Cell 2 and Cell 11 level according to the row "T3" in Table 8.6.2.3.3.2-1. | - | - | - | - |
| 37-42 | Steps 1 to 6 of generic test procedure in TS 36.508 subclause 6.4.2.7. And the UE move to idle mode on Cell 11. | - | - | - | - |
| 43 | The SS transmits a Paging message on Cell 11. | <-- | Paging | - | - |
| 44 | The UE transmits an RRCConnectionRequest message on Cell 11. | --> | RRCConnectionRequest | - | - |
| 45 | The SS transmit an RRCConnectionSetup message on Cell 11. | <-- | RRCConnectionSetup | - | - |
| - | EXCEPTION: In case the UE had performed a logging before the cell re-selection to Cell 11 then the steps 46a1-46a7 are executed. | - | - | - | - |
| 46a1 | The UE transmit an RRCConnectionSetupComplete message including logMeasAvailable IE set it to true. | --> | RRCConnectionSetupComplete | - | - |
| 46a2-46a5 | Steps 6 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 11. | - | - | - | - |
| 46a6 | The SS send a UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest | - | - |
| 46a7 | Check: Does the UE send an UEInformationResponse message with logged serving cell measurements of Cell 11? | --> | UEInformationResponse | 4 | F |
| - | EXCEPTION: In case the UE had not performed any logging before the cell re-selection to Cell 11 then the step 46b1 is executed. | - | - | - | - |
| 46b1 | Check: Does UE transmit an RRCConnectionSetupComplete message with logMeasAvailable IE not present? | --> | RRCConnectionSetupComplete | 4 | P |
| 47 | The SS transmits an RRCConnectionRelease message to release the RRC connection. | <-- | RRCConnectionRelease | - | - |
| 48 | The SS changes Cell 1, Cell 2 and Cell 11 level according to the row "T4" in Table 8.6.2.3.3.2-1. | - | - | - | - |
| 49-54 | Steps 1 to 6 of generic test procedure in TS 36.508 subclause 6.4.2.7. And the UE move to idle mode on Cell 1. | - | - | - | - |
| 55 | Wait 30 seconds for UE logging interval timer to expire at least once, | - | - | - | - |
| 56-63 | Steps 2 to 9 of the generic radio bearer establishment procedure in TS 36.508 subclause 4.5.3.3 are executed to successfully complete the service request procedure on Cell 1. | - | - | - | - |
| 64 | The SS send an UEInformationRequest message to get logMeasReport. | <-- | UEInformationRequest | - | - |
| 65 | Check: Does the UE send an UEInformationResponse include logMeasReport with logged serving cell measurements of Cell 1? | --> | UEInformationResponse | 3 | P |

8.6.2.3.3.3	Specific message contents

Table 8.6.2.3.3.3-1: SystemInformationBlockType3 for cell 1 (preamble)

| Derivation Path: 36.508 table 4.4.3.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| t-ReselectionEUTRA | 7 | 7 seconds |  |
| } |  |  |  |
| } |  |  |  |

Table 8.6.2.3.3.3-2: LoggedMeasurementConfiguration (step 1, Table 8.6.2.3.3.2-2)

| Derivation path: 36.508 table 4.6.1-4.0A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| areaConfiguration-r10 CHOICE { |  |  |  |  |
| cellGlobalIdList-r10 SEQUENCE { |  | 1 entry |  |  |
| plmn-Identity[1] |  | plmn-Identity of Cell 11 |  |  |
| cellIdentity[1] |  | cellIdentity of Cell 11 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.3.3.3-3: UEInformationRequest (steps 19, 33a6, 46a6 and 64, Table 8.6.2.3.3.2-2)

Derivation Path: 36.508 table 4.6.1-23A,with condition “Logged MDT”

Table 8.6.2.3.3.3-4: UEInformationResponse (steps 20 and 33a7, Table 8.6.2.3.3.2-2)

| Derivation Path: 36.508 table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 1 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF LogMeasInfo-r10 SEQUENCE { |  | At least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 11 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10 |  | Not checked |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.3.3.3-5: RRCConnectionSetupComplete (steps 33a1 and 46a1, Table 8.6.2.3.3.2-2)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| gummei-Type-r10 |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | true |  |  |  |  |  |
| rn-SubframeConfigReq-r10 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.2.3.3.3-6: RRCConnectionSetupComplete (steps 33b1 and 46b1, Table 8.6.2.3.3.2-2)

| Derivation path: 36.508 table 4.6.1-18 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |
| gummei-Type-r10 |  | Not checked |  |  |  |  |  |
| rlf-InfoAvailable-r10 |  | Not checked |  |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |  |  |  |
| rn-SubframeConfigReq-r10 |  | Not checked |  |  |  |  |  |
| nonCriticalExtension |  | Not checked |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.6.2.3.3.3-7: LoggedMeasurementConfiguration (step 34, Table 8.6.2.3.3.2-2)

| Derivation path: 36.508 table 4.6.1-4.0A |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LoggedMeasurementConfiguration-r10 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| loggedMeasurementConfiguration-r10 SEQUENCE { |  |  |  |  |
| areaConfiguration-r10 CHOICE { |  |  |  |  |
| trackingAreaCodeList-r10 SEQUENCE { |  | 1 entry |  |  |
| trackingAreaCode [1] |  | TAC = 1 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| loggingInterval-r10 |  | ms2560 | 2.56 seconds |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.3.3.3-8: UEInformationResponse (step 46a7, Table 8.6.2.3.3.2-2)

| Derivation Path: 36.508 table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF LogMeasInfo-r10 SEQUENCE { |  | At least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 2 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10 |  | Not checked |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 8.6.2.3.3.3-9: UEInformationResponse (step 65, Table 8.6.2.3.3.2-2)

| Derivation Path: 36.508 table 4.6.1-23B |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE { |  |  |  |  |
| absoluteTimeStamp-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| traceReference-r10 SEQUENCE { |  |  |  |  |
| plmn-Identity-r10 SEQUENCE { |  |  |  |  |
| mcc SEQUENCE (SIZE (3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| mnc SEQUENCE (SIZE (2..3)) OF MCC-NMC-Digit |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| } |  |  |  |  |
| traceId-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| } |  |  |  |  |
| traceRecordingSessionRef-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| tce-Id-r10 |  | Same value as sent by SS in LoggedMeasurementConfiguration in step 34 |  |  |
| logMeasInfoList-r10 SEQUENCE (SIZE (1..maxLogMeasReport-r10)) OF LogMeasInfo-r10 SEQUENCE { |  | At least one entry complies to entry with index ‘x’ below. SS records the relativeTimeStamp value for each entry |  |  |
| locationInfo-r10[x] |  | Not checked |  |  |
| relativeTimeStamp-r10 [x] |  | SS record the value |  |  |
| servCellIdentity-r10 [x] |  | Same as Cell 1 |  |  |
| measResultServCell-r10 [x] SEQUENCE { |  |  |  |  |
| rsrpResult-r10 |  | (0..97) |  |  |
| rsrqResult-r10 |  | (0..34) |  |  |
| } |  |  |  |  |
| measResultNeighCells-r10 |  | Not checked |  |  |
| } |  |  |  |  |
| logMeasAvailable-r10 |  | Not present |  |  |
| } |  |  |  |  |
| nonCriticalExtension |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

[End of change]