Title:	Discussion on test point selection for NS_06 A-MPR in FR1, including PC1 for n14 and PC1 for n85

Document for:	Discussion and approval

# Background

The general agreement in RAN5 is that FR1 testing should follow what is being tested for LTE. This is also valid for test point selection for A-MPR in FR1.

This discussion paper is very similar to the TP analysis of NS_35 version 2 [6]. Only operating bands and channel bandwidths differentiate between NS_35 and NS_06. Which means that the proposals approved for NS_35 should be applicable to NS_06.

Both NS_06 and NS_35 should be used for A-MPR and A-SEM testing, where A-MPR is defined as N/A for power classes 1.5, 2 and 3 and power class 1 for n14, which means that the MPR requirement should be used instead of A-MPR. For band n85, A-MPR is specified.

For NS_06 power class 1 for n14 one modulation DFT-s-OFDM Pi / 2 BPSK w Pi / 2 BPSK DMRS is added to the other 9 modulation types.

# Discussion

The selection of test points below is partly based on approved test point analyzes for SEM [2, 3] and for FR1. In general for A-MPR the emissions minimum requirements are SEM and/or Spurious. Specifically, for NS_06 only SEM requirements apply and no A-MPR is allowed.

For NS_06 power class 1 operation on band n85, A-MPR is specified in Clause 6.2.3.32 of TS 38.101-1 [7] as following:

## Test Environment

Based on the baseline agreed in [2], the test environment for NR A-MPR measurement for FR1 can be defined as in LTE.

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR in FR1.

## Test Frequencies

The selection of test frequencies can be based on the agreed proposals for SEM [2], i.e. Low and High frequency.

PROPOSAL 2: Select Low range and High range test frequencies for NS_06 A-MPR in FR1.

## Test Channel Bandwidth

Minimum requirements for NS_06 applies for all channel bandwidths. Select channel bandwidth to test as for SEM [2]

PROPOSAL 3: Select Lowest and Highest test channel bandwidth for NS_06 A-MPR in FR1.

## Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for SEM [2], i.e. lowest and highest supported subcarrier spacing.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_06 A-MPR in FR1.

## UL Modulation

The selection of UL modulation can be based on the agreed proposals for SEM [2], i.e. all defined UL waveforms (5 DFT-s OFDM and 4 CP-OFDM waveforms). For NS_06 power class 1 for n14 one modulation (DFT-s-OFDM Pi / 2 BPSK w Pi / 2 BPSK DMRS) is added to the other 9 modulation types.

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_06 A-MPR in FR1.

## Uplink RB allocation

Since there is no additional MPR requirements for NS_06, the selection of UL RB allocation can be based on the agreed proposals for SEM [2]. This applies to power classes 1.5, 2 and 3 and power class 1 for n14.

PROPOSAL 6: For NS_06 A-MPR base the uplink RB allocation on the agreed proposal for SEM for power classes 1.5, 2 and 3 and power class 1 for n14.

## Uplink RB allocation for NS_06 PC1 on band n85

NS_06 can be signalled for a UE supporting band n85, where FUL_low = 698 MHz and where FUL_high = 716 MHz. The Maximum transmission bandwidth for NR is shown in figure 1 from [7].

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Figure 1: Maximum transmission badwidth.

The following channel bandwidths (BWChannel) are supported for NR band n85 in [7]. 3 MHz CBW is optional in Rel-18 version of the specification.

Figure 2: Channel bandwiths for n85.

The A -MPR requirement for NS_06 PC1 on band n85 is shown in clause 2. RB allocations are calculated in Table 1 and Table 2.

Table 1: A-MPR regions for ( LCRB ≤ 0.20 ∙ NRB and ( RBstart = 0 or RBstart + LCRB = NRB ) )

| CBW | SCS | NRB | 0.20 ∙ NRB | LCRB | RBstart, high | RBstart, low |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | 15 | 3 | 3 | 12 | 0 |
| 5 | 15 | 25 | 5 | 5 | 20 | 0 |
| 10 | 15 | 52 | 10.4 | 10 | 15 | 0 |
| 15 | 15 | 79 | 15.8 | 15 | 64 | 0 |
| 10 | 30 | 24 | 4.8 | 4 | 20 | 0 |
| 15 | 30 | 38 | 7.6 | 7 | 31 | 0 |

Select RB allocations RBstart, low and RBstart, high fulfilling Table 1 based on the agreed proposals for SEM [2].

- Edge_1RB_Left is selected for Low frequency, A-MPR apply

- Edge_1RB_Right is selected for High frequency, A-MPR apply

- Outer_Full is selected for both Low and High, no A-MPR.

Table 2: A-MPR regions for ( LCRB = 1 and 5 ∙ | RBstart + 0.5 – NRB / 2 | ∙ 12 ∙ SCS ≥ 1.5 ∙ CBW + 5 MHz )

| CBW | SCS | NRB | 1.5 ∙ CBW + 5 MHz | NRB / 2 | RBstart | 5 ∙ ¦ RBstart + 0.5 – NRB / 2 ¦ ∙ 12 ∙ SCS |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | 15 | 15 | 9.5 | 7.5 | 15 | 7.2 |
| 5 | 15 | 25 | 12.5 | 12.5 | 25 | 11.7 |
| 10 | 15 | 52 | 20 | 26 | 48 | 20.25 |
| 15 | 15 | 79 | 27.5 | 39.5 | 70 | 27.9 |
| 10 | 30 | 24 | 20 | 12 | 23 | 20.7 |
| 15 | 30 | 38 | 27.5 | 19 | 34 | 27.9 |

Lowest possible RB allocations RBstart are shown in Table 2. Considering the higher edge of the band as worst case it would be sufficient to test requirement with Edge_1RB_Right allocation only.

PROPOSAL 7: For NS_06 power class 1 operation on band n85, the uplink RB allocation is according to existing test configuration.

2.7.	Number of Test Points

The number of test points for NS_06 AMPR is equal to the number of test points is 144 for power class 3 and 160 for power class 1 for n14.

For NS_06 power class 1 operation on band n85, the number of test points is 144.

2.8.	Exceptions for A-MPR UL-MIMO test

The requirements of A-MPR for UL-MIMO is the same as the requirements of single antenna A-MPR test case 6.2.3, therefore the test point selection in proposal 1~6 can be reused without any change.

CP-OFDM is mandatory in case UL-MIMO transmission with two-layer data, only CP-OFDM test points selected in proposal 5 are valid for UL-MIMO.

PROPOSAL 8: Test points selected in proposals 1~6 can be reused by A-MPR UL-MIMO testing, and only CP-OFDM test points selected in proposal 5 are reused for UL-MIMO.

# Proposal

It is proposed that RAN5 discusses and agrees the following proposals for open areas described in section 2 of this document to progress on SA FR1 NS_06 A-MPR in FR1 test case definition:

PROPOSAL 1: Select Test Environment as Normal Conditions (NC) for A-MPR in FR1.

PROPOSAL 2: Select Low range and High range test frequencies for NS_06 A-MPR in FR1.

PROPOSAL 3: Select Lowest and Highest test channel bandwidth for NS_06 A-MPR in FR1.

PROPOSAL 4: Select Lowest and Highest supported subcarrier spacing for NS_06 A-MPR in FR1.

PROPOSAL 5: Test all UL modulation waveforms supported by the UE for NS_06 A-MPR in FR1

PROPOSAL 6: For NS_06 A-MPR base the uplink RB allocation on the agreed proposal for SEM for power classes 1.5, 2 and 3 and power class 1 for n14.

PROPOSAL 7: For NS_06 power class 1 operation on band n85, the uplink RB allocation is according to existing test configuration.

PROPOSAL 8: Test points selected in proposals 1~6 can be reused by A-MPR UL-MIMO testing, and only CP-OFDM test points selected in proposal 5 are reused for UL-MIMO.

# References

- R5-180065, "Discussion about Test parameters for NR TRx TCs " (NTT DOCOMO).

- R5-180886, " Discussion on test point selection for NR SEM in FR1” (Keysight Technologies).

- R5-182019, “Discussion of NR FR1 Test Point for TX Spurious Emission test cases”, (Qualcomm Inc.)

- R5-186256, “Update test points analysis for multiple FR1 test cases”, (Huawei, HiSilicon, Keysight, CAICT)

- TS 38.101-1 v15.5.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Rel-15)

- R5-194169, “Discussion on test point selection for NS_35 A-MPR in FR1, Ericsson, Huawei, Hisilicon, RAN5#83, Reno, Athens, May.2019

- TS 38.101-1 v18.6.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Rel-18)