3GPP TSG-RAN5 Meeting #106	R5-250258

# Athens, Greece, 17 – 21 Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.509 | CR | - | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification of Beamlock test function to support locking two TX beams from two panels simultaneously |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Missing support locking two TX beams from two panels simultaneously |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extending the current FR2 UE Beamlock test function (UBF) to lock UE two Tx beams from two panels simultaneously by - adding two TX beam support for UBF in clause 5.4 - adding beamlock message content in clause 6.4.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Locking two TX beams from two panels simultaneously in FR2 UBF will not be specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4, 6.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.508-1 ... CR 3421 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Discusion document in R5-250256. R5-250257 (CR 3421) to 38.508-1. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------- Start of changes --------

## 5.4	UE Beamlock test Function (UBF)

### 5.4.1	General

The UE Beamlock test function is intended for making the UE to lock the UE antenna pattern(s) once it has formed beam(s) towards the base station (SS) direction(s) following the cell identification procedure in preparation for subsequent test procedures. Activating the UBF shall lock the antenna pattern(s) of all active intra-band component carriers and all MIMO layers affected by the test function.

The Beamlock test function is mandatory for applicable UEs operating in Frequency Range 2 (FR2).

The SS uses the UE Beamlock test mode activation procedure to command the UE to lock the UE antenna pattern(s). The Beamlock activation procedure can apply to UE transmitter and UE receiver beams either simultaneously or independently.

Figure 5.4.1-1: UE Beamlock test mode activation procedure

The SS uses the UE Beamlock test mode deactivation procedure to command the UE to re-tracking the beam(s) towards the base station direction(s). The Beamlock deactivation procedure can apply to UE transmitter and UE receiver beams either simultaneously or independently.

Figure 5.4.1-2: UE Beamlock test mode deactivation procedure

### 5.4.2	Activate Beamlock procedure

#### 5.4.2.1	Initiation

The SS requests the UE to activate beamlock by transmitting an ACTIVATE BEAMLOCK message.

#### 5.4.2.2	Reception of ACTIVATE BEAMLOCK message by UE

When UE receives ACTIVATE BEAMLOCK message then the UE shall:

1>	if the UE is operating in FR2 AND is in RRC_CONNECTED state:

2>	if bits 2 and 1 in UE Beamlock test Function = 01

3>	Lock the UE antenna pattern with Tx only

2>	else if bits 2 and 1 in UE Beamlock test Function = 10

3>	Lock the UE antenna pattern with Rx only

2>	else if bits 2 and 1 in UE Beamlock test Function = 11

3>	Lock the UE antenna pattern with both TxRx

2>	if bit 3 in UE Beamlock test Function = 0

3>	Lock the best UE antenna pattern with Tx

2>	else if bit 3 in UE Beamlock test Function = 1

3>	Lock the best two UE antenna patterns with Tx

2>	Transmit ACTIVATE BEAMLOCK COMPLETE message

1>	else:

2>	the UE behaviour is unspecified.

### 5.4.3	Deactivate Beamlock procedure

#### 5.4.3.1	Initiation

The SS requests the UE to deactivate beamlock by transmitting a DEACTIVATE BEAMLOCK message. The SS should do this when the UE is in RRC_CONNECTED state.

#### 5.4.3.2	Reception of DEACTIVATE BEAMLOCK message by UE

When UE receives DEACTIVATE BEAMLOCK message then the UE shall:

1>	if the UE is operating in FR2 AND is in RRC_CONNECTED state AND the UE Beamlock test function is active:

2>	unlock all active UE antenna pattern(s) and transmit DEACTIVATE BEAMLOCK COMPLETE message;

1> else:

2>	the UE behaviour is unspecified.

#### 5.4.3.3	Release of antenna beamlock by UE

When the UE leaves the RRC_CONNECTED state, the UE shall:

1>	if the UE is operating in FR2 AND the UE Beamlock test Function is active

2>	unlock all active UE antenna pattern(s);

-------- Skipped unchanged clauses --------

## 6.4	Beamlock messages

### 6.4.1	ACTIVATE BEAMLOCK

This message is only sent in the direction SS to UE.

| Information Element | Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- |
| Protocol discriminator | TS 24.007 [5], sub clause 11.2.3.1.1 | M | V | ½ |
| Skip indicator | TS 24.007 [5], sub clause 11.2.3.1.2 | M | V | ½ |
| Message type |  | M | V | 1 |
| UE Beamlock test Function |  | M | V | 1 |

where message type is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | octet 1 |

where UE Beamlock test Function is:

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | bit no. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | X1 | X2 | octet 1 |

where X1,X2 = 01 for activate beamlock of Tx only, 10 for activate beamlock of Rx only and 11 for activate beamlock of both TxRx,

where T1,T2 = 00 for activate beamlock of best Tx antenna pattern, 01 for activate beamlock of best two Tx antenna patterns.

NOTE: X1,X2 = 00 is not used

-------- End of changes --------