3GPP TSG-RAN5 Meeting #106	R5-250480

Athens, Greece, 17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 3683 | rev |  | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to Annexure to address less than 5 MHz test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | # NR_FR1_lessthan_5MHz_BW-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Misalignment with TS 38.133 v18.8.0 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aligning the CORESET for RMSI and RMC scheduling to TS 38.133 v18.8.0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between RAN4 and RAN5 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1.2, A.1.3, C.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## A.1.2	CORESET for RMSI scheduling

### A.1.2.1	FDD

--- Start of Change 1 ---

Table A.1.2.1-1: RMSI CORESET Reference Measurement Channels for SCS = 15 kHz for FDD

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | CR.1.1 FDD | CR.1.2 FDD | CR.1.3 FDD | CR.1.4 FDD |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case |
| Subcarrier spacing for RMSI CORESET | kHz | 15 | 15 | 15 | 15 |
| Allocated resource blocks for RMSI CORESETNote 7 |  | 24 | 12 | 15 | 20 |
| Subcarrier spacing for SSB | kHz | 15 | 15 | 15 | 15 |
| SSB and RMSI CORESET multiplexing configuration |  | Pattern 1 | Pattern 1 | Pattern 1 | Pattern 1 |
| Offset between SSB and RMSI CORESET Note 3, 7 | RB | 0 (Note 8) | 0 (Note8) | 0 (Note8) | 0 (Note8) |
| Configuration of PDCCH monitoring occasions for RMSI CORESET Note 4 |  | Index 4 | Index 4 | Index 4 | Index 4 |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 |
| Duration of RMSI CORESETNote 7 | symbols | 2 | 2 | 3 | 3 |
| DCI Format Note 1 |  | Note 2 | Note 2 | Note 2 | Note 2 |
| Aggregation level | CCE | 8 | 4 | 8 | 8 |
| DMRS precoder granularity |  | 6 | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| REG bundle size |  | 6 | 6 | 6 | 6 |
| Mapping from REG to CCE |  | Distributed | Distributed | Non-Distributed | Distributed |
| Cell ID |  | Note 5 | Note 5 | Note 5 | Note 5 |
| Payload (without CRC) | Bits | Note 6 | Note 6 | Note 6 | Note 6 |
| NOTE 1: DCI formats are defined in TS 38.212 [31]. NOTE 2: DCI format shall depend upon the test configuration. NOTE 3: The offset is defined with respect to the subcarrier spacing of the CORESET from the smallest RB index of RMSI CORESET to the smallest RB index of the common RB overlapping with the first RB of the SS/PBCH block. NOTE 4: The configuration of PDCCH monitoring occasions for RMSI CORESET is defined in Table 13-11 in TS 38.213 [8]. NOTE 5: Cell ID shall depend upon the test configuration. NOTE 6: Payload size shall depend upon the test configuration. NOTE 7: The configuration of set of resource blocks and slot symbols of control resource set for Type0-PDCCH search space corresponds to index 0 in Table 13-1 in TS 38.213 [8]. NOTE 8: Other values can be used to align with GSCN as long as SSB does not overlap the RMC |  |  |  |  |  |

--- End of Change 1 ---

--- Unchanged Clauses Skipped ---

## A.1.3	CORESET for RMC scheduling

### A.1.3.1	FDD

Table A.1.3.1-1: Control Channel RMC for SCS = 15 kHz for FDD

--- Start of Change 2 ---

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | CCR.1.1 FDD | CCR.1.2 FDD | CCR.1.3 FDD | CCR.1.4 FDD | CCR.1.5 FDD | CCR.1.6 FDD | CCR.1.7 FDD | CCR.1.8 FDD |
| Channel bandwidth | MHz | Defined in test case | Defined in test case | Defined in test case | Defined in test case | 10 | 3MHz | 3MHz | 5MHz |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| Allocated resource blocks for CORESET Note 3 |  | 24 | 18 | 24 | 18 | 24 | 12 | 12 | 18 |
| Number of transmitter antennas |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Duration of CORESET | symbols | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 |
| monitoringSymbolsWithinSlot |  | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 | 0010000 0000000 | 1000000 0000000 | 1000000 0000000 | 1000000 0000000 |
| REG bundle size |  | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 |
| DMRS precoder granularity |  | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size | Same as REG bundle size |
| CCE to REG mapping |  | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Interleaved | Non-Interleaved | Interleaved |
| Interleave n_shift |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Interleave size |  | 2 | 2 | 2 | 2 | 2 | 2 | N/A | 2 |
| Beamforming Pre-Coder |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A |
| Aggregation level | CCE | 4 | 2 | 8 | 4 | 4 | 4 | 4 | 8 |
| DCI formats |  | Note 1 | Note 1 | Note 1 | Note 1 | Note 1 | 1-0 | 1-0 | 1-0 |
| Payload size (without CRC) | bits | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 | Note 2 |
| Note 1: DCI format shall depend upon the test configuration. Note 2: Payload size shall depend upon the test configuration Note 3: Allocated in the resource blocks where the associated RMC is scheduled. |  |  |  |  |  |  |  |  |  |

--- End of Change 2 ---

--- Unchanged Clauses Skipped ---

# C.1	Downlink physical channels

## C.1.1	General

The following clauses describe the downlink physical channels that are needed for setting a connection and channels that are needed during a connection.

## C.1.2	Default downlink signal levels

The downlink power settings in Table C.1.2-1 and Table C.1.2-2 are used unless otherwise specified in a test case for FR1 cell and FR2 cell, respectively. The downlink power settings in Table C.1.2-1 and Table C.1.2-2 are also used for the initial registration for NR /5GC test cases in clauses 6 and 7. For EN-DC test cases in clauses 4 and 5, the E-UTRA power settings used for initial registration are defined in Annex A.6.

For FR1, if the UE has more than one Rx antenna, the downlink signal is applied to each one. All UE Rx antennas shall be connected.

---Start of Change 3---

Table C.1.2-1: Default Downlink power levels for NR (FR1)

| SCS (kHz) | Parameter | Unit | Value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel bandwidth | MHz | 3 |  | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 |
| 15 | Number of RBs |  | 12 | 15 | 25 | 50 | 75 | 100 | 128 | 160 | 215 | 270 | N/A | N/A | N/A | N/A |
|  | Channel BW power | dBm | -63 | -62 | -60 | -57 | -55 | -54 | -53 | -52 | -51 | -50 | N/A | N/A | N/A | N/A |
| 30 | Number of RBs |  | N/A | N/A | 10 | 24 | 36 | 50 | 64 | 75 | 100 | 128 | 162 | 216 | 243 | 270 |
|  | Channel BW power | dBm | N/A | N/A | -61 | -57 | -55 | -54 | -53 | -52 | -51 | -50 | -49 | -48 | -47 | -47 |
| 60 | Number of RBs |  | N/A | N/A | N/A | 10 | 18 | 24 | 30 | 36 | 50 | 64 | 75 | 100 | 120 | 135 |
|  | Channel BW power | dBm | N/A | N/A | N/A | -58 | -56 | -54 | -53 | -52 | -51 | -50 | -49 | -48 | -47 | -47 |
|  | RS EPRE | dBm/ 15kHz | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 | -85 |
| NOTE 1: The channel bandwidth powers are informative, based on -85dBm/15kHz SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. NOTE 2: The power level is specified at each UE Rx antenna. NOTE 3: DL level is applied for any of the Subcarrier Spacing configuration ( ) with the same power spectrum density of -85dBm/15kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

--- End of Change 3 ---