3GPP TSG-RAN5 Meeting #106	R5-250570

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

Source:	Huawei, HiSilicon

Title:	Discussion on XR UTO-UCI test case

Agenda item:	6.3.13.7

Document for:	Discussion

Background

“How to specify rules for the determination of unused CG occasions” is discussed in RAN2#124, and the agreements is below[1], the UE determines the unused CG PUSCH by consider the amount of buffered data available for CGO.

In RAN2#125, DP R2-2400148[3] is raised by Qualcomm and RAN2 agree that UE may make the future buffer data determination. CR R2-2406092 [4] remove the restriction of only considering the already buffered data in UE’s determination of unused CG occasions.

[38.321 5.8.2]

The latest core spec indicated that UE determines the unused CG PUSCH by consider at least the amount of buffered data available for CGO . The amount of buffered data available is not the only factors, and details for the rule of others factors is not indicated in the spec. For example, how long UE has to wait to make sure UE is confident that no more data will arrive before the concerned CG occasion.

Therefore, it is not easy to check the value of UTO-UCI in RAN5 test case.

Discussion

At present, the test configurations of UTO-UCI test case (38.523-1 7.1.1.6.8) are listed below:

1. CG period is 40ms and Multi-CG does not configured (nrofSlotsInCG-Period=0).

2. UE need to report 8 bits UTO-UCI in CG PUSCH (nrofBitsInUTO-UCI=8).

3. SS transmits 4 RLC SDU (4 X 99 Bytes), and UE loops back the 4 RLC SDU in 4 CG PUSCH.

4. SS check the UTO-UCI in the 4 CG PUSCH.

The problem is how to check the 8 bit UTO-UCI reported by UE. The check for unused bits of UTO-UCI is FFS now in the test spec (Figure.2).

For example, If the UE set UTO-UCI in CGO1 only based on the amount of buffered data that can be transmitted at that time (i.e UEA in the Figure.1),  UTO-UCI is {00011111} which means CGO2/3/4 may be used and CGO 5/6/7/8/9 are unused. If the UE set UTO-UCI in CGO1 not only based on the amount of buffered data that can be transmitted at that time, but also prefering to wait more time to make sure that no new data will come before CGO5(i.e UEB in the Figure.1)., UTO-UCI is {00000000} which means CGO2/3/4/5/6/7/8/9 may be used.

Figure.1 UTO-UCI Check

Figure.2

Option1: SS does not check the unused bits in UTO-UCI for all kinds of UE.

Option2: Create a new PICS in 38.508-1 to distinguish different UE implementation, only check the unused bits in UTO-UCI for the UE which only consider the current buffer data when it determine UTO-UCI.

Conclusion

From detailed analysis above, two options are listed for the UTO-UCI test case design.

Option1: SS does not check the unused bits in UTO-UCI for all kinds of UE.

Option2: Create a new PICS in 38.508-1 to distinguish different UE implementation, only check the unused bits in UTO-UCI for the UE which only consider the current buffer data when it determine UTO-UCI.

- References

- R2-2400002, “Report of 3GPP TSG RAN WG2 meeting #124”

- R2-2403994, “Report of 3GPP TSG RAN WG2 meeting #125”

- R2-2400148, "Correction to the determination of unused CG occasions" Qualcomm Incorporated, Apple, MediaTek, RAN2#125

- R2-2406092, "Correction for XR" Qualcomm Incorporated, RAN2#126

- R5-247669, “New testcase 7.1.1.6.8 Correct handling of UL grant / Unused transmission occasion / Configured grant type 1”,Nokia, RAN5 #105