3GPP TSG-RAN5 Meeting #106	R5-250577

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5422 | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of LTE eCall capable test case 11.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NW does not support eCall over IMS, and eCall capable UE is in EMM-REGISTERED state and EMM-IDLE mode. When eCall is initiated, according to 23.167 Table H.2 case B, the first eCall attempt on CS. UE has successfully perform combine attach in LTE, that means LTE NW support PS and CS, so UE can perform CS eCall based on CSFB. However, the test case only allowed UE to perform CS eCall directly on GSM or UTRAN. Therefore conformant UE may fail test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update ATTACH ACCEPT to inform UE that CS for LTE is not avaialbe, and EPS only attach is accepted. Under this condition, CSFB cannot be used, when UE perform CS eCall, , the only choice is to directly access to GSM or UTRAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | conformant UE may fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

11.3.3	eCall capable / EPS supports IMS voice over PS session / EPS supports emergency service / eCall over IMS is not supported / eCall using the CS domain / emergency call over IMS if eCall using the CS domain is not available / UTRA or GERAN

11.3.3.1	Test Purpose (TP)

(1)

with { The UE in EMM-REGISTERED state and EMM-IDLE mode }

ensure that {

when { UE is requested to make an automatic eCall}

then { UE establishes the eCall using the CS domain (UTRA or GERAN)}

}

(2)

with { The UE is in EMM-REGISTERED state and EMM-IDLE mode }

ensure that {

when { UE is requested to make an automatic eCall}

then { UE establishes the normal emergency call on LTE, UE establishes the RRC connection with the RRC establishmentCause set to ‘emergency’ and sends a SERVICE REQUEST message }

}

11.3.3.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clauses 6.2.2, 5.2.2.7 and TS 23.167 Annex H.6

[TS 36.331 clause 5.2.2.7]

Upon receiving the SystemInformationBlockType1 or SystemInformationBlockType1-BR either via broadcast or via dedicated signalling, the UE shall:

1>	if in RRC_IDLE or in RRC_CONNECTED while T311 is running; and

…

3>	forward the eCallOverIMS-Support to upper layers, if present;

[TS 36.331 clause 6.2.2]

eCallOverIMS-Support

Indicates whether the cell supports eCall over IMS services for UEs as defined in TS 23.401 [41]. If absent, eCall over IMS is not supported by the network in the cell. NOTE 2.

[TS 23.167 Annex H.6]

This clause details the domain priority and selection (see clause 7.3) for a UE that attempts to make an eCall over IMS session using E-UTRAN radio access networks based on the availability of the CS or PS domains and the network support for IMS emergency, eCall over IMS and IMS voice over PS.

The following table (Table H.2) defines these rules based on the UE (last 2 columns) for different initial conditions (first 4 columns) when an eCall over IMS session is initiated and when the UE is not in limited service state.

Table H.2: Domain Selection Rules for eCall over IMS session attempts for E-UTRAN radio access networks

|  | PS Available | VoIMS | EMS | ECL | First eCall Attempt | Second eCall Attempt |
| --- | --- | --- | --- | --- | --- | --- |
| A | Y | Y | Y | Y | PS | CS if available |
| B | Y | Y | Y | N | CS if available | PS (UE establishes IMS emergency session) |
| C | Y | Y or N | N | N | CS if available | No attempt is made in the PS domain |
| D | Y | N | Y | Y | PS or CS if available | CS if first attempt in PS PS if first attempt in CS |
| E | Y | N | Y | N | CS if available | PS (UE establishes IMS emergency session) |
| F | N |  | - | - | CS if available |  |
| VoIMS = Voice over IMS over PS sessions support as indicated by IMS Voice over PS session supported indication as defined in TS 23.401 [28]. EMS = IMS Emergency Services supported as indicated by Emergency Service Support indicator as defined in TS 23.401 [28]. ECL = eCall Over IMS support as indicated by the eCall support indicator defined in TS 23.401 [28]. |  |  |  |  |  |  |

11.3.3.3	Test description

11.3.3.3.1	Pre-test conditions

System Simulator:

-	Cell A is set to “Serving cell”

-	If (px_RATComb_Tested = EUTRA_UTRA AND pc_CS_Em_Call_in_UTRA)

-	cell 5 is configured as ''Suitable Neighbour cell''

- 	If (px_RATComb_Tested = EUTRA_GERAN AND pc_CS_Em_Call_in_GERAN)

-	cell 24 is configured as ''Suitable Neighbour cell''

- 	System information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells.

- 	The SS broadcasts IMS Emergency Call support in System Information Block Type 1 on Cell A.

- 	The SS broadcasts eCall over IMS is not supported in System Information Block Type 1 on Cell A.

UE:

- 	The eCall capable UE is equipped with ‘eCall support’ enabled USIM configured as per TS 36.508 [18] Table 4.9.3.5-1.

Preamble:

- 	The UE is in state Registered, Idle mode (state 2) according to [18] on Cell A.

11.3.3.3.2	Test procedure sequence

Table 11.3.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Cause the UE to originate Automatic eCall. (Note 1) | - | - | - | - |
| - | EXCEPTION: Steps 2a1 to 2b21 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 2a1 | IF (px_RATComb_Tested = EUTRA_UTRA AND pc_CS_Em_Call_in_UTRA) | - | - | - | - |
| 2a2 | Check: Does the UE transmit an RRC CONNECTION REQUEST message on Cell 5 with Establishment cause: Emergency Call? | --> | RRC CONNECTION REQUEST | 1 | P |
| 2a3 | The SS transmits an RRC CONNECTION SETUP message. | <-- | RRC CONNECTION SETUP | - | - |
| 2a4 | The UE transmits an RRC CONNECTION SETUP COMPLETE message. | --> | RRC CONNECTION SETUP COMPLETE | - | - |
| 2a5 | Check: Does the UE transmit a CM SERVICE REQUEST with CM service type IE indicating “Emergency call establishment”? | --> | CM SERVICE REQUEST | 1 | P |
| 2a6 | The SS transmits an AUTHENTICATION REQUEST. | <-- | AUTHENTICATION REQUEST | - | - |
| 2a7 | The UE transmits AUTHENTICATION RESPONSE. | --> | AUTHENTICATION RESPONSE | - | - |
| 2a8 | The SS transmits a SECURITY MODE COMMAND message for the CS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 2a9 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 2a10 | Check: Does the UE transmit an EMERGENCY SETUP message with Emergency Service Category IE bit 7 set to 1 and all other bits are set to 0? | --> | EMERGENCY SETUP | 1 | P |
| 2a11-2a16 | Steps 11 to 16 of the generic test procedure in TS 34.108 subclause 7.2.3.2.3 are performed on Cell 5. NOTE: the CS call setup is completed. | - | - | - | - |
| 2a17 | Traffic channel is kept active for at least 5 seconds. | - | - | - | - |
| 2a18 | The SS transmits DISCONNECT. | <-- | DISCONNECT | - | - |
| 2a19 | The UE transmits RELEASE. | --> | RELEASE | - | - |
| 2a20 | The SS transmits RELEASE COMPLETE. | <-- | RELEASE COMPLETE | - | - |
| 2a21 | The SS transmits an RRC CONNECTION RELEASE message. | <-- | RRC CONNECTION RELEASE | - | - |
| 2a22 | The UE transmits RRC CONNECTION RELEASE COMPLETE. | --> | RRC CONNECTION RELEASE COMPLETE | - | - |
| 2b1 | IF (px_RATComb_Tested = EUTRA_GERAN AND pc_CS_Em_Call_in_GERAN) | - | - | - | - |
| 2b2 | Check: Does the UE transmit a CHANNEL REQUEST message on Cell 24 with Establishment cause: Emergency call? | --> | CHANNEL REQUEST | 1 | P |
| 2b3 | The SS transmits an IMMEDIATE ASSIGNMENT message. | <-- | IMMEDIATE ASSIGNMENT | - | - |
| 2b4 | Check: Does the UE transmit a CM SERVICE REQUEST with CM service type IE indicating “Emergency call establishment”? | --> | CM SERVICE REQUEST | 1 | P |
| 2b5 | The SS transmits an AUTHENTICATION REQUEST message. | <-- | AUTHENTICATION REQUEST | - | - |
| 2b6 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 2b7 | The SS transmits a CIPHERING MODE COMMAND. | <-- | CIPHERING MODE COMMAND | - | - |
| 2b8 | The UE transmits a CIPHERING MODE COMPLETE. | --> | CIPHERING MODE COMPLETE | - | - |
| 2b9 | Check: Does the UE transmit an EMERGENCY SETUP message with Emergency Service Category IE bit 7 set to 1 and all other bits are set to 0? | --> | EMERGENCY SETUP | 1 | P |
| 2b10-2b16 | Steps 11 to 17 of the generic test procedure in TS 51.010-1 subclause 10.2.3 are performed on Cell 24. NOTE: the CS call setup is completed. | - | - | - | - |
| 2b17 | Traffic channel is kept active for at least 5 seconds. | - | - | - | - |
| 2b18 | The SS transmits DISCONNECT. | <-- | DISCONNECT | - | - |
| 2b19 | The UE transmits RELEASE. | --> | RELEASE | - | - |
| 2b20 | The SS transmits RELEASE COMPLETE. | <-- | RELEASE COMPLETE | - | - |
| 2b21 | The SS transmits CHANNEL RELEASE | <-- | CHANNEL RELEASE | - | - |
| 3 | if possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 4a1 describes behaviour that depends on the UE capability. | - | - | - | - |
| 4a1 | If pc_SwitchOnOff or pc_USIM_Removal then UE sends DETACH REQUEST message | --> | DETACH REQUEST | - | - |
| 5 | Set the power level of Cell 5 and Cell 24 to Non-suitable" Off" level. | - | - | - | - |
| 6 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 7-23 | Steps 2 to 18 of the registration procedure described in TS 36.508 subclause 4.5.2.3 are performed on Cell A. NOTE: The UE performs registration and the RRC connection is released. | - | - | - | - |
| 24 | Cause the UE to originate Automatic eCall. (see Note 1) | - | - | - | - |
| 25 | Check: Does the UE transmit an RRCConnectionRequest message on Cell A with Establishment cause: Emergency? | --> | RRCConnectionRequest | 2 | P |
| 26-39 | Steps 2 to 15 of the generic test procedure for IMS Emergency call establishment in EUTRA: in EUTRA: Normal Service (TS 36.508 subclause 4.5A.4.3-1). (Note 2) | - | - | - | - |
| 40 | Release IMS Call (Note 3) | - | - | - | - |
| 41 | The SS releases the RRC connection. | - | - | - | - |
| Note 1: The request to originate an automatic eCall may be performed by MMI or AT command. Note 2: The UE shall set the “Request URI and To header” to "urn:service:sos.ecall.automatic" and shall not include the initial MSD in the SIP INVITE message. Note 3: The IMS Call is released using the generic procedure in TS 34.229-1 [35] subclause C.33. |  |  |  |  |  |

11.3.3.3.3	Specific message contents

Table 11.3.3.3.3-0: Message ATTACH ACCEPT (preamble and step 19, able 11.3.3.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS attach result | 001 | "EPS only" |  |
| Location area identification | Not present | SS doesn’t provide LAI |  |
| MS identity | Not Present | SS doesn’t provide TMSI |  |
| EMM cause | 00010010 | #18 "CS domain not available" | combined_EPS_IMSI |

| Condition | Explanation |
| --- | --- |
| combined_EPS_IMSI | This condition applies if the UE is configured to initiate combined EPS/IMSI attach or if explicitly specified. |

Table 11.3.3.3.3-1: RRC CONNECTION REQUEST (Step 2a2, Table 11.3.3.3.2-1)

| Derivation Path: TS 34.108 clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Establishment cause | Emergency Call |  |  |

Table 11.3.3.3.3-2: CM SERVICE REQUEST (Steps 2a5 and 2b4, Table 11.3.3.3.2-1)

| Derivation Path: TS 24.008 Table 9.2.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CM service type | 0010 | Emergency call establishment |  |

Table 11.3.3.3.3-3: CHANNEL REQUEST (Step 2b2, Table 11.3.3.3.2-1)

| Derivation Path: TS 44.018 Table 9.1.8.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Establishment cause | 101 | Emergency call |  |

Table 11.3.3.3.3-4: RRCConnectionRequest (step 25, Table 11.3.3.3.2-1)

| Derivation Path: 36.508, Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| establishmentCause | emergency |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>