3GPP TSG-RAN5 Meeting #106	R5-250578

# Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5423 | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of LTE eCall only mode test case 11.3.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1.The step number in the EXCEPTION after step1 is incorrect 2.The test step number for "The UE transmits RRC CONNECTION RELEASE COMPLETE" is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the step number in the EXCEPTION - Add test number 2a22 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect test step |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

11.3.4	eCall Only mode / EPS supports IMS voice over PS session / EPS does not support emergency service / eCall over IMS is not supported / eCall using CS domain / eCall failure if CS domain is not available

11.3.4.1	Test Purpose (TP)

(1)

with { UE in eCall Only mode is in the state EMM-DEREGISTERED.eCALL-INACTIVE  }

ensure that {

when { When EPS supports IMS voice over PS but does not support emergency service and does not support eCall over IMS and when UE is requested to make an automatic eCall }

then { UE establishes the eCall using the CS domain (UTRA or GERAN)}

}

(2)

with { UE in eCall Only mode is in the state EMM-DEREGISTERED.eCALL-INACTIVE }

ensure that {

when { When EPS supports IMS voice over PS but does not support emergency service and does not support eCall over IMS and CS domain is not available and when UE is requested to make an automatic eCall }

then { UE does not establishes an normal emergency call, eCall over IMS or eCall over CS domain }

}

11.3.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 23.122, clause 4.4.3.1.1 and TS 23.167 Annex H.6. Unless otherwise stated these are Rel-14 requirements.

[TS 23.122 clause 4.4.3.1.1]

The MS selects and attempts registration on other PLMN/access technology combinations, if available and allowable, in the following order:

i)	either the HPLMN (if the EHPLMN list is not present or is empty) or the highest priority EHPLMN that is available (if the EHPLMN list is present) ;

ii)	each PLMN/access technology combination in the "User Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iii)	each PLMN/access technology combination in the "Operator Controlled PLMN Selector with Access Technology" data file in the SIM (in priority order);

iv)	other PLMN/access technology combinations with received high quality signal in random order;

v)	other PLMN/access technology combinations in order of decreasing signal quality.

When following the above procedure the following requirements apply:

…

l)	In i to v, if the MS is in eCall only mode, the MS shall not consider PLMNs which do not advertise support for eCall over IMS, unless such PLMNs are available in GERAN or UTRAN.

NOTE 6:	As an implementation option, an MS in eCall only mode that was not able to select any PLMN according to l) can perform a second iteration of i to v with no restriction.

…

[TS 23.167 Annex H.6]

This clause details the domain priority and selection (see clause 7.3) for a UE that attempts to make an eCall over IMS session using E-UTRAN radio access networks based on the availability of the CS or PS domains and the network support for IMS emergency, eCall over IMS and IMS voice over PS.

The following table (Table H.2) defines these rules based on the UE (last 2 columns) for different initial conditions (first 4 columns) when an eCall over IMS session is initiated and when the UE is not in limited service state.

Table H.2: Domain Selection Rules for eCall over IMS session attempts for E-UTRAN radio access networks

|  | PS Available | VoIMS | EMS | ECL | First eCall Attempt | Second eCall Attempt |
| --- | --- | --- | --- | --- | --- | --- |
| A | Y | Y | Y | Y | PS | CS if available |
| B | Y | Y | Y | N | CS if available | PS (UE establishes IMS emergency session) |
| C | Y | Y or N | N | N | CS if available | No attempt is made in the PS domain |
| D | Y | N | Y | Y | PS or CS if available | CS if first attempt in PS PS if first attempt in CS |
| E | Y | N | Y | N | CS if available | PS (UE establishes IMS emergency session) |
| F | N |  | - | - | CS if available |  |
| VoIMS = Voice over IMS over PS sessions support as indicated by IMS Voice over PS session supported indication as defined in TS 23.401 [28]. EMS = IMS Emergency Services supported as indicated by Emergency Service Support indicator as defined in TS 23.401 [28]. ECL = eCall Over IMS support as indicated by the eCall support indicator defined in TS 23.401 [28]. |  |  |  |  |  |  |

11.3.4.3	Test description

11.3.4.3.1	Pre-test conditions

System Simulator:

-	Cell A is set to “Serving cell”

-	If (px_RATComb_Tested = EUTRA_UTRA AND pc_CS_Em_Call_in_UTRA)

-	cell 5 is configured as ''Suitable Neighbour cell''

-	If (px_RATComb_Tested = EUTRA_GERAN AND pc_CS_Em_Call_in_GERAN)

-	cell 24 is configured as ''Suitable Neighbour cell''

-	System information combination 10 as defined in TS 36.508[18] clause 4.4.3.1 is used in E-UTRA cells. -	The SS broadcasts IMS Emergency Call is not supported in System Information Block Type 1 on Cell A.

-	The SS broadcasts eCall over IMS is not supported in System Information Block Type 1 on Cell A.

UE:

-	The eCall capable UE is equipped with ‘eCall only’ enabled USIM configured as per TS 36.508 [18] Table 4.9.3.5-2.

Preamble:

-	The UE is in state OFF (state 1)

11.3.4.3.2	Test procedure sequence

Table 11.3.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The UE is switched on and SS waits 15 sec to allow the UE to camp on the serving cell and enter and remain in substate EMM-DEREGISTERED.eCALL-INACTIVE. | - | - | - | - |
| 1 | Cause the UE to originate Automatic eCall. (Note 1) | - | - | - | - |
| - | EXCEPTION: Steps 2a1 to 2b21 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. | - | - | - | - |
| 2a1 | IF (px_RATComb_Tested = EUTRA_UTRA AND pc_CS_Em_Call_in_UTRA) | - | - | - | - |
| 2a2 | Check: Does the UE transmit an RRC CONNECTION REQUEST message on Cell 5 with Establishment cause: Emergency Call? | --> | RRC CONNECTION REQUEST | 1 | P |
| 2a3 | The SS transmits an RRC CONNECTION SETUP message. | <-- | RRC CONNECTION SETUP | - | - |
| 2a4 | The UE transmits an RRC CONNECTION SETUP COMPLETE message. | --> | RRC CONNECTION SETUP COMPLETE | - | - |
| 2a5 | Check: Does the UE transmit a CM SERVICE REQUEST with CM service type IE indicating “Emergency call establishment”? | --> | CM SERVICE REQUEST | 1 | P |
| 2a6 | The SS transmits an AUTHENTICATION REQUEST. | <-- | AUTHENTICATION REQUEST | - | - |
| 2a7 | The UE transmits AUTHENTICATION RESPONSE. | --> | AUTHENTICATION RESPONSE | - | - |
| 2a8 | The SS transmits a SECURITY MODE COMMAND message for the CS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 2a9 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 2a10 | Check: Does the UE transmit an EMERGENCY SETUP message with Emergency Service Category IE bit 7 set to 1 and all other bits are set to 0? | --> | EMERGENCY SETUP | 1 | P |
| 2a11-2a16 | Steps 11 to 16 of the generic test procedure in TS 34.108 subclause 7.2.3.2.3 are performed on Cell 5. NOTE: the CS call setup is completed. | - | - | - | - |
| 2a17 | Traffic channel is kept active for at least 5 seconds. | - | - | - | - |
| 2a18 | The SS transmits DISCONNECT. | <-- | DISCONNECT | - | - |
| 2a19 | The UE transmits RELEASE. | --> | RELEASE | - | - |
| 2a20 | The SS transmits RELEASE COMPLETE. | <-- | RELEASE COMPLETE | - | - |
| 2a21 | The SS transmits an RRC CONNECTION RELEASE message. | <-- | RRC CONNECTION RELEASE | - | - |
| 2a22 | The UE transmits RRC CONNECTION RELEASE COMPLETE. | --> | RRC CONNECTION RELEASE COMPLETE | - | - |
| 2b1 | IF (px_RATComb_Tested = EUTRA_GERAN AND pc_CS_Em_Call_in_GERAN) | - | - | - | - |
| 2b2 | Check: Does the UE transmit a CHANNEL REQUEST message on Cell 24 with Establishment cause: Emergency call? | --> | CHANNEL REQUEST | 1 | P |
| 2b3 | The SS transmits an IMMEDIATE ASSIGNMENT message. | <-- | IMMEDIATE ASSIGNMENT | - | - |
| 2b4 | Check: Does the UE transmit a CM SERVICE REQUEST with CM service type IE indicating “Emergency call establishment”? | --> | CM SERVICE REQUEST | 1 | P |
| 2b5 | The SS transmits an AUTHENTICATION REQUEST message. | <-- | AUTHENTICATION REQUEST | - | - |
| 2b6 | The UE transmits an AUTHENTICATION RESPONSE message. | --> | AUTHENTICATION RESPONSE | - | - |
| 2b7 | The SS transmits a CIPHERING MODE COMMAND. | <-- | CIPHERING MODE COMMAND | - | - |
| 2b8 | The UE transmits a CIPHERING MODE COMPLETE. | --> | CIPHERING MODE COMPLETE | - | - |
| 2b9 | Check: Does the UE transmit an EMERGENCY SETUP message with Emergency Service Category IE bit 7 set to 1 and all other bits are set to 0? | --> | EMERGENCY SETUP | 1 | P |
| 2b10-2b16 | Steps 11 to 17 of the generic test procedure in TS 51.010-1 subclause 10.2.3 are performed on Cell 24. NOTE: the CS call setup is completed. | - | - | - | - |
| 2b17 | Traffic channel is kept active for at least 5 seconds. | - | - | - | - |
| 2b18 | The SS transmits DISCONNECT. | <-- | DISCONNECT | - | - |
| 2b19 | The UE transmits RELEASE. | --> | RELEASE | - | - |
| 2b20 | The SS transmits RELEASE COMPLETE. | <-- | RELEASE COMPLETE | - | - |
| 2b21 | The SS transmits CHANNEL RELEASE | <-- | CHANNEL RELEASE | - | - |
| 3 | if possible (see ICS) switch off is performed or the USIM is removed. Otherwise the power is removed. | - | - | - | - |
| - | EXCEPTION: Step 4a1 describes behaviour that depends on the UE capability. | - | - | - | - |
| 4a1 | If pc_SwitchOnOff or pc_USIM_Removal then UE sends DETACH REQUEST message | --> | DETACH REQUEST | - | - |
| 5 | Set the power level of Cell 5 and Cell 24 to Non-suitable" Off" level. | - | - | - | - |
| 6 | The UE is brought back to operation or the USIM is inserted. | - | - | - | - |
| 7 | SS waits 15 sec to allow the UE to camp on the serving cell and enter and remain in substate EMM-DEREGISTERED.eCALL-INACTIVE. | - | - | - | - |
| 8 | Cause the UE to originate Automatic eCall. (see Note 1) | - | - | - | - |
| 9 | Check: Does the UE transmit an RRCConnectionRequest message on Cell A with Establishment cause: Emergency in the next 30 sec? | --> | RRCConnectionRequest | 2 | F |
| Note 1: The request to originate an automatic eCall may be performed by MMI or AT command. |  |  |  |  |  |

11.3.4.3.3	Specific message contents

Table 11.3.4.3.3-1: SystemInformationBlockType1 for Cell A (preamble and all steps)

| Derivation Path: 36.508 clause 4.4.3.2, Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| ims-EmergencySupport-r9 | Not present | Not support IMS emergency call |  |
| eCallOverIMS-Support-r14 | Not present | Not support eCall over IMS services. |  |
| } |  |  |  |

Table 11.3.4.3.3-2: RRC CONNECTION REQUEST (Step 2a2, Table 11.3.4.3.2-1)

| Derivation Path: TS 34.108 clause 9.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Establishment cause | Emergency Call |  |  |

Table 11.3.4.3.3-3: CM SERVICE REQUEST (Steps 2a5 and 2b4, Table 11.3.4.3.2-1)

| Derivation Path: TS 24.008 Table 9.2.11 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CM service type | 0010 | Emergency call establishment |  |

Table 11.3.4.3.3-4: CHANNEL REQUEST (Step 2b2, Table 11.3.4.3.2-1)

| Derivation Path: TS 44.018 Table 9.1.8.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Establishment cause | 101 | Emergency call |  |

<End of modified section 1>