3GPP TSG RAN WG5 Meeting #106	R5-250629

# Athens, Greece，17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3201 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial correction for some referenced Table numbers and Annex clause numbers in V2X receiver test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some referenced Table numbers and Annex clause numbers in V2X receiver test cases are incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The incorrect referenced Table numbers and Annex clause numbers in V2X receiver test cases were corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The incorrect referenced Table numbers and Annex clause numbers in V2X receiver test cases will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3E, 7.6E, 7.7E, 7.8E |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Editorial correction. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.3E.2.3	Minimum conformance requirements

When UE is configured for NR V2X reception non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2 with parameters specified in Table 7.3E.2.3-1.

<< Unchanged sections omitted >>

7.3E.2.4.2	Test procedure

1.	The UE starts to perform the NR V2X sidelink communication according to SL-V2X-Preconfiguration and to schedule the V2X RMC according to Table 7.3G.1.4.1-1.

2.	Set the signal level of V2X to the appropriate REFSENS value defined in Table 7.3G.1.3-1.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.

<< Unchanged sections omitted >>

7.3E.2.5	Test requirement

When UE is configured for NR V2X reception non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2 with parameters specified in Table 7.3E.2.5-1.

<< Unchanged sections omitted >>

7.6E.2.1.4.2	Test procedure

1.	The UE starts to perform the NR V2X sidelink communication according to SL-V2X-Preconfiguration and to schedule the V2X RMC according to Table 7.6E.2.1.4.1-1. Set the downlink signal level according to Table 7.6E.2.1.5-1.

2.	Set the parameters of the signal generator for an interfering signal below the wanted signal in Case 1 according to Tables 7.6E.2.1.5-1 and 7.6E.2.1.5-2.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.

4.	Repeat step 3, using an interfering signal above the wanted signal in Case 1 at step 2.

5.	Repeat step 3, using interfering signals in Case 2 at step2 and 4. The ranges of case 2 are covered in steps equal to the interferer bandwidth. The test frequencies are chosen in analogy to Table 7.6E.2.1.5-2.

<< Unchanged sections omitted >>

7.6E.2.1.5	Test Requirement

When UE is configured for NR V2X reception non-concurrent with NR uplink transmissions for NR V2X operating bands specified in Table 5.2E.1-1, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.7.2 with parameters specified in Tables 7.6E.2.1.5-1 and 7.6E.2.1.5-2.

<< Unchanged sections omitted >>

7.6E.3.1.4.2	Test procedure

1.	The UE starts to perform the NR V2X sidelink communication according to SL-V2X-Preconfiguration and to schedule the V2X RMC according to Table 7.6E.3.1.4.1-1. Set the downlink signal level according to Table 7.6E.3.1.5-1.

2.	Set the parameters of the CW signal generator for an interfering signal below the wanted signal according to Table 7.6E.3.1.5-2. The frequency step size is  MHz with BWChannel the bandwidth of the frequency channel in MHz.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H. Record the frequencies for which the throughput doesn't meet the requirements.

4.	Repeat step 2 and 3, using an interfering signal above the wanted signal at step 2.

<< Unchanged sections omitted >>

7.7E.1.4.2	Test procedure

1.	The UE starts to perform the NR V2X sidelink communication according to SL-V2X-Preconfiguration and to schedule the V2X RMC according to Table 7.6E.3.1.4.1-1. Set the downlink signal level according to Table 7.7E.1.5-1.

2.	Set the parameters of the CW signal generator for an interfering signal according to Table 7.7E.1.5-2. The spurious frequencies are taken from records in the final step of test procedures in clause 7.6E.3.1.4.2.

3.	For the spurious frequency, measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.

<< Unchanged sections omitted >>

7.8E.2.1.4.2	Test procedure

1.	The UE starts to perform the NR V2X sidelink communication according to SL-V2X-Preconfiguration and to schedule the V2X RMC according to Table 7.8E.2.1.4.1-1. Set the downlink signal level according to Table 7.8E.2.1.5-1.

2.	Set the parameters of the signal generators for the two interfering signals below the wanted signal according to Table 7.8E.2.1.5-1.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex H.

4.	Repeat step 3, using two interfering signals above the wanted signal at step 2.

<< End of changes >>