3GPP TSG RAN WG5 Meeting #106	R5-250631

# Athens, Greece，17th – 21st February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3203 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new test case 7.6E.2.2 In-band blocking for V2X con-current operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 7.6E.2.2 In-band blocking for V2X / con-current operation is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 7.6E.2.2 In-band blocking for V2X / con-current operation was added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 7.6E.2.2 will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6E.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

7.6E.2.2	In-band blocking for V2X / con-current operation

Editor’s Note: The following aspects are not yet determined:

- TP analysis, Test Configuration Table and connection diagram are FFS.

7.6E.2.2.1	Test Purpose

In-band blocking is defined for an unwanted interfering signal falling into the range from 30MHz below to 30MHz above the V2X band, or into the range from 15 MHz below to 15 MHz above the UE NR receive band, at which the relative throughput shall meet or exceed the requirement for the specified measurement channels.

The lack of in-band blocking ability will decrease the coverage area when other transmitters exist (except in the adjacent channels and spurious response).

7.6E.2.2.2	Test Applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink communication.

7.6E.2.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 7.6E.2.0.2.

7.6E.2.2.4	Test Description

7.6E.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on V2X operating bands specified in Table 5.2E.2-1 and Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 7.6E.2.2.4.1-1. The details of the uplink, downlink and V2X reference measurement channels (RMCs) are specified in Annexes A.2, A.3 and A.7.2 respectively. Configuration of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 7.6E.2.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1[5] subclause 4.1 |  |  |  | [Normal] |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | [Mid range for NR Uu and V2X carrier] |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | [Lowest for NR Uu and V2X carrier] [Highest for NR Uu and V2X carrier] |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | [Lowest for NR Uu and V2X carrier] |  |  |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |  |  |  |
|  | NR Uu Configuration |  |  |  |  | V2X Configuration |  |
| Test ID | Downlink Configuration |  | Uplink Configuration |  |  | Modulation | PSSCH RB allocation |
|  | Mod'n | RB allocation | Mod'n |  | RB allocation |  |  |
| 1 | [CP-OFDM QPSK] | [Full RB] (NOTE 1) | [DFT-s-OFDM QPSK] |  | [NOTE 2] | [CP-OFDM QPSK] | [Full RB] (NOTE 3) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2. NOTE 2: Refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band NOTE 3: The specific configuration is defined in Table 7.3E.2.4.1-2. |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure FFS for TE diagram and section A.3.2.7 for UE diagram.

2.	The parameter settings for the NR sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause 4.10.

3.	The parameter settings for the NR cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

4.	Message content exceptions are defined in clause 7.6E.2.2.4.3.

5.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and C.3.1, and uplink signals according to Annex G.0, G.1, G.2, and G.3.1.

6.	The reference measurement channel is set according to Table 7.6E.2.2.4.1-1.

7.	Propagation conditions are set according to Annex B.0.

8.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Sidelink On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 7.3.2.4.3.

7.6E.2.2.4.2	Test procedure

1.	The UE starts to perform the NR V2X sidelink communication according to SL-V2X-Preconfiguration and to schedule the V2X RMC according to Table 7.6E.2.2.4.1-1. Set the downlink signal level according to Table 7.6E.2.1.5-1.

2.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 7.6E.2.2.4.1-1. The SS sends downlink MAC padding bits on the DL RMC.

3.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 7.6E.2.2.4.1-1. Since the UL has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

4.	Set the downlink signal level according to the table 7.6.2.5-1 as appropriate. Send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as -MU to -(MU + Uplink power control window size) dB of the target power level in Table 7.6.2.5-1 for at least the duration of the Throughput measurement, where:

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.3-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified for test case 6.3.4.3 in Table F.1.2-1.

-	For UEs supporting Tx diversity, the transmit power is measured as the sum of the output power from both UE antenna connectors.

5.	Swich the interference signal generator to NR sidelink. Set the parameters of the signal generator for an interfering signal below the wanted signal in Case 1 according to Tables 7.6E.2.1.5-1 and 7.6E.2.1.5-2.

6.	Measure the average throughput of NR sidelink for a duration sufficient to achieve statistical significance according to Annex H.

7.	Repeat step 6, using an interfering signal above the wanted signal in Case 1 at step 5.

8.	Repeat step 6, using interfering signals in Case 2 at step5 and 7. The ranges of case 2 are covered in steps equal to the interferer bandwidth. The test frequencies are chosen in analogy to Table 7.6E.2.1.5-2.

9.	Swich the interference signal generator to NR downlink. Set the parameters of the signal generator for an interfering signal below the wanted signal in Case 1 according to Tables 7.6.2.5-1 and 7.6.2.5-2.

10.	Measure the average throughput of NR downlink for a duration sufficient to achieve statistical significance according to Annex H.

11.	Repeat step 10, using an interfering signal above the wanted signal in Case 1 at step 9.

12.	Repeat step 10, using interfering signals in Case 2 at step 9 and 11. The ranges of case 2 are covered in steps equal to the interferer bandwidth. Interferer frequencies should be chosen starting with an offset nearest to the centre frequency and sweep outwards towards the band edges. In order to ensure that full range is tested for interferer frequency, run last test steps at frequency equal to FInterferer range limit defined at the corresponding band edge.

13.	Repeat steps from 9 to 10, using interfering signals in Case 3 at step 9.

NOTE:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no greater than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.3.

7.6E.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with TRANSFORM_PRECODER_ENABLED condition in Table 4.6.3-118 PUSCH-Config.

7.6E.2.2.5	Test Requirement

When UE is configured for inter-band con-current NR V2X operation for operating bands specified in Table 5.2E.2-1, the throughput of NR sidelink shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.7.2 with parameters specified in Tables 7.6E.2.1.5-1 and 7.6E.2.1.5-2, the throughput of NR downlink shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.3.2  with parameters specified in Tables 7.6.2.5-1 and 7.6.2.5-2.

<< End of changes >>