3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210078

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1512 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GMM test case 9.3.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The UL NAS count obtained through f_EUTRA_NB_SS_NAS_CountGet() is the count of the last UL NAS message, this should be incremented to get the current NAS count - The UE registration is rejected during preamble, so before step 2 the UE has no valid 5GC security context and no valid 5G-GUTI. So, in the Registration Request at step 2, - only cleartext IEs are included - Additional GUTI shall not be included - The current implementation is not generating the mapped 5GC security context from the S1 security context. Note: According to 24.301 s5.5.5, If the UE has a current EPS security context, the UE shall integrity protect the TRACKING AREA UPDATE REQUEST message with the current EPS security context and increase the uplink NAS COUNT by one. - In SECURITY MODE COMPLETE the expected “nasMsg” is omit. Before step 2, UE has no valid 5GC security context, so UE shall include “nasMsg” in SECURITY MODE COMPLETE |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Increment v_NASCountInfo.UL before sending to NR PTC - Do not check non-cleartext IEs and Additional GUTI in Registration Request - increase the uplink EPS NAS COUNT by one, and generate mapped 5GC security key - change 2nd input parameter to Initial_NoSecurity while calling f_Check_NG_SecurityModeCompleteMsg |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_EUTRA38_Send_AuthParametersToNR() |
| --- | --- |
| Reason for change | The UL NAS count obtained through f_EUTRA_NB_SS_NAS_CountGet() is the count of the last UL NAS message, this should be incremented to get the current NAS count |
| Summary of change | Increment v_NASCountInfo.UL before sending to NR PTC |
| TTCN module | EUTRA38_NR5GCProcedures |
| MCC160 Comment |  |

Before Change

function f_EUTRA38_Send_AuthParametersToNR () runs on EUTRA_5GS_PTC { var EUTRA_SecurityParams_Type v_NAS_SecurityParams := f_EUTRA_Security_Get(); var NasCountInfo_Type v_NASCountInfo := f_EUTRA_NB_SS_NAS_CountGet(NASCTRL); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Security(omit, omit, v_NAS_SecurityParams.KASME, v_NAS_SecurityParams.KSIasme, v_NASCountInfo.UL, v_NASCountInfo.DL, // @sic R5w190307 sic@ omit, omit, v_NAS_SecurityParams.NH, // @sic R5s201078 sic@ v_NAS_SecurityParams.NCC)); // @sic R5s201078 sic@ }

After Change

function f_EUTRA38_Send_AuthParametersToNR () runs on EUTRA_5GS_PTC { var EUTRA_SecurityParams_Type v_NAS_SecurityParams := f_EUTRA_Security_Get(); var NasCountInfo_Type v_NASCountInfo := f_EUTRA_NB_SS_NAS_CountGet(NASCTRL); v_NASCountInfo.UL := f_NasSecurity_IncrementCOUNT(v_NASCountInfo.UL); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Security(omit, omit, v_NAS_SecurityParams.KASME, v_NAS_SecurityParams.KSIasme, v_NASCountInfo.UL, v_NASCountInfo.DL, // @sic R5w190307 sic@ omit, omit, v_NAS_SecurityParams.NH, // @sic R5s201078 sic@ v_NAS_SecurityParams.NCC)); // @sic R5s201078 sic@ }

## Change 2

| Function name | fl_TC_9_3_1_2_NR5GC_TestBody() |
| --- | --- |
| Reason for change | - The UE registration is rejected during preamble, so before step 2 the UE has no valid 5GC security context and no valid 5G-GUTI. So, in the Registration Request at step 2, - only cleartext IEs are included - Additional GUTI shall not be included - The current implementation is not generating the mapped 5GC sexurity context from the S1 security context. Note: According to 24.301 s5.5.5, If the UE has a current EPS security context, the UE shall integrity protect the TRACKING AREA UPDATE REQUEST message with the current EPS security context and increase the uplink NAS COUNT by one. - In SECURITY MODE COMPLETE the expected “nasMsg” is omit. Before step 2, UE has no valid 5GC security context, so UE shall include “nasMsg” in SECURITY MODE COMPLETE |
| Summary of change | - Do not check non-cleartext IEs and Additional GUTI in Registration Request - increase the uplink EPS NAS COUNT by one, and generate mapped 5GC security key - change 2nd input parameter to Initial_NoSecurity while calling f_Check_NG_SecurityModeCompleteMsg |
| TTCN module | Mobility_Registration_NR5GC_NR |
| MCC160 Comment |  |

Before Change

function fl_TC_9_3_1_2_NR5GC_TestBody() runs on NR5GC_PTC return NR_RadioBearerId_Type { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_RadioBearerId_Type v_SRB; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; // @sic R5s201060 sic@ var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get(); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_Result; var NasCount_Type v_NasCountUL; //@siclog "Step 1" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 2" siclog@ // Steps 1 - 3 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security(EUTRA); // @sic R5-205142 sic@ v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(EUTRA); // @sic R5s201060 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, f_NR_Convert_CoOrdTo5G_GUTI(v_CoOrd_Guti, omit), // @sic R5s201060 sic@ '111'B, '?'B, omit, cdr_NG_GMM_Cap_S1, -,-, *, cdr_NR5GC_UENetworkCap('17'O), -, cr_PDU_SessionStatusAny, -, cr_UEStatus_S1, f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams), -, -, -, ?, -, -, -, -, -, -, -, omit)) { // @sic R5-206290 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Send TAU Req for checking in EUTRA f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_OctetData(v_ReceivedMsg.Pdu.Msg.registration_Request.epsMessage.epsMsg, "TAUReq")); // 5GMM security // Do NAS Security Mode procedure v_SecurityParams := f_NR5GC_Security_Get(); f_SS_NG_NAS_ConfigureSecurity(NASCTRL, v_SecurityParams.NAS_Integrity, v_SecurityParams.NAS_Ciphering, true); /* download security keys and reset NAS COUNT */ v_MsgToSend := f_Get_NG_SecurityModeCmdMsg(v_SecurityParams, Mobility, v_GMM_MobilityInfo); // Mapped security context with Kasme @sic R5-205142 sic@ v_MsgToSend.security_Mode_Command.ngNasKSI := cs_NAS_KeySetIdentifier_lv(v_SecurityRx.KSI, tsc_NasKsi_MappedSecurityContext); // send SECURITY MODE COMMAND SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_NewSecurityContext, v_MsgToSend))); // receive SECURITY MODE COMPLETE SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered_NewSecurityContext))) -> value v_ReceivedAsp; v_Result := f_Check_NG_SecurityModeCompleteMsg(v_ReceivedAsp.Signalling.Nas[0].Pdu, Mobility); v_NasCountUL := v_ReceivedAsp.Signalling.Nas[0].SecurityProtection.NasCount; if (not v_Result) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "NAS Security Mode Procedure Failed"); } f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); f_NR_RRC_ActivateSecurity (v_NGC_NASCellA,v_NasCountUL); f_NR5GC_MobilityRegistration_Steps4_6(v_NGC_NASCellA, v_GMM_MobilityInfo, - , -, noRrcConnectionRelease, cs_NG_NetworkFeatureSupport_Def); // @sic R5-205142 sic@ Now finish the procedure v_SRB := f_NR5GC_IRAT_S1ToN1_Steps14(v_NGC_NASCellA, noRrcConnectionRelease); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @sic R5s201005 sic@ return v_SRB; }

After Change

function fl_TC_9_3_1_2_NR5GC_TestBody() runs on NR5GC_PTC return NR_RadioBearerId_Type { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NR_RadioBearerId_Type v_SRB; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (v_NGC_NASCellA); var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (v_NGC_NASCellA); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); var EUTRA_NR_CoOrd_Security_Type v_SecurityRx; var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti; // @sic R5s201060 sic@ var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get(); var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_Result; var NasCount_Type v_NasCountUL; //@siclog "Step 1" siclog@ f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog "Step 2" siclog@ // Steps 1 - 3 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellA, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security(EUTRA); // @sic R5-205142 sic@ v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(EUTRA); // @sic R5s201060 sic@ if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility, f_NR_Convert_CoOrdTo5G_GUTI(v_CoOrd_Guti, omit), // @sic R5s201060 sic@ '111'B, '?'B, omit, -, -,-, *, -, -, -, -, cr_UEStatus_S1, -, -, -, -, -, -, -, -, -, -, -, -, omit)) { // @sic R5-206290 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // Send TAU Req for checking in EUTRA f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_OctetData(v_ReceivedMsg.Pdu.Msg.registration_Request.epsMessage.epsMsg, "TAUReq")); // derive 5G parameters from values received from EUTRA v_SecurityRx.NASCountUL := f_NasSecurity_IncrementCOUNT(v_SecurityRx.NASCountUL); f_NR5GC_MappedContextFromS1_InitNAS (v_SecurityRx, false); // 5GMM security // Do NAS Security Mode procedure v_SecurityParams := f_NR5GC_Security_Get(); REMOVE //f_SS_NG_NAS_ConfigureSecurity(NASCTRL, v_SecurityParams.NAS_Integrity, v_SecurityParams.NAS_Ciphering, true); /* download security keys and reset NAS COUNT */ v_MsgToSend := f_Get_NG_SecurityModeCmdMsg(v_SecurityParams, Mobility, v_GMM_MobilityInfo); // Mapped security context with Kasme @sic R5-205142 sic@ v_MsgToSend.security_Mode_Command.ngNasKSI := cs_NAS_KeySetIdentifier_lv(v_SecurityRx.KSI, tsc_NasKsi_MappedSecurityContext); // send SECURITY MODE COMMAND SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_NewSecurityContext, v_MsgToSend))); // receive SECURITY MODE COMPLETE SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered_NewSecurityContext))) -> value v_ReceivedAsp; v_Result := f_Check_NG_SecurityModeCompleteMsg(v_ReceivedAsp.Signalling.Nas[0].Pdu, Initial_NoSecurity);//KS R5sxxx change_9312 v_NasCountUL := v_ReceivedAsp.Signalling.Nas[0].SecurityProtection.NasCount; if (not v_Result) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "NAS Security Mode Procedure Failed"); } f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); f_NR_RRC_ActivateSecurity (v_NGC_NASCellA,v_NasCountUL); f_NR5GC_MobilityRegistration_Steps4_6(v_NGC_NASCellA, v_GMM_MobilityInfo, - , -, noRrcConnectionRelease, cs_NG_NetworkFeatureSupport_Def); // @sic R5-205142 sic@ Now finish the procedure v_SRB := f_NR5GC_IRAT_S1ToN1_Steps14(v_NGC_NASCellA, noRrcConnectionRelease); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @sic R5s201005 sic@ return v_SRB; }