3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210128

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1546 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC test case 8.2.2.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Disabling RLC ACK for EUTRA SRB 1 is needed only for test case 8.2.2.2.1 and is not necessary for test case 8.2.2.3.1. - In f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5, TTCN will release EUTRA SRBs and DRBs at Step 2 of 36.523-3 cl. 7.16.2 and then setup SRBs and DRBs again at Step 4. As RBs are released at Step 2, full configuration of PDCP, RLC, and MAC are needed for Step 4. - Based on latest prose CR R5-206340, at Step 1, EUTRA SRB 1 will be proxy PDCP while EUTRA SRB 2 will just be configured as NR PDCP. RLC Bearer Routing of EUTRA SRB2 should be None := True. - LTE handover happened at Step 1 with sk_counter = 1. Before Step 3, it is necessary to re-calculate S_KGNB key based on sk_counter = 1. - From R5s201348 Change 7, it was agreed to use cs_NR_RlcBearerConfig_SRB1SRB2SRB3 instead of cs_NR_RlcBearerConfig_AM but it was not reflected in D20wk50 TTCN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Use p_DuplicatePDU to distinguish test case 8.2.2.2.1 and 8.2.2.3.1. - Add full configuration of PDCP, RLC, and MAC layer for variable v_SrbDrbList. - Set SRB1 to cs_RlcBearerRouting_NR(nr_Cell1) while SRB2 is set to crs_RlcBearerRouting_None. - Add f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys with sk_counter=1 and f_ENDC_GetKeysActivateSecurity before Step 3. - Use cs_NR_RlcBearerConfig_SRB1SRB2SRB3 instead of cs_NR_RlcBearerConfig_AM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.3.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_EUTRA38_ReconfigureSRBs_NR_PDCPSplit |
| --- | --- |
| Reason for change | - Disabling RLC ACK for EUTRA SRB 1 is needed only for test case 8.2.2.2.1 and is not necessary for test case 8.2.2.3.1. - In f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5, TTCN will release EUTRA SRBs and DRBs at Step 2 of 36.523-3 cl. 7.16.2 and then setup SRBs and DRBs again at Step 4. As RBs are released at Step 2, full configuration of PDCP, RLC, and MAC are needed for Step 4. - Based on latest prose CR R5-206340, at Step 1, EUTRA SRB 1 will be proxy PDCP while EUTRA SRB 2 will just be configured as NR PDCP. RLC Bearer Routing of EUTRA SRB2 should be None := True. |
| Summary of change | - Use p_DuplicatePDU to distinguish test case 8.2.2.2.1 and 8.2.2.3.1. - Add full configuration of PDCP, RLC, and MAC layer for variable v_SrbDrbList. - Set SRB1 to cs_RlcBearerRouting_NR(nr_Cell1) while SRB2 is set to crs_RlcBearerRouting_None. |
| TTCN module | RRCReconfig_ENDC_EUTRA |
| MCC160 Comment |  |

Before change:

function f_EUTRA38_ReconfigureSRBs_NR_PDCPSplit(EUTRA_CellId_Type   p_CellId,

boolean             p_Receive_RBConfig2 := false,

integer p_SK_Counter := 0,

boolean             p_DuplicatePDU := true

) runs on EUTRA_5GS_PTC

{// @sic R5s201348 sic@

var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get();

var template (value) octetstring v_NR_RBConfig1;

var template (omit) octetstring v_NR_RBConfig2 := omit;

var octetstring v_OctetData;

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var integer v_MCG_DRBId := f_EUTRA38_ENDC_GetDRBIdOfMCG();

var template (value) RadioBearerList_Type v_ConfigDrbSS := {cs_OneDRB_ConfigAM(v_MCG_DRBId)}; //DRB on MCG - unchanged through the TC

var template (value) RadioBearerList_Type v_SrbDrbList;

var template (omit) octetstring v_NR_SCGConfig := omit;

var template (value) SecurityActTimeList_Type v_SecurityActTimeList;

var DRB_IdentityList_AllAndAMonly_Type v_DRB_IdentityList:= f_EUTRA_DRB_IdentityList(v_ConfigDrbSS);

var SubFrameTiming_Type v_Timing;

if (p_SK_Counter != 0)

{

v_Security_Params.SK_Counter := p_SK_Counter;

}

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Ciphering Alg");

f_EUTRA_AS_CipheringAlgorithm_Set(f_EUTRA_ConvertCiphAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3)));

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Integrity Alg");

f_EUTRA_AS_IntegrityAlgorithm_Set(f_EUTRA_ConvertIntAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3)));

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig1 := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

if (p_Receive_RBConfig2) {

v_NR_RBConfig2 := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

}

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig1, v_Security_Params.SK_Counter,fl_NR_GetP_Max(), v_NR_RBConfig2);

f_EUTRA_SS_PdcpCount_GetAmDrbs(p_CellId, v_DRB_IdentityList.AMonly, f_EUTRA_SS_PdcpCount_GetAll(p_CellId));

v_SecurityActTimeList := f_EUTRA_RRC_CipherActTime_GetForHO(p_CellId, v_DRB_IdentityList.AMonly, f_EUTRA_SS_PdcpCount_GetAll(p_CellId));

v_SrbDrbList := {cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB1, nr_Cell1, cs_RLC_Configuration_TestModeAckProhibit(Prohibit)),

cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB2, nr_Cell1),

v_ConfigDrbSS[0]

};

v_Timing :=  f_EUTRA_GetNextSendOccasion(p_CellId, tsc_EUTRA_DelayBeforeIntraCellHO);

f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5(p_CellId,

cs_TimingInfo_EUTRA_NB(v_Timing),

v_ConfigDrbSS,

-,  //default value tsc_C_RNTI_Def2

-, //MeasConfig,

-,

v_SecurityActTimeList,

-,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbDrbList

);

if (p_DuplicatePDU)

{

// Check: Does the UE send a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message on split SRB1 duplicated over the SCG path?

interleave {

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1))) {}

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, cr_RlcBearerRouting_NR(nr_Cell1))) {}

}

} else {

// Check: Does the UE send a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message on split SRB1 over the MCG path?

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

}

//After steps in table 8.2.2.2.1.3.2-2 are completed the SS is configured to send RLC ACK on split SRB1 and the last message received in step 2 is acknowledged.

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB1, Continue)});

//Complete the HO procedure steps 7

// Step 7: configure RACH procedure back to default

f_EUTRA_SS_ConfigRachProcedure_Def(eutra_Cell1);

// enable TA transmissions

f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_Auto_Def));

}

After change:

function f_EUTRA38_ReconfigureSRBs_NR_PDCPSplit(EUTRA_CellId_Type   p_CellId,

boolean             p_Receive_RBConfig2 := false,

integer p_SK_Counter := 0,

boolean             p_DuplicatePDU := true

) runs on EUTRA_5GS_PTC

{// @sic R5s201348 sic@

var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get();

var template (value) octetstring v_NR_RBConfig1;

var template (omit) octetstring v_NR_RBConfig2 := omit;

var octetstring v_OctetData;

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var integer v_MCG_DRBId := f_EUTRA38_ENDC_GetDRBIdOfMCG();

var template (value) RadioBearerList_Type v_ConfigDrbSS := {cs_OneDRB_ConfigAM(v_MCG_DRBId)}; //DRB on MCG - unchanged through the TC

var template (value) RadioBearerList_Type v_SrbDrbList;

var template (omit) octetstring v_NR_SCGConfig := omit;

var template (value) SecurityActTimeList_Type v_SecurityActTimeList;

var DRB_IdentityList_AllAndAMonly_Type v_DRB_IdentityList:= f_EUTRA_DRB_IdentityList(v_ConfigDrbSS);

var SubFrameTiming_Type v_Timing;

if (p_SK_Counter != 0)

{

v_Security_Params.SK_Counter := p_SK_Counter;

}

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Ciphering Alg");

f_EUTRA_AS_CipheringAlgorithm_Set(f_EUTRA_ConvertCiphAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3)));

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(NR, "NR Integrity Alg");

f_EUTRA_AS_IntegrityAlgorithm_Set(f_EUTRA_ConvertIntAlg2Bitstring(substr(oct2bit(v_OctetData), 5, 3)));

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig1 := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

if (p_Receive_RBConfig2) {

v_NR_RBConfig2 := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

}

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig1, v_Security_Params.SK_Counter,fl_NR_GetP_Max(), v_NR_RBConfig2);

f_EUTRA_SS_PdcpCount_GetAmDrbs(p_CellId, v_DRB_IdentityList.AMonly, f_EUTRA_SS_PdcpCount_GetAll(p_CellId));

v_SecurityActTimeList := f_EUTRA_RRC_CipherActTime_GetForHO(p_CellId, v_DRB_IdentityList.AMonly, f_EUTRA_SS_PdcpCount_GetAll(p_CellId));

if (p_DuplicatePDU){

v_SrbDrbList := {cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB1, nr_Cell1, cs_RlcBearerRouting_NR(nr_Cell1), cs_RLC_Configuration_TestModeAckProhibit(Prohibit)),

cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB2, nr_Cell1, cs_RlcBearerRouting_NR(nr_Cell1)),

v_ConfigDrbSS[0]

};

} else {

v_SrbDrbList := {cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB1, nr_Cell1, cs_RlcBearerRouting_NR(nr_Cell1), cs_RLC_Configuration_AM(cs_TX_AM_RLC_SRB1_Def, cs_RX_AM_RLC_SRB1_Def), cs_MAC_Configuration(tsc_SRB1_LogicalChannelPriority, infinity_), 1),

cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(tsc_SRB2, nr_Cell1, crs_RlcBearerRouting_None, cs_RLC_Configuration_AM(cs_TX_AM_RLC_SRB2_Def, cs_RX_AM_RLC_SRB2_Def), cs_MAC_Configuration(tsc_SRB2_LogicalChannelPriority, infinity_), 2)

};

}

v_Timing :=  f_EUTRA_GetNextSendOccasion(p_CellId, tsc_EUTRA_DelayBeforeIntraCellHO);

f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5(p_CellId,

cs_TimingInfo_EUTRA_NB(v_Timing),

v_ConfigDrbSS,

-,  //default value tsc_C_RNTI_Def2

-, //MeasConfig,

-,

v_SecurityActTimeList,

-,

v_RRCConnectionReconfiguration_v890_IEs,

v_SrbDrbList

);

if (p_DuplicatePDU)

{

// Check: Does the UE send a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message on split SRB1 duplicated over the SCG path?

interleave {

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1))) {}

[]SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, cr_RlcBearerRouting_NR(nr_Cell1))) {}

}

} else {

// Check: Does the UE send a RRCConnectionReconfigurationComplete message containing NR RRCReconfigurationComplete message on split SRB1 over the MCG path?

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

}

//After steps in table 8.2.2.2.1.3.2-2 are completed the SS is configured to send RLC ACK on split SRB1 and the last message received in step 2 is acknowledged.

if (p_DuplicatePDU){

f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, {cs_RB_Reconfig_TestModeAckProhibit( tsc_RbId_SRB1, Continue)});

}

//Complete the HO procedure steps 7

// Step 7: configure RACH procedure back to default

f_EUTRA_SS_ConfigRachProcedure_Def(eutra_Cell1);

// enable TA transmissions

f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_Auto_Def));

}

## 2.2 Change 2

| Function name | cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA |
| --- | --- |
| Reason for change | - As state in Change 1: In f_EUTRA_508RRC_IntraLTE_HO_IntraCell_Step1_5, TTCN will release EUTRA SRBs and DRBs at Step 2 of 36.523-3 cl. 7.16.2 and then setup SRBs and DRBs again at Step 4. As RBs are released at Step 2, full configuration of PDCP, RLC, and MAC are needed for Step 4. - Based on latest prose CR R5-206340, at Step 1, EUTRA SRB 1 will be proxy PDCP while EUTRA SRB 2 will just be configured as NR PDCP. RLC Bearer Routing of EUTRA SRB2 should be None := True. |
| Summary of change | - Add parameters for PDCP, RLC, and MAC configuration. - Add RlcBearerRouting_Type parameter to distinguish between proxy PDCP and non-proxy PDCP RBs. |
| TTCN module |  |
| MCC160 Comment | RRCReconfig_ENDC_EUTRA |

Before change:

template (value) RadioBearer_Type cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(

SRB_Identity_Type p_SRB_Id,

template (value) NR_CellId_Type p_NR_CellId,

template(omit) RLC_Configuration_Type p_RLC_Configuration := omit) :=

cs_RadioBearer_Reconfig(cs_RadioBearerId_SRB(p_SRB_Id),

cs_PDCP_Configuration_NR_Terminating(cs_NR_PDCP_RbConfig_ParamsSRB,

cs_RlcBearerRouting_NR(p_NR_CellId)), p_RLC_Configuration);

After change:

template (value) RadioBearer_Type cs_RadioBearer_Reconfig_SRBwithNR_PDCPonEUTRA(

SRB_Identity_Type p_SRB_Id,

template (value) NR_CellId_Type p_NR_CellId,

template (value) RlcBearerRouting_Type p_RlcBearerRouting,

template(omit) RLC_Configuration_Type p_RLC_Configuration := omit,

template(omit) MAC_Configuration_Type p_MAC_Configuration := omit,

template (omit) LogicalChannelId_Type p_LogicalChannelId := omit) :=

cs_RadioBearer_Reconfig(cs_RadioBearerId_SRB(p_SRB_Id),

cs_PDCP_Configuration_NR_Terminating(cs_NR_PDCP_RbConfig_ParamsSRB,

p_RlcBearerRouting), p_RLC_Configuration,

p_MAC_Configuration, p_LogicalChannelId);

## 2.3 Change 3

| Function name | fl_TC_8_2_2_3_1_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | LTE handover happened at Step 1 with sk_counter = 1. Before Step 3, it is necessary to re-calculate S_KGNB key based on sk_counter = 1. |
| Summary of change | Add f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys before Step 3. |
| TTCN module |  |
| MCC160 Comment | RRCReconfig_ENDC_EUTRA |

Before change:

function fl_TC_8_2_2_3_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{// @sic R5-202647, R5-206340, R5s201348 sic@

var template (omit) octetstring v_NR_SCGConfig := omit;

var template (omit) octetstring v_NR_RBConfig := omit;

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get();

// @siclog "Steps 1-2" siclog@

f_EUTRA38_ReconfigureSRBs_NR_PDCPSplit(eutra_Cell1, true, (v_Security_Params.SK_Counter + 1), false);

// @siclog "Step 3" siclog@

// The SS transmits on SRB3/SCG an NR RRCReconfiguration message to reconfigure SCG DRB.

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

// @siclog "Step 5" siclog@

// The SS transmits over split SRB1 SCG path/NR Cell 1 an RRCConnectionReconfiguration message to release SRB3.

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig);

SRB.send(cas_SRB1_RrcNasPduList_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_RRCConnectionReconfiguration_Common(tsc_RRC_TI_Def,-,-,-,-,v_RRCConnectionReconfiguration_v890_IEs),

omit));

// @siclog "Step 6" siclog@

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

// @siclog "Step 7" siclog@

// The SS transmits an RRCConnectionReconfiguration message to release SRB1 over SCG path/NR Cell 1

// Keep split SRB configured at SS side to ensure UE is not using the SCG path

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig);

SRB.send(cas_SRB1_RrcNasPduList_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_RRCConnectionReconfiguration_Common(tsc_RRC_TI_Def,-,-,-,-,v_RRCConnectionReconfiguration_v890_IEs),

omit));

// @siclog "Step 8" siclog@

// Check: Does the UE send RRCConnectionReconfigurationComplete containing NR RRCReconfigurationComplete message on SRB1 over the MCG path?

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1, tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8");

}

After change:

function fl_TC_8_2_2_3_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{// @sic R5-202647, R5-206340, R5s201348 sic@

var template (omit) octetstring v_NR_SCGConfig := omit;

var template (omit) octetstring v_NR_RBConfig := omit;

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get();

// @siclog "Steps 1-2" siclog@

f_EUTRA38_ReconfigureSRBs_NR_PDCPSplit(eutra_Cell1, true, (v_Security_Params.SK_Counter + 1), false);

// @siclog "Step 3" siclog@

// The SS transmits on SRB3/SCG an NR RRCReconfiguration message to reconfigure SCG DRB.

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR, 1);

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

// @siclog "Step 5" siclog@

// The SS transmits over split SRB1 SCG path/NR Cell 1 an RRCConnectionReconfiguration message to release SRB3.

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig);

SRB.send(cas_SRB1_RrcNasPduList_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_RRCConnectionReconfiguration_Common(tsc_RRC_TI_Def,-,-,-,-,v_RRCConnectionReconfiguration_v890_IEs),

omit));

// @siclog "Step 6" siclog@

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

// @siclog "Step 7" siclog@

// The SS transmits an RRCConnectionReconfiguration message to release SRB1 over SCG path/NR Cell 1

// Keep split SRB configured at SS side to ensure UE is not using the SCG path

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig);

SRB.send(cas_SRB1_RrcNasPduList_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_RRCConnectionReconfiguration_Common(tsc_RRC_TI_Def,-,-,-,-,v_RRCConnectionReconfiguration_v890_IEs),

omit));

// @siclog "Step 8" siclog@

// Check: Does the UE send RRCConnectionReconfigurationComplete containing NR RRCReconfigurationComplete message on SRB1 over the MCG path?

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1, tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8");

}

## 2.4 Change 4

| Function name | fl_TC_8_2_2_3_1_ENDC_NR_TestBody |
| --- | --- |
| Reason for change | - From R5s201348 Change 7, it was agreed to use cs_NR_RlcBearerConfig_SRB1SRB2SRB3 instead of cs_NR_RlcBearerConfig_AM but it was not reflected in D20wk50 TTCN. - LTE handover happened at Step 1 with sk_counter = 1. Before Step 3, it is necessary to re-calculate S_KGNB key based on sk_counter = 1. |
| Summary of change | - Use cs_NR_RlcBearerConfig_SRB1SRB2SRB3 instead of cs_NR_RlcBearerConfig_AM. - Add f_ENDC_GetKeysActivateSecurity before Step 3. |
| TTCN module |  |
| MCC160 Comment | RRCReconfig_ENDC_NR |

Before change:

function fl_TC_8_2_2_3_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC

{// @sic R5-202647, R5-206340, R5s201348 sic@

var template (value) CellGroupConfig v_SCGConfig;

var template (value) RadioBearerConfig v_RadioBearerConfig1;

var template (value) RadioBearerConfig v_RadioBearerConfig2;

var template (omit) octetstring v_EncodedSCG_Config := omit;

var template (value) SRB_ToAddModList v_SRB_ToAddModList;

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList;

var template (value) NR_RadioBearerList_Type v_RadioBearerList;

var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1);

var template (value) UL_AM_RLC v_ParametersTX := f_NR_UL_AM_RLC(v_IsFR1);

var template (value) DL_AM_RLC v_ParametersRX := f_NR_DL_AM_RLC(v_IsFR1);

// @siclog "Step 1" siclog@

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertCiphAlg2Bitstring(f_NR_AS_CipheringAlgorithm_Get())), "NR Ciphering Alg"));

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertIntAlg2Bitstring(f_NR_AS_IntegrityAlgorithm_Get())), "NR Integrity Alg"));

//Set the NR ENDC Configuration for SRB1

v_SRB_ToAddModList := {

cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB1, cs_NR_PDCP_Config_SplitSRB(-, f_NR_LogicalChannelId_SRB(tsc_SRB1))),

cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)

};

v_RadioBearerConfig1 := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod(tsc_SRB3)}, -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary));

v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(v_SRB_ToAddModList, -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), master));

v_RLC_BearerToAddModList := {

cs_RLC_BearerConfig_SRB_ReestablishRLC(tsc_SRB1),

cs_38508_RLC_BearerConfig_SRB(tsc_SRB3),

cs_38508_RLC_BearerConfig_DRB(tsc_NR_DRB2,

cs_38508_RLC_Config_AM_DRB(v_ParametersTX, v_ParametersRX),

cs_38508_LogicalChannelConfig_DRB,

true_)

};

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, v_RLC_BearerToAddModList);

//SS : configure SRB1 with RLCAckProhibit

v_RadioBearerList := {

cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB1, cs_NR_PDCP_Configuration_Proxy(crs_RlcBearerRouting_Eutra(eutra_Cell1)), cs_NR_RlcBearerConfig_AM(v_IsFR1, -, f_NR_LogicalChannelId_SRB(tsc_SRB1))),

cs_NR_SS_SRB3_Config

};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_RadioBearerList, cs_TimingInfo_Now); //Configure SRB1 in NR with PDCP set to proxy + SRB3 on NR

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig1, v_SCGConfig, -, -, false);  // Now send octetstrings to EUTRA

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig2, omit);  // Now send octetstrings to EUTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

// @siclog "Step 3" siclog@

// SS transmits an NR RRCReconfiguration message to reconfigure SCG DRB on SRB3/NR Cell 1

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, -, cs_NR_MAC_CellGroupConfig(-, -, cs_NR_BSR_ConfigDef(sf5, sf40)));

v_EncodedSCG_Config := bit2oct(encvalue(v_SCGConfig));

SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, v_EncodedSCG_Config)));

// @siclog "Step 4" siclog@

// Check: Does the UE transmit an NR RRCReconfigurationComplete message on SRB3/NR Cell 1?

SRB.receive(car_NR_SRB3_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

// @siclog "Step 5" siclog@

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, { f_NR_LogicalChannelId_SRB(tsc_SRB3) });

// Now send octetstrings to EUTRA

f_NR_SendRRCReconfigurationContentsToEUTRA (cs_NR_RadioBearer_ReleaseSRB3, v_SCGConfig, -, -, false);

// @siclog "Step 7-8" siclog@

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, { f_NR_LogicalChannelId_SRB(tsc_SRB1) });

f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig, -, -, false); // Now send octetstrings to EUTRA

}

After change:

function fl_TC_8_2_2_3_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC

{// @sic R5-202647, R5-206340, R5s201348 sic@

var template (value) CellGroupConfig v_SCGConfig;

var template (value) RadioBearerConfig v_RadioBearerConfig1;

var template (value) RadioBearerConfig v_RadioBearerConfig2;

var template (omit) octetstring v_EncodedSCG_Config := omit;

var template (value) SRB_ToAddModList v_SRB_ToAddModList;

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList;

var template (value) NR_RadioBearerList_Type v_RadioBearerList;

var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1);

var template (value) UL_AM_RLC v_ParametersTX := f_NR_UL_AM_RLC(v_IsFR1);

var template (value) DL_AM_RLC v_ParametersRX := f_NR_DL_AM_RLC(v_IsFR1);

// @siclog "Step 1" siclog@

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertCiphAlg2Bitstring(f_NR_AS_CipheringAlgorithm_Get())), "NR Ciphering Alg"));

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertIntAlg2Bitstring(f_NR_AS_IntegrityAlgorithm_Get())), "NR Integrity Alg"));

//Set the NR ENDC Configuration for SRB1

v_SRB_ToAddModList := {

cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB1, cs_NR_PDCP_Config_SplitSRB(-, f_NR_LogicalChannelId_SRB(tsc_SRB1))),

cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)

};

v_RadioBearerConfig1 := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod(tsc_SRB3)}, -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary));

v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(v_SRB_ToAddModList, -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), master));

v_RLC_BearerToAddModList := {

cs_RLC_BearerConfig_SRB_ReestablishRLC(tsc_SRB1),

cs_38508_RLC_BearerConfig_SRB(tsc_SRB3),

cs_38508_RLC_BearerConfig_DRB(tsc_NR_DRB2,

cs_38508_RLC_Config_AM_DRB(v_ParametersTX, v_ParametersRX),

cs_38508_LogicalChannelConfig_DRB,

true_)

};

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, v_RLC_BearerToAddModList);

//SS : configure SRB1 with RLCAckProhibit

v_RadioBearerList := {

cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB1, cs_NR_PDCP_Configuration_Proxy(crs_RlcBearerRouting_Eutra(eutra_Cell1)), cs_NR_RlcBearerConfig_SRB1SRB2SRB3(tsc_NR_SRB1_LogicalChannelId, tsc_NR_SRB1_LogicalChannelPriority)),

cs_NR_SS_SRB3_Config

};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_RadioBearerList, cs_TimingInfo_Now); //Configure SRB1 in NR with PDCP set to proxy + SRB3 on NR

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig1, v_SCGConfig, -, -, false);  // Now send octetstrings to EUTRA

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig2, omit);  // Now send octetstrings to EUTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

f_ENDC_GetKeysActivateSecurity(nr_Cell1, EUTRA);

// @siclog "Step 3" siclog@

// SS transmits an NR RRCReconfiguration message to reconfigure SCG DRB on SRB3/NR Cell 1

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, -, cs_NR_MAC_CellGroupConfig(-, -, cs_NR_BSR_ConfigDef(sf5, sf40)));

v_EncodedSCG_Config := bit2oct(encvalue(v_SCGConfig));

SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, v_EncodedSCG_Config)));

// @siclog "Step 4" siclog@

// Check: Does the UE transmit an NR RRCReconfigurationComplete message on SRB3/NR Cell 1?

SRB.receive(car_NR_SRB3_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

// @siclog "Step 5" siclog@

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, { f_NR_LogicalChannelId_SRB(tsc_SRB3) });

// Now send octetstrings to EUTRA

f_NR_SendRRCReconfigurationContentsToEUTRA (cs_NR_RadioBearer_ReleaseSRB3, v_SCGConfig, -, -, false);

// @siclog "Step 7-8" siclog@

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, { f_NR_LogicalChannelId_SRB(tsc_SRB1) });

f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig, -, -, false); // Now send octetstrings to EUTRA

}