3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210475

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1696 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Re-verification of NR5GC IRAT test case 11.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To re-verify NR5GC IRAT test case 11.1.3 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC IRAT test case 11.1.3 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	7

3	Branches executed	8

4	Execution Log Files	9

4.1	Qualcomm SM8350 + SDX60	9

5	References	9

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.1.3 which is part of the 5G NRGC_IRAT test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Narendra Kalahasti

Narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	11.1.3.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk12

System Simulator used:	Keysight S8704A Protocol Conformance Toolset and Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM8350 + SDX60

Verification Status:	PASS

# Corrections required

In addition to the below changes, changes 2 and 7 from R5s210414 are required and applied.

## Change 1

| Function name | f_TC_11_1_3_NR5GC_EUTRA() |
| --- | --- |
| Reason for change | RTP loopback needs to be enabled for the IMS voice call established during this test case. |
| Summary of change | Add HANDLE_UM_DATA to the f_EUTRA_Init() function which controls RTP loopback configuration. Note : Same change required for all other EPS fallback test cases |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_11_1_3_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { //MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / Success f_EUTRA38_IRAT_Init(c31); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_11_1_3_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); // @sic R5-204399 sic@ }

After Change

function f_TC_11_1_3_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { //MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with handover / Single registration mode with N26 interface / Success f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_11_1_3_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); // @sic R5-204399 sic@ }

## Change 2

| Function name | fl_TC_11_1_3_NR5GC_EUTRA_TestBody() |
| --- | --- |
| Reason for change | Currently the handover command is sent tp NR PTC before EUTRA SS configuration is done, this can cause a race condition where NR side initiates the handover and RRCConnectionReconfigurationComplete comes form the UE while SS configuration is still being done. |
| Summary of change | Move triggering of handover to later after EUTRA SS side configuration is done. |
| TTCN module |  |
| MCC160 Comment |  |

Before Change

function fl_TC_11_1_3_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { …………….. // Send EUTRA HO command to NR f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_HOCmd, "HO Cmd")); v_Auth_Params := f_EUTRA38_MappedContextFromN1_InitNAS(eutra_Cell1, v_SecurityRx, true); v_Auth_Params.AS_Ciphering.ActTimeList := f_EUTRA_RRC_CipherActTime_GetForHO(eutra_Cell1, tsc_DrbList_Def); //f_EUTRA_NB_SS_NAS_ConfigureSecurity(NASCTRL, v_Auth_Params.NAS_Integrity, v_Auth_Params.NAS_Ciphering); f_EUTRA_SS_RRC_EnableIntProt_CiphULandDL(eutra_Cell1, v_Auth_Params.AS_Integrity, v_Auth_Params.AS_Ciphering, cs_TimingInfo_Now); f_EUTRA_Security_Set(v_Auth_Params); //Configure UL grant configuration ("OnSR", periodic TA is NOT started) in target cell (cell 1) f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_ULGrantAllocation_Def_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_None)); // @siclog "Step 18" siclog@ SRB.receive( car_SRB1_RrcPdu_IND( eutra_Cell1, cr_508_RRCConnectionReconfigurationComplete( tsc_RRC_TI_Def ) ) ); /* SS automatically checks that UE transmit an RRCConnectionReconfigurationComplete using the indicated security key and the indicated security algorithms */ f_EUTRA_PreliminaryPass( __FILE__, __LINE__, "Step 18" ); ………….. }

After Change

function fl_TC_11_1_3_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { …………….. // Send EUTRA HO command to NR //MOVED f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_HOCmd, "HO Cmd")); v_Auth_Params := f_EUTRA38_MappedContextFromN1_InitNAS(eutra_Cell1, v_SecurityRx, true); v_Auth_Params.AS_Ciphering.ActTimeList := f_EUTRA_RRC_CipherActTime_GetForHO(eutra_Cell1, tsc_DrbList_Def); //f_EUTRA_NB_SS_NAS_ConfigureSecurity(NASCTRL, v_Auth_Params.NAS_Integrity, v_Auth_Params.NAS_Ciphering); f_EUTRA_SS_RRC_EnableIntProt_CiphULandDL(eutra_Cell1, v_Auth_Params.AS_Integrity, v_Auth_Params.AS_Ciphering, cs_TimingInfo_Now); f_EUTRA_Security_Set(v_Auth_Params); //Configure UL grant configuration ("OnSR", periodic TA is NOT started) in target cell (cell 1) f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_ULGrantAllocation_Def_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_None)); f_EUTRA_NR_SendCoOrd(NR, cms_EUTRA_NR_OctetData (v_HOCmd, "HO Cmd")); // @siclog "Step 18" siclog@ SRB.receive( car_SRB1_RrcPdu_IND( eutra_Cell1, cr_508_RRCConnectionReconfigurationComplete( tsc_RRC_TI_Def ) ) ); /* SS automatically checks that UE transmit an RRCConnectionReconfigurationComplete using the indicated security key and the indicated security algorithms */ f_EUTRA_PreliminaryPass( __FILE__, __LINE__, "Step 18" ); ………….. }

## Change 3

| Function name | f_TC_11_1_3_NR5GC_IMS1() |
| --- | --- |
| Reason for change | E.g. The function fl_IMSFallback_RegistrationToMOSpeechCall() will end with the sending of 200 OK for PRACK. A short delay should be wllowed so this can be sent from the SS and associated TCP ACK is received, before triggering NR and EUTRA PTCs to switch RAT and update the IP routing table. |
| Summary of change | Add short delay to allow all IP packet exchange associated with IMS signalling to complete before triggering RAT change. Note : Same change required for other EPS fallback test cases |
| TTCN module | EPS_Fallabck_NR5GC_IMS.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_11_1_3_NR5GC_IMS1()runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; v_InviteRequestWithSdp := fl_IMSFallback_RegistrationToMOSpeechCall(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_EPS_Fallback_Part2(v_InviteRequestWithSdp); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Parallel Behaviour Step 1a8/1b6"); //Send coordination to EUTRA for Step 27 f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); // @siclog "Step 29" siclog@ f_IMS_IPCAN_WaitForTrigger(OtherIPCAN); f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

After Change

function f_TC_11_1_3_NR5GC_IMS1()runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; v_InviteRequestWithSdp := fl_IMSFallback_RegistrationToMOSpeechCall(); f_Delay(0.5); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_EPS_Fallback_Part2(v_InviteRequestWithSdp); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Parallel Behaviour Step 1a8/1b6"); //Send coordination to EUTRA for Step 27 f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); // @siclog "Step 29" siclog@ f_IMS_IPCAN_WaitForTrigger(OtherIPCAN); f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

# Branches executed

The test case was executed on EUTRAN<>NR band combinations 3<>n79 and 66<>n41.

# Execution Log Files

## Qualcomm SM8350 + SDX60

The Qualcomm SM8350 + SDX60 UE passed this test case on the Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_11_1_3_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_11_1_3_PIXIT.html

The Qualcomm SM8350 + SDX60 UE also passed this test case on the Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log:

Anritsu/ TC_11_1_3_LOGS.xml

- PICS/PIXIT parameter file:

Anritsu/TC_11_1_3_PIXIT.txt

# References

| [1] | R5s210474 : Supporting information for re-verification of NR5GC IRAT test case 11.1.3 |
| --- | --- |