3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210579

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1730 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.3.1.2 (ENDC and NR5GC) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current TTCN implementation of 7.1.3.1.2 supports below cases as mentioned in the workshop document R5w210114. 1) Single NR_DRB_Common_IND with 32 PDCP SDU list (Received in same slot) or 2) 32 NR_DRB_Common_IND with 1 PDCP SDU (32 SDUs are received in same slot) The current loop structure works well for the case 2, whereas for the case 1 , the loop structure is not doing right comparison between expected and Received PdcpSduList |  |  |  |  |  |  |  |  |
| Summary of change: |  | Loop is corrected so that Received and Expected PdcpSduList comparison is rightly done in case of SS receiving Single NR_DRB_Common_IND with 32 PDCP SDU list |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.1.2.ENDC and 7.1.3.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function fl_TC_7_1_3_1_1and2	2

# Corrections required

## Correction to function fl_TC_7_1_3_1_1and2

| Function name | function fl_TC_7_1_3_1_1and2 |
| --- | --- |
| Reason for change | The current TTCN implementation of 7.1.3.1.2 supports below cases as mentioned in the workshop document R5w210114. 1) Single NR_DRB_Common_IND with 32 PDCP SDU list (Received in same slot) or 2) 32 NR_DRB_Common_IND with 1 PDCP SDU (32 SDUs are received in same slot) The current loop structure works well for the case 2, whereas for the case 1 , the loop structure is not doing right comparison between expected and Received PdcpSduList |
| Summary of change | Loop is corrected so that Received and Expected PdcpSduList comparison is rightly done in case of SS receiving Single NR_DRB_Common_IND with 32 PDCP SDU list |
| TTCN module | ttcn\develop\NR_TC_Common\7_1_3\PDCP_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_7_1_3_1_1and2(NR_PDCP_SS_State_Type p_NR_PDCP_State, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC {//@sic R5-211387 sic@ <<< SKIPPED CODE >>> // EXCEPTION: : Steps 1 - 2 shall be repeated from j = 0 to j = FLOOR(Maximum_PDCP_SN/iteration_size). (Note 1, 4) for (j := 0; j < p_OuterLoop; j:= j + 1) { // @sic R5-197663, R5-197664, R5-202613 sic@ //@siclog "Step 1" siclog@ // SS transmits several PDCP PDUs in a PDCP PDU list without header compression, the number of PDCP PDUs sent in each PDCP PDU list is defined by the iteration_size. (Note 4, 5). v_PdcpPduList := f_GeneratePdcpPDUList(p_NR_PDCP_State, v_NumberOfPdu,v_Sdu); f_NR_PDCPPDUList_Send(p_NR_PDCP_State,nr_Cell1,v_PdcpPduList); // EXCEPTION: In Step 2, SS may receive a PDCP PDU or several PDCP PDUs, then step 2 may be repeated multiple times until all PDCP PDUs with SN=j*iteration_size to SN=(((j+1)* iteration_size)-1) for each iteration are received. // @siclog "Step 2" siclog@ // CHECK: Does UE transmit PDCP PDUs with SN=0 for the first iteration and all PDCP PDUs for each iteration?(Note 2) for (i := 0; i < p_InnerLoop; i:= i + 1) { // @sic R5-211387 sic@ // Get next list of PDCP PDUs received v_NR_L2DataList := f_NR_PDCP_PDU_GetList(p_NR_PDCP_State); //R5s210395 v_LenPDUList := lengthof(v_NR_L2DataList.PdcpPdu); for (k := 0; k < v_LenPDUList; k := k + 1) { // set i to account for k i := i + k; if (i == p_InnerLoop) {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, too many PDCP PDUs received");}; // check ok ??? // extract the PDCP PDU to be handled v_NR_PDCP_PDU_Received := v_NR_L2DataList.PdcpPdu[k]; // build the PDCP PDU expected (template) v_NR_PDCP_PDU_Expected := v_PdcpPduList[i]; // ith element in the currently used v_PdcpPduList // match the PDCP PDU to be handled against the PDCP PDU expected if (not match (v_NR_PDCP_PDU_Received, v_NR_PDCP_PDU_Expected)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, mismatch of received and expected PDUs"); }; if ((i == 0) and (j == 0)) { // SN = 0 f_NR_PreliminaryPass(__FILE__, __LINE__, " SN=0 "); } else { // SN = 1 .. if ((((j * p_InnerLoop) + i + 1) mod v_VerdictFilter) == 0) {f_NR_PreliminaryPass(__FILE__, __LINE__, " SN = " & int2str((j * p_InnerLoop) + i) )}; // @sic R5-185083, R5-187106, R5s200315, R5s200957 sic@ } } } } //@siclog "Step 3" siclog@ // SS transmits a PDCP PDU containing one PDCP SDU without header compression. f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_Sdu); //@siclog "Step 4" siclog@ // CHECK: Does UE transmit a PDCP PDU with SN=0? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_Sdu, cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ //SS transmits a PDCP PDU containing one PDCP SDU without header compression. f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1, v_Sdu); //@siclog "Step 6" siclog@ //CHECK: Does UE transmit a PDCP PDU with SN=1? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_Sdu, cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); }

After Change:

function fl_TC_7_1_3_1_1and2(NR_PDCP_SS_State_Type p_NR_PDCP_State, integer p_OuterLoop, integer p_InnerLoop) runs on NR_BASE_PTC {//@sic R5-211387 sic@ <<< SKIPPED CODE >>> // EXCEPTION: In Step 2, SS may receive a PDCP PDU or several PDCP PDUs, then step 2 may be repeated multiple times until all PDCP PDUs with SN=j*iteration_size to SN=(((j+1)* iteration_size)-1) for each iteration are received. // @siclog "Step 2" siclog@ // CHECK: Does UE transmit PDCP PDUs with SN=0 for the first iteration and all PDCP PDUs for each iteration?(Note 2) for (i := 0; i < p_InnerLoop; i:= i + 1) { // @sic R5-211387 sic@ // Get next list of PDCP PDUs received v_NR_L2DataList := f_NR_PDCP_PDU_GetList(p_NR_PDCP_State); //R5s210395 v_LenPDUList := lengthof(v_NR_L2DataList.PdcpPdu); for (k := 0; k < v_LenPDUList; k := k + 1) { // set i to account for k if(v_LenPDUList > 1) // Single NR_DRB_Common_IND with 32 PDCP SDU list { i := k; } else //this condition is to receive 32 NR_DRB_Common_IND with 1 PDCP SDU { i := i + k; } if (i == p_InnerLoop) {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, too many PDCP PDUs received");}; // check ok ??? // extract the PDCP PDU to be handled v_NR_PDCP_PDU_Received := v_NR_L2DataList.PdcpPdu[k]; // build the PDCP PDU expected (template) v_NR_PDCP_PDU_Expected := v_PdcpPduList[i]; // ith element in the currently used v_PdcpPduList // match the PDCP PDU to be handled against the PDCP PDU expected if (not match (v_NR_PDCP_PDU_Received, v_NR_PDCP_PDU_Expected)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2, mismatch of received and expected PDUs"); }; if ((i == 0) and (j == 0)) { // SN = 0 f_NR_PreliminaryPass(__FILE__, __LINE__, " SN=0 "); } else { // SN = 1 .. if ((((j * p_InnerLoop) + i + 1) mod v_VerdictFilter) == 0) {f_NR_PreliminaryPass(__FILE__, __LINE__, " SN = " & int2str((j * p_InnerLoop) + i) )}; // @sic R5-185083, R5-187106, R5s200315, R5s200957 sic@ } } } } //@siclog "Step 3" siclog@ // SS transmits a PDCP PDU containing one PDCP SDU without header compression. f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_Sdu); //@siclog "Step 4" siclog@ // CHECK: Does UE transmit a PDCP PDU with SN=0? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_Sdu, cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ //SS transmits a PDCP PDU containing one PDCP SDU without header compression. f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1, v_Sdu); //@siclog "Step 6" siclog@ //CHECK: Does UE transmit a PDCP PDU with SN=1? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_Sdu, cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); }