3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210894

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1889 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EN-DC FR1 : Addition of NR MAC test case 7.1.1.3.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR MAC test case 7.1.1.3.8.1 to the 5G/ENDC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR MAC test case 7.1.1.3.8.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.8.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	8

4	Execution Log Files	8

4.1	HiSilicon Balong 5000	8

5	References	8

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.1.1.3.8.1which is part of the 5G ENDC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	7.1.1.3.8.1.ENDC

ATS Version:	iwd-TTCN3-B2020-09_D12wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | TC_7_1_1_3_8_1_ENDC() |
| --- | --- |
| Reason for change | - Since multi PHR implementation is taken care of in the test case body, we should avoid calling f_NR_InitialiseMultiplePHR() to avoid conflicts. - PHR reporting on SS SYSIND port needs to be enabled before step 5 |
| Summary of change | - Pass UL_CA as false to f_ENDC_NR_Init_CA() to avoid calling f_NR_InitialiseMultiplePHR() - Add calls to f_NR_SS_SystemIndCtrlConfig() to enabled SYSIND reporting of PHR before step 5, and disable again at the end of the test case body. |
| TTCN module | MAC_CA_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_3_8_ENDC_Common() runs on ENDC_NR_PTC { var template (value) CellGroupConfig v_SCGConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP; var SubFrameTiming_Type v_TimingUL; // Multi PR initialisation is taken care in Test body. f_ENDC_NR_Init_CA(p_NR_CA_Tested, {p_PSCell, p_SCell}, true);//@sic R5s211385 sic@ //Create and configure NR cell1s f_NR_CellConfig_PCell_Def(p_PSCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(p_PSCell, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the SCG DRB in test mode f_NR_ENDC_ConfigAM_UM(p_PSCell, 1, 0, -, -, -, -, -, -); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_SetCellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_TestBody_Set(true); //@siclog "Steps 1 - 2" siclog@ f_NR_AddMod_1SCell_ENDC(p_PSCell, p_SCell,-,-,-,-,true); v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_PSCell, tsc_NR_BWP_Id); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(136, v_NR_UplinkBWP); //The SS transmits UL grant to the UE at every 10ms in PDCCH occasion. //@siclog "Step 3 " siclog@ v_TimingUL := f_NR_GetNextSendOccasion_UL(p_PSCell, 100); f_NR_ULGrantConfiguration_StartPeriodicGrant(p_PSCell, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 10); //@siclog "Steps 4&5" siclog@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_PSCell); v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_71138); f_NR_CellInfo_SetMAC_CellGroupConfig(p_PSCell, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, omit, omit, v_MAC_CellGroupConfig); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig); // Now send octetstrings to EUTRA //@siclog "Steps 5" siclog@ SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_PSCell, cr_NR_MAC_CE_MultiEntryPHR(?))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); //@siclog "Steps 7-20" siclog@ f_TC_7_1_1_3_8_NR_TestBody(p_PSCell, p_SCell,cr_NR_MAC_CE_MultiEntryPHR_3Entries(?),tsc_SCellIndex_2); //@siclog "Steps 21-22" siclog@ v_MAC_CellGroupConfig.phr_Config := { release := NULL }; f_NR_CellInfo_SetMAC_CellGroupConfig(p_PSCell, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, //@sic R5s200753 sic@ omit, omit, v_MAC_CellGroupConfig); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig); // Now send octetstrings to EUTRA f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change

import from NR_ASP_Templates_SysInd all; ………… function f_TC_7_1_1_3_8_ENDC_Common() runs on ENDC_NR_PTC { var template (value) CellGroupConfig v_SCGConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP; var SubFrameTiming_Type v_TimingUL; // Multi PR initialisation is taken care in Test body. f_ENDC_NR_Init_CA(p_NR_CA_Tested, {p_PSCell, p_SCell}, false);//@sic R5s211385 sic@ //Create and configure NR cell1s f_NR_CellConfig_PCell_Def(p_PSCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active); // Get NR capabilities sent during EUTRA preamble f_NR_ENDC_PreambleOnEUTRA(p_PSCell, MCG_Only); f_NR_AS_CipheringAlgorithm_Set(nea0); //Configure on SS the SCG DRB in test mode f_NR_ENDC_ConfigAM_UM(p_PSCell, 1, 0, -, -, -, -, -, -); // Wait for Trigger from EUTRA indicating start of Test Body f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_SetCellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_TestBody_Set(true); //@siclog "Steps 1 - 2" siclog@ f_NR_AddMod_1SCell_ENDC(p_PSCell, p_SCell,-,-,-,-,true); v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_PSCell, tsc_NR_BWP_Id); v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(136, v_NR_UplinkBWP); //The SS transmits UL grant to the UE at every 10ms in PDCCH occasion. //@siclog "Step 3 " siclog@ v_TimingUL := f_NR_GetNextSendOccasion_UL(p_PSCell, 100); f_NR_ULGrantConfiguration_StartPeriodicGrant(p_PSCell, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 10); //@siclog "Steps 4&5" siclog@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_PSCell); v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_71138); f_NR_CellInfo_SetMAC_CellGroupConfig(p_PSCell, v_MAC_CellGroupConfig); f_NR_SS_SystemIndCtrlConfig(p_PSCell, cds_NR_SystemIndCtrl_MAC_PHR(enable)); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, omit, omit, v_MAC_CellGroupConfig); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig); // Now send octetstrings to EUTRA //@siclog "Steps 5" siclog@ SYSIND.receive(car_NR_SYSTEM_MultiEntryPHR_IND(p_PSCell, cr_NR_MAC_CE_MultiEntryPHR(?))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); //@siclog "Steps 7-20" siclog@ f_TC_7_1_1_3_8_NR_TestBody(p_PSCell, p_SCell,cr_NR_MAC_CE_MultiEntryPHR_3Entries(?),tsc_SCellIndex_2); //@siclog "Steps 21-22" siclog@ v_MAC_CellGroupConfig.phr_Config := { release := NULL }; f_NR_CellInfo_SetMAC_CellGroupConfig(p_PSCell, v_MAC_CellGroupConfig); v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG, //@sic R5s200753 sic@ omit, omit, v_MAC_CellGroupConfig); f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig); // Now send octetstrings to EUTRA f_NR_SS_SystemIndCtrlConfig(p_PSCell, cds_NR_SystemIndCtrl_MAC_PHR(disable)); f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

# Branches executed

The test case was executed on NR band n78 uing NR ciphering nea1 and integrity algorithm nia1

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_1_1_3_8_1_LOG.html

- PICS/PIXIT parameter file:

TC_7_1_1_3_8_1_PIXIT.xml

# References

| [1] | R5s210895 : EN-DC FR1 : Supporting information for addition of NR MAC test case 7.1.1.3.8.1 |
| --- | --- |