3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210960

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1789 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test case 8.1.1.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Following the implementation of R5-213466, steps 18 – 21 are added. After Step 21, the UE should be registered on Cell 3, so this cell should be used for the postamble sequence. - In the step 20, the UE acquired the new system information and reselected to Cell 3. At this time, the cell power level meets the cell reselection conditions of the same priority between Cell 1 and Cell 3, which means the UE cannot reselect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Pass cell 3 to postamble function - Added the step 18A and changed the cellReselectionPriority to 7 in SIB2 of Cell 3. Note : An associated prose CR on 38.523-1 will be raised at RAN5#92 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_8_1_1_2_1_NR5GC |
| --- | --- |
| Reason for change | Following the implementation of R5-213466, steps 18 – 21 are added. After Step 21, the UE should be registered on Cell 3, so this cell should be used for the postamble sequence. |
| Summary of change | Pass cell 3 to postamble function |
| TTCN module | RRC_ConnEst_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_1_2_1_NR5GC() runs on NR5GC_PTC //@sic R5-190680 sic@ { // RRC connection establishment / Return to idle state after T300 expiry var DeltaValues_Type v_DeltaPrimaryBand; f_NR5GC_Init(NR_4);//@sic R5-201216 sic@ f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -78, -82);//@sic R5-201216 sic@ if (not f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); f_NR_CellInfo_SetAndCalculateQ_RxLevMin(nr_Cell1, -91.0, v_DeltaPrimaryBand.DeltaNRf1); } if (not f_NR_CellInfo_GetIsFR1(nr_Cell3)) { v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); f_NR_CellInfo_SetAndCalculateQ_RxLevMin(nr_Cell3, -100.0, v_DeltaPrimaryBand.DeltaNRf2); } //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3);//@sic R5-201216 sic@ f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_8_1_1_2_1_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A);//@sic R5-201216 sic@ }

After Change:

function f_TC_8_1_1_2_1_NR5GC() runs on NR5GC_PTC //@sic R5-190680 sic@ { // RRC connection establishment / Return to idle state after T300 expiry var DeltaValues_Type v_DeltaPrimaryBand; f_NR5GC_Init(NR_4);//@sic R5-201216 sic@ f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -78, -82);//@sic R5-201216 sic@ if (not f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); f_NR_CellInfo_SetAndCalculateQ_RxLevMin(nr_Cell1, -91.0, v_DeltaPrimaryBand.DeltaNRf1); } if (not f_NR_CellInfo_GetIsFR1(nr_Cell3)) { v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); f_NR_CellInfo_SetAndCalculateQ_RxLevMin(nr_Cell3, -100.0, v_DeltaPrimaryBand.DeltaNRf2); } //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell3);//@sic R5-201216 sic@ f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_8_1_1_2_1_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell3, STATE_IDLE_1A);//@sic R5-201216 sic@ }

## 2.2 Change 2

| Function name | fl_TC_8_1_1_2_1_TestBody() |
| --- | --- |
| Reason for change | In the step 20, the UE acquired the new system information and reselected to Cell 3. At this time, the cell power level meets the cell reselection conditions of the same priority between Cell 1 and Cell 3, which means the UE cannot reselect. |
| Summary of change | Added the step 18A and changed the cellReselectionPriority to 7 in SIB2 of Cell 3. Note : An associated prose CR on 38.523-1 will be raised at RAN5#92 |
| TTCN module | RRC_ConnEst_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_8_1_1_2_1_TestBody() runs on NR5GC_PTC { ………………… //@sic R5-213466 sic@ //@siclog "Step 18" siclog //The SS changes NR Cell 3 to high priority in SIB4. v_InterFreqCarrierFreqList := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell1); v_InterFreqCarrierFreqList[0].cellReselectionPriority := 7; f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell1, {v_InterFreqCarrierFreqList[0]}); //@siclog "Step 19" siclog //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1); //@siclog "Step 20" siclog //Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods (nr_Cell1); //@siclog "Step 21" siclog //Check: Does the test result of generic test procedure step 1-9a1 in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is in RRC_IDLE on NR Cell 3? f_NR5GC_CheckRRCIdle(nr_Cell3); }

After Change

function fl_TC_8_1_1_2_1_TestBody() runs on NR5GC_PTC { ……………………………… //@sic R5-213466 sic@ //@siclog "Step 18" siclog //The SS changes NR Cell 3 to high priority in SIB4. v_InterFreqCarrierFreqList := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell1); v_InterFreqCarrierFreqList[0].cellReselectionPriority := 7; f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell1, {v_InterFreqCarrierFreqList[0]}); //@siclog "Step 18A" siclog f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell3, 7); //@siclog "Step 19" siclog //The SS transmits a Short message on PDCCH using P-RNTI indicating a systemInfoModification. f_NR_ModifySysinfo(nr_Cell1); f_NR_ModifySysinfo(nr_Cell3); //@siclog "Step 20" siclog //Wait for 2.1* modification period to allow the new system information to take effect. f_NR_WaitModificationPeriods (nr_Cell1); //@siclog "Step 21" siclog //Check: Does the test result of generic test procedure step 1-9a1 in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is in RRC_IDLE on NR Cell 3? f_NR5GC_CheckRRCIdle(nr_Cell3); }