3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211292

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0146 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Positioning Functions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation, after changes introduced in R5s210633, test case may fail if UE does not report GNSS measurements for every GNSS signal supported. Before R5s210633, the implementation was checking that any GNSS signal measurement is reported, and those signals reported have to be matching the pics configuration. This criteria should be enough for the UE to pass the test case. Note: R5s210633 proposed change in TDoc does not match the effective CR in 37.571-4. Other files and test cases as the ones proposed in the TDoc were affected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of checking that every GNSS supported signal is reported as measurement, the proposed implementation checks that any signal that is reported matches the pics configuration (as it was before R5s210633) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.4.2.15s.NR5GC, 9.3.4.4.15s.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

Correction to f_POS_NR_CheckSignalsGps	2

Correction to f_POS_NR_CheckSignalsGlonass	3

Correction to f_POS_NR_CheckSignalsGalileo	4

Correction to f_POS_NR_CheckSignalsQzss	5

Correction to f_POS_NR_CheckSignalsBds	6

Correction to f_POS_CheckSignalsGps	6

Correction to f_POS_CheckSignalsGlonass	7

Correction to f_POS_CheckSignalsGalileo	8

Correction to f_POS_CheckSignalsQzss	9

Correction to f_POS_CheckSignalsBds	10

# Corrections required

## Correction to f_POS_NR_CheckSignalsGps

| Function name | f_POS_NR_CheckSignalsGps |
| --- | --- |
| Reason for change | In the current TTCN implementation, after changes introduced in R5s210633, test case may fail if UE does not report GNSS measurements for every GPS signal supported. |
| Summary of change | Instead of checking that every GPS supported signal is reported as measurement, the proposed implementation checks that any signal that is reported matches the pics configuration (as it was before R5s210633) |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

function f_POS_NR_CheckSignalsGps(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_A_GPS_L1C_A_Reported := false;

var boolean v_A_GPS_L1C_Reported := false;

var boolean v_A_GPS_L2C_Reported := false;

var boolean v_A_GPS_L5_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_A_GPS_L1C_A_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_A_GPS_L1C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_A_GPS_L2C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

v_A_GPS_L5_Reported := true;

}

if (not((v_A_GPS_L1C_A_Reported == pc_A_GPS_L1C_A) and (v_A_GPS_L1C_Reported == pc_A_GPS_L1C) and (v_A_GPS_L2C_Reported == pc_A_GPS_L2C) and (v_A_GPS_L5_Reported == pc_A_GPS_L5)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "A-GPS signals not reported correctly");

}

}

}

After

function f_POS_NR_CheckSignalsGps(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966 sic@

var integer i;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (not(((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) and pc_A_GPS_L2C) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) and pc_A_GPS_L5) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) and pc_A_GPS_L1C_A))) //@sic R5s201460 sic@

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "A-GPS signals not reported correctly");

}

}

}

## Correction to f_POS_NR_CheckSignalsGlonass

| Function name | f_POS_NR_CheckSignalsGlonass |
| --- | --- |
| Reason for change | In the current TTCN implementation, after changes introduced in R5s210633, test case may fail if UE does not report GNSS measurements for every GLONASS signal supported. |
| Summary of change | Instead of checking that every GLONASS supported signal is reported as measurement, the proposed implementation checks that any signal that is reported matches the pics configuration (as it was before R5s210633) |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

function f_POS_NR_CheckSignalsGlonass(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_GLONASS_G1_Reported := false;

var boolean v_GLONASS_G2_Reported := false;

var boolean v_GLONASS_G3_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_GLONASS_G1_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_GLONASS_G2_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_GLONASS_G3_Reported := true;

}

if (not((v_GLONASS_G1_Reported == pc_GLONASS_G1) and (v_GLONASS_G2_Reported == pc_GLONASS_G2) and (v_GLONASS_G3_Reported == pc_GLONASS_G3)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "GLONASS signals not reported correctly");

}

}

}

After

function f_POS_NR_CheckSignalsGlonass(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966 sic@

var integer i;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (not(((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) and pc_GLONASS_G2) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) and pc_GLONASS_G3) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) and pc_GLONASS_G1)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "GLONASS signals not reported correctly");

}

}

}

## Correction to f_POS_NR_CheckSignalsGalileo

| Function name | f_POS_NR_CheckSignalsGalileo |
| --- | --- |
| Reason for change | In the current TTCN implementation, after changes introduced in R5s210633, test case may fail if UE does not report GNSS measurements for every Galileo signal supported. |
| Summary of change | Instead of checking that every Galileo supported signal is reported as measurement, the proposed implementation checks that any signal that is reported matches the pics configuration (as it was before R5s210633) |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

function f_POS_NR_CheckSignalsGalileo(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_GALILEO_E1_Reported := false;

var boolean v_GALILEO_E5a_Reported := false;

var boolean v_GALILEO_E5b_Reported := false;

var boolean v_GALILEO_E6_Reported := false;

var boolean v_GALILEO_E5aE5b_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_GALILEO_E1_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_GALILEO_E5a_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_GALILEO_E5b_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

v_GALILEO_E6_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(4))) {

v_GALILEO_E5aE5b_Reported := true;

}

if (not((v_GALILEO_E1_Reported == pc_GALILEO_E1) and (v_GALILEO_E5a_Reported == pc_GALILEO_E5a) and (v_GALILEO_E5b_Reported == pc_GALILEO_E5b) and

(v_GALILEO_E6_Reported == pc_GALILEO_E6) and (v_GALILEO_E5aE5b_Reported == pc_GALILEO_E5aE5b)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "GLONASS signals not reported correctly");

}

}

}

After

function f_POS_NR_CheckSignalsGalileo(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966 sic@

var integer i;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (not(((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) and pc_GALILEO_E5a) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) and pc_GALILEO_E5b) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) and pc_GALILEO_E6) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(4))) and pc_GALILEO_E5aE5b) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) and pc_GALILEO_E1)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Galileo signals not reported correctly");

}

}

}

## Correction to f_POS_NR_CheckSignalsQzss

| Function name | f_POS_NR_CheckSignalsQzss |
| --- | --- |
| Reason for change | In the current TTCN implementation, after changes introduced in R5s210633, test case may fail if UE does not report GNSS measurements for every QZSS signal supported. |
| Summary of change | Instead of checking that every QZSS supported signal is reported as measurement, the proposed implementation checks that any signal that is reported matches the pics configuration (as it was before R5s210633) |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

function f_POS_NR_CheckSignalsQzss(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_QZSS_QZS_L1_Reported := false;

var boolean v_QZSS_QZS_L1C_Reported := false;

var boolean v_QZSS_QZS_L2C_Reported := false;

var boolean v_QZSS_QZS_L5_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_QZSS_QZS_L1_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_QZSS_QZS_L1C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_QZSS_QZS_L2C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

v_QZSS_QZS_L5_Reported := true;

}

if (not ((v_QZSS_QZS_L1_Reported == pc_QZSS_QZS_L1) and (v_QZSS_QZS_L1C_Reported == pc_QZSS_QZS_L1C) and

(v_QZSS_QZS_L2C_Reported == pc_QZSS_QZS_L2C) and (v_QZSS_QZS_L5_Reported == pc_QZSS_QZS_L5)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "QZSS signals not reported correctly");

}

}

}

}

After

function f_POS_NR_CheckSignalsQzss(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966 sic@

var integer i;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (not(((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) and pc_QZSS_QZS_L1C) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) and pc_QZSS_QZS_L2C) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) and pc_QZSS_QZS_L5) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) and pc_QZSS_QZS_L1)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "QZSS signals not reported correctly");

}

}

}

## Correction to f_POS_NR_CheckSignalsBds

| Function name | f_POS_NR_CheckSignalsBds |
| --- | --- |
| Reason for change | In the current TTCN implementation, after changes introduced in R5s210633, test case may fail if UE does not report GNSS measurements for every Beidou signal supported. |
| Summary of change | Instead of checking that every Beidou supported signal is reported as measurement, the proposed implementation checks that any signal that is reported matches the pics configuration (as it was before R5s210633) |
| TTCN module | NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

function f_POS_NR_CheckSignalsBds(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5s210633 sic@

var integer i;

var boolean v_BDS_B1I_Reported := false;

var boolean v_BDS_B1C_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_BDS_B1I_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(7,(9,10,11)))) {

v_BDS_B1C_Reported := true;

}

}

if (not ((v_BDS_B1I_Reported == pc_BDS_B1I) and (v_BDS_B1C_Reported == pc_BDS_B1C)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "BDS signals not reported correctly");

}

}

After

function f_POS_NR_CheckSignalsBds(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on NR_BASE_PTC

{// @sic R5-198966 sic@

var integer i;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (not(((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(7,(9,10,11)))))) and pc_BDS_B1C) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) and pc_BDS_B1I)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "BDS signals not reported correctly");

}

}

}

## Correction to f_POS_CheckSignalsGps

| Function name | f_POS_CheckSignalsGps |
| --- | --- |
| Reason for change | In the current TTCN implementation, after changes introduced in R5s210633, test case may fail if UE does not report GNSS measurements for every GPS signal supported. |
| Summary of change | Instead of checking that every GPS supported signal is reported as measurement, the proposed implementation checks that any signal that is reported matches the pics configuration (as it was before R5s210633) |
| TTCN module | EUTRA_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

function f_POS_CheckSignalsGps(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on EUTRA _PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_A_GPS_L1C_A_Reported := false;

var boolean v_A_GPS_L1C_Reported := false;

var boolean v_A_GPS_L2C_Reported := false;

var boolean v_A_GPS_L5_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_A_GPS_L1C_A_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_A_GPS_L1C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_A_GPS_L2C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

v_A_GPS_L5_Reported := true;

}

if (not((v_A_GPS_L1C_A_Reported == pc_A_GPS_L1C_A) and (v_A_GPS_L1C_Reported == pc_A_GPS_L1C) and (v_A_GPS_L2C_Reported == pc_A_GPS_L2C) and (v_A_GPS_L5_Reported == pc_A_GPS_L5)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "A-GPS signals not reported correctly");

}

}

}

After

function f_POS_CheckSignalsGps(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on EUTRA_PTC

{// @sic R5-198966 sic@

var integer i;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (not(((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) and pc_A_GPS_L2C) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) and pc_A_GPS_L5) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) and pc_A_GPS_L1C_A))) //@sic R5s201460 sic@

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "A-GPS signals not reported correctly");

}

}

}

## Correction to f_POS_CheckSignalsGlonass

| Function name | f_POS_CheckSignalsGlonass |
| --- | --- |
| Reason for change | In the current TTCN implementation, after changes introduced in R5s210633, test case may fail if UE does not report GNSS measurements for every GLONASS signal supported. |
| Summary of change | Instead of checking that every GLONASS supported signal is reported as measurement, the proposed implementation checks that any signal that is reported matches the pics configuration (as it was before R5s210633) |
| TTCN module | EUTRA_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

function f_POS_CheckSignalsGlonass(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on EUTRA_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_GLONASS_G1_Reported := false;

var boolean v_GLONASS_G2_Reported := false;

var boolean v_GLONASS_G3_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_GLONASS_G1_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_GLONASS_G2_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_GLONASS_G3_Reported := true;

}

if (not((v_GLONASS_G1_Reported == pc_GLONASS_G1) and (v_GLONASS_G2_Reported == pc_GLONASS_G2) and (v_GLONASS_G3_Reported == pc_GLONASS_G3)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "GLONASS signals not reported correctly");

}

}

}

After

function f_POS_CheckSignalsGlonass(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on EUTRA_PTC

{// @sic R5-198966 sic@

var integer i;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (not(((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) and pc_GLONASS_G2) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) and pc_GLONASS_G3) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) and pc_GLONASS_G1)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "GLONASS signals not reported correctly");

}

}

}

## Correction to f_POS_CheckSignalsGalileo

| Function name | f_POS_CheckSignalsGalileo |
| --- | --- |
| Reason for change | In the current TTCN implementation, after changes introduced in R5s210633, test case may fail if UE does not report GNSS measurements for every Galileo signal supported. |
| Summary of change | Instead of checking that every Galileo supported signal is reported as measurement, the proposed implementation checks that any signal that is reported matches the pics configuration (as it was before R5s210633) |
| TTCN module | EUTRA_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

function f_POS_CheckSignalsGalileo(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on EUTRA_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_GALILEO_E1_Reported := false;

var boolean v_GALILEO_E5a_Reported := false;

var boolean v_GALILEO_E5b_Reported := false;

var boolean v_GALILEO_E6_Reported := false;

var boolean v_GALILEO_E5aE5b_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_GALILEO_E1_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_GALILEO_E5a_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_GALILEO_E5b_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

v_GALILEO_E6_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(4))) {

v_GALILEO_E5aE5b_Reported := true;

}

if (not((v_GALILEO_E1_Reported == pc_GALILEO_E1) and (v_GALILEO_E5a_Reported == pc_GALILEO_E5a) and (v_GALILEO_E5b_Reported == pc_GALILEO_E5b) and

(v_GALILEO_E6_Reported == pc_GALILEO_E6) and (v_GALILEO_E5aE5b_Reported == pc_GALILEO_E5aE5b)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "GLONASS signals not reported correctly");

}

}

}

After

function f_POS_CheckSignalsGalileo(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on EUTRA_PTC

{// @sic R5-198966 sic@

var integer i;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (not(((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) and pc_GALILEO_E5a) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) and pc_GALILEO_E5b) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) and pc_GALILEO_E6) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(4))) and pc_GALILEO_E5aE5b) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) and pc_GALILEO_E1)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Galileo signals not reported correctly");

}

}

}

## Correction to f_POS_CheckSignalsQzss

| Function name | f_POS_CheckSignalsQzss |
| --- | --- |
| Reason for change | In the current TTCN implementation, after changes introduced in R5s210633, test case may fail if UE does not report GNSS measurements for every QZSS signal supported. |
| Summary of change | Instead of checking that every QZSS supported signal is reported as measurement, the proposed implementation checks that any signal that is reported matches the pics configuration (as it was before R5s210633) |
| TTCN module | EUTRA_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

function f_POS_CheckSignalsQzss(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on EUTRA_PTC

{// @sic R5-198966, R5s210633 sic@

var integer i;

var boolean v_QZSS_QZS_L1_Reported := false;

var boolean v_QZSS_QZS_L1C_Reported := false;

var boolean v_QZSS_QZS_L2C_Reported := false;

var boolean v_QZSS_QZS_L5_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_QZSS_QZS_L1_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) {

v_QZSS_QZS_L1C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) {

v_QZSS_QZS_L2C_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) {

v_QZSS_QZS_L5_Reported := true;

}

if (not ((v_QZSS_QZS_L1_Reported == pc_QZSS_QZS_L1) and (v_QZSS_QZS_L1C_Reported == pc_QZSS_QZS_L1C) and

(v_QZSS_QZS_L2C_Reported == pc_QZSS_QZS_L2C) and (v_QZSS_QZS_L5_Reported == pc_QZSS_QZS_L5)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "QZSS signals not reported correctly");

}

}

}

}

After

function f_POS_CheckSignalsQzss(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on EUTRA_PTC

{// @sic R5-198966 sic@

var integer i;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (not(((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(1))) and pc_QZSS_QZS_L1C) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(2))) and pc_QZSS_QZS_L2C) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(3))) and pc_QZSS_QZS_L5) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) and pc_QZSS_QZS_L1)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "QZSS signals not reported correctly");

}

}

}

## Correction to f_POS_CheckSignalsBds

| Function name | f_POS_CheckSignalsBds |
| --- | --- |
| Reason for change | In the current TTCN implementation, after changes introduced in R5s210633, test case may fail if UE does not report GNSS measurements for every Beidou signal supported. |
| Summary of change | Instead of checking that every Beidou supported signal is reported as measurement, the proposed implementation checks that any signal that is reported matches the pics configuration (as it was before R5s210633) |
| TTCN module | EUTRA_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

function f_POS_CheckSignalsBds(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on EUTRA_PTC

{// @sic R5s210633 sic@

var integer i;

var boolean v_BDS_B1I_Reported := false;

var boolean v_BDS_B1C_Reported := false;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) {

v_BDS_B1I_Reported := true;

}

if (match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(7,(9,10,11)))) {

v_BDS_B1C_Reported := true;

}

}

if (not ((v_BDS_B1I_Reported == pc_BDS_B1I) and (v_BDS_B1C_Reported == pc_BDS_B1C)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "BDS signals not reported correctly");

}

}

After

function f_POS_CheckSignalsBds(GNSS_SgnMeasList p_GNSS_SgnMeasList) runs on EUTRA_PTC

{// @sic R5-198966 sic@

var integer i;

for (i:=0; i<lengthof(p_GNSS_SgnMeasList); i:=i+1) {

if (not(((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(7,(9,10,11)))))) and pc_BDS_B1C) or

((match(p_GNSS_SgnMeasList[i], cr_GNSS_SgnMeasElement(0))) and pc_BDS_B1I)))

{

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "BDS signals not reported correctly");

}

}

}