3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s211323

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2077 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC multilayer test case 11.4.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC multilayer test case 11.4.9 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC multilayer test case 11.4.9 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

3	Branches executed	6

4	Execution Log Files	7

4.1	HiSilicon Balong 5000	7

5	References	7

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.4.9 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.4.9.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk37

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_11_4_9_NR5GC_IMS2 |
| --- | --- |
| Reason for change | For 11.4.9, the emergency call will be established without emergency registration |
| Summary of change | Replace f_IMS_EmergencyCallAndRelease() with f_IMS_EmergencyCallAndReleaseWithoutRegistration() |
| TTCN module | Emergency_NR5GC_IMS |
| MCC160 Comment |  |

Before Change

function f_TC_11_4_9_NR5GC_IMS2()runs on IMS_PTC { // Emergency PDU session f_IMS_PTC_Init(PDN_2); f_IMS_EmergencyCallAndRelease(); }

After Change

function f_TC_11_4_9_NR5GC_IMS2()runs on IMS_PTC { // Emergency PDU session f_IMS_PTC_Init(PDN_2); f_IMS_EmergencyCallAndReleaseWithoutRegistration(); }

## Change 2

| Function name | f_TC_11_4_9_NR5GC |
| --- | --- |
| Reason for change | Following implenmentation of agreed prose CR R5-214620, thee new postamble state EMERGENCY_RELEASE_IDLE should be used for this test case. |
| Summary of change | Use postamble state STATE_EMERGENCY_RELEASE_IDLE |
| TTCN module | Emergency_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_11_4_9_NR5GC() runs on NR5GC_PTC { //5GMM-DEREGISTERED.LIMITED-SERVICE No suitable cells in tracking area / Emergency call establishment and release f_NR5GC_Init(NR_1); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); //The UE is in test state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.2 on NR Cell 1 //In the list of tracking areas provided by the AMF (IE 'TAI list') contains only the TAI of NR Cell 1. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); fl_TC_11_4_9_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

After Change

function f_TC_11_4_9_NR5GC() runs on NR5GC_PTC { //5GMM-DEREGISTERED.LIMITED-SERVICE No suitable cells in tracking area / Emergency call establishment and release f_NR5GC_Init(NR_1); //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); //The UE is in test state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A.2 on NR Cell 1 //In the list of tracking areas provided by the AMF (IE 'TAI list') contains only the TAI of NR Cell 1. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); fl_TC_11_4_9_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_EMERGENCY_RELEASE_IDLE); }

## Change 3

| Function name | fl_TC_11_4_9_NR5GC_TestBody |
| --- | --- |
| Reason for change | Following agreed prose CR R5-216202, TTCN execution should wait for 10s at step 14 |
| Summary of change | Change the time at step 14 to 10s |
| TTCN module | Emergency_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_TC_11_4_9_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; ……. //@siclog "Step 13" siclog@ //SS releases the RRC connection f_NR_RRCRelease(nr_Cell1); //@siclog "Step 13A" siclog@//@sic R5-216202 sic@ //Make the UE attempt an IMS none-emergency call. (NOTE 2) f_UT_RequestIMSCall(UT); //@siclog "Step 14" siclog@ //Check: Does the UE transmit an RRCSetupRequest message in the next 30second? //NOTE: This is an arbitrary value to wait for catching not allowed UE behaviour. f_NR_CheckNoRRCSetupReq (nr_Cell1, 30.0); }

After Change

function fl_TC_11_4_9_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; …….. //@siclog "Step 13" siclog@ //SS releases the RRC connection f_NR_RRCRelease(nr_Cell1); //@siclog "Step 13A" siclog@//@sic R5-216202 sic@ //Make the UE attempt an IMS none-emergency call. (NOTE 2) f_UT_RequestIMSCall(UT); //@siclog "Step 14" siclog@ //Check: Does the UE transmit an RRCSetupRequest message in the next 10second? //NOTE: This is an arbitrary value to wait for catching not allowed UE behaviour. f_NR_CheckNoRRCSetupReq (nr_Cell1, 10.0); }

# Branches executed

The test case was executed on NR band n41 uing NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_11_4_9_LOG.html

- PICS/PIXIT parameter file:

TC_11_4_9_PIXIT.xml

# References

| [1] | R5s211324 : NR5GC FR1 : Supporting information for addition of NR5GC multilayer test case 11.4.9 |
| --- | --- |