3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230090

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2927 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC MDT Test Case 8.1.6.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-01-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Current implementation for SIB5 is not aligned with 38.523-1 Table 8.1.6.2.4.3.3-2: SIB5 of NR Cell 1. carrierFreqListEUTRA should only have 1 entry, but the other entries of the default SIB5 carrying multiple entries are left unchanged. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set SIB5 carrierFreqListEUTRA to reflect only the first entry. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function f_TC_8_1_6_2_4_NR5GC() |
| --- | --- |
| Reason for change | Current implementation for SIB5 is not aligned with 38.523-1 Table 8.1.6.2.4.3.3-2: SIB5 of NR Cell 1. carrierFreqListEUTRA should only have 1 entry, but the other entries of the default SIB5 carrying multiple entries are left unchanged. |
| Summary of change | Set SIB5 carrierFreqListEUTRA to reflect only the first entry. |
| TTCN module | RRC_MDT_InterRAT_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_6_2_4_NR5GC() runs on NR5GC_PTC { // Inter-RAT MDT / Connection Establishment Failure / Logging and reporting / Reporting of E-UTRA Inter-RAT handover var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; f_NR5GC_IRAT_Init(NR_6); v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); //Set Table 8.1.6.2.4.3.3-1: SIB2 of NR Cell 1 f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell1, 10); //Table 8.1.6.2.4.3.3-2: SIB5 of NR Cell 1 f_NR_CellInfo_SetSIB5_CarrierFreq(nr_Cell1, 0, v_CoOrd_SysInfo.Eutra[0].Arfcn); f_NR5GC_CellInfo_SetSIB5_CellReselectionPriority(nr_Cell1, 0, 5); f_NR5GC_CellInfo_SetSIB5_ThreshXHigh(nr_Cell1, 0, 10); f_NR5GC_CellInfo_SetSIB5_ThreshXLow(nr_Cell1, 0, 20); f_NR5GC_CellInfo_SetSIB5_TReselectionEUTRA(nr_Cell1, 7); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -78, -78); //Maximum power level for FR2 FFS //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower (nr_Cell1, -78, -78); // Power levels on FR2 are FFS f_NR_TestBody_Set(true); f_TC_8_1_6_2_4_TestBody(v_CoOrd_SysInfo); // @sic R5s221162 sic@ f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Postamble")); }// End of f_TC_8_1_6_2_4_NR5GC()

After Change:

function f_TC_8_1_6_2_4_NR5GC() runs on NR5GC_PTC { // Inter-RAT MDT / Connection Establishment Failure / Logging and reporting / Reporting of E-UTRA Inter-RAT handover var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var CarrierFreqListEUTRA v_CarrierFreqListEUTRA; f_NR5GC_IRAT_Init(NR_6); v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); //Set Table 8.1.6.2.4.3.3-1: SIB2 of NR Cell 1 f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell1, 10); //Table 8.1.6.2.4.3.3-2: SIB5 of NR Cell 1 //f_NR_CellInfo_SetSIB5_CarrierFreq(nr_Cell1, 0, v_CoOrd_SysInfo.Eutra[0].Arfcn); // REMOVED v_CarrierFreqListEUTRA := f_NR5GC_CellInfo_GetSIB5_CarrierFreqListEUTRA(nr_Cell1); f_NR5GC_CellInfo_SetSIB5_CarrierFreqListEUTRA(nr_Cell1, {v_CarrierFreqListEUTRA[0]}); // only 1 entry f_NR5GC_CellInfo_SetSIB5_CellReselectionPriority(nr_Cell1, 0, 5); f_NR5GC_CellInfo_SetSIB5_ThreshXHigh(nr_Cell1, 0, 10); f_NR5GC_CellInfo_SetSIB5_ThreshXLow(nr_Cell1, 0, 20); f_NR5GC_CellInfo_SetSIB5_TReselectionEUTRA(nr_Cell1, 7); f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, -78, -78); //Maximum power level for FR2 FFS //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A); f_NR_SetCellPower (nr_Cell1, -78, -78); // Power levels on FR2 are FFS f_NR_TestBody_Set(true); f_TC_8_1_6_2_4_TestBody(v_CoOrd_SysInfo); // @sic R5s221162 sic@ f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Postamble")); }// End of f_TC_8_1_6_2_4_NR5GC()