3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230144

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2960 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC idle mode test case 6.2.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test case is failing when running this test case with IMS mode. The problem is at step 1 the UE is switched on from NR5GC PTC (function f_NR5GC_SwitchOnAndResetIMS() which also resets IMS PTC) but then (if pc_Available_PLMNs_AcT_Ind is set true) the device actually attaches on EUTRA. But the IMS default handler remembers on which port it has received the reset trigger and takes this to be the active RAT. So when REGISTER is received it sends a trigger to NR5GC PTC to stop the 10s timer and get some RAT-specific information needed to process the REGISTER, but because the registration is happening on EUTRA this is not handled, so IMS PTC never cgenerates the 401 Unauthorised response. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Code change done in such a way that IMS got reset at ENDC side. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_2_1_4_NR5GC_EUTRA() |
| --- | --- |
| Reason for change | The test case is failing when running this test case with IMS mode. The problem is at step 1 the UE is switched on from NR5GC PTC (function f_NR5GC_SwitchOnAndResetIMS() which also resets IMS PTC) but then (if pc_Available_PLMNs_AcT_Ind is set true) the device actually attaches on EUTRA. But the IMS default handler remembers on which port it has received the reset trigger and takes this to be the active RAT. So when REGISTER is received it sends a trigger to NR5GC PTC to stop the 10s timer and get some RAT-specific information needed to process the REGISTER, but because the registration is happening on EUTRA this is not handled, so IMS PTC never cgenerates the 401 Unauthorised response. |
| Summary of change | Code change done in such a way that IMS got reset at ENDC side. |
| TTCN module | Idle_PLMNSelection_NR5GC_EUTRA.TTCN |
| MCC160 Comment |  |

Before Change

function f_TC_6_2_1_4_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT PLMN Selection / Selection of correct RAT from the OPLMN list / Manual mode var NAS_MSG_Indication_Type v_NAS_Ind; var charstring v_PLMN; f_EUTRA38_IRAT_Init(c1); .............................. // @siclog "Step 2a1" siclog@ // IF pc_Available_PLMNs_AcT_Ind[29](Support of Access Technology Indication in available PLMNs list) THEN PLMN2 (E-UTRAN) is selected if(pc_Available_PLMNs_AcT_Ind){ f_EUTRA_NR_WaitForCoOrd_Trigger(NR); f_UT_ManualPLMN_Select(UT, f_Asn2Nas_PlmnId(valueof(cs_HPLMN_002_11)), tsc_ManualPLMN_EUTRA); // @sic R5s200772 sic@ // @siclog "Step 2a2" siclog@ // Does the test result of generic test procedure in TS 38.508-1[4] table 4.5.2.2-5 is performed on E-UTRA Cell 1 v_NAS_Ind := f_EUTRA_InitialRegistration_Step1_4(eutra_Cell1, PREAMBLE, NORMAL); // do extra checks now on Attach Req message if (not ispresent (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueAddSecurityCap should be present for ENDC"); } select (f_EUTRA38_NR_Config_Get()) { case (EN_DC) { if (not match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_EUTRA_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for ENDC"); } } case else { if (not match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for NR"); } } } f_EUTRA38_MultiPDN_InitialRegistration_Step5_18b1 (eutra_Cell1, TESTMode_OFF, v_NAS_Ind); // @sic R5-204399 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.2.1.4 Step 2a2"); //@siclog "Step 2a3" siclog@ .............................. }

After Change

function f_TC_6_2_1_4_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT PLMN Selection / Selection of correct RAT from the OPLMN list / Manual mode var NAS_MSG_Indication_Type v_NAS_Ind; var charstring v_PLMN; f_EUTRA38_IRAT_Init(c1); .............................. // @siclog "Step 2a1" siclog@ // IF pc_Available_PLMNs_AcT_Ind[29](Support of Access Technology Indication in available PLMNs list) THEN PLMN2 (E-UTRAN) is selected if(pc_Available_PLMNs_AcT_Ind){ f_EUTRA_NR_WaitForCoOrd_Trigger(NR); // Switch on UE f_EUTRA_SwitchOnAndResetIMS(); //KS#23xxxx f_UT_ManualPLMN_Select(UT, f_Asn2Nas_PlmnId(valueof(cs_HPLMN_002_11)), tsc_ManualPLMN_EUTRA); // @sic R5s200772 sic@ // @siclog "Step 2a2" siclog@ // Does the test result of generic test procedure in TS 38.508-1[4] table 4.5.2.2-5 is performed on E-UTRA Cell 1 v_NAS_Ind := f_EUTRA_InitialRegistration_Step1_4(eutra_Cell1, PREAMBLE, NORMAL); // do extra checks now on Attach Req message if (not ispresent (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueAddSecurityCap should be present for ENDC"); } select (f_EUTRA38_NR_Config_Get()) { case (EN_DC) { if (not match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_EUTRA_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for ENDC"); } } case else { if (not match (v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap(omit))){ f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for NR"); } } } f_EUTRA38_MultiPDN_InitialRegistration_Step5_18b1 (eutra_Cell1, TESTMode_OFF, v_NAS_Ind); // @sic R5-204399 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.2.1.4 Step 2a2"); //@siclog "Step 2a3" siclog@ .............................. }