3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230296

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3031 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the NR testcase 7.1.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the Current TTCN Implementation, at Step 17A, the reconfiguration message is transmitted ; at this point UE is expected to send Reconfiguration complete and RLC ACK for reconfiguration message based on Rach procedure ( Msg 3 ). Before the rach procedure gets completed, there is a possibility that NW RLC can retransmit the reconfiguration message due to the expiry of tPollRetransmit timer. In the current TTCN implementation, Enable SR reception indication is done in the function f_TC_7_1_1_1_2_NR_TestBody2() as an immediate first step, after the reconfiguration procedure completion. As a result of reconfiguration getting retransmitted, UE can send Scheduling Request to send RLC ACK, and it results in the test case failure due to unexpected reception of the Scheduling Request Indication SR reception indication is mainly required for Step 20, so it is proposed to move the Enable SR reception indication just before Step 19. It also helps with the stability improvement of this test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SR reception indication is mainly required for Step 20, so it is proposed to move the Enable SR reception indication just before Step 19 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case due to stability Issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.2.ENDC , 7.1.1.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This change is needed for both NR5GC and ENDC variants |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the Function f_TC_7_1_1_1_2_NR_TestBody2	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_TC_7_1_1_1_2_NR_TestBody2

| Function name | f_TC_7_1_1_1_2_NR_TestBody2 |
| --- | --- |
| Reason for change | In the Current TTCN Implementation, at Step 17A, the reconfiguration message is transmitted ; at this point UE is expected to send Reconfiguration complete and RLC ACK for reconfiguration message based on Rach procedure ( Msg 3 ). Before the rach procedure gets completed, there is a possibility that NW RLC can retransmit the reconfiguration message due to the expiry of tPollRetransmit timer. In the current TTCN implementation, Enable SR reception indication is done in the function f_TC_7_1_1_1_2_NR_TestBody2() as an immediate first step, after the reconfiguration procedure completion. As a result of reconfiguration getting retransmitted, UE can send Scheduling Request to send RLC ACK, and it results in the test case failure due to unexpected reception of the Scheduling Request Indication SR reception indication is mainly required for Step 20, so it is proposed to move the Enable SR reception indication just before Step 19. It also helps with the stability improvement of this test case. |
| Summary of change | SR reception indication is mainly required for Step 20, so it is proposed to move the Enable SR reception indication just before Step 19 |
| TTCN module | ttcn/develop/NR_TC_Common/7_1_1/MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_1_2_NR_TestBody2( DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var SubFrameTiming_Type v_TimingDL; var template (present) integer v_PRACH_PreamblesGroupB := (28,29,30,31,32,33,34,35,36,37,38,39,40,41); //@sic R5s200823 sic@ var SubFrameTiming_Type v_TimingDRB; var SubFrameTiming_Type v_TimingPRACH; var integer v_DurationPRACH; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var boolean v_DRB_Flag := false; var boolean v_PRACH_Flag := false; var NR_SYSTEM_IND v_Received_NR_SysInd; timer t_Watchdog := 5.0; // Enable SR reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); // Enable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_Now, cs_NR_UplinkTimeAlignment_Stop); //@sic R5s20046 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,100); // Timing of DL f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_UplinkTimeAlignment_Start); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 600); // @siclog "Step 18" siclog@ // Configure SS transmission of Timing advance //@sic R5s20046 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,400); // Timing of DL f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_MacCommandTrigger_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_TimingAdvance_CE(-,'011111'B))); //@sic R5s210694 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_UplinkTimeAlignment_Stop); // default C-RNTI based RACH procedure suitable for steps 21 to 24. //@sic R5s210694 sic@ // @siclog "Steps 19" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 300); //@sic R5s20046 sic@ // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits >208 bits t_Watchdog.start; DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), {crs_NR_PDCP_SDU_36B} )); << SKIPPED CODE >> }

After Change:

function f_TC_7_1_1_1_2_NR_TestBody2( DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var SubFrameTiming_Type v_TimingDL; var template (present) integer v_PRACH_PreamblesGroupB := (28,29,30,31,32,33,34,35,36,37,38,39,40,41); //@sic R5s200823 sic@ var SubFrameTiming_Type v_TimingDRB; var SubFrameTiming_Type v_TimingPRACH; var integer v_DurationPRACH; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var boolean v_DRB_Flag := false; var boolean v_PRACH_Flag := false; var NR_SYSTEM_IND v_Received_NR_SysInd; timer t_Watchdog := 5.0; // Enable SR reception indication //f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); //WA#WI=1078705 // Enable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable)); // Stop UL Grant configuration f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_Now, cs_NR_UplinkTimeAlignment_Stop); //@sic R5s20046 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,100); // Timing of DL f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_UplinkTimeAlignment_Start); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 600); // @siclog "Step 18" siclog@ // Configure SS transmission of Timing advance //@sic R5s20046 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,400); // Timing of DL f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_MacCommandTrigger_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_TimingAdvance_CE(-,'011111'B))); //@sic R5s210694 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_UplinkTimeAlignment_Stop); // default C-RNTI based RACH procedure suitable for steps 21 to 24. //@sic R5s210694 sic@ f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); //WA#WI=1078705 // @siclog "Steps 19" siclog@ v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 300); //@sic R5s20046 sic@ // PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header =44B=352 bits >208 bits t_Watchdog.start; DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_TimingDL), {crs_NR_PDCP_SDU_36B} )); << SKIPPED CODE >> }