3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230300

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3034 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-03-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During actual testing, the explicitly configured NPRB = 7 and MCS = 2 with SLIV 53 and with the mcs_Table = omit enforcing normal 64QAM Table as per the MCC160 comments on TTCN CR R5s230088 is resulting in RIV (Frequency Allocation) of 1638 with TB Size = 288 bits with QPSK modulation for Type A PDSCH mappingType as per the calculation for Modulation order and target code rate determination mentioned in TS 38.214 section 5.1.3.1-1 (64QAM Table) MCS index table 1 for PDSCH from the SS Stack rather than the expected 352 bits Hence, this TB Size of 288 bits is still not enough to transmit the Data to the UE. It is recommended to increase the MCS = 3 to get the TB Size = 368 bits which is enough to transmit the DL Data of 352 bits |  |  |  |  |  |  |  |  |
| Summary of change: |  | Increased the MCS = 3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.1.ENDC, 7.1.1.6.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the Function f_TC_7_1_1_6_1_NR_TestBody1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the Function f_TC_7_1_1_6_1_NR_TestBody1

| Function name | f_TC_7_1_1_6_1_NR_TestBody1 |
| --- | --- |
| Reason for change | During actual testing, the explicitly configured NPRB = 7 and MCS = 2 with SLIV 53 and with the mcs_Table = omit enforcing normal 64QAM Table as per the MCC160 comments on TTCN CR R5s230088 is resulting in RIV (Frequency Allocation) of 1638 with TB Size = 288 bits with QPSK modulation for Type A PDSCH mappingType as per the calculation for Modulation order and target code rate determination mentioned in TS 38.214 section 5.1.3.1-1 (64QAM Table) MCS index table 1 for PDSCH from the SS Stack rather than the expected 352 bits Hence, this TB Size of 288 bits is still not enough to transmit the Data to the UE. It is recommended to increase the MCS = 3 to get the TB Size = 368 bits which is enough to transmit the DL Data of 352 bits |
| Summary of change | Increased the MCS = 3 |
| TTCN module | MAC_SPS_TC_Common_NR |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { <<SKIPPED CODE>> timer t_Watchdog := 10.0; // Configure for indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // Initialise Nprb and Imcs for TBsize=352 if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_N_PRB := 7; v_I_MCS:= 2;} //38.508 Table 4.6.3-51 Table 4.6.3-51 =1, so 38.523-3 Table B.1.1.2-3 else {v_N_PRB :=11 ; v_I_MCS:= 0}//38.508 Table 4.6.3-51 Table 4.6.3-51 =0, so 38.523-3 Table B.1.1.1-3 <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC return SubFrameTiming_Type { <<SKIPPED CODE>> timer t_Watchdog := 10.0; // Configure for indication of UL HARQ ACK/NACK f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // Initialise Nprb and Imcs for TBsize=352 if ( f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_N_PRB := 7; v_I_MCS:= 3;} //38.508 Table 4.6.3-51 Table 4.6.3-51 =1, so 38.523-3 Table B.1.1.2-3 WA# else {v_N_PRB :=11 ; v_I_MCS:= 0}//38.508 Table 4.6.3-51 Table 4.6.3-51 =0, so 38.523-3 Table B.1.1.1-3 <<SKIPPED CODE>>