3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230361

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3074 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC testcase 7.1.1.6.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation of f_TC_7_1_1_6_1_NR_TestBody1, at step 13, the time slot of ACK for C-RNTI(v_TimingUL_5_CRNTI) does not take care of different SCS, this causes timing mismatches when the HARQ ACK is received at Step 13 leading to TC failure, when the TC is run in FDD mode or in FR2 TDD mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | At step 13, calculate the the time slot of ACK for C-RNTI (v_TimingUL_5_CRNTI) based on SCS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.6.1.NR5GC, 7.1.1.6.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_6_1_NR_TestBody1() |
| --- | --- |
| Reason for change | In the current TTCN implementation of f_TC_7_1_1_6_1_NR_TestBody1, at step 13, the time slot of ACK for C-RNTI(v_TimingUL_5_CRNTI) does not take care of different SCS, this causes timing mismatches when the HARQ ACK is received at Step 13 leading to TC failure, when the TC is run in FDD mode or in FR2 TDD mode. |
| Summary of change | At step 13, calculate the the time slot of ACK for C-RNTI (v_TimingUL_5_CRNTI) based on SCS. |
| TTCN module | MAC_SPS_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { <SKIPPED CODE>> //@siclog "Step 11-12" siclog@ //C-RNTI grant auto transferred by SS //@sic R5s221197 R5s230088 sic@ v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 + 1); v_TimingUL_5_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingUL_4,const_SPS_Period/2 -1); v_TimingUL_5_CRNTI.Slot.SlotOffset.Numerology1 := 1; // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_MAC_PDUListDL)); //@siclog "Step 14 " siclog@ <SKIPPED CODE>> }//End of f_TC_7_1_1_6_1_NR_TestBody1

After Change:

function f_TC_7_1_1_6_1_NR_TestBody1(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { <SKIPPED CODE>> //@siclog "Step 11-12" siclog@ //C-RNTI grant auto transferred by SS //@sic R5s221197 R5s230088 sic@ select(v_SCS) { case(kHz15) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2); v_TimingUL_5_CRNTI := v_TimingDL_CRNTI; v_TimingUL_5_CRNTI.Symbol := valueof(cs_SymbolTimingInfo_Any); v_TimingUL_5_CRNTI.Subframe.Number := v_TimingDL_CRNTI.Subframe.Number + 7; //FDD K1 =7 } case(kHz30) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 + 1); v_TimingUL_5_CRNTI := v_TimingDL_CRNTI; v_TimingUL_5_CRNTI.Symbol := valueof(cs_SymbolTimingInfo_Any); v_TimingUL_5_CRNTI.Subframe.Number := v_TimingDL_CRNTI.Subframe.Number + 2; //TDD K1 =5 v_TimingUL_5_CRNTI.Slot.SlotOffset.Numerology1 := 1; } case(kHz120) { v_TimingDL_CRNTI := f_SubFrameTiming_AddMilliSeconds(v_TimingDL,const_SPS_Period/2 + 1); v_TimingUL_5_CRNTI := v_TimingDL_CRNTI; v_TimingUL_5_CRNTI.Symbol := valueof(cs_SymbolTimingInfo_Any); v_TimingUL_5_CRNTI.Subframe.Number := v_TimingDL_CRNTI.Subframe.Number + 1; //FR2 K1 =8 } case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } // PDCP SDU =36B +3B PDCP Header+ 1B RLC UM Header + 2B MAC Header =42B=336 bits (an allowed TB size as per 38.523-3 Annex B) v_EncodedRlcPdu1 := f_NR_RLC_UMD_FullPDU_Encvalue(f_NR_PDCP_PDU_Encvalue(f_NR_ReturnDefaultPDCP_PDU (3, crs_NR_PDCP_SDU_36B, 1)));//@sic R5-227518 sic@ v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) }; v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) }; DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR( v_TimingDL_CRNTI), v_MAC_PDUListDL)); //@siclog "Step 14 " siclog@ <SKIPPED CODE>> }//End of f_TC_7_1_1_6_1_NR_TestBody1