3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230091

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2928 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 6.2.3.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang LinkTester, MEDIATEK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-1-6 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to 38.523-1 Table 6.2.3.8.3.3-2, It should be configured threshX-High-r15 to10 and threshX-Low-r15 to 20 in sib24, instead of thresh-HighQ-r15 and thresh-LowQ-r15. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Deleted f_EUTRA_CellInfo_SetSIB24_threshX_Q_r15, used f_EUTRA_CellInfo_SetSIB24_ThreshX_High_r15 and. f_EUTRA_CellInfo_SetSIB24_ThreshX_Low_r15. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.8.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to the Function f_TC_6_2_3_8_NR5GC_EUTRA	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk49 related to the title of this CR.

Contact: 	Pengchong Shen
	shenpengchong@cictmobile.com

# Corrections required

## Correction to the Function f_TC_6_2_3_8_NR5GC_EUTRA

| Function name | f_TC_6_2_3_8_NR5GC_EUTRA |
| --- | --- |
| Reason for change | According to 38.523-1 Table 6.2.3.8.3.3-2, It should be configured threshX-High-r15 to10 and threshX-Low-r15 to 20 in sib24, instead of thresh-HighQ-r15 and thresh-LowQ-r15. |
| Summary of change | Deleted f_EUTRA_CellInfo_SetSIB24_threshX_Q_r15, used f_EUTRA_CellInfo_SetSIB24_ThreshX_High_r15 and. f_EUTRA_CellInfo_SetSIB24_ThreshX_Low_r15. |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_6_2_3_8_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT cell reselection / From E-UTRA_Idle to NR RRC_IDLE according to RAT priority provided by dedicated signalling (RRConnRelease) //@sic R5s220708 sic@ var template (value) EUTRA_CellInfo_Type v_CellInfo; var template (value) SystemInformationBlockType24_r15 v_SIB24; var template (omit) CarrierFreqListNR_r15 v_CarrierFreqListNR_r15 := omit; var ReselectionThreshold v_ThreshServingLow_Cell1 := 3; var ReselectionThreshold v_S_NonIntraSearch_Cell1 := 3; f_EUTRA38_IRAT_Init(c31); // System information combination 31 as defined in TS 36.508-1 clause 4.4.3.1 is used in E-UTRA Cell f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // sends Freq. Info to the other RAT // Table 6.2.3.8.3.3-1: SystemInformationBlockType3 of EUTRA Cell 1 f_EUTRA_CellInfo_SetSysInfo_ThreshServingLow (eutra_Cell1, v_ThreshServingLow_Cell1); f_EUTRA_CellInfo_SetSysInfo_S_NonIntraSearch(eutra_Cell1, v_S_NonIntraSearch_Cell1); // Table 6.2.3.8.3.3-2: SystemInformationBlockType24 of EUTRA Cell 1 v_CellInfo := f_EUTRA_CellInfo_Get(eutra_Cell1); v_CarrierFreqListNR_r15[0] := v_CellInfo.Sysinfo.BCCH_Info.SIs[2].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib24_v1530.carrierFreqListNR_r15[0]; v_SIB24 := v_CellInfo.Sysinfo.BCCH_Info.SIs[2].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib24_v1530; v_SIB24.carrierFreqListNR_r15 := v_CarrierFreqListNR_r15; f_EUTRA_CellInfo_SetSIB24(eutra_Cell1,v_SIB24); f_EUTRA_CellInfo_SetSIB24_threshX_Q_r15(eutra_Cell1, 10, 20); f_EUTRA_CellInfo_SetSIB24_t_ReselectionNR_r15(eutra_Cell1, 7); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Preamble: finishes with EUTRA cell on (NR cell off) and co-ord msg sent to say UE has completed registration on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); // Wait for UE to register on NR first f_EUTRA38_NR5GC_Preamble (eutra_Cell1, TESTMode_OFF, RRC_IDLE); // Go on to EUTRA RRC_Idle while NR is already off f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble")); // inform NR side about registration on both sides f_EUTRA_TestBody_Set(true); fl_TC_6_2_3_8_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); // take down the cell now, UE finished postamble on NR side f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); f_EUTRA_ReleaseAllCells(); }

After Change:

function f_TC_6_2_3_8_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT cell reselection / From E-UTRA_Idle to NR RRC_IDLE according to RAT priority provided by dedicated signalling (RRConnRelease) //@sic R5s220708 sic@ var template (value) EUTRA_CellInfo_Type v_CellInfo; var template (value) SystemInformationBlockType24_r15 v_SIB24; var template (omit) CarrierFreqListNR_r15 v_CarrierFreqListNR_r15 := omit; var ReselectionThreshold v_ThreshServingLow_Cell1 := 3; var ReselectionThreshold v_S_NonIntraSearch_Cell1 := 3; f_EUTRA38_IRAT_Init(c31); // System information combination 31 as defined in TS 36.508-1 clause 4.4.3.1 is used in E-UTRA Cell f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); // sends Freq. Info to the other RAT // Table 6.2.3.8.3.3-1: SystemInformationBlockType3 of EUTRA Cell 1 f_EUTRA_CellInfo_SetSysInfo_ThreshServingLow (eutra_Cell1, v_ThreshServingLow_Cell1); f_EUTRA_CellInfo_SetSysInfo_S_NonIntraSearch(eutra_Cell1, v_S_NonIntraSearch_Cell1); // Table 6.2.3.8.3.3-2: SystemInformationBlockType24 of EUTRA Cell 1 v_CellInfo := f_EUTRA_CellInfo_Get(eutra_Cell1); v_CarrierFreqListNR_r15[0] := v_CellInfo.Sysinfo.BCCH_Info.SIs[2].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib24_v1530.carrierFreqListNR_r15[0]; v_SIB24 := v_CellInfo.Sysinfo.BCCH_Info.SIs[2].message_.c1.systemInformation.criticalExtensions.systemInformation_r8.sib_TypeAndInfo[0].sib24_v1530; v_SIB24.carrierFreqListNR_r15 := v_CarrierFreqListNR_r15; f_EUTRA_CellInfo_SetSIB24(eutra_Cell1,v_SIB24); f_EUTRA_CellInfo_SetSIB24_ThreshX_High_r15(eutra_Cell1, 10); f_EUTRA_CellInfo_SetSIB24_ThreshX_Low_r15(eutra_Cell1, 20); f_EUTRA_CellInfo_SetSIB24_t_ReselectionNR_r15(eutra_Cell1, 7); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); //Preamble: finishes with EUTRA cell on (NR cell off) and co-ord msg sent to say UE has completed registration on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); // Wait for UE to register on NR first f_EUTRA38_NR5GC_Preamble (eutra_Cell1, TESTMode_OFF, RRC_IDLE); // Go on to EUTRA RRC_Idle while NR is already off f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble")); // inform NR side about registration on both sides f_EUTRA_TestBody_Set(true); fl_TC_6_2_3_8_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); // take down the cell now, UE finished postamble on NR side f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble"); f_EUTRA_ReleaseAllCells(); }