3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230214

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2990 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 7.1.2.3.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang LinkTester Technology |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If the RLC SDU length is 984 bits, RedCap UE will transmit 204 AMD PDUs with the poll bit set only in the last (204th) one because of (984 bits x 204PDUs) / 8 = 25092 > 25 KB at step 1 in Table 7.1.2.3.6.3.2-4. The length of RLC SDU length shall be 976 bits for both RedCap and non-Redcap UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update RLC SDU length. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Prose CR required |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to f_TC_7_1_2_3_6_NR_TestBody	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D22wk49 related to the title of this CR.

Contact: 	Pengchong Shen
	shenpengchong@cictmobile.com

# Corrections required

## Correction to f_TC_7_1_2_3_6_NR_TestBody

| Function name | f_TC_7_1_2_3_6_NR_TestBody |
| --- | --- |
| Reason for change | If the RLC SDU length is 984 bits, RedCap UE will transmit 204 AMD PDUs with the poll bit set only in the last (204th) one because of (984 bits x 204PDUs) / 8 = 25092 > 25 KB at step 1 in Table 7.1.2.3.6.3.2-4. The length of RLC SDU length shall be 976 bits for both RedCap and non-Redcap UE. |
| Summary of change | Update RLC SDU length. |
| TTCN module | RLC_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_2_3_6_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var integer v_RLC_SDUsize := 122; var B1_Type v_Poll; var integer i; var integer v_SlotsNumber; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DataPerSlotList; var template (value) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU; var integer v_Duration; var SubFrameTiming_Type v_SubFrameTimingTa; var SubFrameTiming_Type v_SubFrameTimingTb; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var integer v_SequenceNumber := 0; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RLC_Status_NackSN18Bit_Type v_NR_NackSN18Bit_1; var template (value) NR_RLC_Status_NackSN18Bit_Type v_NR_NackSN18Bit_2; var template (value) NR_RLC_Status_NackSN18Bit_Type v_NR_NackSN18Bit_3; var template (value) NR_RLC_Status_NackListSN18Bit_Type v_NR_NackSN18Bit_List; var template (value) NR_RLC_Status_NackSN12Bit_Type v_NR_NackSN12Bit_1; var template (value) NR_RLC_Status_NackSN12Bit_Type v_NR_NackSN12Bit_2; var template (value) NR_RLC_Status_NackSN12Bit_Type v_NR_NackSN12Bit_3; var template (value) NR_RLC_Status_NackListSN12Bit_Type v_NR_NackSN12Bit_List; var template (present) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU_Any; var template (present) NR_RLC_PDUList_Type v_NR_RLC_PDUList; var UL_t_PollRetransmit_Type v_T_PollRetransmit; var DL_t_Reassembly_Type v_T_Reassembly; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var SN_FieldLengthAM v_SN_FieldLengthAM := size18; var boolean v_PDCP_LongSN := not pc_supportOfRedCap_r17; // @sic R5-226947 sic@ redcap: short PDCP header and one more data octet timer t_WatchDog; if (pc_supportOfRedCap_r17 == true) { // @sic R5-226947 sic@ v_SN_FieldLengthAM := size12; v_RLC_SDUsize := 123; }; // TBS = RLC PDU data (122 octets) + RLC header (3 octets)+ MAC BSR CE header (2 octets) + MAC PDU subheader (2 octets) = 122 + 3 + 2 + 2 = 129 octets = 1032 bits => TBS = 1032 bits // TBS = RLC PDU data (123 octets) + RLC header (2 octets)+ MAC BSR CE header (2 octets) + MAC PDU subheader (2 octets) = 123 + 2 + 2 + 2 = 129 octets = 1032 bits => TBS = 1032 bits (Redcap) v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(1032, v_NR_UplinkBWP); v_SlotsNumber := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1); // generate 424 RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 424, -, v_PDCP_LongSN); // @sic R5-226947 sic@ // @siclog "Step 1" siclog@ // During the whole test sequence, the SS should not allocate UL grants unless when explicitly stated so in the procedure. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); ………… }

After Change:

function f_TC_7_1_2_3_6_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var integer v_RLC_SDUsize := 122; var B1_Type v_Poll; var integer i; var integer v_SlotsNumber; var template (value) NR_DRB_DataPerSlotList_DL_Type v_DataPerSlotList; var template (value) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU; var integer v_Duration; var SubFrameTiming_Type v_SubFrameTimingTa; var SubFrameTiming_Type v_SubFrameTimingTb; var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND; var integer v_SequenceNumber := 0; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RLC_Status_NackSN18Bit_Type v_NR_NackSN18Bit_1; var template (value) NR_RLC_Status_NackSN18Bit_Type v_NR_NackSN18Bit_2; var template (value) NR_RLC_Status_NackSN18Bit_Type v_NR_NackSN18Bit_3; var template (value) NR_RLC_Status_NackListSN18Bit_Type v_NR_NackSN18Bit_List; var template (value) NR_RLC_Status_NackSN12Bit_Type v_NR_NackSN12Bit_1; var template (value) NR_RLC_Status_NackSN12Bit_Type v_NR_NackSN12Bit_2; var template (value) NR_RLC_Status_NackSN12Bit_Type v_NR_NackSN12Bit_3; var template (value) NR_RLC_Status_NackListSN12Bit_Type v_NR_NackSN12Bit_List; var template (present) NR_RLC_AMD_PDU_Type v_NR_RLC_AMD_PDU_Any; var template (present) NR_RLC_PDUList_Type v_NR_RLC_PDUList; var UL_t_PollRetransmit_Type v_T_PollRetransmit; var DL_t_Reassembly_Type v_T_Reassembly; var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; var template (omit) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var SN_FieldLengthAM v_SN_FieldLengthAM := size18; var boolean v_PDCP_LongSN := not pc_supportOfRedCap_r17; // @sic R5-226947 sic@ redcap: short PDCP header and one more data octet timer t_WatchDog; if (pc_supportOfRedCap_r17 == true) { // @sic R5-226947 sic@ v_SN_FieldLengthAM := size12; //v_RLC_SDUsize := 123; }; // TBS = RLC PDU data (122 octets) + RLC header (3 octets)+ MAC BSR CE header (2 octets) + MAC PDU subheader (2 octets) = 122 + 3 + 2 + 2 = 129 octets = 1032 bits => TBS = 1032 bits // TBS = RLC PDU data (123 octets) + RLC header (2 octets)+ MAC BSR CE header (2 octets) + MAC PDU subheader (2 octets) = 123 + 2 + 2 + 2 = 129 octets = 1032 bits => TBS = 1032 bits (Redcap) v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(1032, v_NR_UplinkBWP); v_SlotsNumber := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1); // generate 424 RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 424, -, v_PDCP_LongSN); // @sic R5-226947 sic@ // @siclog "Step 1" siclog@ // During the whole test sequence, the SS should not allocate UL grants unless when explicitly stated so in the procedure. f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); ………… }