3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230309

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3042 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR/5GC FR1: re-verification of Rel-16 power saving test case 7.1.1.12.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add test case 7.1.1.12.3 to the 5G/NR5GC ATS for FR1 path |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.1.12.3 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.12.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.508-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Changes 4-6 were requested by Rohde&Schwarz |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	10

2.3	Change 3	12

3	Branches executed	14

4	Execution Log Files	14

4.1	MediaTek MT6879	14

5	References	15

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 7.1.1.12.3, which is part of the NR5GC test suite in iwd-TTCN3-B2022-09_D23wk12.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	7.1.1.12.3.NR5GC

ATS Version:	iwd-TTCN3-B2022-09_D23wk12

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	MediaTek MT6879

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_12_3_NR5GC |
| --- | --- |
| Reason for change | 1, According to R5-231902, searchspace ID for DCI_2_6 is 7. But it is configured to 6 2, searchSpacesToAddModList, searchSpacesToAddModListExt_r16 in rrcReconfiguration at step1 is incorrect 3, spCellConfigDedicated.tag_Id is not configured 4, v_DownlinkBWP.Pdcch.SearchSpaceArray is filed incorrect 5, SearchSpaceExt is not configured at SS 6, at step 7-8. v_PhysicalCellGroupConfig is changed, but it is not saved 7, at step 19A, the current ps_PositionDCI_2_6_r16 is 0. So the DCI_2_6 sent at this step should be configured with the 1st block 8, at step 20, Measurement Gap is configured. According to TP4, UE shall always starts the drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe and monitors the PDCCH. So, it is not needed to send DCI_2_6 explicitly before postamble |
| Summary of change | 1, change searchSpaceId to 7 2, correct the context of rrcReconfiguration 3, set spCellConfigDedicated.tag_Id 4, correct v_DownlinkBWP.Pdcch.SearchSpaceArray 5, configure SearchSpaceExt at SS (also see Change 3) 6, call f_NR_CellInfo_SetPhysicalCellGroupConfig to save PhysicalCellGroupConfig at step 7-8 7, correct the configuration of DCI_2_6 blocks at step 19A 8, remove the DCI_2_6 before postamble |
| TTCN module | MAC_PowerSaving_NR5GC |
| MCC160 Comment | - Accepted - Accepted as per approved R5-233317 - Appepted - Accepted as per approved R5-233317 - Accepted, alternative implementation is proposed. All relevant changes and mofidications will be moved to a separate function f_TC_7_1_1_12_3_NR_GetCellGroupConfigForStep1() that also can be called for the same test case running under ENDC configuration - Accepted - Accepted - Accepted MCC160 additional change: to reduce the code created a function f_NR_GetDefaultDRXConfigSetup to assign v_MAC_CellGroupConfig.drx_Config variable |

Before Change

function f_TC_7_1_1_12_3_NR5GC() runs on NR5GC_PTC { //DRX adaptation / UE wakeup indication ……… ……… var template (value) NR_DownlinkBWP_Type v_DownlinkBWP ; var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_NextOccasion; var template(value) SearchSpace v_SearchSpace; //Init Cell parameters f_NR5GC_Init(NR_1); ……… ……… //Table 7.1.1.12.3.3.3-4 v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_ControlResourceSet := f_NR_CellInfo_GetControlResourceSet(nr_Cell1, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_ControlResourceSet.controlResourceSetId := 1; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.controlResourceSetToAddModList := {v_ControlResourceSet} v_SearchSpace := f_NR_CellInfo_GetSearchSpaceConfigAtUE(nr_Cell1, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_SearchSpace.searchSpaceId := 6; v_SearchSpace.controlResourceSetId := 1; v_SearchSpace.searchSpaceType := cs_NR_SearchSpace_SearchSpaceType_CSS_AllOmit; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModList[1] := v_SearchSpace; //@sic R5s230001 Ch1.3, R5-231902 sic@ v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModListExt_r16 := {cs_SearchSpaceExt_r16_AllOmit, //@sic R5s230001 Ch.1.3, R5-231902 sic@ cs_SearchSpaceExt_r16_Dci_Format2_6} v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_DownlinkBWP.Pdcch.SearchSpaceArray[1].SearchSpaceConfigAtUE.R15 := v_SearchSpace; v_DownlinkBWP.Pdcch.SearchSpaceArray[0].SearchSpaceConfigAtUE.R16 := cs_SearchSpaceExt_r16_AllOmit; v_DownlinkBWP.Pdcch.SearchSpaceArray[1].SearchSpaceConfigAtUE.R16 := cs_SearchSpaceExt_r16_Dci_Format2_6; f_NR_CellInfo_SetDownlinkBWP(nr_Cell1, v_DownlinkBWP, tsc_NR_BWP_Id); //@sic R5s230001 Ch.1.3 sic@ v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, -, -, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig, v_SpCellConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); v_LongDRXCycle := 10240; //@sic R5s230001 Ch.1.4 sic@ v_DrxStartOffset := 0; //@siclog Steps 3-6 siclog@ f_TC_7_1_1_12_3_NR_Steps3_6(v_LongDRXCycle, v_DrxStartOffset, v_DRBId); //@siclog Steps 7-8 siclog@ //SS transmits RRCReconfiguration to configure ps-wakeup with value true. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_WakeUp_r16 := true_; v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); //@siclog Steps 9-11 siclog@ ………………………………………………….. //@siclog Step 19A siclog@ //The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_NextOccasion, nr_Cell1, v_LongDRXCycle, v_DrxStartOffset, 7); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_6Items_FirstOrLast(-,'1'B))); //@sic R5s230001, R5-231906 sic@ v_Timing := f_SubFrameTiming_AddMilliSeconds(v_NextOccasion, 5); // add 5ms //@siclog Step 20 siclog@ //SS transmits RRCReconfiguration to configure specific measonfig parameters. (Note 1) ……… ………. //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); // leaving UE test state 3 v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_NextOccasion, nr_Cell1, v_LongDRXCycle, v_DrxStartOffset, 7); //@sic R5s230001 Ch1.9 sic@ f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_6Items_FirstOrLast( -,'1'B))); //@sic R5s230001 sic@ f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def), cs_TimingInfo_NR(v_NextOccasion)); //@sic R5s230001 sic@ f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

After Change

function f_TC_7_1_1_12_3_NR5GC() runs on NR5GC_PTC { //DRX adaptation / UE wakeup indication ………. ………. var template (value) NR_DownlinkBWP_Type v_DownlinkBWP ; var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_NextOccasion; var template(value) SearchSpace v_SearchSpace; var template (value) NR_SearchSpace_NrofCandidates_Type v_NrofCandidates; //Init Cell parameters f_NR5GC_Init(NR_1); ………. ……… //Table 7.1.1.12.3.3.3-4 v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_ControlResourceSet := f_NR_CellInfo_GetControlResourceSet(nr_Cell1, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_ControlResourceSet.controlResourceSetId := 1; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.controlResourceSetToAddModList := {v_ControlResourceSet} v_SearchSpace := f_NR_CellInfo_GetSearchSpaceConfigAtUE(nr_Cell1, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_SearchSpace.searchSpaceId := 7; v_SearchSpace.controlResourceSetId := 1; v_SearchSpace.searchSpaceType := cs_NR_SearchSpace_SearchSpaceType_CSS_AllOmit; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModList[0] := v_SearchSpace; //@sic R5s230001 Ch1.3, R5-231902 sic@ v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModListExt_r16 := {cs_SearchSpaceExt_r16_Dci_Format2_6} //@sic R5s230001 Ch.1.3, R5-231902 sic@ v_SpCellConfig.spCellConfigDedicated.tag_Id := 0; v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_DownlinkBWP.Pdcch.SearchSpaceArray[0].SearchSpaceConfigAtUE.R15 := v_SearchSpace; v_DownlinkBWP.Pdcch.SearchSpaceArray[0].SearchSpaceConfigAtUE.R16 := cs_SearchSpaceExt_r16_Dci_Format2_6; f_NR_CellInfo_SetDownlinkBWP(nr_Cell1, v_DownlinkBWP, tsc_NR_BWP_Id); //@sic R5s230001 Ch.1.3 sic@ if(f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_NrofCandidates := cs_38508_NrofCandidates_Def; } else { v_NrofCandidates := cds_38508_NrofCandidates_FR2 } f_NR_SS_SearchSpaceExtConfig(nr_Cell1, -, v_NrofCandidates); v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, -, -, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig, v_SpCellConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); v_LongDRXCycle := 10240; //@sic R5s230001 Ch.1.4 sic@ v_DrxStartOffset := 0; //@siclog "Steps 3-6" siclog@ f_TC_7_1_1_12_3_NR_Steps3_6(v_LongDRXCycle, v_DrxStartOffset, v_DRBId); //@siclog "Steps 7-8" siclog@ //SS transmits RRCReconfiguration to configure ps-wakeup with value true. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_WakeUp_r16 := true_; v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); //@siclog "Steps 9-11" siclog@ …………………………………………….. //@siclog Step 19A siclog@ //The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_NextOccasion, nr_Cell1, v_LongDRXCycle, v_DrxStartOffset, 7); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_6Items_FirstOrLast('1'B))); //@sic R5s230001, R5-231906 sic@ v_Timing := f_SubFrameTiming_AddMilliSeconds(v_NextOccasion, 5); // add 5ms //@siclog Step 20 siclog@ //SS transmits RRCReconfiguration to configure specific measonfig parameters. (Note 1) ……… ………. //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); // leaving UE test state 3 v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_NextOccasion, nr_Cell1, v_LongDRXCycle, v_DrxStartOffset, 7); //@sic R5s230001 Ch1.9 sic@ //REMOVE f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_6Items_FirstOrLast( -,'1'B))); //@sic R5s230001 sic@ f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def), cs_TimingInfo_NR(v_NextOccasion)); //@sic R5s230001 sic@ f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); };

MCC proposed implementation

function f_TC_7_1_1_12_3_NR5GC() runs on NR5GC_PTC { //DRX adaptation / UE wakeup indication ………. ………. var template (value) NR_DownlinkBWP_Type v_DownlinkBWP ; var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_NextOccasion; var template(value) SearchSpace v_SearchSpace; //Init Cell parameters f_NR5GC_Init(NR_1); ………. ……… v_CellGroupConfig := f_TC_7_1_1_12_3_NR_GetCellGroupConfigForStep1(); //@sic R5s230001, R5s230309 sic@ v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); v_LongDRXCycle := 10240; //@sic R5s230001 Ch.1.4 sic@ v_DrxStartOffset := 0; //@siclog "Steps 3-6" siclog@ f_TC_7_1_1_12_3_NR_Steps3_6(v_LongDRXCycle, v_DrxStartOffset, v_DRBId); //@siclog "Steps 7-8" siclog@ //SS transmits RRCReconfiguration to configure ps-wakeup with value true. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); //@sic R5s230309 sic@ v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_WakeUp_r16 := true_; v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); //@siclog "Steps 9-11" siclog@ …………………………………………….. //@siclog Steps 12-13 siclog@ //SS transmits RRCReconfiguration to configure ps-wakeup with value true. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) v_NR_DRX_OnDurationTimer := { milliSeconds := ms10 }; v_NR_DRX_LongCycleStartOffset := {ms20 := 0}; v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); //@sic R5s230309 sic@ v_MAC_CellGroupConfig.drx_Config.setup := cs_NR_DRX_Config_Def( v_NR_DRX_OnDurationTimer, ms6, 56, 56, sl320, sl320, v_NR_DRX_LongCycleStartOffset, -, 0); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); ………………………… //@siclog Step 19A siclog@ //The SS transmits DCI 2-6 on the PDCCH within the PS-offset time before the start of next long DRX drx-onDurationTimer and the DCI 2-6 indicates to start the next Drx-onDurationTimer v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_NextOccasion, nr_Cell1, v_LongDRXCycle, v_DrxStartOffset, 7); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_6Items_FirstOrLast('1'B))); //@sic R5s230001, R5-231906 sic@ v_Timing := f_SubFrameTiming_AddMilliSeconds(v_NextOccasion, 5); // add 5ms //@siclog Step 20 siclog@ //SS transmits RRCReconfiguration to configure specific measonfig parameters. (Note 1) ……… ………. //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config // configure SS Mac IN NORMAL TEST MODE. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); // leaving UE test state 3 v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_NextOccasion, nr_Cell1, v_LongDRXCycle, v_DrxStartOffset, 7); //@sic R5s230001 Ch1.9 sic@ //REMOVE f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DciTrigger_WakeUP_REQ(nr_Cell1, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_6Items_FirstOrLast( -,'1'B))); //@sic R5s230001 sic@ f_NR_RRCRelease_Common(nr_Cell1, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def), cs_TimingInfo_NR(v_NextOccasion)); //@sic R5s230001 sic@ f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); };

New Function and new template in MAC_PowerSaving_TC_Common_NR module

//---------------------------------------------------------------------------- /* * @desc REFERENCE TS 38.523-1 clause 7.1.1.12.3, generates cell group config to transmit in step 1 * @status */ function f_TC_7_1_1_12_3_NR_GetCellGroupConfigForStep1(integer p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId) runs on NR_BASE_PTC return template (value) CellGroupConfig { var template (value) CellGroupConfig v_CellGroupConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig; var template(value) ControlResourceSet v_ControlResourceSet; var template (value) SpCellConfig v_SpCellConfig; var template (value) NR_DownlinkBWP_Type v_DownlinkBWP ; var template(value) SearchSpace v_SearchSpace; var integer v_SearchSpaceIndex_DCI_2_6; //@sic R5s230309 sic@ //Table 7.1.1.12.3.3.3-2 v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.drx_Config := f_NR_GetDefaultDRXConfigSetup(nr_Cell1); //@sic R5s230309 sic@ f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.dcp_Config_r16.setup := cs_DCP_Config_r16_Def; f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); //@sic R5s230001 Ch1.1 sic@ f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig))); //@sic R5s230001 Ch1.2 sic@ //Table 7.1.1.12.3.3.3-4 v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_ControlResourceSet := f_NR_CellInfo_GetControlResourceSet(nr_Cell1, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_ControlResourceSet.controlResourceSetId := 1; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.controlResourceSetToAddModList := {v_ControlResourceSet} v_SearchSpace := f_NR_CellInfo_GetSearchSpaceConfigAtUE(nr_Cell1, tsc_NR_SearchSpaceId_CSS, tsc_NR_BWP_Id); //@sic R5s230309 sic@ v_SearchSpace.searchSpaceId := tsc_NR_SearchSpaceId_DCI_2_6; //@sic R5s230309 sic@ v_SearchSpace.controlResourceSetId := 1; v_SearchSpace.searchSpaceType := cs_NR_SearchSpace_SearchSpaceType_CSS_AllOmit; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModList[0] := v_SearchSpace; //@sic R5s230001 Ch1.3, R5-231902 sic@ v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModListExt_r16 [0] := cs_SearchSpaceExt_r16_Dci_Format2_6; //@sic R5s230309, R5-233317 sic@ v_SpCellConfig.spCellConfigDedicated.tag_Id := 0; //@sic R5s230309 sic@ v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(nr_Cell1, tsc_NR_BWP_Id); v_SearchSpaceIndex_DCI_2_6 := lengthof(v_DownlinkBWP.Pdcch.SearchSpaceArray); //@sic 230309 sic@ v_DownlinkBWP.Pdcch.SearchSpaceArray[v_SearchSpaceIndex_DCI_2_6] := cs_NR_BWP_SearchSpaceConfig_CSS_DCI_2_6(v_SearchSpace); //@sic R5s230309 sic@ f_NR_CellInfo_SetDownlinkBWP(nr_Cell1, v_DownlinkBWP, tsc_NR_BWP_Id); //@sic R5s230001 Ch.1.3 sic@ f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_DL_BWP_ConfigCommon_REQ(nr_Cell1, -,{v_DownlinkBWP})); //@sic R5s230309 Ch.1.5 sic@ v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG, -, -, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig, v_SpCellConfig); return v_CellGroupConfig; } template (value) NR_SYSTEM_CTRL_REQ cads_NR_DL_BWP_ConfigCommon_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_DownlinkBWP_List_Type p_NR_DownlinkBWP_List_Type) modifies cas_NR_CellConfigAllOmit_REQ := { /* @status */ Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := omit, Downlink:= { BWPs := { ActiveBWP := omit, BwpArray := p_NR_DownlinkBWP_List_Type, InitialBWP_RedCap := omit } }, Uplink := omit } } } } }; //---------------------------------------------------------------------------- /* * @desc Returns default DRX Config setup based on 38.508-1 Table 4.6.3-56 for a given cell * @param p_NR_CellId * @return MAC_CellGroupConfig.drx_Config * @status */ function f_NR_GetDefaultDRXConfigSetup(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC return template (value) MAC_CellGroupConfig.drx_Config { var template (value) MAC_CellGroupConfig.drx_Config v_DRX_Config; if(f_NR_CellInfo_GetIsFR1(p_NR_CellId)) { v_DRX_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR1); } else { v_DRX_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR2); } return v_DRX_Config }

## Change 2

| Function name | f_TC_7_1_1_12_3_NR_Steps14_19 |
| --- | --- |
| Reason for change | 1, according to Table 7.1.1.12.3.3.2-1, should delay 1280ms at step 14 2, p_LongDRXCycle is in unit ms. But f_NR_SubFrameTiming_AddSlots is called at step 18 |
| Summary of change | 1, delay 1.28s at step 14 2, replace f_NR_SubFrameTiming_AddSlots with f_SubFrameTiming_AddMilliSeconds at step18 |
| TTCN module | MAC_PowerSaving_TC_Common_NR |
| MCC160 Comment | - Rejected will be implemented as per R5-233467 - Accepted |

Before Change

function f_TC_7_1_1_12_3_NR_Steps14_19(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication ……… ……… //@siclog Step 14 siclog@ //Wait 20ms to ensure UE is out DRX active time. f_Delay(0.02); //@siclog Step 15 siclog@ ……… ……… //@siclog "Step 18" siclog@ //In the PDCCH occasion when the next Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing ,p_LongDRXCycle*2); //@sic R5s230001 sic@ f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_NR(v_WakeUpTiming))); //@sic R5s230001 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment)); ……… ……… }

After Change

function f_TC_7_1_1_12_3_NR_Steps14_19(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication ……… ……… //@siclog Step 14 siclog@ //Wait 1280ms to ensure UE is out DRX active time. f_Delay(1.28); //@siclog Step 15 siclog@ ……… ……… //@siclog "Step 18" siclog@ //In the PDCCH occasion when the next Drx-onDurationTimer is running, the SS indicates the transmission of a DL MAC PDU on the PDCCH. v_Timing := f_SubFrameTiming_AddMilliSeconds(v_Timing ,p_LongDRXCycle*2); //@sic R5s230001 sic@ f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_NR(v_WakeUpTiming))); //@sic R5s230001 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_DRBId, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment)); ……… ……… }

## Change 3

| Function name | (new) f_NR_SS_SearchSpaceExtConfig (new) cs_NR_BWP_SearchSpaceConfig_CSS_DCI_2_6 (new) cads_NR_SSExtConfig_REQ |
| --- | --- |
| Reason for change | For change 1.5 |
| Summary of change | new function and template to configure searchspaceExt at SS |
| TTCN module | MAC_PowerSaving_TC_Common_NR |
| MCC160 Comment | Rejected new f_NR_SS_SearchSpaceExtConfig due to alternative implementation, see Ch.1.5 Accepted (new) cs_NR_BWP_SearchSpaceConfig_CSS_DCI_2_6 , but will be implemented differently Rejected new cads_NR_SSExtConfig_REQ due to alternative implementation, see Ch.1.5 |

(New)

template (value) NR_BWP_SearchSpaceConfig_Type cs_NR_BWP_SearchSpaceConfig_CSS_DCI_2_6(template (value) NR_SearchSpace_NrofCandidates_Type p_NrofCandidates, SearchSpaceId p_SearchSpaceId := 7, ControlResourceSetId p_ControlResourceSetId := tsc_NR_ControlResourceSetId) := { TypeAndPriorityList :={ tsc_NR_SearchSpaceTypeAndPriority_CssType3}, AggregationLevel := AggregationLevel4, SearchSpaceConfigAtUE := { R15 := cs_38508_SearchSpace(p_SearchSpaceId, p_ControlResourceSetId, cs_NR_SearchSpace_SearchSpaceType_CSS_AllOmit, p_NrofCandidates), R16 := cs_SearchSpaceExt_r16_Dci_Format2_6 } }; template (value) NR_SYSTEM_CTRL_REQ cads_NR_SSExtConfig_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo, template (value) NR_SearchSpace_NrofCandidates_Type p_NrofCandidates ) modifies cas_NR_CellConfigAllOmit_REQ := { Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo), Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := omit, Downlink := { FrequencyInfoDL := omit, SSPbchBlock := omit, PdschCellLevelConfig := omit, BWPs := omit, BWPs := { ActiveBWP := omit, BwpArray := { { Id := omit, BWP := omit, Pdcch := { SearchSpaceArray := {cs_NR_BWP_SearchSpaceConfig_CSS_DCI_2_6(p_NrofCandidates)}, CoresetArray := omit, RelativeTxPower := omit, Coreset0_OffsetRBs := omit }, Pdsch := omit, Sps := omit } } }, CsiConfig := omit }, Uplink := omit } } } } }; function f_NR_SS_SearchSpaceExtConfig(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) NR_SearchSpace_NrofCandidates_Type p_NrofCandidates) runs on NR_BASE_PTC { f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_SSExtConfig_REQ(p_NR_CellId, p_TimingInfo, p_NrofCandidates)); }

MCC160 alternative implementation

template (value) NR_BWP_SearchSpaceConfig_Type cs_NR_BWP_SearchSpaceConfig_CSS_DCI_2_6(template (value) SearchSpace p_SearchSpace) := { TypeAndPriorityList :={ tsc_NR_SearchSpaceTypeAndPriority_CssType3}, AggregationLevel := AggregationLevel4, SearchSpaceConfigAtUE := { R15 := p_SearchSpace, R16 := cs_SearchSpaceExt_r16_Dci_Format2_6 } };

## Change 4 additional comment from Rohde&Schwarz

| Function name | f_TC_7_1_1_12_3_NR5GC() |
| --- | --- |
| Reason for change | - The DRX Config parameters are mainly intended to be used by the UE to configure DRX. The local end SS configuration at Steps 1- 2, Step 12 – 13 and Step 20 should not configure DRX parameters as the TTCN controls the timing of the Long DRX Cycles and the sending of the DCI 2_6. By configuring the DRX parameters in the SS Stack, it cannot be guaranteed that the timings as calculated by TTCN would be followed. - The information to be sent to the UE in the RRCReconfiguration message should first be got from the function f_NR_GetServinCellConfig to be then set via the variable v_SpCellConfig. - The local end configuration of the Measurement Gap at Step 20 should not be done as this would not ensure effective test of Step 22A as the Prose expects the DCI 2_6 with WakeUpBit = 0 to be transmitted to check if the UE receives this DCI 2_6, however, ignores it as it has been received during measurement gap thereby testing and validating TP 4 |
| Summary of change | - Local end SS config is modified to omit the DRX parameters at Step 1 – 2 ,Step 12 – 13 and Step 20 - Function f_NR_GetServinCellConfig called to retrieve the necessary information - Removed the local end configuration of the Measurement Gap |
| TTCN module | MAC_PowerSaving_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_12_3_NR5GC() runs on NR5GC_PTC { //DRX adaptation / UE wakeup indication <<SKIPPED CODE>> //@siclog Steps 1-2 siclog@ //SS transmits RRCReconfiguration to configure specific DCP parameters. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) //Table 7.1.1.12.3.3.3-2 v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); if(f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR1); } else { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR2); } f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.dcp_Config_r16.setup := cs_DCP_Config_r16_Def; f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); //@sic R5s230001 Ch1.1 sic@ //f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig))); //@sic R5s230001 Ch1.2 sic@ //Table 7.1.1.12.3.3.3-4 v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_ControlResourceSet := f_NR_CellInfo_GetControlResourceSet(nr_Cell1, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_ControlResourceSet.controlResourceSetId := 1; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.controlResourceSetToAddModList := {v_ControlResourceSet} <<SKIPPED CODE>> //@siclog Steps 12-13 siclog@ //SS transmits RRCReconfiguration to configure ps-wakeup with value true. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) v_NR_DRX_OnDurationTimer := { milliSeconds := ms10 }; v_NR_DRX_LongCycleStartOffset := {ms20 := 0}; v_MAC_CellGroupConfig.drx_Config.setup := cs_NR_DRX_Config_Def( v_NR_DRX_OnDurationTimer, ms6, 56, 56, sl320, sl320, v_NR_DRX_LongCycleStartOffset, -, 0); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_CellGroupConfig.mac_CellGroupConfig := v_MAC_CellGroupConfig; v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_WakeUp_r16 := omit; v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_Offset_r16 := 40; v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); //@sic R5s230001 Ch.1.1 sic@ //f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig))); //@sic R5s230001 Ch 1.2 sic@ <<SKIPPED CODE>> //@siclog Step 20 siclog@ //SS transmits RRCReconfiguration to configure specific measonfig parameters. (Note 1) SRB.send(cas_NR_RrcNasPduList_REQ(nr_Cell1, tsc_NR_RbId_SRB1, cs_TimingInfo_NR(v_NextOccasion), cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, v_MeasConfig, v_V1530Ext)), -, -, -, cs_RlcBearerRouting_NR(nr_Cell1))); f_NR_SS_MeasGapCtrlConfig(nr_Cell1, cs_NR_MeasGapCtrl_Config(v_MeasGapUE), cs_TimingInfo_NR(v_Timing)); f_NR_SS_ServingCellConfig(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig))); //@sic R5s230001 Ch1.6 sic@ <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_12_3_NR5GC() runs on NR5GC_PTC { //DRX adaptation / UE wakeup indication <<SKIPPED CODE>> //@siclog Steps 1-2 siclog@ //SS transmits RRCReconfiguration to configure specific DCP parameters. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) //Table 7.1.1.12.3.3.3-2 v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); if(f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR1); } else { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR2); } f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.dcp_Config_r16.setup := cs_DCP_Config_r16_Def; f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); //@sic R5s230001 Ch1.1 sic@ f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); //WA# //Table 7.1.1.12.3.3.3-4 //WA# v_ServingCellConfig := f_NR_GetServinCellConfig(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1, v_ServingCellConfig); //WA# v_ControlResourceSet := f_NR_CellInfo_GetControlResourceSet(nr_Cell1, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_ControlResourceSet.controlResourceSetId := 1; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.controlResourceSetToAddModList := {v_ControlResourceSet} <<SKIPPED CODE>> //@siclog Steps 12-13 siclog@ //SS transmits RRCReconfiguration to configure ps-wakeup with value true. (Note 1) //The UE transmits RRCReconfigurationComplete. (Note 2) v_NR_DRX_OnDurationTimer := { milliSeconds := ms10 }; v_NR_DRX_LongCycleStartOffset := {ms20 := 0}; v_MAC_CellGroupConfig.drx_Config.setup := cs_NR_DRX_Config_Def( v_NR_DRX_OnDurationTimer, ms6, 56, 56, sl320, sl320, v_NR_DRX_LongCycleStartOffset, -, 0); f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); v_CellGroupConfig.mac_CellGroupConfig := v_MAC_CellGroupConfig; v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_WakeUp_r16 := omit; v_PhysicalCellGroupConfig.dcp_Config_r16.setup.ps_Offset_r16 := 40; v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); //@sic R5s230001 Ch.1.1 sic@ f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); //@sic R5s230001 Ch 1.2 sic@ WA# <<SKIPPED CODE>> //@siclog Step 20 siclog@ //SS transmits RRCReconfiguration to configure specific measonfig parameters. (Note 1) SRB.send(cas_NR_RrcNasPduList_REQ(nr_Cell1, tsc_NR_RbId_SRB1, cs_TimingInfo_NR(v_NextOccasion), cs_NR_RRC_MSG_Request_DCCH(cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, -, v_MeasConfig, v_V1530Ext)), -, -, -, cs_RlcBearerRouting_NR(nr_Cell1))); //WA# //REMOVED f_NR_SS_MeasGapCtrlConfig(nr_Cell1, cs_NR_MeasGapCtrl_Config(v_MeasGapUE), cs_TimingInfo_NR(v_Timing)); f_NR_SS_ServingCellConfig(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); //@sic R5s230001 Ch1.6 sic@ WA# <<SKIPPED CODE>>

## Change 5 additional comment from Rohde&Schwarz

| Function name | f_TC_7_1_1_12_3_NR_Steps22_24 |
| --- | --- |
| Reason for change | It is recommended to enable the reception of the HARQ ACK in f_TC_7_1_1_12_3_NR_Steps22_24 after the delay of 10ms to ensure that the HARQ ACK for the transmitted RLC ACK for the received RRC Reconfiguration Complete at Step 21 does not get forwarded to TTCN thereby causing TC failure. |
| Summary of change | Moved the call to enable the HARQ ACK after the delay of 10ms |
| TTCN module | MAC_PowerSaving_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_12_3_NR_Steps22_24(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId ) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_NextOccasion; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //@sic R5s230001 sic@ //@siclog Step 22 siclog@ //Wait 10ms to ensure UE is out DRX active time. f_Delay(0.01); <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_12_3_NR_Steps22_24(UInt_Type p_LongDRXCycle, UInt_Type p_DrxStartOffset, DRB_Identity p_DRBId ) runs on NR_BASE_PTC {//DRX adaptation / UE wakeup indication var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_NextOccasion; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; //@siclog Step 22 siclog@ //Wait 10ms to ensure UE is out DRX active time. f_Delay(0.01); //WA# MOVED After Delay f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); //@sic R5s230001 sic@ <<SKIPPED CODE>>

## Change 6 additional comment from Rohde&Schwarz

| Function name | SubFrameTiming_SubstractMilliSeconds |
| --- | --- |
| Reason for change | 1. The implementation of the function f_SubFrameTiming_SubstractMilliSeconds is causing a divide by zero RUNTIME error when the following equation is performed: For e.g: v_TimingInfo.Slot.SlotOffset.Numerology1:= (v_SlotOffset.Numerology1 - (v_Slots_To_Substract - v_SlotOffset.Numerology1)) mod v_SlotOffset.Numerology1; Here, as per the calculations done in function f_NR_GetNextSendOccasion_7_1_1_5_X to calculate DRX Timing, when long DRX cycle is 10240 ms, the resulting timing stored in variable v_DrxActiveTiming in function f_NR_GetWakeUpSignalTiming is all zero for SFN, SF and Slot. This is leading to a divide by zero RUNTIME error when a MOD is performed. |
| Summary of change | Alternative function implementation is proposed |
| TTCN module | MAC_PowerSaving_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted as an improvement. Original code is corrected replacing by mod v_SlotsInSubframe and will be kept as a backup f_SubFrameTiming_SubstractMilliSeconds_MCC160 untill the new function is tested and verified |

Before Change:

function f_SubFrameTiming_SubstractMilliSeconds(SubFrameTiming_Type p_TimingInfo, float p_MilliSeconds) return SubFrameTiming_Type {//@sic R5s230001 sic@ var SubFrameTiming_Type v_TimingInfo := p_TimingInfo; //@sic R5w220211 sic@ var integer v_Diff; var SlotOffset_Type v_SlotOffset; var integer v_PS_Offset_Slots; var integer v_SlotsInSubframe; var integer v_Slots_To_Substract; var integer v_Subframes_To_Substract; var integer v_Frames_To_Substract; var integer v_Reminding := 0; if (not (ischosen(p_TimingInfo.Subframe.Number) and ischosen(p_TimingInfo.SFN.Number) and ischosen(p_TimingInfo.HSFN.Number) and ischosen(p_TimingInfo.Slot.SlotOffset))) { FatalError(__FILE__, __LINE__, "Invalid SubFrameTiming"); } v_SlotOffset := p_TimingInfo.Slot.SlotOffset; select union (v_SlotOffset) { case (Numerology0) { v_PS_Offset_Slots := float2int (p_MilliSeconds); v_SlotsInSubframe := 1; v_Slots_To_Substract := 0; } case (Numerology1) { v_PS_Offset_Slots := float2int (p_MilliSeconds/0.5); v_SlotsInSubframe := 2; v_Slots_To_Substract := v_PS_Offset_Slots mod v_SlotsInSubframe; if ((v_SlotOffset.Numerology1 - v_Slots_To_Substract) <0){ v_Reminding := 1;} v_TimingInfo.Slot.SlotOffset.Numerology1:= (v_SlotOffset.Numerology1 - (v_Slots_To_Substract - v_SlotOffset.Numerology1))mod v_SlotOffset.Numerology1; } case (Numerology2) { v_PS_Offset_Slots := float2int (p_MilliSeconds/0.25); v_SlotsInSubframe := 4; v_Slots_To_Substract := v_PS_Offset_Slots mod v_SlotsInSubframe; if ((v_SlotOffset.Numerology2 - v_Slots_To_Substract) <0){ v_Reminding := 1;} v_TimingInfo.Slot.SlotOffset.Numerology2:= (v_SlotOffset.Numerology2 - (v_Slots_To_Substract - v_SlotOffset.Numerology2))mod v_SlotOffset.Numerology2; } case (Numerology3) { v_PS_Offset_Slots := float2int (p_MilliSeconds/0.125); v_SlotsInSubframe := 8; v_Slots_To_Substract := v_PS_Offset_Slots mod v_SlotsInSubframe; if ((v_SlotOffset.Numerology3 - v_Slots_To_Substract) <0){ v_Reminding := 1;} v_TimingInfo.Slot.SlotOffset.Numerology3:= (v_SlotOffset.Numerology3 - (v_Slots_To_Substract - v_SlotOffset.Numerology3))mod v_SlotOffset.Numerology3; } case (Numerology4) { v_PS_Offset_Slots := float2int (p_MilliSeconds/0.0625); v_SlotsInSubframe := 16; v_Slots_To_Substract := v_PS_Offset_Slots mod v_SlotsInSubframe; if ((v_SlotOffset.Numerology4 - v_Slots_To_Substract) <0){ v_Reminding := 1;} v_TimingInfo.Slot.SlotOffset.Numerology4:= (v_SlotOffset.Numerology4 - (v_Slots_To_Substract - v_SlotOffset.Numerology4))mod v_SlotOffset.Numerology4; } case else { FatalError(__FILE__, __LINE__, "Invalid numerology");} } v_Subframes_To_Substract := ((v_PS_Offset_Slots - (v_Slots_To_Substract) ) / v_SlotsInSubframe) + v_Reminding; v_Diff := v_Subframes_To_Substract - p_TimingInfo.Subframe.Number; v_TimingInfo.Subframe.Number := (10 - (v_Subframes_To_Substract - p_TimingInfo.Subframe.Number))mod 10; if (v_Diff <= 0) { v_Reminding := 0; } else { v_Reminding := 1; } v_Frames_To_Substract := (v_Subframes_To_Substract - (v_Subframes_To_Substract mod 10)) / 10 + v_Reminding; v_Diff := v_Frames_To_Substract - p_TimingInfo.SFN.Number; v_TimingInfo.SFN.Number := (1024 - (v_Frames_To_Substract - p_TimingInfo.SFN.Number))mod 1024; if (v_Diff>0) { v_TimingInfo.HSFN.Number := (1024 - (1 - p_TimingInfo.HSFN.Number)) mod 1024; } return v_TimingInfo; }

After Change:

function f_SubFrameTiming_SubstractMilliSeconds( NR_CellId_Type p_NR_CellId, SubFrameTiming_Type p_TimingInfo, float p_MilliSeconds) runs on NR_BASE_PTC return SubFrameTiming_Type { var SubFrameTiming_Type v_TimingInfo; var UInt_Type v_Diff; var integer v_SFN_Previous; var integer v_Add_MilliSeconds; var UInt_Type v_Diff_Slots; var UInt_Type v_Diff_SFN; var UInt_Type v_Diff_HSFN; var UInt_Type v_ExactSlotNumber; var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(p_NR_CellId); if (not (ischosen(p_TimingInfo.Subframe.Number) and ischosen(p_TimingInfo.SFN.Number) and ischosen(p_TimingInfo.HSFN.Number))) { FatalError(__FILE__, __LINE__, "Invalid SubFrameTiming"); } v_TimingInfo := p_TimingInfo; select (v_SCS){ case (kHz15) {v_ExactSlotNumber := p_TimingInfo.Subframe.Number;} case (kHz30) {v_ExactSlotNumber := (p_TimingInfo.Subframe.Number * 2)+p_TimingInfo.Slot.SlotOffset.Numerology1;} case (kHz60) {v_ExactSlotNumber := (p_TimingInfo.Subframe.Number * 4)+p_TimingInfo.Slot.SlotOffset.Numerology2; } case (kHz120){v_ExactSlotNumber := (p_TimingInfo.Subframe.Number * 8)+p_TimingInfo.Slot.SlotOffset.Numerology3; } case else {FatalError(__FILE__, __LINE__, "Invalid SubCarrierSpacing");} } if (v_ExactSlotNumber >= float2int (p_MilliSeconds)) { //Same SFN v_Diff_Slots := v_ExactSlotNumber - float2int (p_MilliSeconds); select (v_SCS){ case (kHz15) {v_TimingInfo.Subframe.Number := v_Diff_Slots;} case (kHz30) {v_TimingInfo.Subframe.Number := v_Diff_Slots / 2;} case (kHz60) {v_TimingInfo.Subframe.Number := v_Diff_Slots / 4; } case (kHz120){v_TimingInfo.Subframe.Number := v_Diff_Slots / 8; } case else {FatalError(__FILE__, __LINE__, "Invalid SubCarrierSpacing");} } } else { //in previous SFN select (v_SCS){ case (kHz15) { v_Diff_Slots := 10 - float2int (p_MilliSeconds); v_TimingInfo.Subframe.Number := v_Diff_Slots; } case (kHz30) { v_Diff_Slots := 20 - float2int (p_MilliSeconds); v_TimingInfo.Subframe.Number := v_Diff_Slots / 2; } case (kHz60) { v_Diff_Slots := 40 - float2int (p_MilliSeconds); v_TimingInfo.Subframe.Number := v_Diff_Slots / 4; } case (kHz120){ v_Diff_Slots := 80 - float2int (p_MilliSeconds); v_TimingInfo.Subframe.Number := v_Diff_Slots / 8; } case else {FatalError(__FILE__, __LINE__, "Invalid SubCarrierSpacing");} } if (v_TimingInfo.SFN.Number == 0) { //in previous HSFN v_TimingInfo.SFN.Number := 1022; v_TimingInfo.Subframe.Number := 5; v_Diff_HSFN := v_TimingInfo.HSFN.Number - 1; if (v_Diff_HSFN > 0) { v_TimingInfo.HSFN.Number := p_TimingInfo.HSFN.Number -1; } else { v_TimingInfo.HSFN.Number := 1023; } } else { v_Diff_SFN := v_TimingInfo.SFN.Number - 1; if (v_Diff_SFN > 0) { v_TimingInfo.SFN.Number := p_TimingInfo.SFN.Number -1; } } } v_TimingInfo.Slot := p_TimingInfo.Slot; return v_TimingInfo; }

# Branches executed

The test case was executed on NR band n41 using NR ciphering nea2 and integrity algorithm nia2

# Execution Log Files

## MediaTek MT6879

The MediaTek MT6879UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_7_1_1_12_3_LOG.html

- PICS/PIXIT parameter file:

TC_7_1_1_12_3_PIXIT.xml

# References

| [1] | R5s230310: NR/5GC FR1: Supporting information for re-verification of Rel-16 power saving test case 7.1.1.12.3 |
| --- | --- |