3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230342

# Online, , 9th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4727 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.5.20 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, NB_IOTenh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-04-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current TTCN implementation, at Step 11 the SS transmits the rrcRelese with re-direction to frequency of Ncell 1 alongwith extended wait time set to 30s. However, at this time the Ncell 1 is not set to serving cell power, and a FAST UE can retry to camp on Ncell 23 again by sending a rrcConnectionREQ. This causes TC failure and needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Before sending the rrcConnection Release on Ncell23, called the function f_NBIOT_SS_ConfigRachProcedureWithNoMsg4(nbiot_Cell23); This ensures that the UE never gets RAR, thus it cannot send rrcConnectionREQ on Ncell 23. Once the power level change is effective and the UE detects that, it moves to Ncell 1 as expected by the testcase. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to function fl_TC_22_5_20_Body	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk12 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function fl_TC_22_5_20_Body

| Function name | fl_TC_22_5_20_Body |
| --- | --- |
| Reason for change | In the current TTCN implementation, at Step 11 the SS transmits the rrcRelese with re-direction to frequency of Ncell 1 alongwith extended wait time set to 30s. However, at this time the Ncell 1 is not set to serving cell power, and a FAST UE can retry to camp on Ncell 23 again by sending a rrcConnectionREQ. This causes TC failure and needs to be addressed. |
| Summary of change | Before sending the rrcConnection Release on Ncell23, called the function f_NBIOT_SS_ConfigRachProcedureWithNoMsg4(nbiot_Cell23); This ensures that the UE never gets RAR, thus it cannot send rrcConnectionREQ on Ncell 23. Once the power level change is effective and the UE detects that, it moves to Ncell 1 as expected by the testcase. |
| TTCN module | ENBIOT_NAS.ttcn |
| MCC160 Comment | Accepted as improvment. The proposed implementation could help a wrong UE to pass the test. See MCC160 proposed implementation below. |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 11" siclog@ //SS transmits an RRCConnectionRelease-NB message (IE redirectedCarrierInfo including carrierFreq-r13 of Ncell 1). f_NBIOT_RRC_ConnectionRelease_Common(nbiot_Cell23, CONTROL_PLANE, cs_RRCConnectionRelease_NB_r14(tsc_RRC_TI_Def, other, omit , omit, 30, v_CarrierFreq_Cell1.DL), cs_TimingInfo_Now); // @sic R5-206436 sic@ // @sic R5s210634 sic@ //@siclog "Step 12" siclog@ //SS configures Ncell 23 as non-suitable cell and Ncell 1 as serving cell. v_CellPowerList := { cs_NB_CellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE), cs_NB_CellPower(nbiot_Cell23, tsc_NonSuitableNBIOTCellRS_EPRE) }; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 11" siclog@ //SS transmits an RRCConnectionRelease-NB message (IE redirectedCarrierInfo including carrierFreq-r13 of Ncell 1). //WA#22_5_20 COMMENTED OUT f_NBIOT_RRC_ConnectionRelease_Common(nbiot_Cell23, CONTROL_PLANE, cs_RRCConnectionRelease_NB_r14(tsc_RRC_TI_Def, other, omit , omit, 30, v_CarrierFreq_Cell1.DL), cs_TimingInfo_Now); // @sic R5-206436 sic@ // @sic R5s210634 sic@ //WA#22_5_20 This is put in place to handle a FAST UE trying to camp again on Ncell 23 before the power level change is detected f_NBIOT_SS_ConfigRachProcedureWithNoMsg4(nbiot_Cell23); SRB.send(cas_NB_SRB_RrcPdu_REQ(nbiot_Cell23, f_NBIOT_GetSrb1Id(CONTROL_PLANE), cs_TimingInfo_EUTRA_NB(f_NBIOT_GetNextSearchSpace(nbiot_Cell23, 300)), cs_RRCConnectionRelease_NB_r14(tsc_RRC_TI_Def, other, omit , omit, 30, v_CarrierFreq_Cell1.DL))); //@siclog "Step 12" siclog@ //SS configures Ncell 23 as non-suitable cell and Ncell 1 as serving cell. v_CellPowerList := { cs_NB_CellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE), cs_NB_CellPower(nbiot_Cell23, tsc_NonSuitableNBIOTCellRS_EPRE) }; <<SKIPPED CODE>>

MCC160 proposed implementation

function fl_TC_22_5_20_Body() runs on NBIOT_PTC

{

var O1_Type v_UserData := 'F0'O;

var NAS_MSG_Indication_Type v_NAS_Ind;

var template (value) NB_CellPowerList_Type v_CellPowerList;

var template (omit) EPS_BearerContextStatus v_EPS_ContextStatus;

var GutiParameters_Type v_Guti_Params_Cell1 := f_NBIOT_CellInfo_GetGuti (nbiot_Cell1);

var NAS_PlmnId v_PLMN_Cell1 := f_Asn2Nas_PlmnId (v_Guti_Params_Cell1.PLMN_Identity);

var EUTRA_ASN1_TrackingAreaCode_Type v_TAC_Cell1 := f_NBIOT_CellInfo_GetTAC (nbiot_Cell1);

//The timer uplink data delay is configured in CLOSE UE TEST LOOP.

//When UE receives downlink ESM DATA TRANSPORT, UE will start this timer and loopback the uplink user data when this timer expires.

//But timer uplink data delay = 8s may be too short when RRC connnection release procedure takes 10s

//Hence this is set to 15s

var NB_CarrierFreq_Type v_CarrierFreq_Cell1 := f_NBIOT_CellInfo_GetEARFCN(nbiot_Cell1); // @sic R5-206436 sic@ // @sic R5s210634 sic@

var integer v_DelayLoopbackDataTime := 5;

timer t_T3448 := 30.0;

timer t_Wait;

var SubFrameTiming_Type v_TimingAtT;

var template(value) TimingInfo_Type v_TimingInfoNCell1;

//@siclog "Step 1" siclog@

SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1,

tsc_SRB1bis,

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_ESM_DATA_TRANSPORT(tsc_EpsDefaultBearerId, tsc_PTI_Unassigned, cs_UserDataContainer(v_UserData&v_UserData&v_UserData))))); // @sic R5s170462 sic@

//@siclog "Step 2" siclog@

//Wait for 1 s after the IP packet has been transmitted.

//Note 1: The 1 second delay is used to secure that the UE have received and forwarded the IP Packet transmitted by the SS to the UE test loop function before the RRCConnectionRelease-NB message is sent by the SS.

f_Delay(1.0);

…

//@siclog "Step 9" siclog@

//SS configures Ncell 1 as non-suitable cell and Ncell 23 as serving cell.

v_CellPowerList := {

cs_NB_CellPower(nbiot_Cell23, tsc_ServingCellRS_EPRE),

cs_NB_CellPower(nbiot_Cell1, tsc_NonSuitableNBIOTCellRS_EPRE)

};

f_NBIOT_SetCellPowerList(v_CellPowerList);

//@siclog "Step 10" siclog@

v_NAS_Ind := f_NBIOT_RRC_ConnEst_DefWithNas(nbiot_Cell23,

cr_EstablishmentCause_NB_Any,

cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cr_508_TAU_Request (tsc_EpsUpdate_TaUpdate,

f_NBIOT_SecurityKSIasme_Get(),

cr_DRXparameter_Any ('5C'O) ifpresent,

cr_AdditionalUpdateType_CIOT ifpresent)));

if (not f_Check_UE_NetworkCap_CIOT (v_NAS_Ind.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueNetworkCapability, CP_Backoff)) {

f_NBIOT_SetVerdictFailOrInconc (__FILE__, __LINE__, "UE Network Cap failed CIOT bit check");

}

//@siclog "Step 11" siclog@

//SS transmits an RRCConnectionRelease-NB message (IE redirectedCarrierInfo including carrierFreq-r13 of Ncell 1).

v_TimingAtT := f_NBIOT_GetNextSearchSpace(nbiot_Cell23, 300);

v_TimingInfoNCell1 := f_NBIOT_TimingOtherCell(nbiot_Cell23, cs_TimingInfo_EUTRA_NB(v_TimingAtT), nbiot_Cell1);

v_CellPowerList := { cs_NB_CellPower(nbiot_Cell1, tsc_ServingCellRS_EPRE) };

f_NBIOT_SetCellPowerList(v_CellPowerList, v_TimingInfoNCell1 );

f_NBIOT_RRC_ConnectionRelease_Common(nbiot_Cell23, CONTROL_PLANE, cs_RRCConnectionRelease_NB_r14(tsc_RRC_TI_Def, other, omit , omit, 30, v_CarrierFreq_Cell1.DL), cs_TimingInfo_EUTRA_NB(v_TimingAtT)); // @sic R5-206436 sic@ // @sic R5s210634 sic@

//@siclog "Step 12" siclog@

//SS configures Ncell 23 as non-suitable cell and Ncell 1 as serving cell.

v_CellPowerList := {

cs_NB_CellPower(nbiot_Cell23, tsc_NonSuitableNBIOTCellRS_EPRE)

};

f_NBIOT_SetCellPowerList(v_CellPowerList);  // @sic R5s190574 sic@

//@siclog "Step 13" siclog@

…

//@siclog "Step 30 Void" siclog@

//@siclog "Step 31" siclog@

//The SS transmits an RRCConnectionRelease-NB message

f_NBIOT_RRC_ConnectionRelease(nbiot_Cell1);   // @sic R5s190574 sic@

} // End of fl_TC_22_5_20_Body