3GPP TSG-RAN5 Meeting #2023-TTCN email	R5s230544

# Online, , 12th Dec 2022 - 31st Dec 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3143 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC emergency call test case 11.4.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-07-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Wrong step number passed into function fl_EmergencyCallAndRelease() for preliminary pass verdict |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct step number |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inaccurate test case logging |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk24 related to the title of this CR.

Contact:	Shaun Harry 
	shaun.harry@keysight.com

- Corrections required

  - Change 1

| Function name | fl_TC_11_4_6_NR5GC_TestBody() |
| --- | --- |
| Reason for change | Wrong step number passed into function fl_EmergencyCallAndRelease() for preliminary pass verdict |
| Summary of change | Correct step number |
| TTCN module | Emergency_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function fl_TC_11_4_6_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s210770 sic@ var TrackingAreaCode v_TAC_cell11 := f_NR_CellInfo_GetTAC (nr_Cell11); var NAS_PlmnId v_PLMN_cell11 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var template (value) ServiceAreaIdList v_ServiceAreaIdList := cs_NG_ServiceAreaIdList(v_PLMN_cell11, {bit2oct(v_TAC_cell11)}); //@siclog "Steps 1" siclog@ // NR Cell 11 as "Serving cell", NR Cell 1/2 as "Non-Suitable "Off" cell" f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Steps 2" siclog@ // Check: Does in the next 5 sec the UE transmit an RRCSetupRequest message? f_NR5GC_MobilityRegistration(nr_Cell11, -, -, -, -, -, -, v_ServiceAreaIdList); // @sic R5s210635 R5s210770 sic@ f_IP_ChangeNrCell(IP, nr_Cell11); // @sic R5s210635 sic@ //@siclog "Steps 3 - 7" siclog@ fl_EmergencyCallAndRelease(nr_Cell11, px_EmergencyCallNumber, "Step 2"); //@siclog "Steps 8" siclog@ // Make the UE attempt an IMS none-emergency call. f_UT_RequestIMSCall(UT); //@siclog "Steps 9" siclog@ // Check: Does the UE transmit in the next 10 sec an RRCSetupRequest message? f_NR_CheckNoRRCSetupReq(nr_Cell11, 10.0); }

After Change:

function fl_TC_11_4_6_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s210770 sic@ var TrackingAreaCode v_TAC_cell11 := f_NR_CellInfo_GetTAC (nr_Cell11); var NAS_PlmnId v_PLMN_cell11 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); var template (value) ServiceAreaIdList v_ServiceAreaIdList := cs_NG_ServiceAreaIdList(v_PLMN_cell11, {bit2oct(v_TAC_cell11)}); //@siclog "Steps 1" siclog@ // NR Cell 11 as "Serving cell", NR Cell 1/2 as "Non-Suitable "Off" cell" f_NR_SetCellPower (nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE); //@siclog "Steps 2" siclog@ // Check: Does in the next 5 sec the UE transmit an RRCSetupRequest message? f_NR5GC_MobilityRegistration(nr_Cell11, -, -, -, -, -, -, v_ServiceAreaIdList); // @sic R5s210635 R5s210770 sic@ f_IP_ChangeNrCell(IP, nr_Cell11); // @sic R5s210635 sic@ //@siclog "Steps 3 - 7" siclog@ fl_EmergencyCallAndRelease(nr_Cell11, px_EmergencyCallNumber, "Step 4"); //@siclog "Steps 8" siclog@ // Make the UE attempt an IMS none-emergency call. f_UT_RequestIMSCall(UT); //@siclog "Steps 9" siclog@ // Check: Does the UE transmit in the next 10 sec an RRCSetupRequest message? f_NR_CheckNoRRCSetupReq(nr_Cell11, 10.0); }