3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240046

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3320 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_TC_8_2_3_8_1x_TestBody |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-12-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the 23wk49 TTCN implementation of of the function : fl_TC_8_2_3_8_1x_TestBody v_MeasGapUE is initialized as var template (value) MeasGapConfig v_MeasGapUE; var template (value) MeasGapConfig is causing run time issue in case of 8.2.3.8.1.ENDC FR2 because MeasGapUE will only have the values if cell 3 / cell 10 are present,and this case does not have any of these cells configured. This change is needed for FR2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to have var template (omit) MeasGapConfig v_MeasGapUE as in the earlier ATS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Causes Runtime exception UninitializedValueException |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.8.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This TTCN change is made in 23wk49 without having any CR tagged |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change to the Function fl_TC_8_2_3_8_1x_TestBody	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the Function fl_TC_8_2_3_8_1x_TestBody

| Function name | fl_TC_8_2_3_8_1x_TestBody |
| --- | --- |
| Reason for change | In the 23wk49 TTCN implementation of of the function : fl_TC_8_2_3_8_1x_TestBody v_MeasGapUE is initialized as var template (value) MeasGapConfig v_MeasGapUE; var template (value) MeasGapConfig is causing run time issue in case of 8.2.3.8.1.ENDC FR2 because MeasGapUE will only have the values if cell 3 / cell 10 are present,and this case does not have any of these cells configured |
| Summary of change | It is proposed to have var template (omit) MeasGapConfig v_MeasGapUE as in the earlier ATS. |
| TTCN module | \Common\NG_NAS\NG_NAS_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_8_2_3_8_1x_TestBody(NR_CellId_Type p_NR_CellId) runs on ENDC_NR_PTC { << SKIPPED CODE >> var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (value) MeasGapConfig v_MeasGapUE; << SKIPPED CODE >>

After Change:

function fl_TC_8_2_3_8_1x_TestBody(NR_CellId_Type p_NR_CellId) runs on ENDC_NR_PTC { << SKIPPED CODE >> var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasGapConfig v_MeasGapUE := omit; //WA#WI=1230100 << SKIPPED CODE >>