3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240647

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3573 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC SOR test case 6.3.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In the current TTCN implementation of template cr_NG_SORTransparentContainer - template (present) O2_Type p_Counter := ?, is checking as AnyValue. However, it can be AnyOrOmit as Counter in SOR is a optional IE. This causes a runtime error. - template (omit) IEI8_Type p_IEI := '73'O, is mendate for all containertype. However, it can be Omit as IEI is not present in some Payload container which use the same templete implementation. This causes a runtime error. - In the function f_NR5GC_SorRegistration_Step5_20, parameters of cr_NG_SORTransparentContainer template needs to be updated with IEI while using in cr_NG_REGISTRATION_COMPLETE as it is a mandatory parameter. - In function fl_TC_6_3_1_1_TestBody, parameters of cr_NG_SORTransparentContainer need to be updated with IEI while using in cr_NG_REGISTRATION_COMPLETE as it is a mandatory parameter. - In the TTCN implementation of function f_NG_DecodeAndMatchSORTransportContainer - f_NG_DecodeAndMatchSORTransportContainer, length of v_SORPayloadRxBytes is missed in v_SORPayloadRx to decode the SORPayLoadContainer. - Instead of matching the Header within SORContainer, the whole SORContainer is being matched. However as per the Prose, only the header should be checked. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated the p_Counter as * to accept AnyValueOrOmit. - Updated the p_IEI as Omit to accept AnyValueOrOmit. - Passed the IEI value to this template which is mandatory for REGISTRATION_COMPLETE. - Passed the IEI value to this template which is mandatory for REGISTRATION_COMPLETE. - Added the length of v_SORPayloadRx for IEL of SORTransportContainer. - Updated the function impelmentation to check for IE Header. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required.	4

2.1	Change 1	4

2.2	Change 2	5

2.3	Change 3	7

2.4	Change 4	8

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | template (present) SORTransparentContainer cr_NG_SORTransparentContainer |
| --- | --- |
| Reason for change | In the current TTCN implementation of template cr_NG_SORTransparentContainer - template (present) O2_Type p_Counter := ?, is checking as AnyValue. However, it can be AnyOrOmit as Counter in SOR is a optional IE. This causes a runtime error. - template (omit) IEI8_Type p_IEI := '73'O, is mendate for all containertype. However, it can be Omit as IEI is not present in some Payload container which use the same templete implementation. This causes a runtime error. |
| Summary of change | - Updated the p_Counter as * to accept AnyValueOrOmit. - Updated the p_IEI as Omit to accept AnyValueOrOmit. |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

template (present) SORTransparentContainer cr_NG_SORTransparentContainer(template (present) SOR_Header p_Header, template (present) O16_Type p_Mac := ?, template (present) O2_Type p_Counter := ?, template octetstring p_Data := *, template (omit) IEI8_Type p_IEI := '73'O, template (omit) Type6Length_Type p_IEL := '0011'O):= { /* @status APPROVED (NR5GC) */ /* @sic R5s240531 sic@ */ iei := p_IEI, iel := p_IEL, header := p_Header, mac := p_Mac, counter := p_Counter, data := p_Data };

After Change:

template (present) SORTransparentContainer cr_NG_SORTransparentContainer(template (present) SOR_Header p_Header, template (present) O16_Type p_Mac := ?, template (present) O2_Type p_Counter := *, template octetstring p_Data := *, template (omit) IEI8_Type p_IEI := omit, template (omit) Type6Length_Type p_IEL := '0011'O):= { /* @status APPROVED (NR5GC) */ /* @sic R5s240531 sic@ */ iei := p_IEI, iel := p_IEL, header := p_Header, mac := p_Mac, counter := p_Counter, data := p_Data };

## Change 2

| Function name | f_NR5GC_SorRegistration_Step5_20 |
| --- | --- |
| Reason for change | In the function f_NR5GC_SorRegistration_Step5_20, parameters of cr_NG_SORTransparentContainer template needs to be updated with IEI while using in cr_NG_REGISTRATION_COMPLETE as it is a mandatory parameter. |
| Summary of change | Passed the IEI value to this template which is mandatory for REGISTRATION_COMPLETE |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_SorRegistration_Step5_20(NR_CellId_Type p_NR_CellId, octetstring p_Data, template (value) SOR_Header p_Header, template (present) SOR_Header p_Header_UE := ?, boolean p_MacError:= false, TypeOfRegistration_Type p_RegType := Initial_NoSecurity, GMM_MobilityInfo_Type p_GMM_MobilityInfo, NG_NAS_MSG_Indication_Type p_ReceivedMsg, boolean p_AckSor:= true) runs on NR5GC_PTC { //@sic R5s230902 sic@ var NR_SRB_COMMON_IND v_ReceivedAsp; var O16_Type v_Mac_UE; v_Mac_UE:=f_NR5GC_SorRegistration_Step5_13 (p_NR_CellId, p_GMM_MobilityInfo, p_ReceivedMsg, p_Data, p_Header, p_RegType, p_MacError); //@sic R5s210236 sic@ if(p_AckSor) { SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE(cr_NG_SORTransparentContainer(p_Header_UE,v_Mac_UE)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } f_NR5GC_RRC_Idle_Steps16_20(p_NR_CellId,-, TESTMode_OFF,rrcConnectionRelease); } else{ f_NR5GC_RRC_Idle_Steps15_20(p_NR_CellId, TESTMode_OFF, rrcConnectionRelease); } }

After Change:

function f_NR5GC_SorRegistration_Step5_20(NR_CellId_Type p_NR_CellId, octetstring p_Data, template (value) SOR_Header p_Header, template (present) SOR_Header p_Header_UE := ?, boolean p_MacError:= false, TypeOfRegistration_Type p_RegType := Initial_NoSecurity, GMM_MobilityInfo_Type p_GMM_MobilityInfo, NG_NAS_MSG_Indication_Type p_ReceivedMsg, boolean p_AckSor:= true) runs on NR5GC_PTC { //@sic R5s230902 sic@ var NR_SRB_COMMON_IND v_ReceivedAsp; var O16_Type v_Mac_UE; v_Mac_UE:=f_NR5GC_SorRegistration_Step5_13 (p_NR_CellId, p_GMM_MobilityInfo, p_ReceivedMsg, p_Data, p_Header, p_RegType, p_MacError); //@sic R5s210236 sic@ if(p_AckSor) { SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE(cr_NG_SORTransparentContainer(p_Header_UE,v_Mac_UE,-,-,'73'O)))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } f_NR5GC_RRC_Idle_Steps16_20(p_NR_CellId,-, TESTMode_OFF,rrcConnectionRelease); } else{ f_NR5GC_RRC_Idle_Steps15_20(p_NR_CellId, TESTMode_OFF, rrcConnectionRelease); } }

## Change 3

| Function name | function fl_TC_6_3_1_1_TestBody() |
| --- | --- |
| Reason for change | In function fl_TC_6_3_1_1_TestBody, parameters of cr_NG_SORTransparentContainer need to be updated with IEI while using in cr_NG_REGISTRATION_COMPLETE as it is a mandatory parameter |
| Summary of change | Passed the IEI value to this template which is mandatory for REGISTRATION_COMPLETE |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_6_3_1_1_TestBody() runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202596 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202596 sic@ var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202596 sic@ <<SKIPPED CODE>> //@siclog "Step 17" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message carrying acknowledgement of successful reception of the steering of roaming information SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell13, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE(cr_NG_SORTransparentContainer(cr_NG_SOR_Header('0'B,'0'B,?,'1'B),v_Mac_UE)))) {//@sic R5s230347 R5s230547 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } <<SKIPPED CODE>> };

After Change:

function fl_TC_6_3_1_1_TestBody() runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202596 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202596 sic@ var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202596 sic@ <<SKIPPED CODE>> //@siclog "Step 17" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message carrying acknowledgement of successful reception of the steering of roaming information SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell13, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE(cr_NG_SORTransparentContainer(cr_NG_SOR_Header('0'B,'0'B,?,'1'B),v_Mac_UE,-,-,'73'O)))) {//@sic R5s230347 R5s230547 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } <<SKIPPED CODE>> };

## Change 4

| Function name | f_NG_DecodeAndMatchSORTransportContainer |
| --- | --- |
| Reason for change | In the TTCN implementation of function f_NG_DecodeAndMatchSORTransportContainer - f_NG_DecodeAndMatchSORTransportContainer, length of v_SORPayloadRxBytes is missed in v_SORPayloadRx to decode the SORPayLoadContainer. - Instead of matching the Header within SORContainer, the whole SORContainer is being matched. However as per the Prose, only the header should be checked. |
| Summary of change | - Added the length of v_SORPayloadRx for IEL of SORTransportContainer. - Updated the function impelmentation to check for IE Header. |
| TTCN module | NR5GC_CommonFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NG_DecodeAndMatchSORTransportContainer (NR_SRB_COMMON_IND p_ReceivedAsp, template SOR_Header p_SORContainerRX := cr_NG_SOR_Header (?, ?, ?, ?)) runs on NR_BASE_PTC { var SORTransparentContainer v_SORContainerRX; var octetstring v_SORPayloadRxBytes; var bitstring v_SORPayloadRx; v_SORPayloadRxBytes := p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.payload.payload; v_SORPayloadRx := oct2bit(v_SORPayloadRxBytes); if (decvalue(v_SORPayloadRx, v_SORContainerRX) != 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "SOR Container Failed"); } // If SOR Header template to be tested if (ispresent(p_SORContainerRX)) { if (not match (v_SORContainerRX, cr_NG_SORTransparentContainer (p_SORContainerRX))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "SOR Header Failed"); } } }

After Change:

function f_NG_DecodeAndMatchSORTransportContainer (NR_SRB_COMMON_IND p_ReceivedAsp, template SOR_Header p_SORContainerRX := cr_NG_SOR_Header (?, ?, ?, ?)) runs on NR_BASE_PTC { var SORTransparentContainer v_SORContainerRX; var octetstring v_SORPayloadRxBytes; var bitstring v_SORPayloadRx; var template SORTransparentContainer v_SORTransparentContainer := cr_NG_SORTransparentContainer (p_SORContainerRX); v_SORPayloadRxBytes := p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.payload.payload; v_SORPayloadRx := oct2bit(int2oct(lengthof(v_SORPayloadRxBytes), 2) & v_SORPayloadRxBytes); if (decvalue(v_SORPayloadRx, v_SORContainerRX) != 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "SOR Container Failed"); } // If SOR Header template to be tested if (ispresent(p_SORContainerRX)) { if (not match (v_SORContainerRX.header, v_SORTransparentContainer.header)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "SOR Header Failed"); } } }