3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240649

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3575 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC IRAT EPS Fallback test cases 11.1.5 and 11.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - For 11.1.5 and 11.1.6, the DHCP and ICMPv6 servers don’t have to be started again for this PDN as they are already running (the same PDN has been established already at the beginning of the test). - For 11.1.5, the send coordination message for IMS is not at the correct place, it needs to be sent after the transfer if PDU sessions |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Skipped “f_IP_Handling_StartPDN” by passing parameter p_PduInfoFromNR in f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2 - The send coordination message for IMS is not at the correct place, it needs to be sent after the transfer if PDU sessions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.5.NR5GC, 11.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

2.2	Change 2	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function fl_TC_11_1_5_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC |
| --- | --- |
| Reason for change | - The DHCP and ICMPv6 servers don’t have to be started again for this PDN as they are already running (the same PDN has been established already at the beginning of the test). - The send coordination message for IMS is not at the correct place, it needs to be sent after the transfer if PDU sessions |
| Summary of change | - Skipped “f_IP_Handling_StartPDN” by passing parameter p_PduInfoFromNR in f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2 - Moved the coordination message before step 30-32 |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_1_5_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var EPS_BearerIdentity v_EpsBearerId; // @sic R5s211660 sic@ var SRB_COMMON_IND v_ReceivedAsp; // @sic R5s211660 sic@ timer t_Wait := 5.0; // @sic R5s211660 sic@ // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell3})); // @sic R5s211660 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 9A" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE_FR2); // @sic R5s211660 sic@ f_EUTRA38_SetCellPower(eutra_Cell3, tsc_CellRS_EPRE_73, tsc_ServingCellRS_EPRE_FR2); // @sic R5-221453 sic@ // @siclog "Step 14 - 27" siclog@ f_TAUReject_AttachWithoutN26 (eutra_Cell1); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); // @sic R5s211660 sic@ t_Wait.start; // @sic R5s211660 sic@ alt { // @sic R5s211660 sic@ [] SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, ?, -, -, *)))) -> value v_ReceivedAsp // @sic R5-226605 sic@ { t_Wait.stop; // @sic R5s220177 sic@ v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2 (eutra_Cell1, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pDN_CONNECTIVITY_REQUEST); a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(eutra_Cell1); a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(eutra_Cell1, v_EpsBearerId); } [] t_Wait.timeout {} } // @siclog "Step 30 -32" siclog@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); /*******************Skipping the code ***************************************/ }

After Change:

function fl_TC_11_1_5_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var EPS_BearerIdentity v_EpsBearerId; // @sic R5s211660 sic@ var SRB_COMMON_IND v_ReceivedAsp; // @sic R5s211660 sic@ timer t_Wait := 5.0; // @sic R5s211660 sic@ timer t_Wait_timer := 1.0; var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell3})); // @sic R5s211660 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 9A" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE_FR2); // @sic R5s211660 sic@ f_EUTRA38_SetCellPower(eutra_Cell3, tsc_CellRS_EPRE_73, tsc_ServingCellRS_EPRE_FR2); // @sic R5-221453 sic@ // @siclog "Step 14 - 27" siclog@ v_PduSessionInfoList := f_TAUReject_AttachWithoutN26 (eutra_Cell1); t_Wait.start; // @sic R5s211660 sic@ alt { // @sic R5s211660 sic@ [] SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, ?, -, -, *)))) -> value v_ReceivedAsp // @sic R5-226605 sic@ { t_Wait.stop; // @sic R5s220177 sic@ f_EUTRA38_MsgInDefault_StorePDNConnectivityReq (v_ReceivedAsp); // @siclog "Parallel Behaviour" siclog@ v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2 (eutra_Cell1, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pDN_CONNECTIVITY_REQUEST,-,-,v_PduSessionInfoList); a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(eutra_Cell1); a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(eutra_Cell1, v_EpsBearerId); } [] t_Wait.timeout {} } f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); // @sic R5s211660 sic@ // @siclog "Step 30 -32" siclog@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); /*******************Skipping the code ***************************************/ }

## Change 2

| Function name | function fl_TC_11_1_6_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC |
| --- | --- |
| Reason for change | The DHCP and ICMPv6 servers don’t have to be started again for this PDN as they are already running (the same PDN has been established already at the beginning of the test). |
| Summary of change | Skipped “f_IP_Handling_StartPDN” by passing parameter p_PduInfoFromNR in f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2 |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_1_6_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var EPS_BearerIdentity v_EpsBearerId; // @sic R5s211708 sic@ var SRB_COMMON_IND v_ReceivedAsp; // @sic R5s211708 sic@ timer t_Wait := 5.0; // @sic R5s211708 sic@ // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 5A" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); f_TAUReject_AttachWithoutN26 (eutra_Cell1); t_Wait.start; alt { // @sic R5s211708 sic@ // @sic R5-221454 sic@ [] SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, ?, -, -, *)))) -> value v_ReceivedAsp // @sic R5s230082 sic@ // @sic R5-231415 sic@ { t_Wait.stop; v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2(eutra_Cell1, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pDN_CONNECTIVITY_REQUEST); a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(eutra_Cell1); a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(eutra_Cell1, v_EpsBearerId); } [] t_Wait.timeout { } } /*******************Skipping the code ***************************************/ }

After Change:

function fl_TC_11_1_6_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var EPS_BearerIdentity v_EpsBearerId; // @sic R5s211708 sic@ var SRB_COMMON_IND v_ReceivedAsp; // @sic R5s211708 sic@ timer t_Wait := 5.0; // @sic R5s211708 sic@ var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1})); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Change Power Level"); // @sic R5-216190 sic@ // @siclog "Step 5A" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE, tsc_Suitable_NeighbourIntraFreq_CellRS_EPRE); v_PduSessionInfoList := f_TAUReject_AttachWithoutN26 (eutra_Cell1); t_Wait.start; alt { // @sic R5s211708 sic@ // @sic R5-221454 sic@ [] SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, ?, -, -, *)))) -> value v_ReceivedAsp // @sic R5s230082 sic@ // @sic R5-231415 sic@ { t_Wait.stop; v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2 (eutra_Cell1, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pDN_CONNECTIVITY_REQUEST,-,-,v_PduSessionInfoList); a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(eutra_Cell1); a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(eutra_Cell1, v_EpsBearerId); } [] t_Wait.timeout { } } /*******************Skipping the code ***************************************/ }