3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240686

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3592 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC testcase 7.1.1.12.4.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 7.1.1.12.4.1 to the IWD_24wk37 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 7.1.1.12.4.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.12.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	6

2.3	Change 3	10

2.4	Change 4	11

2.5	Change 5	13

2.6	Change 6	16

3 Branches executed	17

4	Execution Log Files	17

4.1	MediaTek MT6983 UE	17

4.2	HiSilicon Balong 5000 UE	17

5	References	17

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

## Verification Test Summary

Test Case: 	7.1.1.12.4.1.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D24wk37

System Simulator used:	R&S® 5G Protocol Conformance Test platform

UE used:	MediaTek MT6983, HiSilicon Balong 5000

Verification Status:	      PASS

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_12_4_NR_Common |
| --- | --- |
| Reason for change | - Step 5 of TS 38.523-1 indicates that a MAC PDU is transmitted in the DL. Hence, the pre-test conditions cannot have the Test loop function (off) - The ciphering should be set to NULL after the Preamble is completed - The RRC Reconfiguration message at Step 1 should have the MacBearerRouting filled |
| Summary of change | - TTCN code is added to complete the preamble with TESTModeA_ON with the resulting state in STATE_IDLE_1A. Further, the function f_NR5GC_RRC_ConnectedState3N_Def is called to establish the loopback set to return no data in uplink NOTE: A Prose CR for this shall be raised at the next RAN5 - Ciphering is set to NULL after the preamble is completed - MacBearerRouting is filled. |
| TTCN module | MAC_PowerSaving_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_12_4_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested ) runs on NR5GC_PTC {//DRX adaptation / SCell dormancy indication var template (value) CellGroupConfig v_CellGroupConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var SubFrameTiming_Type v_SendTiming; var SubFrameTiming_Type v_ConfigTiming; var DRB_Identity v_DRBId; //Same Pre-test conditions as in clause 7.1.1.0 except that Test loop function(Off) System information combination NR-4 and in addition NR Cell 3 is configured as NR Active Scell f_NR5GC_Init_CA(NR_4, {p_PCell, p_SCell}, p_NR_CA_Tested); //Table 7.1.0-1 f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_SCell); // Create and configure cells f_NR_CellConfig_PCell_Def(p_PCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active); // NR Cell to add is configured as NR Active Scell //Same Pre-test conditions as in clause 7.1.1.0 except that Test loop function(Off) f_NR_AS_CipheringAlgorithm_Set(nea0); f_NR5GC_Preamble (p_PCell, STATE_CONNECTED_3A, TESTMode_OFF); f_NR_TestBody_Set(true); //@siclog Step 0 siclog@ //Set the power levels according to "T1" as per Table 7.1.1.12.4.1.3.2-1/2. f_NR_SetCellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog Steps 1-2 siclog@ //SS transmits an RRCReconfiguration message. (Note 1) //The UE transmits the RRCReconfigurationComplete message v_CellGroupConfig := f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1(p_PCell, p_SCell, tsc_NR_CellGroupId_MCG); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); v_SendTiming := f_NR_GetNextSendOccasion_DL(p_PCell); v_ConfigTiming := f_SubFrameTiming_AddMilliSeconds(v_SendTiming, 5); // 5 ms later //configure cells f_NR_ConfigPCell_AddRelSCells (p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_ConfigTiming), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_CellGroupConfig, cs_TimingInfo_NR(v_ConfigTiming)); // Transmit RRCReconfiguration with condition SCell_Add to UE f_NR_SendRRCReconfiguration(p_PCell, tsc_NR_RbId_SRB1, -, v_V1530Ext, -, -, cs_TimingInfo_NR(v_SendTiming)); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); f_TC_7_1_1_12_4_NR_Steps3_9(p_PCell, p_SCell, v_DRBId); f_NR_TestBody_Set(false); //Postamble f_NR5GC_Rel_1SCell(p_PCell, p_SCell); f_NR_Postamble(p_PCell, STATE_CONNECTED_3A); } //end of f_TC_7_1_1_12_4_NR_Common

After Change:

function f_TC_7_1_1_12_4_NR_Common (NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, NR_CA_Tested_Type p_NR_CA_Tested ) runs on NR5GC_PTC {//DRX adaptation / SCell dormancy indication var template (value) CellGroupConfig v_CellGroupConfig; var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext; var SubFrameTiming_Type v_SendTiming; var SubFrameTiming_Type v_ConfigTiming; var DRB_Identity v_DRBId; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; //WA# //Same Pre-test conditions as in clause 7.1.1.0 except that Test loop function(Off) System information combination NR-4 and in addition NR Cell 3 is configured as NR Active Scell f_NR5GC_Init_CA(NR_4, {p_PCell, p_SCell}, p_NR_CA_Tested); //Table 7.1.0-1 f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_SCell); // Create and configure cells f_NR_CellConfig_PCell_Def(p_PCell); f_NR_CellConfig_SCell(p_SCell,Scell_Active); // NR Cell to add is configured as NR Active Scell //Same Pre-test conditions as in clause 7.1.1.0 except that Test loop function(Off) f_NR5GC_Preamble (p_PCell, STATE_IDLE_1A, TESTModeA_ON); //WA# //WA# f_NR_AS_CipheringAlgorithm_Set(nea0); //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); //Initialise for No Header Manipulation in UL. v_SS_Drb_ConfigList := {cs_NR_SS_RadioBearer_MAC_TestMode(v_DRBId, cs_NR_MAC_TestModeInfo_NormalMode)}; //Configure f_NR5GC_RRC_ConnectedState3N_Def(p_PCell, TEST_LOOPModeA_ON, cs_UE_TestLoopModeA_NR_LB_Setup_1DRB(0, v_DRBId), -, //p_IpPduDelayTime -, -, //MAC_CellGroupConfig -, v_SS_Drb_ConfigList ); f_NR_TestBody_Set(true); //@siclog Step 0 siclog@ //Set the power levels according to "T1" as per Table 7.1.1.12.4.1.3.2-1/2. f_NR_SetCellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@siclog Steps 1-2 siclog@ //SS transmits an RRCReconfiguration message. (Note 1) //The UE transmits the RRCReconfigurationComplete message v_CellGroupConfig := f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1(p_PCell, p_SCell, tsc_NR_CellGroupId_MCG); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); v_SendTiming := f_NR_GetNextSendOccasion_DL(p_PCell); v_ConfigTiming := f_SubFrameTiming_AddMilliSeconds(v_SendTiming, 5); // 5 ms later //configure cells f_NR_ConfigPCell_AddRelSCells (p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_ConfigTiming), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_CellGroupConfig, cs_TimingInfo_NR(v_ConfigTiming)); // Transmit RRCReconfiguration with condition SCell_Add to UE f_NR_SendRRCReconfiguration(p_PCell, tsc_NR_RbId_SRB1, -, v_V1530Ext, -, -, cs_TimingInfo_NR(v_SendTiming), -, -, crs_MacBearerRouting(p_PCell)); //WA# //Retrieve DRB Id of the first PDU session v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); f_TC_7_1_1_12_4_NR_Steps3_9(p_PCell, p_SCell, v_DRBId); f_NR_TestBody_Set(false); //Postamble f_NR5GC_Rel_1SCell(p_PCell, p_SCell); f_NR_Postamble(p_PCell, STATE_CONNECTED_3A); } //end of f_TC_7_1_1_12_4_NR_Common

## Change 2

| Function name | f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1 |
| --- | --- |
| Reason for change | There are several changes required in this function to initialise the CellGroupConfig: - To initialise the DRX parameters, the MAC cell group needs to be extracted from the TTCN database and the DRX parameters initialised - The spCellConfigDedicated on the PCell needs to be configured with the DCI 2_6 search space along with searchSpacesToAddModListExt_r16 where the second entry belongs to the DCI 2_6 search space. - The additional BWP (bwp = 1) parameters of the SCell, BWP Common and BWP Dedicated need to be initialised which includes the searchSpacesToAddModList for USS and CSS |
| Summary of change | - Initialised the DRX parameters for the peer signalling to the UE via the RRC Reconfiguration message - TTCN code modifications to PCell to configure DCI 2_6 search space as well as added the entries searchSpacesToAddModListExt_r16. Created a new function f_NR_SS_SearchSpaceConfig for this (Please see Change 3). - Initialised the additional BWP parameters for the SCell |
| TTCN module | MAC_PowerSaving_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, CellGroupId p_CellGroupId ) runs on NR_BASE_PTC return template (value) CellGroupConfig {//DRX adaptation / SCell dormancy indication var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SCellConfig v_SCellConfig; var template (omit) ServingCellConfigCommon v_ServingCellConfigCommon; var template (value) BWP_DownlinkDedicated v_BWP_DownlinkDedicated; var template (value) BWP_Downlink v_BWP_Downlink; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig; var template (value) SpCellConfig v_SpCellConfig; var template (value) ControlResourceSet v_ControlResourceSet; var template (value) ServingCellConfig v_SCellConfigDedicated; var template (value) DormantBWP_Config_r16 v_DormantBWP_Config; var SearchSpace v_SearchSpaceUSS; var template (omit) PDCCH_Config.controlResourceSetToAddModList v_ControlResourceSetToAddModList; var template (omit) BWP_DownlinkDedicated.pdcch_Config v_PDCCH_Config; //Table 7.1.1.12.4.1.3.3-2 v_CellGroupConfig := f_NR_GetSCGConfig_SCellAdd(p_SCell, p_CellGroupId); //configure DRX if(f_NR_CellInfo_GetIsFR1(p_PCell)) { v_CellGroupConfig.mac_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR1); } else { v_CellGroupConfig.mac_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR2); } f_NR_CellInfo_SetMAC_CellGroupConfig(p_PCell, v_CellGroupConfig.mac_CellGroupConfig); //compiler warning may be ignored //configure DCP v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(p_PCell); v_PhysicalCellGroupConfig.dcp_Config_r16.setup := cs_DCP_Config_r16_Def; f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); f_NR_SS_ServingCellConfig(p_PCell, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, v_CellGroupConfig.mac_CellGroupConfig, v_PhysicalCellGroupConfig))); v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; //configure PDCCH in SpCellConfig as per Table 7.1.1.12.4.1.3.3-3 v_SpCellConfig := f_NR_GetSpCellConfigDef(p_PCell); v_ControlResourceSet := f_NR_CellInfo_GetControlResourceSet(p_PCell, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_ControlResourceSet.controlResourceSetId := tsc_NR_ControlResourceSetId; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.controlResourceSetToAddModList := {v_ControlResourceSet}; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModListExt_r16 := {cs_SearchSpaceExt_r16_Dci_Format2_6}; v_CellGroupConfig.spCellConfig := v_SpCellConfig; //configure ServingCellConfigCommon as per Table 7.1.1.12.4.1.3.3-4 v_ServingCellConfigCommon := f_NR_GetServingCellConfigCommon_CA(p_SCell); //configure BPW to add as per Table 7.1.1.12.4.1.3.3-6 v_SearchSpaceUSS := f_NR_CellInfo_GetSearchSpaceConfigAtUE(p_SCell, tsc_NR_SearchSpaceId_USS, tsc_NR_BWP_Id); v_ControlResourceSetToAddModList := f_NR_GetPDCCH_ConfigControlResourceSet(p_SCell); v_PDCCH_Config := {setup := cs_38508_PDCCH_Config (v_SearchSpaceUSS, -, v_ControlResourceSetToAddModList)}; v_BWP_DownlinkDedicated := cs_BWP_DownlinkDedicated_Def(v_PDCCH_Config); v_BWP_Downlink := cs_BWP_Downlink_Def (tsc_NR_BWP_Id1, omit, v_BWP_DownlinkDedicated); v_BWP_Downlink.bwp_Common.pdcch_ConfigCommon := omit; //@sic R5-235285 sic@ //configure serving cell as per Table 7.1.1.12.4.1.3.3-5 v_SCellConfigDedicated := f_NR_GetServingCellConfig(p_SCell); v_SCellConfigDedicated.uplinkConfig := omit; //@sic R5-235285 sic@ v_SCellConfigDedicated.initialDownlinkBWP.radioLinkMonitoringConfig := omit; //@sic R5-235285 sic@ v_SCellConfigDedicated.downlinkBWP_ToAddModList := {v_BWP_Downlink}; v_DormantBWP_Config := cs_NR_DormantBWP_Config_r16_Setup_OutsideActiveTime(tsc_NR_BWP_Id1); v_SCellConfigDedicated.dormantBWP_Config_r16.setup := v_DormantBWP_Config; //v_SCellConfigDedicated.pdsch_ServingCellConfig.setup := {pucch_Cell := 1}; //@sic R5-225297, R5-235285 sic@ if (ispresent (v_SCellConfigDedicated)) //compiler warning may be ignored due to check { v_SCellConfig := cs_SCellConfig_Def(v_ServingCellConfigCommon,v_SCellConfigDedicated, tsc_NR_SCellIndex_1); v_CellGroupConfig.sCellToAddModList := {v_SCellConfig}; } return v_CellGroupConfig } //f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1

After Change:

function f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, CellGroupId p_CellGroupId ) runs on NR_BASE_PTC return template (value) CellGroupConfig {//DRX adaptation / SCell dormancy indication var template (value) CellGroupConfig v_CellGroupConfig; var template (value) SCellConfig v_SCellConfig; var template (omit) ServingCellConfigCommon v_ServingCellConfigCommon; var template (value) BWP_DownlinkDedicated v_BWP_DownlinkDedicated; var template (value) BWP_Downlink v_BWP_Downlink; var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig; var template (value) SpCellConfig v_SpCellConfig; var template (value) ControlResourceSet v_ControlResourceSet; var template (value) ServingCellConfig v_SCellConfigDedicated; var template (value) DormantBWP_Config_r16 v_DormantBWP_Config; var template (value) SearchSpace v_SearchSpaceUSS; //WA# var template (omit) PDCCH_Config.controlResourceSetToAddModList v_ControlResourceSetToAddModList; //WA# var template (omit) ServingCellConfig v_ServingCellConfig; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var template(value) SearchSpace v_SearchSpace; var template (value) DMRS_DownlinkConfig v_DMRS_DownlinkConfig; var template (value) PDSCH_TimeDomainResourceAllocationList v_PDSCH_TimeDomainResourceAllocationList; var template (value) NR_SearchSpace_NrofCandidates_Type v_NrofCandidates; var template (value) NR_DownlinkBWP_Type v_DownlinkBWP ; var template (omit) NR_BWP_DownlinkDedicated_RadioLinkMonitoringConfig_Type v_RadioLinkMonitoringConfig; var template (value) BWP_DownlinkCommon v_BWP_DownlinkCommon; var SubcarrierSpacing v_SubcarrierSpacing; var BWP.locationAndBandwidth v_LocationAndBandwidth; var template (value) SearchSpace v_SearchSpaceCSS; var B45_Type v_Coreset_FrequencyDomain := '111100000000000000000000000000000000000000000'B; var template (value) PDCCH_Config v_PDCCH_Config; var template (value) PDSCH_Config v_PDSCH_Config; var SSB_Index v_SSB_Index := f_NR_CellInfo_GetSSB_Index(p_SCell); //Table 7.1.1.12.4.1.3.3-2 v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_PCell); //WA# //configure DRX if(f_NR_CellInfo_GetIsFR1(p_PCell)) { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR1); //WA# } else { v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup (cs_38508_DRX_Config_FR2); //WA# } f_NR_CellInfo_SetMAC_CellGroupConfig(p_PCell, v_MAC_CellGroupConfig); //WA# //WA# v_CellGroupConfig := cs_38508_CellGroupConfig (f_NR_CellInfo_Get_CellGroupId(p_PCell), omit, omit, omit, omit, omit ); v_CellGroupConfig.mac_CellGroupConfig := v_MAC_CellGroupConfig; //WA# //configure DCP v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(p_PCell); v_PhysicalCellGroupConfig.dcp_Config_r16.setup := cs_DCP_Config_r16_Def; f_NR_CellInfo_SetPhysicalCellGroupConfig(nr_Cell1, v_PhysicalCellGroupConfig); f_NR_SS_ServingCellConfig(p_PCell, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); //WA# v_CellGroupConfig.physicalCellGroupConfig := v_PhysicalCellGroupConfig; //configure PDCCH in SpCellConfig as per Table 7.1.1.12.4.1.3.3-3 v_ServingCellConfig := f_NR_GetServingCellConfig(p_PCell); //WA# v_SpCellConfig := f_NR_GetSpCellConfigDef(p_PCell, v_ServingCellConfig); //WA# v_ControlResourceSet := f_NR_CellInfo_GetControlResourceSet(p_PCell, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_ControlResourceSet.controlResourceSetId := tsc_NR_ControlResourceSetId; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.controlResourceSetToAddModList := {v_ControlResourceSet}; //WA# v_SearchSpace := f_NR_CellInfo_GetSearchSpaceConfigAtUE(p_PCell, tsc_NR_ControlResourceSetId, tsc_NR_BWP_Id); v_SearchSpace.searchSpaceId := tsc_NR_SearchSpaceId_DCI_2_6; v_SearchSpace.controlResourceSetId := 1; v_SearchSpace.searchSpaceType := cs_NR_SearchSpace_SearchSpaceType_CSS_AllOmit; v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModList[1] := v_SearchSpace; //@sic R5s230001 Ch1.3, R5-231902 sic@ v_SpCellConfig.spCellConfigDedicated.initialDownlinkBWP.pdcch_Config.setup.searchSpacesToAddModListExt_r16 := {cs_SearchSpaceExt_r16_AllOmit, cs_SearchSpaceExt_r16_Dci_Format2_6}; v_SpCellConfig.spCellConfigDedicated.tag_Id := 0; v_DownlinkBWP := f_NR_CellInfo_GetDownlinkBWP(p_PCell, tsc_NR_BWP_Id); v_DownlinkBWP.Pdcch.SearchSpaceArray[1].SearchSpaceConfigAtUE.R15 := v_SearchSpace; v_DownlinkBWP.Pdcch.SearchSpaceArray[0].SearchSpaceConfigAtUE.R16 := cs_SearchSpaceExt_r16_AllOmit; v_DownlinkBWP.Pdcch.SearchSpaceArray[1].SearchSpaceConfigAtUE.R16 := cs_SearchSpaceExt_r16_Dci_Format2_6; f_NR_CellInfo_SetDownlinkBWP(p_PCell, v_DownlinkBWP, tsc_NR_BWP_Id); //@sic R5s230001 Ch.1.3 sic@ if(f_NR_CellInfo_GetIsFR1(p_PCell)) { v_DMRS_DownlinkConfig := cs_38508_DMRS_DownlinkConfig_FR1; v_PDSCH_TimeDomainResourceAllocationList := cs_38508_PDSCH_TimeDomainResourceAllocationList_FR1; v_NrofCandidates := cs_38508_NrofCandidates_Def; } else { v_DMRS_DownlinkConfig := cs_38508_DMRS_DownlinkConfig_FR2; v_PDSCH_TimeDomainResourceAllocationList := cs_38508_PDSCH_TimeDomainResourceAllocationList_FR2; v_NrofCandidates := cds_38508_NrofCandidates_FR2; } v_CellGroupConfig.spCellConfig := v_SpCellConfig; f_NR_SS_SearchSpaceConfig(p_PCell, -, v_DMRS_DownlinkConfig, v_PDSCH_TimeDomainResourceAllocationList, v_NrofCandidates, v_SearchSpace); v_PDSCH_Config := cs_38508_PDSCH_Config(v_DMRS_DownlinkConfig, -, v_SSB_Index); // Table 7.1.1.8.1.3.3-2C: PDSCH-Config v_PDSCH_Config.pdsch_TimeDomainAllocationList.setup := v_PDSCH_TimeDomainResourceAllocationList; //configure ServingCellConfigCommon as per Table 7.1.1.12.4.1.3.3-4 v_ServingCellConfigCommon := f_NR_GetServingCellConfigCommon_CA(p_SCell); v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(p_SCell); v_LocationAndBandwidth := 1381; v_BWP_DownlinkCommon := cs_BWP_DownlinkCommon_GenericParametersOnly(cs_38508_BWP(v_SubcarrierSpacing, v_LocationAndBandwidth)); //configure BPW to add as per Table 7.1.1.12.4.1.3.3-6 v_ControlResourceSetToAddModList := { cs_38508_ControlResourceSet(9, -, v_Coreset_FrequencyDomain) }; v_SearchSpaceUSS := cs_38508_SearchSpace(37, 9, cs_NR_SearchSpace_SearchSpaceType_USS(formats0_1_And_1_1), cs_NR_SearchSpace_NrofCandidates_AggregationLevel2Only); v_PDCCH_Config := cs_38508_PDCCH_Config(v_SearchSpaceUSS, -, v_ControlResourceSetToAddModList); v_SearchSpaceCSS := cs_38508_SearchSpace(30, 9, cs_NR_SearchSpace_SearchSpaceType_CSS, cs_NR_SearchSpace_NrofCandidates_AggregationLevel2Only); v_PDCCH_Config.searchSpacesToAddModList[1]:= v_SearchSpaceCSS; v_BWP_DownlinkDedicated := cs_38508_BWP_DownlinkDedicated(v_PDCCH_Config, v_PDSCH_Config); v_BWP_Downlink := cs_BWP_Downlink_Def (tsc_NR_BWP_Id1, v_BWP_DownlinkCommon, v_BWP_DownlinkDedicated); v_BWP_Downlink.bwp_Common.pdcch_ConfigCommon := omit; //@sic R5-235285 sic@ //configure serving cell as per Table 7.1.1.12.4.1.3.3-5 v_SCellConfigDedicated := f_NR_GetServingCellConfig(p_SCell); v_SCellConfigDedicated.uplinkConfig := omit; //@sic R5-235285 sic@ v_SCellConfigDedicated.initialDownlinkBWP.radioLinkMonitoringConfig := omit; //@sic R5-235285 sic@ v_SCellConfigDedicated.downlinkBWP_ToAddModList := {v_BWP_Downlink}; v_DormantBWP_Config := cs_NR_DormantBWP_Config_r16_Setup_OutsideActiveTime(tsc_NR_BWP_Id1); v_SCellConfigDedicated.dormantBWP_Config_r16.setup := v_DormantBWP_Config; //v_SCellConfigDedicated.pdsch_ServingCellConfig.setup := {pucch_Cell := 1}; //@sic R5-225297, R5-235285 sic@ if (ispresent (v_SCellConfigDedicated)) //compiler warning may be ignored due to check { v_SCellConfig := cs_SCellConfig_Def(v_ServingCellConfigCommon,v_SCellConfigDedicated, tsc_NR_SCellIndex_1); v_CellGroupConfig.sCellToAddModList := {v_SCellConfig}; } return v_CellGroupConfig } //f_TC_7_1_1_12_4_NR_GetCellGroupConfigForStep1

## Change 3

| Function name | f_NR_SS_SearchSpaceConfig |
| --- | --- |
| Reason for change | Refer to Change 2 |
| Summary of change | New Change |
| TTCN module | NR_ConfigurationSteps.ttcn |

function f_NR_SS_SearchSpaceConfig(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now, template (value) DMRS_DownlinkConfig p_DMRS_DownlinkConfig, template (value) PDSCH_TimeDomainResourceAllocationList p_PDSCH_TimeDomainResourceAllocationList, template (value) NR_SearchSpace_NrofCandidates_Type p_NrofCandidates, template(value) SearchSpace p_SearchSpace) runs on NR_BASE_PTC { var SubcarrierSpacing v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(p_NR_CellId); var NR_FrequencyBand_Type v_NR_FrequencyBand := f_NR_CellInfo_GetFrequencyBand(p_NR_CellId); var BWP.locationAndBandwidth v_LocationAndBandwidth := f_NR_InitLocationAndBandwidth(v_NR_FrequencyBand.FrequencyBandIndicator, v_NR_FrequencyBand.FrequencyInfoDL.scs_SpecificCarrierList[0].subcarrierSpacing, v_NR_FrequencyBand.ChannelBandwidth); f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_SS_Config_REQ(p_NR_CellId, p_TimingInfo, p_DMRS_DownlinkConfig, p_PDSCH_TimeDomainResourceAllocationList, p_NrofCandidates, v_SubcarrierSpacing, v_NR_FrequencyBand,v_LocationAndBandwidth, p_SearchSpace)); }

## Change 4

| Template name | cads_NR_SS_Config_REQ |
| --- | --- |
| Reason for change | This is needed for newly introduced function f_NR_SS_SearchSpaceConfig (Refer to Change 3) |
| Summary of change | New Change |
| TTCN module | NR_CellCfg_Templates.ttcn |

template (value) NR_SYSTEM_CTRL_REQ cads_NR_SS_Config_REQ(NR_CellId_Type p_NR_CellId, template (value) TimingInfo_Type p_TimingInfo, template (value) DMRS_DownlinkConfig p_DMRS_DownlinkConfig, template (value) PDSCH_TimeDomainResourceAllocationList p_PDSCH_TimeDomainResourceAllocationList, template (value) NR_SearchSpace_NrofCandidates_Type p_NrofCandidates, template (value) SubcarrierSpacing p_SubcarrierSpacing, template (value) NR_FrequencyBand_Type p_NR_FrequencyBand, BWP.locationAndBandwidth p_LocationAndBandwidth, template(value) SearchSpace p_SearchSpace ) modifies cas_NR_CellConfigAllOmit_REQ := { Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo), Request := { Cell := { AddOrReconfigure := { PhysicalLayer := { Common := omit, Downlink := { FrequencyInfoDL := omit, SSPbchBlock := omit, PdschCellLevelConfig := omit, BWPs := { ActiveBWP := omit, BwpArray := { { Id := tsc_NR_BWP_Id, BWP := { R15 := cs_38508_BWP(p_SubcarrierSpacing, p_LocationAndBandwidth) }, Pdcch := { SearchSpaceArray := {cs_NR_BWP_SearchSpaceConfig_SearchSpaceZero(p_SubcarrierSpacing), cs_NR_BWP_SearchSpaceConfig_CSS(p_NrofCandidates), cs_NR_BWP_SearchSpaceConfig_USS(p_NrofCandidates), cs_NR_BWP_SearchSpaceConfig_SISS(p_NrofCandidates), cs_NR_BWP_SearchSpaceConfig_CSS_DCI_2_6(p_SearchSpace)}, CoresetArray := { cs_NR_ASN1_ControlResourceSet(cs_NR_ControlResourceSetZeroAtSS(0)), // CORESET#0 according to 38.211 clause 7.3.2.2 with shift index being the physical layer cell id @sic R5-190786, RP-182523 sic@ */ cs_NR_ASN1_ControlResourceSet(cs_38508_ControlResourceSet()) // default CORESET as according to 38.508-1 Table 4.6.3-17 }, RelativeTxPower := cs_NR_PDCCH_EPREs_Def, Coreset0_OffsetRBs := p_NR_FrequencyBand.Coreset0_OffsetRBs }, Pdsch := { ConfigCommon := { R15 := cs_38508_PDSCH_ConfigCommon(p_PDSCH_TimeDomainResourceAllocationList) }, ConfigDedicated := { R15 := cs_38508_PDSCH_Config(p_DMRS_DownlinkConfig, omit, tsc_NR_SSB_Index1) }, RelativeTxPower := cs_NR_PDSCH_EPREs_Def }, Sps := omit, SL_Pdcch := omit, NCD_SSPbchBlock := omit } }, InitialBWP_RedCap := omit }, CsiConfig := omit }, Uplink := omit } } } } };

## Change 5

| Function name | f_TC_7_1_1_12_4_NR_Steps3_9 |
| --- | --- |
| Reason for change | - The enabling of the reception of the HARQ ACK on PCell is not needed as this would then feedback the HARQ ACK for the SCell Activation MAC CE at Step 3 which is not expected - As there is no uplink data expected, there is no need to start the default onSR UL grant config - It is recommended to have a small delay after f_SS_CellConfig_ActivateSCell_CA before enabling HARQ ACK reception on PCell so that it does not get sent to TTCN - The enabling of the reception of the HARQ ACK on the SCell is not necessary as the Steps 6 and 9 check for reception of HARQ ACK on the PCell. - The Long DRX cycle should be 10240 rather than 1024 - As the sizeDCI2_6_r16 is setup as 6 entries, the local end SS configuration should have 6 entries when the Wakeup and SCellDormancyIndicationBitValue is configured - A delay of 11.0s needs to happen so that the complete long drx cycle of 10.240s is finished before Step 7 is started. - The DRB transmissions at Step 5 and Step 8 should happen on BWP#0 with data routed via the PDCCH of the SCell which does not necessitate the switching of the BWP. |
| Summary of change | - Removed the enabling of the HARQ reception - Removed the starting of the UL Grant configuration - Added a delay of 100ms - Removed the enabling of the HARQ reception on the SCell - Corrected the long DRX cycle to 10240 - Used a new template called cs_NR_DciFormat_2_6_Block_List_6Items which matches the sizeDCI2_6_r16 - A delay of 11.0s is added to ensure the DRX long cycle is completed so that the DCI 2-6 is transmitted within the ps-offset time before the start of next long DRX drx-onDurationTimer on NR Cell 1 - Removed the configuration for the BWP switch. |
| TTCN module | MAC_PowerSaving_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_12_4_NR_Steps3_9(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, DRB_Identity p_DRBId ) runs on NR_BASE_PTC {//DRX adaptation / SCell dormancy indication var UInt_Type v_LongDRXCycle; var UInt_Type v_DrxStartOffset; var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_SendTiming; var SubFrameTiming_Type v_ConfigTiming; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; timer t_Watchdog := 5.0; f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); f_NR_ULGrantConfiguration_Start(p_SCell); //@siclog Step 3 siclog@ //The SS transmits a SCell Activation MAC-CE to activate SCell (NR Cell 3). f_SS_CellConfig_ActivateSCell_CA(p_PCell); // suppress number of DL retransmissions: f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(p_PCell); // to configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(p_PCell, cds_NR_SystemIndCtrl_UL_HARQ(enable)); f_NR_SS_SystemIndCtrlConfig(p_SCell, cds_NR_SystemIndCtrl_UL_HARQ(enable)); v_LongDRXCycle := 1024; v_DrxStartOffset := 0; v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_SendTiming, p_PCell, v_LongDRXCycle, v_DrxStartOffset); v_ConfigTiming := f_SubFrameTiming_AddMilliSeconds(v_WakeUpTiming, 3); //@siclog Step 4 siclog@ //The SS transmits DCI 2-6 within ps-Offset time before the start of next long DRX drx-onDurationTimer on NR Cell 1. (Note 3) //The Wake-up indication is value 1 and the SCell dormancy indication is value 0 in the DCI 2-6 f_NR_SS_CommonCellConfig(p_PCell, cads_NR_DciTrigger_WakeUP_REQ(p_PCell, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_5Items('1'B, '0'B))); v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(0, p_DRBId); //@siclog Steps 5 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the initial BWP (BWP#0) when the Drx-onDurationTimer is running. f_NR_SS_ConfigureBWPs_DL(p_SCell, cs_TimingInfo_NR(v_ConfigTiming), tsc_NR_BWP_Id); //@sic R5-225297 sic@ DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); //@siclog Step 6 siclog@ //Check: Does the UE transmit a HARQ ACK on the PCell for the DL MAC PDU in Step 5 within 5 seconds t_Watchdog.start; alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_PCell, ack)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; } [] t_Watchdog.timeout {} } v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_SendTiming, p_PCell, v_LongDRXCycle, v_DrxStartOffset); v_ConfigTiming := f_SubFrameTiming_AddMilliSeconds(v_WakeUpTiming, 3); //@siclog Step 7 siclog@ //The SS transmits DCI 2-6 within the ps-offset time before the start of next long DRX drx-onDurationTimer on NR Cell 1. (Note 4) //Note 4: The Wake-up indication is value 1 and the SCell dormancy indication is value 1 in the DCI 2-6 f_NR_SS_CommonCellConfig(p_PCell, cads_NR_DciTrigger_WakeUP_REQ(p_PCell, cs_TimingInfo_NR(v_WakeUpTiming), cs_NR_DciFormat_2_6_Block_List_5Items('1'B, '1'B))); //@siclog Steps 8 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the active BWP (BWP#0) when the Drx-onDurationTimer is running. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(1, p_DRBId); f_NR_SS_ConfigureBWPs_DL(p_SCell, cs_TimingInfo_NR(v_ConfigTiming), tsc_NR_BWP_Id); DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_SCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); //@siclog Steps 9 siclog@ //Check: Does the UE transmit a HARQ ACK on the PCell NR Cell 1 for the DL MAC PDU in Step 8? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_PCell, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); }//f_TC_7_1_1_12_4_NR_Steps3_9

After Change:

function f_TC_7_1_1_12_4_NR_Steps3_9(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell, DRB_Identity p_DRBId ) runs on NR_BASE_PTC {//DRX adaptation / SCell dormancy indication var UInt_Type v_LongDRXCycle; var UInt_Type v_DrxStartOffset; var SubFrameTiming_Type v_WakeUpTiming; var SubFrameTiming_Type v_SendTiming; var SubFrameTiming_Type v_ConfigTiming; var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL; timer t_Watchdog := 5.0; //@siclog Step 3 siclog@ //The SS transmits a SCell Activation MAC-CE to activate SCell (NR Cell 3). f_SS_CellConfig_ActivateSCell_CA(p_PCell); // suppress number of DL retransmissions: f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(p_PCell); f_Delay(0.1); //WA# // to configure SS to report reception of HARQ ACK or NACK f_NR_SS_SystemIndCtrlConfig(p_PCell, cds_NR_SystemIndCtrl_UL_HARQ(enable));//WA# v_LongDRXCycle := 10240; //WA# v_DrxStartOffset := 0; v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_SendTiming, p_PCell, v_LongDRXCycle, v_DrxStartOffset); v_ConfigTiming := f_SubFrameTiming_AddMilliSeconds(v_WakeUpTiming, 3); //@siclog Step 4 siclog@ //The SS transmits DCI 2-6 within ps-Offset time before the start of next long DRX drx-onDurationTimer on NR Cell 1. (Note 3) //The Wake-up indication is value 1 and the SCell dormancy indication is value 0 in the DCI 2-6 f_NR_SS_CommonCellConfig(p_PCell, cads_NR_DciTrigger_WakeUP_REQ(p_PCell, cs_TimingInfo_NR(v_WakeUpTiming),cs_NR_DciFormat_2_6_Block_List_6Items('1'B, '0'B))); //WA# v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(0, p_DRBId); //@siclog Steps 5 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the initial BWP (BWP#0) when the Drx-onDurationTimer is running. //WA# DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_PCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); //@siclog Step 6 siclog@ //Check: Does the UE transmit a HARQ ACK on the PCell for the DL MAC PDU in Step 5 within 5 seconds t_Watchdog.start; alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_PCell, ack)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; } [] t_Watchdog.timeout {} } f_Delay(11.0); //WA# v_WakeUpTiming := f_NR_GetWakeUpSignalTiming (v_SendTiming, p_PCell, v_LongDRXCycle, v_DrxStartOffset); v_ConfigTiming := f_SubFrameTiming_AddMilliSeconds(v_WakeUpTiming, 3); //@siclog Step 7 siclog@ //The SS transmits DCI 2-6 within the ps-offset time before the start of next long DRX drx-onDurationTimer on NR Cell 1. (Note 4) //Note 4: The Wake-up indication is value 1 and the SCell dormancy indication is value 1 in the DCI 2-6 f_NR_SS_CommonCellConfig(p_PCell, cads_NR_DciTrigger_WakeUP_REQ(p_PCell, cs_TimingInfo_NR(v_WakeUpTiming), cs_NR_DciFormat_2_6_Block_List_6Items('1'B, '1'B))); //WA# //@siclog Steps 8 siclog@ //The SS indicates a new transmission on PDCCH of SCell and transmits a MAC PDU on the active BWP (BWP#0) when the Drx-onDurationTimer is running. v_MAC_PDUListDL := f_NR_GetMAC_PDUListDL(1, p_DRBId); //WA# DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(p_PCell, p_DRBId, cs_TimingInfo_NR(v_SendTiming), v_MAC_PDUListDL, -, -, crs_MacBearerRouting(p_SCell))); //@siclog Steps 9 siclog@ //Check: Does the UE transmit a HARQ ACK on the PCell NR Cell 1 for the DL MAC PDU in Step 8? SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(p_PCell, ack)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); }//f_TC_7_1_1_12_4_NR_Steps3_9

## Change 6

| Function name | cs_NR_DciFormat_2_6_Block_List_6Items |
| --- | --- |
| Reason for change | Refer to change 5 |
| Summary of change | New Change |
| TTCN module | MAC_PowerSaving_TC_Common_NR.ttcn |

template (value) NR_DciFormat_2_6_Block_List_Type cs_NR_DciFormat_2_6_Block_List_6Items ( B1_Type p_WakeUP := '1'B, bitstring p_SCellDormacyIndicationBitValue := '0'B ) := { cs_NR_DciFormat_2_6_Block_Type(p_WakeUP, cs_NR_DciFormat_2_6_SCellDormancyIndication_Value(p_SCellDormacyIndicationBitValue)), cs_NR_DciFormat_2_6_Block_Type, cs_NR_DciFormat_2_6_Block_Type, cs_NR_DciFormat_2_6_Block_Type, cs_NR_DciFormat_2_6_Block_Type, cs_NR_DciFormat_2_6_Block_Type }

# 3 Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “CA_n78c” using NEA0/NIA1 algorithms.

# Execution Log Files

## MediaTek MT6983 UE

The MediaTek MT6983 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_7_1_1_12_4_1_NR5GC_FR1_MTK.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

## HiSilicon Balong 5000 UE

The HiSilicon Balong 5000 UE passed this test case on R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

Test case execution log file:
TC_7_1_1_12_4_1_NR5GC_FR1_HiSi.log
(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file

# References

| [1] | R5s240687: Supporting information for addition of NR5GC testcase 7.1.1.12.4.1 in FR1 |
| --- | --- |