3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240688

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4869 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE RACS testcase 9.2.5.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | At Steps 1-14 the UE has performed successful Attach on Eutra Cell A, and has not been power cycled. Thus, a full security context is established. Hence the UE will transmit the TAU REQ at Step 15Ab1 integrity protected and ciphered. However, the current TTCN implementation expects the TAU REQ at Step 15Ab1 to be only integrity protected. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the TTCN to expect the TAU REQ to be integrity protected and ciphered. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant shall fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | fl_TC_9_2_5_3_Body() |
| --- | --- |
| Reason for change | At Steps 1-14 the UE has performed successful Attach on Eutra Cell A, and has not been power cycled. Thus, a full security context is established. Hence the UE will transmit the TAU REQ at Step 15Ab1 integrity protected and ciphered. However, the current TTCN implementation expects the TAU REQ at Step 15Ab1 to be only integrity protected. This needs to be addressed. |
| Summary of change | Updated the TTCN to expect the TAU REQ to be integrity protected and ciphered. Please see below. |
| TTCN module | NAS_RACS.ttcn |

Before Change:

function fl_TC_9_2_5_3_Body() runs on EUTRA_PTC { <<SKIPPED CODE>> // @siclog "Step 15Ab1" siclog@ // ELSE Check: Does the UE indicate UE radio Capability ID not available in UE radio capability availability indication IE in TRACKING AREA UPDATE REQUEST message? SRB.receive(car_SRB2_NasPdu_IND(eutra_CellA, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cdr_TAU_Request_ActiveFlag(v_EpsUpdate_TypeValue, f_EUTRA_SecurityKSIasme_Get(), cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType)))) -> value v_ReceivedAsp;//@sic R5s240354 sic@ v_NAS_Ind:= v_ReceivedAsp.Signalling.Nas[0]; <<SKIPPED CODE>> }

After Change:

function fl_TC_9_2_5_3_Body() runs on EUTRA_PTC { <<SKIPPED CODE>> // @siclog "Step 15Ab1" siclog@ // ELSE Check: Does the UE indicate UE radio Capability ID not available in UE radio capability availability indication IE in TRACKING AREA UPDATE REQUEST message? SRB.receive(car_SRB2_NasPdu_IND(eutra_CellA, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cdr_TAU_Request_ActiveFlag(v_EpsUpdate_TypeValue, f_EUTRA_SecurityKSIasme_Get(), cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType)))) -> value v_ReceivedAsp;//@sic R5s240354 sic@ WA#WI-1317171 v_NAS_Ind:= v_ReceivedAsp.Signalling.Nas[0]; <<SKIPPED CODE>> }