3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240704

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3601 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.3.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation, access barring is removed at Step 3 and function f_NR_WaitModificationPeriods is called at Step 4 so that the UE can read updated sysino. This funciton enforces a timer that is 2.1*Modification period. If a UE reads the updated sysinfo after first modification period, and tried to initiate a rrcSetupREQ with cause emergency then the testcase fails because this situation is not handled. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Created a default handler function that “absorbs” any rrcSetupREQ received during the time when function f_NR_WaitModificationPeriods is active. Also, included testcase 11_3_7 in function f_GetTestcaseAttrib_EarlyContentionResolution so that pre-configured rrcSetup message is not sent by SS upon reception of a rrcSetupREQ in the default handler. Please see below for detailed implementation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | fl_TC_11_3_7_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, access barring is removed at Step 3 and function f_NR_WaitModificationPeriods is called at Step 4 so that the UE can read updated sysino. This funciton enforces a timer that is 2.1*Modification period. If a UE reads the updated sysinfo after first modification period, and tried to initiate a rrcSetupREQ with cause emergency then the testcase fails because this situation is not handled. |
| Summary of change | Declared a default variable and activated it before call to function f_NR_ModifySysinfo, and deactivated it after function f_NR_WaitModificationPeriods. Created a default handler function that “absorbs” any rrcSetupREQ received during the time when function f_NR_WaitModificationPeriods is active. |
| TTCN module | UAC_NR5GC.ttcn |

Before Change:

<<SKIPPED CODE>> var SubFrameTiming_Type v_TimingOfLocalRelease; timer t_WaitTime; // @sic R5-213504, R5s210424 sic@ timer t_T390; //@siclog "Step 1" siclog@ //Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). //NOTE 1: This could be done by e.g. MMI or AT command. f_UT_RequestIMSEmergencyCall (UT, -, LOCAL_CNF_REQUIRED); // @sic R5s230789 sic@ t_T390.start(v_T390Max); // @sic R5-213504, R5s210424 sic@ //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' within 30 s'? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_TimerValue, emergency, "Step 2"); //@siclog "Step 3" siclog@ //@sic R5-211410, R5-213504, R5s210424 sic@ //SS modifies NR Cell 1 SIB1 to remove Access barring for AC2 and sends short message. (NOTE 2, NOTE 3) //NOTE 2: Since barring is alleviated, UE may re-attempt an emergency call, triggered at step 1, automatically after T390 stop/expiry without manual intervention. //NOTE 3: Steps 3 to 9 are there to ensure that UE under test supports emergency call and is not initiating it at Step 2 due to access barring for AC2. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); f_NR_ModifySysinfo(nr_Cell1); //@siclog "Step 4" siclog@ //@sic R5-211410 sic@ //Wait for 2.1* modification period to allow the new system information to take effect f_NR_WaitModificationPeriods(nr_Cell1); //@siclog "Step 5" siclog@ //@sic R5-211410, R5-213504, R5s210424, R5-224716 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> var SubFrameTiming_Type v_TimingOfLocalRelease; var default v_MyDefaultVar := null; //WA#WI1363573 WA#11_3_7 timer t_WaitTime; // @sic R5-213504, R5s210424 sic@ timer t_T390; //@siclog "Step 1" siclog@ //Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). //NOTE 1: This could be done by e.g. MMI or AT command. f_UT_RequestIMSEmergencyCall (UT, -, LOCAL_CNF_REQUIRED); // @sic R5s230789 sic@ t_T390.start(v_T390Max); // @sic R5-213504, R5s210424 sic@ //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' within 30 s'? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_TimerValue, emergency, "Step 2"); //@siclog "Step 3" siclog@ //@sic R5-211410, R5-213504, R5s210424 sic@ //SS modifies NR Cell 1 SIB1 to remove Access barring for AC2 and sends short message. (NOTE 2, NOTE 3) //NOTE 2: Since barring is alleviated, UE may re-attempt an emergency call, triggered at step 1, automatically after T390 stop/expiry without manual intervention. //NOTE 3: Steps 3 to 9 are there to ensure that UE under test supports emergency call and is not initiating it at Step 2 due to access barring for AC2. f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, omit); v_MyDefaultVar := activate(a_NR_ReceiveRRCSetupRequest(nr_Cell1)); //WA#WI1363573 WA#11_3_7 f_NR_ModifySysinfo(nr_Cell1); //@siclog "Step 4" siclog@ //@sic R5-211410 sic@ //Wait for 2.1* modification period to allow the new system information to take effect f_NR_WaitModificationPeriods(nr_Cell1); deactivate (v_MyDefaultVar); //WA#WI1363573 WA#11_3_7 //@siclog "Step 5" siclog@ //@sic R5-211410, R5-213504, R5s210424, R5-224716 sic@ <<SKIPPED CODE>>

## Change 2

| Template name | a_NR_ReceiveRRCSetupRequest |
| --- | --- |
| Reason for change | Please see change 1 |
| Summary of change | New default handler function |
| TTCN module | UAC_NR5GC.ttcn |

altstep a_NR_ReceiveRRCSetupRequest(NR_CellId_Type p_CellId) runs on NR_BASE_PTC { //WA#WI1363573 WA#11_3_7 [] SRB.receive(car_NR_SRB0_RrcPdu_IND(p_CellId, cr_38508_RRCSetupRequest(?,? ))) { repeat; } }

## Change 3

| Template name | f_GetTestcaseAttrib_EarlyContentionResolution |
| --- | --- |
| Reason for change | If the UE transmits a rrcSetupREQ as described in Change 1, then the SS will transmit a preconfigured rrcSetup message, which will result in the UE transmitting a rrcSetupComplete which will cause testcase failure. This needs to be handled. |
| Summary of change | Included testcase 11_3_7 in function f_GetTestcaseAttrib_EarlyContentionResolution so that pre-configured rrcSetup message is not sent by SS upon reception of a rrcSetupREQ in the default handler |
| TTCN module | TestcaseProperties.ttcn |

Before Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_11_NR5GC") { return true; } case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_11_3_6a_NR5GC") { return true; } case ("TC_11_7_2_NR5GC") { return true; } case ("TC_11_7_3_NR5GC") { return true; } <<SKIPPED CODE>>

After Change:

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_11_NR5GC") { return true; } case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_11_3_6a_NR5GC") { return true; } case ("TC_11_3_7_NR5GC") { return true; } //WA#WI1363573 WA#11_3_7 case ("TC_11_7_2_NR5GC") { return true; } case ("TC_11_7_3_NR5GC") { return true; } <<SKIPPED CODE>>