3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240582

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3539 | rev | - | Current version: | 18.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 7.1.1.4.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1 According to TS38.523-1Table 7.1.1.4.1.0-1, startSymbolAndLength should set with S=2. 2 According to TS 38.211 Table 6.4.1.1.3-3 and Table 7.4.1.1.2-3,function fl_NR_TBS_Calculation_NoOfREsPerPrb_Step1 need to by modified. 3 According to TS 38.212, 5.2.2 Low density parity check coding, the calculation of v_EffectiveCodeRate didn’t take the segmentation into account in current algorithm. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1 Pass 2 as p_StartSymbol in f_NR_TBS_Calculation. 2 In case TypeA, set v_ActualNumberOfSymbolsWithDMRS according to the actual No Of Symbols. 3 Modify the algorithm. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.4.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	6

2.3	Change 3	9

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Ting Gao
	gaoting@starpointcomm.com

# Corrections required

## Change 1

| Functionname | f_NR_TBS_Calculation |
| --- | --- |
| Reason for change | According to TS 38.212, 5.2.2 Low density parity check coding The input bit sequence to the code block segmentation is denoted by, where . If is larger than the maximum code block size , segmentation of the input bit sequence is performed and an additional CRC sequence of bits is attached to each code block. The calculation of v_EffectiveCodeRate didn’t take the segmentation into account in current algorithm. |
| Summary of change | 1 Define a new veriable v_Kcb; 2 Modify the algorithm; 3 Pass in p_StartSymbol for fl_NR_TBS_Calculation_NoOfREsPerPrb_Step1 for change 2 and change3; |
| TTCN module | NR_TBS_Calculation.ttcn |
| MCC160 Comment | Accepted as improvement. |

Before Change:

function f_NR_TBS_Calculation(template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, integer p_NoOfSymbols, integer p_NoOfRBs, integer p_Imcs, boolean p_IsDownlink := false) return integer { var NR_TBS_CalculationParameters_Type v_Parameters := valueof(p_CalculationParameters); var NR_MCS_TableEntry_Type v_MCS_TableEntry := v_Parameters.MCS_Table[p_Imcs]; var float v_CodeRate := v_MCS_TableEntry.CodeRate_x_1024 / 1024.0; var integer v_Qm := v_MCS_TableEntry.Qm; var integer v_NoOfREsPerPrb; var integer v_NoOfREs; var float v_Nmax; var float v_Ninfo; var integer v_TBS; var float v_EffectiveCodeRate; var integer v_CRCSize; // Step 1: v_NoOfREsPerPrb := fl_NR_TBS_Calculation_NoOfREsPerPrb_Step1(v_Parameters, p_NoOfSymbols); v_NoOfREs := f_MinInt(156, v_NoOfREsPerPrb) * p_NoOfRBs; /* NOTE: for p_NoOfSymbols=14, p_Parameters.NoOfCdmGroups=1 and p_Parameters.MaxNoOfDmrsAddPos=0 we get v_NoOfREsPerPrb = 162, for p_NoOfSymbols=14, p_Parameters.NoOfCdmGroups=1 and p_Parameters.MaxNoOfDmrsAddPos=1 we get v_NoOfREsPerPrb = 156 => f_MinInt(156, v_NoOfREsPerPrb) results in 156 for both configurations and the TBS' are the same */ //log("v_NoOfREsPerPrb = " & int2str(v_NoOfREsPerPrb)); //log("v_NoOfREs = " & int2str(v_NoOfREs)); // Step 2: v_Nmax := v_Parameters.ScalingFactor * int2float(v_NoOfREs) * int2float(v_Qm) * int2float(v_Parameters.NoOfLayers); v_Ninfo := v_Nmax * v_CodeRate; //log("v_Ninfo = " & int2str(float2int(v_Ninfo)) & "." & int2str(float2int(v_Ninfo * 1000.0) mod 1000)); if (v_Ninfo < 24.0) { /* special case: TBS=24 is the smallest TBS => any Ninfo < 24 would result in TBS=24 according to the calculation in 38.214 => criteria needs to be specified in the prose (either e.g. to skip Ninfo < 24.0 or to skip all {NoOfRBs, Imcs} where there is a greater Imcs value for the same NoOfRBs */ return 0; // !!!! exact criteria needs to be defined and documented !!!! } if (v_Ninfo <= 3824.0) { // Step 3: v_TBS := fl_NR_TBS_Calculation_Step3(v_Ninfo); } else if (v_Ninfo < 3825.0) { //@sic R5-202607, R5-202608 sic@ //There is ambiguity of TBS calculation when 3824.0 < Ninfo < 3825.0 in clause 5.1.3.2 of TS 38.214 v_TBS := 0; } else{ // Step 4: v_TBS := fl_NR_TBS_Calculation_Step4(v_Ninfo, v_CodeRate); } if (p_IsDownlink) { /* @sic R5s190826 change 5: check effective code rate for DL sic@ */ if (v_TBS <= 3824){ v_CRCSize := 16; } else { v_CRCSize := 24; } v_EffectiveCodeRate := int2float(v_TBS + v_CRCSize) / v_Nmax; if (v_EffectiveCodeRate > 0.95) { /* @sic R5s190826 change 5: effective code rate shall be <= 0.95 according to 38.214 clause 5.1.3.4 sic@ */ v_TBS := 0; } } /* if ((v_Ninfo > 3824.0) and (v_Ninfo < 3825.0)) { // @sic R5-200241: Discussion of ambiguity of Ninfo in TBS calculation sic@ */ /* log("v_Ninfo=" & f_Float2Str(v_Ninfo) & ": TBS=" & int2str(v_TBS) & " NoOfRBs=" & int2str(p_NoOfRBs) & " Imcs=" & int2str(p_Imcs)); */ /* } */ return v_TBS; }

After Change:

function f_NR_TBS_Calculation(template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters, integer p_NoOfSymbols, integer p_NoOfRBs, integer p_Imcs, boolean p_IsDownlink := false, integer p_StartSymbol:=0) return integer { var NR_TBS_CalculationParameters_Type v_Parameters := valueof(p_CalculationParameters); var NR_MCS_TableEntry_Type v_MCS_TableEntry := v_Parameters.MCS_Table[p_Imcs]; var float v_CodeRate := v_MCS_TableEntry.CodeRate_x_1024 / 1024.0; var integer v_Qm := v_MCS_TableEntry.Qm; var integer v_NoOfREsPerPrb; var integer v_NoOfREs; var float v_Nmax; var float v_Ninfo; var integer v_TBS; var float v_EffectiveCodeRate; var integer v_CRCSize; var integer v_Kcb; // Step 1: v_NoOfREsPerPrb := fl_NR_TBS_Calculation_NoOfREsPerPrb_Step1(v_Parameters, p_NoOfSymbols,p_StartSymbol); v_NoOfREs := f_MinInt(156, v_NoOfREsPerPrb) * p_NoOfRBs; /* NOTE: for p_NoOfSymbols=14, p_Parameters.NoOfCdmGroups=1 and p_Parameters.MaxNoOfDmrsAddPos=0 we get v_NoOfREsPerPrb = 162, for p_NoOfSymbols=14, p_Parameters.NoOfCdmGroups=1 and p_Parameters.MaxNoOfDmrsAddPos=1 we get v_NoOfREsPerPrb = 156 => f_MinInt(156, v_NoOfREsPerPrb) results in 156 for both configurations and the TBS' are the same */ //log("v_NoOfREsPerPrb = " & int2str(v_NoOfREsPerPrb)); //log("v_NoOfREs = " & int2str(v_NoOfREs)); // Step 2: v_Nmax := v_Parameters.ScalingFactor * int2float(v_NoOfREs) * int2float(v_Qm) * int2float(v_Parameters.NoOfLayers); v_Ninfo := v_Nmax * v_CodeRate; //log("v_Ninfo = " & int2str(float2int(v_Ninfo)) & "." & int2str(float2int(v_Ninfo * 1000.0) mod 1000)); if (v_Ninfo < 24.0) { /* special case: TBS=24 is the smallest TBS => any Ninfo < 24 would result in TBS=24 according to the calculation in 38.214 => criteria needs to be specified in the prose (either e.g. to skip Ninfo < 24.0 or to skip all {NoOfRBs, Imcs} where there is a greater Imcs value for the same NoOfRBs */ return 0; // !!!! exact criteria needs to be defined and documented !!!! } if (v_Ninfo <= 3824.0) { // Step 3: v_TBS := fl_NR_TBS_Calculation_Step3(v_Ninfo); } else if (v_Ninfo < 3825.0) { //@sic R5-202607, R5-202608 sic@ //There is ambiguity of TBS calculation when 3824.0 < Ninfo < 3825.0 in clause 5.1.3.2 of TS 38.214 v_TBS := 0; } else{ // Step 4: v_TBS := fl_NR_TBS_Calculation_Step4(v_Ninfo, v_CodeRate); } if (p_IsDownlink) { /* @sic R5s190826 change 5: check effective code rate for DL sic@ */ if ((v_CodeRate<=0.25)or(v_TBS <= 292)or((v_TBS <= 3824)and(v_CodeRate<=0.67))){ v_Kcb:=3840; } else { v_Kcb:=8448; } if (v_TBS <= 3824){ v_CRCSize := 16; } else { v_CRCSize := 24; } v_EffectiveCodeRate := int2float((v_TBS+(v_TBS/(v_Kcb-24))*24+v_CRCSize))/ v_Nmax; if (v_EffectiveCodeRate > 0.95) { /* @sic R5s190826 change 5: effective code rate shall be <= 0.95 according to 38.214 clause 5.1.3.4 sic@ */ v_TBS := 0; } } /* if ((v_Ninfo > 3824.0) and (v_Ninfo < 3825.0)) { // @sic R5-200241: Discussion of ambiguity of Ninfo in TBS calculation sic@ */ /* log("v_Ninfo=" & f_Float2Str(v_Ninfo) & ": TBS=" & int2str(v_TBS) & " NoOfRBs=" & int2str(p_NoOfRBs) & " Imcs=" & int2str(p_Imcs)); */ /* } */ return v_TBS; }

## Change 2

| Functionname | fl_NR_TBS_Calculation_NoOfREsPerPrb_Step1 |
| --- | --- |
| Reason for change | ##### According to TS 38.211 7.4.1.1.2 Mapping to physical resources, The position(s) of the DM-RS symbols is given by and duration ld where - for PDSCH mapping type A, ld is the duration between the first OFDM symbol of the slot and the last OFDM symbol of the scheduled PDSCH resources in the slot - for PDSCH mapping type B, ld is the duration of the scheduled PDSCH resources ld is different for type A and type B, and in Table 6.4.1.1.3-3 and Table 7.4.1.1.2-3, actual number of symbols with DMRS for mapping type A, if ld is larger than 8, actual Number of Symbols With DMRS should be 2. |
| Summary of change | In case TypeA, set v_ActualNumberOfSymbolsWithDMRS according to the actual No Of Symbols. |
| TTCN module | NR_TBS_Calculation.ttcn |
| MCC160 Comment | Accepted as improvement |

Before Change:

functionfl_NR_TBS_Calculation_NoOfREsPerPrb_Step1(NR_TBS_CalculationParameters_Type p_Parameters, integer p_NoOfSymbols) return integer { var integer v_ActualNumberOfSymbolsWithDMRS; var integer v_TotalNumberOfREs; var integer v_NumberOfREsWithoutData; select (p_Parameters.MappingType) { case ("TypeA") { // Actual number of symbols with DMRS for mapping type A according to TS 38.211 Table 6.4.1.1.3-3 and Table 7.4.1.1.2-3 if ((p_NoOfSymbols < 8) or (p_Parameters.MaxNoOfDmrsAddPos == 0)) { v_ActualNumberOfSymbolsWithDMRS := 1; } else if ((p_NoOfSymbols < 10) or (p_Parameters.MaxNoOfDmrsAddPos == 1)) { v_ActualNumberOfSymbolsWithDMRS := 2; } else if ((p_NoOfSymbols < 12) or (p_Parameters.MaxNoOfDmrsAddPos == 2)) { v_ActualNumberOfSymbolsWithDMRS := 3; } else { v_ActualNumberOfSymbolsWithDMRS := 4; } } case ("TypeB") { select (p_Parameters.PxSCH) { case ("PUSCH") { // Actual number of symbols with DMRS for mapping type B according to TS 38.211 Table 6.4.1.1.3-3 if ((p_NoOfSymbols < 5) or (p_Parameters.MaxNoOfDmrsAddPos == 0)) { v_ActualNumberOfSymbolsWithDMRS := 1; } else if ((p_NoOfSymbols < 8) or (p_Parameters.MaxNoOfDmrsAddPos == 1)) { v_ActualNumberOfSymbolsWithDMRS := 2; } else if ((p_NoOfSymbols < 10) or (p_Parameters.MaxNoOfDmrsAddPos == 2)) { v_ActualNumberOfSymbolsWithDMRS := 3; } else { v_ActualNumberOfSymbolsWithDMRS := 4; } } case ("PDSCH") { // Actual number of symbols with DMRS for mapping type B according to TS 38.211 Table 6.4.1.1.3-3 if ((p_NoOfSymbols < 6) or (p_Parameters.MaxNoOfDmrsAddPos == 0)) { v_ActualNumberOfSymbolsWithDMRS := 1; } else if (p_Parameters.MaxNoOfDmrsAddPos == 1) { v_ActualNumberOfSymbolsWithDMRS := 2; } else { FatalError(__FILE__, __LINE__, "PDSCH mapping type B: invalid configuration"); } } } } } // Total number of REs in an RB with given duration: v_TotalNumberOfREs := p_NoOfSymbols * 12; // number of REs which are not used for PDSCH v_NumberOfREsWithoutData := v_ActualNumberOfSymbolsWithDMRS * p_Parameters.NoOfCdmGroups * 6; // return number of REs which are used for PDSCH return v_TotalNumberOfREs - v_NumberOfREsWithoutData - p_Parameters.XOverhead; }

After Change:

functionfl_NR_TBS_Calculation_NoOfREsPerPrb_Step1(NR_TBS_CalculationParameters_Type p_Parameters, integer p_NoOfSymbols, integer p_StartSymbol:=0) return integer { var integer v_ActualNumberOfSymbolsWithDMRS; var integer v_TotalNumberOfREs; var integer v_NumberOfREsWithoutData; select (p_Parameters.MappingType) { case ("TypeA") { // Actual number of symbols with DMRS for mapping type A according to TS 38.211 Table 6.4.1.1.3-3 and Table 7.4.1.1.2-3 if ((p_NoOfSymbols < 8) or (p_Parameters.MaxNoOfDmrsAddPos == 0)) { if(p_StartSymbol+p_NoOfSymbols >8) { v_ActualNumberOfSymbolsWithDMRS := 2; } else { v_ActualNumberOfSymbolsWithDMRS := 1; } } else if ((p_NoOfSymbols < 10) or (p_Parameters.MaxNoOfDmrsAddPos == 1)) { v_ActualNumberOfSymbolsWithDMRS := 2; } else if ((p_NoOfSymbols < 12) or (p_Parameters.MaxNoOfDmrsAddPos == 2)) { v_ActualNumberOfSymbolsWithDMRS := 3; } else { v_ActualNumberOfSymbolsWithDMRS := 4; } } case ("TypeB") { select (p_Parameters.PxSCH) { case ("PUSCH") { // Actual number of symbols with DMRS for mapping type B according to TS 38.211 Table 6.4.1.1.3-3 if ((p_NoOfSymbols < 5) or (p_Parameters.MaxNoOfDmrsAddPos == 0)) { v_ActualNumberOfSymbolsWithDMRS := 1; } else if ((p_NoOfSymbols < 8) or (p_Parameters.MaxNoOfDmrsAddPos == 1)) { v_ActualNumberOfSymbolsWithDMRS := 2; } else if ((p_NoOfSymbols < 10) or (p_Parameters.MaxNoOfDmrsAddPos == 2)) { v_ActualNumberOfSymbolsWithDMRS := 3; } else { v_ActualNumberOfSymbolsWithDMRS := 4; } } case ("PDSCH") { // Actual number of symbols with DMRS for mapping type B according to TS 38.211 Table 6.4.1.1.3-3 if ((p_NoOfSymbols < 6) or (p_Parameters.MaxNoOfDmrsAddPos == 0)) { v_ActualNumberOfSymbolsWithDMRS := 1; } else if (p_Parameters.MaxNoOfDmrsAddPos == 1) { v_ActualNumberOfSymbolsWithDMRS := 2; } else { FatalError(__FILE__, __LINE__, "PDSCH mapping type B: invalid configuration"); } } } } } // Total number of REs in an RB with given duration: v_TotalNumberOfREs := p_NoOfSymbols * 12; // number of REs which are not used for PDSCH v_NumberOfREsWithoutData := v_ActualNumberOfSymbolsWithDMRS * p_Parameters.NoOfCdmGroups * 6; // return number of REs which are used for PDSCH return v_TotalNumberOfREs - v_NumberOfREsWithoutData - p_Parameters.XOverhead; }

## Change 3

| Functionname | f_TC_7_1_1_4_1_1_NR_TestBody |
| --- | --- |
| Reason for change | According to TS38.523-1Table 7.1.1.4.1.0-1, startSymbolAndLength should set with S=2. |
| Summary of change | Pass 2 as p_StartSymbol in f_NR_TBS_Calculation. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted as improvement |

Before Change:

function f_TC_7_1_1_4_1_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.x: DL-SCH Transport Block Size selection */ var integer v_TBSize:=0; // transport block size in bits var integer v_I_MCS; // Modulation and Coding Scehme Index var integer v_N_PRB; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA; // Time Domain resource allocation var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var NR_ResourceAllocation_Type v_NR_ResourceAllocation := {v_Max_nPRB, 28}; // Max NPRB and Max MCS //@sic R5s190826 sic@ var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var integer v_NoOfSymbols; var SubFrameNumber_Type v_SF := 2;//@sic R5s210885 sic@ var integer v_TBSizeLow := 136; //lowest allowed for normal UE if (v_SCS ==kHz30) {v_SF:= 1;} if(pc_supportOfRedCap_r17) { v_TBSizeLow := 120; } //for Redcap lowest allowed TBSize is 120 bits // Configure SS for UL grant for Max TB size f_NR_ULGrantConfiguration_Start( nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation.Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs)); //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..8 //f_NR_ConfigureULGrant_MaxTBSupported(); for (v_N_PRB := 1; v_N_PRB <= v_Max_nPRB; v_N_PRB := v_N_PRB + 1) { //for nPRB 1 to v_Max_nPRB for (v_I_MCS := 0; v_I_MCS <= 28; v_I_MCS := v_I_MCS + 1) { //for iMCS 0 to 28 for (v_TDRA := 0; v_TDRA <= 1 ; v_TDRA := v_TDRA + 1) { //for Time Domain Resource Allocation 0 to 1 if (v_TDRA == 0) { v_NoOfSymbols := 12; //@sic R5s210885 sic@ } else { v_NoOfSymbols := 7; } //SS looks up iTBS in table 7.1.7.1-1 in TS 36.213 based on the value of iMCS //v_I_TBS := f_NR_Get_iTBS_For_iMCS(v_I_MCS); //The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L, and nPRB. //@sic R5s190826 sic@ v_TBSize := f_NR_TBS_Calculation(p_CalculationParameters, v_NoOfSymbols, v_N_PRB, v_I_MCS,true ); //@sic R5-227163 sic@ if (v_TBSize < v_TBSizeLow ) { //this results in zero number of PDCP SDUs or TBSize has high coding rate¦ continue; } // skip this nprb, Imcs, TDRA combination. // Steps 2 to 5 are performed if TBsize is More than minimum //@siclog "Step 2" siclog@ // SS creates one or more PDCP SDUs, depending on TBsize, in accordance with Table 7.1.1.4.1.1.3.2-2 //Configure SS for DCI Format x with RA type y and a Resource block assignment (RBA) correspondent //to nPRB as specified in 7.1.6.1 in TS 36.213 and Modulation and coding scheme iMCS f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_SubFrame(v_SF),//@sic R5s210885 sic@ resourceAllocationType1, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_N_PRB), int2bit(v_TDRA, 4), cs_NR_TransportBlockRetransmissionList_Def(v_I_MCS), -, //@sic R5s190892 sic@ -, cs_NR_DciFormat_1_0_Params(v_SCS)); //@siclog "Step 3" siclog@ // SS sends MAC PDU (NxPDCP SDUs) v_NR_PDCP_SDUList := fl_NR_GetPdcpSduList_1TB(v_TBSize); //calculates v_N, v_PDCPsize and returns f_NR_Build_N_PDCP_SDUList DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //crs_RlcBearerRouting_NR(nr_Cell1), -, v_NR_PDCP_SDUList )); //@siclog "Step 4" siclog@ // automatically taken by SS //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_Rx_N_PDCP_SDUs(v_NR_PDCP_SDUList, p_NR_DRB_Id); }//End for TDRA 0 to 8 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 }//f_NR_7_1_1_4_1_1

After Change:

function f_TC_7_1_1_4_1_1_NR_TestBody(DRB_Identity p_NR_DRB_Id, template (value) NR_TBS_CalculationParameters_Type p_CalculationParameters) runs on NR_BASE_PTC { /* L2/MAC/7.1 * 7.1.1.4.x: DL-SCH Transport Block Size selection */ var integer v_TBSize:=0; // transport block size in bits var integer v_I_MCS; // Modulation and Coding Scehme Index var integer v_N_PRB; // Number of physical resource Blocks var integer v_Max_nPRB := f_NR_CellInfo_GetCarrierBandwidth_DL(nr_Cell1); // to store the max allowed PRB resources that can be allocated var integer v_TDRA; // Time Domain resource allocation var NR_PDCP_SDUList_Type v_NR_PDCP_SDUList := {''O}; var NR_ResourceAllocation_Type v_NR_ResourceAllocation := {v_Max_nPRB, 28}; // Max NPRB and Max MCS //@sic R5s190826 sic@ var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1); var integer v_NoOfSymbols; var SubFrameNumber_Type v_SF := 2;//@sic R5s210885 sic@ var integer v_TBSizeLow := 136; //lowest allowed for normal UE if (v_SCS ==kHz30) {v_SF:= 1;} if(pc_supportOfRedCap_r17) { v_TBSizeLow := 120; } //for Redcap lowest allowed TBSize is 120 bits // Configure SS for UL grant for Max TB size f_NR_ULGrantConfiguration_Start( nr_Cell1, cs_TimingInfo_Now, v_NR_ResourceAllocation.Imcs, cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs)); //Steps 1 to 5 are repeated for values of nPRB from 1 to v_Max_nPRB and iMCS from 0 to 28, time domain resource allocation v_TDRA 0..8 //f_NR_ConfigureULGrant_MaxTBSupported(); for (v_N_PRB := 1; v_N_PRB <= v_Max_nPRB; v_N_PRB := v_N_PRB + 1) { //for nPRB 1 to v_Max_nPRB for (v_I_MCS := 0; v_I_MCS <= 28; v_I_MCS := v_I_MCS + 1) { //for iMCS 0 to 28 for (v_TDRA := 0; v_TDRA <= 1 ; v_TDRA := v_TDRA + 1) { //for Time Domain Resource Allocation 0 to 1 if (v_TDRA == 0) { v_NoOfSymbols := 12; //@sic R5s210885 sic@ } else { v_NoOfSymbols := 7; } //SS looks up iTBS in table 7.1.7.1-1 in TS 36.213 based on the value of iMCS //v_I_TBS := f_NR_Get_iTBS_For_iMCS(v_I_MCS); //The SS calculates or looks up TBS in TS 38.214 [15] based on the value of S, L, and nPRB. //@sic R5s190826 sic@ v_TBSize := f_NR_TBS_Calculation(p_CalculationParameters, v_NoOfSymbols, v_N_PRB, v_I_MCS,true, 2 ); //@sic R5-227163 sic@ if (v_TBSize < v_TBSizeLow ) { //this results in zero number of PDCP SDUs or TBSize has high coding rate¦ continue; } // skip this nprb, Imcs, TDRA combination. // Steps 2 to 5 are performed if TBsize is More than minimum //@siclog "Step 2" siclog@ // SS creates one or more PDCP SDUs, depending on TBsize, in accordance with Table 7.1.1.4.1.1.3.2-2 //Configure SS for DCI Format x with RA type y and a Resource block assignment (RBA) correspondent //to nPRB as specified in 7.1.6.1 in TS 36.213 and Modulation and coding scheme iMCS f_NR_DLGrantConfiguration(nr_Cell1, cs_TimingInfo_SubFrame(v_SF),//@sic R5s210885 sic@ resourceAllocationType1, cs_NR_FreqDomainResourceAssignmentDL_Explicit(0, v_N_PRB), int2bit(v_TDRA, 4), cs_NR_TransportBlockRetransmissionList_Def(v_I_MCS), -, //@sic R5s190892 sic@ -, cs_NR_DciFormat_1_0_Params(v_SCS)); //@siclog "Step 3" siclog@ // SS sends MAC PDU (NxPDCP SDUs) v_NR_PDCP_SDUList := fl_NR_GetPdcpSduList_1TB(v_TBSize); //calculates v_N, v_PDCPsize and returns f_NR_Build_N_PDCP_SDUList DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, //crs_RlcBearerRouting_NR(nr_Cell1), -, v_NR_PDCP_SDUList )); //@siclog "Step 4" siclog@ // automatically taken by SS //@siclog "Step 5" siclog@ // SS receives NxPDCP SDUs f_NR_Rx_N_PDCP_SDUs(v_NR_PDCP_SDUList, p_NR_DRB_Id); }//End for TDRA 0 to 8 }//End for iMCS 0 to 28 }//End for nPRB 1 to 110 }//f_NR_7_1_1_4_1_1