3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240641

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3568 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_NR5GC_PDUSessionEstablishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In a very specific scenario where more than one PDU session is expected and IMS DNN gets triggered not as the last session then IMS component is not informed via f_NR_DelayForUserPlaneSignalling and if UE triggers IMS signalling before establishing all the PDU sessions then IMS signalling is not handled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the f_NR5GC_PDUSessionEstablishment the part, where “WaitIfIMS” controls whether TTCN will or will not wait for IMS registration, has been moved inside the “while” loop. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In a specific scenario described in “Reason for change” a conformant UE may fail the test case because the IMS registration would not get handled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Any test case in which UE is configured with IMS |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS 	iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_NR5GC_PDUSessionEstablishment |
| --- | --- |
| Reason for change | In a very specific scenario where more than one PDU session is expected and IMS DNN gets triggered not as the last session then IMS component is not informed via f_NR_DelayForUserPlaneSignalling and if UE triggers IMS signalling before establishing all the PDU sessions then IMS signalling is not handled. |
| Summary of change | In the f_NR5GC_PDUSessionEstablishment the part, where “WaitIfIMS” controls whether TTCN will or will not wait for IMS registration, has been moved inside the “while” loop. |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment | Accepted in principle. Similar change made in EUTRA in f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated |

Before Change

function f_NR5GC_PDUSessionEstablishment(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) O1_Type p_PDU_SessionId := ?, template ProcedureTransactionIdentifier p_PTI := omit, template (omit) PDU_SessionType p_PDU_SessionType := omit, template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@ template NG_UE_SM_Cap p_UECap := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template SM_PDU_DN_RequestContainer p_PDUReq := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := cr_NG_Request_Type('001'B), // @sic R5-206290 sic@ template AdditionalInformation p_AdditionalInfo := *, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) EAP_Message p_EAP := omit, template (omit) GPRS_Timer3 p_BackOff := omit, boolean p_PDUSessionAuthenticationRequired := false, template (omit) charstring p_TriggerWithThisAPN := omit, boolean p_FailWhenK_LessN := true, template (omit) charstring p_BuildQosFlow := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, // @sic R5-198184, R5-199075 sic@ boolean p_WaitIfIMS := true, // @sic R5s210515 sic@ integer p_EstReqAlreadyInDefault := 0, boolean p_OmitDNN := false) runs on NR5GC_PTC // @sic R5-223987 sic@ { var SRBs_AlreadyConfigured v_SRBs := SRB1_2; var NR_RadioBearerId_Type v_SRB := p_SRB; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_K_NumOfReqsProcessed := 0; var integer v_L_NumOfReqsReceivedInParallel := 0; var boolean v_EndLoop := false; var boolean v_RxdMsg; var boolean v_IMS_Configured := false; var PDU_PDN_DNN_Type v_PDUType; var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible var default v_MyDefaultVar := null; timer t_Wait := 8.0; // @sic R5-206288 sic@ 8s is less than T3540 if (p_EstReqAlreadyInDefault == 0){ // @sic R5s210520 sic@ // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@ } // Then activate store for any new msgs received v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId)); while ((v_K_NumOfReqsProcessed < p_ExpectedNumberOfNewPDUSessions) and not v_EndLoop) { // @sic R5-199075, R5-204329 sic@ v_RxdMsg := false; t_Wait.start; // @sic R5-204329 sic@ if (f_NR5GC_MsgInDefault_GetNumOfNASMessageReq() > v_L_NumOfReqsReceivedInParallel) { // @sic R5s220174 sic@ // If another msg has been recieved in parallel since the last time we looked v_L_NumOfReqsReceivedInParallel := v_L_NumOfReqsReceivedInParallel + 1; if (v_L_NumOfReqsReceivedInParallel > p_ExpectedNumberOfNewPDUSessions){ // @sic R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "More PDU Session Establishment Messages Received Than Expected"); } v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(v_L_NumOfReqsReceivedInParallel); // @sic R5s220174 sic@ v_RxdMsg := true; t_Wait.stop; // @sic R5s210520 sic@ } else { // no outstanding msgs, so must wait for next PDU Establishment alt { [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { // Stop timer and continue with while loop t_Wait.stop; v_RxdMsg := true; } [] t_Wait.timeout { if (p_FailWhenK_LessN) { // @sic R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Not enough PDU Session Establishment Messages Received"); } else { v_EndLoop := true; } } } } if (v_K_NumOfReqsProcessed > 0) { // @sic R5s200751 sic@ if (isvalue(p_TriggerWithThisAPN)) { // @sic R5-199075 sic@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, valueof(p_TriggerWithThisAPN)); // @sic R5-204439 sic@ } } if (v_RxdMsg) { // @sic R5-204329 sic@ this will be false if either just triggered a new PDN, or about to end the loop // Update K now as latest msg is about to be processed v_K_NumOfReqsProcessed := v_K_NumOfReqsProcessed + 1; if (v_SRB == tsc_NR_RbId_SRB1) { // @sic R5-198923 sic@ v_SRBs := SRB1; } v_PDUType := f_NR5GC_PDUSessionEstablishment_InnerLoop(p_NR_CellId, v_ReceivedAsp, v_SRBs, p_SDAP_Config, p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr, p_PDU_SessionId, p_PTI, p_PDU_SessionType, p_ExpectedSSC_Mode, p_UECap, p_MaxNumPacketFilters, p_IntegrityProtMaxDataRate, p_AlwaysOnPDUSessionReq, p_PDUReq, p_ExtdPCO, p_OldPDU_SessionId, p_RequestType, p_AdditionalInfo, p_Cause, p_RQTimer, p_EAP, p_BackOff, p_PDUSessionAuthenticationRequired, p_BuildQosFlow, p_PduInfoFromLTE, // @sic R5-198184 sic@ p_OmitDNN); // @sic R5-223987 sic@ if (v_SRB == tsc_NR_RbId_SRB1) { // SRB2 will be configured in the 1st instance (if it hasn't already) v_SRB := tsc_NR_RbId_SRB2; v_SRBs := SRB1_2; // @sic R5-198923 sic@ } v_IMS_Configured := v_IMS_Configured or (match (IMS_DNN, v_PDUType)); // @sic R5-206296 sic@ } } if (p_WaitIfIMS) { // @sic R5s210515, R5s220807 sic@ /* Need to delay for IP/IMS signalling if RRCRelease is about to be sent if set to noRrcConnectionRelease, assume the connection will be up long enough to complete IMS signalling without waiting here */ // Wait for IP signalling @sic R5-204329, R5-206296 sic@ if (v_IMS_Configured) { v_WaitForUserplaneSignalling := waitForIMS; } else { // currently all PDUs require IP signalling v_WaitForUserplaneSignalling := dontWaitForIMS; } f_NR_DelayForUserPlaneSignalling(v_WaitForUserplaneSignalling); // @sic R5-206296 sic@ } // all expected msgs have been processed, so deactivate the extra default deactivate(v_MyDefaultVar); }

After Change

function f_NR5GC_PDUSessionEstablishment(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) O1_Type p_PDU_SessionId := ?, template ProcedureTransactionIdentifier p_PTI := omit, template (omit) PDU_SessionType p_PDU_SessionType := omit, template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@ template NG_UE_SM_Cap p_UECap := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template SM_PDU_DN_RequestContainer p_PDUReq := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := cr_NG_Request_Type('001'B), // @sic R5-206290 sic@ template AdditionalInformation p_AdditionalInfo := *, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) EAP_Message p_EAP := omit, template (omit) GPRS_Timer3 p_BackOff := omit, boolean p_PDUSessionAuthenticationRequired := false, template (omit) charstring p_TriggerWithThisAPN := omit, boolean p_FailWhenK_LessN := true, template (omit) charstring p_BuildQosFlow := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, // @sic R5-198184, R5-199075 sic@ boolean p_WaitIfIMS := true, // @sic R5s210515 sic@ integer p_EstReqAlreadyInDefault := 0, boolean p_OmitDNN := false) runs on NR5GC_PTC // @sic R5-223987 sic@ { var SRBs_AlreadyConfigured v_SRBs := SRB1_2; var NR_RadioBearerId_Type v_SRB := p_SRB; var NR_SRB_COMMON_IND v_ReceivedAsp; var integer v_K_NumOfReqsProcessed := 0; var integer v_L_NumOfReqsReceivedInParallel := 0; var boolean v_EndLoop := false; var boolean v_RxdMsg; var boolean v_IMS_Configured := false; var PDU_PDN_DNN_Type v_PDUType; var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible var default v_MyDefaultVar := null; timer t_Wait := 8.0; // @sic R5-206288 sic@ 8s is less than T3540 if (p_EstReqAlreadyInDefault == 0){ // @sic R5s210520 sic@ // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@ } // Then activate store for any new msgs received v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId)); while ((v_K_NumOfReqsProcessed < p_ExpectedNumberOfNewPDUSessions) and not v_EndLoop) { // @sic R5-199075, R5-204329 sic@ v_RxdMsg := false; t_Wait.start; // @sic R5-204329 sic@ if (f_NR5GC_MsgInDefault_GetNumOfNASMessageReq() > v_L_NumOfReqsReceivedInParallel) { // @sic R5s220174 sic@ // If another msg has been recieved in parallel since the last time we looked v_L_NumOfReqsReceivedInParallel := v_L_NumOfReqsReceivedInParallel + 1; if (v_L_NumOfReqsReceivedInParallel > p_ExpectedNumberOfNewPDUSessions){ // @sic R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "More PDU Session Establishment Messages Received Than Expected"); } v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(v_L_NumOfReqsReceivedInParallel); // @sic R5s220174 sic@ v_RxdMsg := true; t_Wait.stop; // @sic R5s210520 sic@ } else { // no outstanding msgs, so must wait for next PDU Establishment alt { [] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { // Stop timer and continue with while loop t_Wait.stop; v_RxdMsg := true; } [] t_Wait.timeout { if (p_FailWhenK_LessN) { // @sic R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Not enough PDU Session Establishment Messages Received"); } else { v_EndLoop := true; } } } } if (v_K_NumOfReqsProcessed > 0) { // @sic R5s200751 sic@ if (isvalue(p_TriggerWithThisAPN)) { // @sic R5-199075 sic@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, valueof(p_TriggerWithThisAPN)); // @sic R5-204439 sic@ } } if (v_RxdMsg) { // @sic R5-204329 sic@ this will be false if either just triggered a new PDN, or about to end the loop // Update K now as latest msg is about to be processed v_K_NumOfReqsProcessed := v_K_NumOfReqsProcessed + 1; if (v_SRB == tsc_NR_RbId_SRB1) { // @sic R5-198923 sic@ v_SRBs := SRB1; } v_PDUType := f_NR5GC_PDUSessionEstablishment_InnerLoop(p_NR_CellId, v_ReceivedAsp, v_SRBs, p_SDAP_Config, p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr, p_PDU_SessionId, p_PTI, p_PDU_SessionType, p_ExpectedSSC_Mode, p_UECap, p_MaxNumPacketFilters, p_IntegrityProtMaxDataRate, p_AlwaysOnPDUSessionReq, p_PDUReq, p_ExtdPCO, p_OldPDU_SessionId, p_RequestType, p_AdditionalInfo, p_Cause, p_RQTimer, p_EAP, p_BackOff, p_PDUSessionAuthenticationRequired, p_BuildQosFlow, p_PduInfoFromLTE, // @sic R5-198184 sic@ p_OmitDNN); // @sic R5-223987 sic@ if (v_SRB == tsc_NR_RbId_SRB1) { // SRB2 will be configured in the 1st instance (if it hasn't already) v_SRB := tsc_NR_RbId_SRB2; v_SRBs := SRB1_2; // @sic R5-198923 sic@ } v_IMS_Configured := v_IMS_Configured or (match (IMS_DNN, v_PDUType)); // @sic R5-206296 sic@ if (p_WaitIfIMS) { // @sic R5s210515, R5s220807 sic@ /* Need to delay for IP/IMS signalling if RRCRelease is about to be sent if set to noRrcConnectionRelease, assume the connection will be up long enough to complete IMS signalling without waiting here */ // Wait for IP signalling @sic R5-204329, R5-206296 sic@ if (v_IMS_Configured) { v_WaitForUserplaneSignalling := waitForIMS; } else { // currently all PDUs require IP signalling v_WaitForUserplaneSignalling := dontWaitForIMS; } f_NR_DelayForUserPlaneSignalling(v_WaitForUserplaneSignalling); // @sic R5-206296 sic@ } } } // all expected msgs have been processed, so deactivate the extra default deactivate(v_MyDefaultVar); }

#### MCC160 Implementation

function f_NR5GC_PDUSessionEstablishment(NR_CellId_Type p_NR_CellId,

integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection,  // @sic R5s201358 sic@,

NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2,

template (omit) SDAP_Config p_SDAP_Config := omit,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template DNN p_ExpectedDNN := *,

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (omit) DNN p_DNN := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (present) O1_Type p_PDU_SessionId := ?,

template ProcedureTransactionIdentifier p_PTI := omit,

template (omit) PDU_SessionType p_PDU_SessionType := omit,

template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@

template NG_UE_SM_Cap p_UECap := *,

template MaxNumPacketFilters p_MaxNumPacketFilters := *,

template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?,

template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *,

template SM_PDU_DN_RequestContainer p_PDUReq := *,

template ExtdProtocolConfigOptions p_ExtdPCO := *,

template NG_PDU_SessionId p_OldPDU_SessionId := *,

template NG_Request_Type p_RequestType := cr_NG_Request_Type('001'B),  // @sic R5-206290 sic@

template AdditionalInformation p_AdditionalInfo := *,

template (omit) GMM_GSM_Cause p_Cause := omit,

template (omit) GPRS_Timer p_RQTimer := omit,

template (omit) EAP_Message p_EAP := omit,

template (omit) GPRS_Timer3 p_BackOff := omit,

boolean p_PDUSessionAuthenticationRequired := false,

template (omit) charstring p_TriggerWithThisAPN := omit,

boolean p_FailWhenK_LessN := true,

template (omit) charstring p_BuildQosFlow := omit,

template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, // @sic R5-198184, R5-199075 sic@

boolean  p_WaitIfIMS := true, // @sic R5s210515 sic@

integer p_EstReqAlreadyInDefault := 0,

boolean p_OmitDNN := false) runs on NR5GC_PTC // @sic R5-223987 sic@

{

var SRBs_AlreadyConfigured v_SRBs := SRB1_2;

var NR_RadioBearerId_Type v_SRB := p_SRB;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var integer v_K_NumOfReqsProcessed := 0;

var integer v_L_NumOfReqsReceivedInParallel := 0;

var boolean v_EndLoop := false;

var boolean v_RxdMsg;

var PDU_PDN_DNN_Type v_PDUType;

var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible

var default v_MyDefaultVar := null;

timer t_Wait := 8.0; // @sic R5-206288 sic@ 8s is less than T3540

if (p_EstReqAlreadyInDefault == 0){ // @sic R5s210520 sic@

// Clear store of msgs received in parallel

f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // @sic R5s220174 sic@

}

// Then activate store for any new msgs received

v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(p_NR_CellId));

while ((v_K_NumOfReqsProcessed < p_ExpectedNumberOfNewPDUSessions) and not v_EndLoop) { // @sic R5-199075, R5-204329 sic@

v_RxdMsg := false;

t_Wait.start; // @sic R5-204329 sic@

if (f_NR5GC_MsgInDefault_GetNumOfNASMessageReq() > v_L_NumOfReqsReceivedInParallel) { // @sic R5s220174 sic@

// If another msg has been recieved in parallel since the last time we looked

v_L_NumOfReqsReceivedInParallel := v_L_NumOfReqsReceivedInParallel + 1;

if (v_L_NumOfReqsReceivedInParallel > p_ExpectedNumberOfNewPDUSessions){ // @sic R5-204329 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "More PDU Session Establishment Messages Received Than Expected");

}

v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(v_L_NumOfReqsReceivedInParallel); // @sic R5s220174 sic@

v_RxdMsg := true;

t_Wait.stop;      // @sic R5s210520 sic@

} else { // no outstanding msgs, so must wait for next PDU Establishment

alt {

[] SRB.receive(car_NR_SRB_NasPdu_IND(p_NR_CellId, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{ // Stop timer and continue with while loop

t_Wait.stop;

v_RxdMsg := true;

}

[] t_Wait.timeout

{

if (p_FailWhenK_LessN) { // @sic R5-204329 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Not enough PDU Session Establishment Messages Received");

} else {

v_EndLoop := true;

}

}

}

}

if (v_K_NumOfReqsProcessed > 0) { // @sic R5s200751 sic@

if (isvalue(p_TriggerWithThisAPN)) { // @sic R5-199075  sic@

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, valueof(p_TriggerWithThisAPN)); // @sic R5-204439 sic@

}

}

if (v_RxdMsg) { // @sic R5-204329 sic@ this will be false if either just triggered a new PDN, or about to end the loop

// Update K now as latest msg is about to be processed

v_K_NumOfReqsProcessed := v_K_NumOfReqsProcessed + 1;

if (v_SRB == tsc_NR_RbId_SRB1) { // @sic R5-198923 sic@

v_SRBs := SRB1;

}

v_PDUType := f_NR5GC_PDUSessionEstablishment_InnerLoop(p_NR_CellId,

v_ReceivedAsp,

v_SRBs,

p_SDAP_Config,

p_ExpectedS_NSSAI,

p_ExpectedDNN,

p_SSC_Mode,

p_DNN,

p_QoS_Rules,

p_Session_AMBR,

p_S_NSSAI,

p_QoSFlowDescr,

p_PDU_SessionId,

p_PTI,

p_PDU_SessionType,

p_ExpectedSSC_Mode,

p_UECap,

p_MaxNumPacketFilters,

p_IntegrityProtMaxDataRate,

p_AlwaysOnPDUSessionReq,

p_PDUReq,

p_ExtdPCO,

p_OldPDU_SessionId,

p_RequestType,

p_AdditionalInfo,

p_Cause,

p_RQTimer,

p_EAP,

p_BackOff,

p_PDUSessionAuthenticationRequired,

p_BuildQosFlow,

p_PduInfoFromLTE, // @sic R5-198184 sic@

p_OmitDNN);  // @sic R5-223987 sic@

if (v_SRB == tsc_NR_RbId_SRB1) { // SRB2 will be configured in the 1st instance (if it hasn't already)

v_SRB := tsc_NR_RbId_SRB2;

v_SRBs := SRB1_2;  // @sic R5-198923 sic@

}

if (p_WaitIfIMS) { // @sic R5s210515, R5s220807 sic@

/* Need to delay for IP/IMS signalling if RRCRelease is about to be sent

if set to noRrcConnectionRelease, assume the connection will be up long enough

to complete IMS signalling without waiting here */

// Wait for IP signalling @sic R5-204329, R5-206296 sic@

if (match (IMS_DNN, v_PDUType)) {

v_WaitForUserplaneSignalling := waitForIMS;

}  else { // currently all PDUs require IP signalling

v_WaitForUserplaneSignalling := dontWaitForIMS;

}

f_NR_DelayForUserPlaneSignalling(v_WaitForUserplaneSignalling);  // @sic R5-206296 sic@

}

}

}

// all expected msgs have been processed, so deactivate the extra default

deactivate(v_MyDefaultVar);

}

function f_EUTRA38_MultiPDN_EstAddPDNs_DefaultActivated(EUTRA_CellId_Type p_CellId,

integer p_ExpectedNoOfNewPDNs,

boolean p_FailWhenK_LessN := true,

integer p_SRB := tsc_SRB2,  // @sic R5s210341 sic@

template (omit) NasCount_Type p_NasCountUL := omit, // Only needed when called after TAU

B3_Type p_RequestType := '001'B,

template AccessPointName p_AccessPointName := cr_AccessPointName_Any,

boolean p_N1disabled := false,

template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit

) runs on EUTRA_Multi5GS_PTC return integer

{

var SRB_COMMON_IND v_ReceivedAsp;

var NAS_MSG_Indication_Type v_NasInd;

var PDN_CONNECTIVITY_REQUEST v_PDNConnReq;

var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible

var EPS_BearerIdentity v_EpsBearerId;

var integer v_L_NumOfReqsReceivedInParallel := 0;

var integer v_K_NumOfPDNsProcessed := 0;

var boolean v_EndLoop := false;

var boolean v_RxdMsg;

var integer v_SRB := p_SRB;

var B3_Type v_PDNType := f_GetPdnType();

var EUTRA38_DRBInfoList_Type v_ExistingDRBList;

var integer v_Index;

timer t_Wait := 10.0;

while ((v_K_NumOfPDNsProcessed < p_ExpectedNoOfNewPDNs) and not v_EndLoop) {

v_RxdMsg := false;

t_Wait.start;

if (f_EUTRA38_MsgInDefault_GetNumOfPDNConnectivityReq() > v_L_NumOfReqsReceivedInParallel) {

t_Wait.stop;

v_L_NumOfReqsReceivedInParallel := v_L_NumOfReqsReceivedInParallel +1;

v_ReceivedAsp := f_EUTRA38_MsgInDefault_GetOnePDNConnectivityReq(v_L_NumOfReqsReceivedInParallel);

v_RxdMsg := true;

} else { // no outstanding msgs, so must wait for next one

alt {

[] SRB.receive(car_SRB_NasPdu_IND(p_CellId, v_SRB, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_PDNConnReqAdditionalPDN(v_PDNType, p_AccessPointName, p_RequestType))))  // @sic R5-216188 sic@

-> value v_ReceivedAsp    /* NOTE: acc. to the prose the ESM information transfer flag is still "Not present or any allowed value"

*       nevertheless if the UE sets the ESM information transfer flag to true this may cause issues

*       as the generic procedure does not handle any requested ESM information transfer */

{

t_Wait.stop;

v_RxdMsg := true;

}

[] t_Wait.timeout

{ // @sic R5-206291 sic@

if (p_FailWhenK_LessN and (v_K_NumOfPDNsProcessed < p_ExpectedNoOfNewPDNs)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Established " & int2str(v_K_NumOfPDNsProcessed) & "PDNs instead of " & int2str(p_ExpectedNoOfNewPDNs));

} else if (not p_FailWhenK_LessN) { // New message not received in time, so just end the loop

v_EndLoop := true;

}

}

}

}

if (v_RxdMsg) { // this will be false if either just triggered a new PDN, or about to end the loop

// Update K now as latest msg is about to be processed

v_K_NumOfPDNsProcessed := v_K_NumOfPDNsProcessed +1;

v_NasInd := v_ReceivedAsp.Signalling.Nas[0];

v_PDNConnReq := v_NasInd.Pdu.Msg.pDN_CONNECTIVITY_REQUEST;

v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2(p_CellId, v_PDNConnReq, (v_SRB != tsc_SRB2), p_NasCountUL, p_PduInfoFromNR, p_N1disabled);  // @sic R5-227443, R5s230015 sic@

a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(p_CellId);

v_SRB := tsc_SRB2;  // SRB2 will have been configured by now, even if not earlier

a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(p_CellId, v_EpsBearerId);

// If the UE has just moved from NR, don't need to wait for IMS signalling

if (not isvalue(p_PduInfoFromNR)) { // @sic R5s230015 sic@

v_ExistingDRBList := f_EUTRA38_MobileInfo_GetDRBInfoList();

// get details of bearer just activated

v_Index := f_EUTRA38_DRBInfo_FindEntry(v_ExistingDRBList, (hex2int(v_EpsBearerId) -4)); // @sic R5s240227 sic@

select (v_ExistingDRBList[v_Index].APN) { // @sic R5-206296, R5s240227 sic@

case (IMS_DNN) {

v_WaitForUserplaneSignalling := waitForIMS;

}

case else {   // @sic R5-245495 sic@

v_WaitForUserplaneSignalling := dontWaitForIMS;

}

}

f_EUTRA_DelayForUserPlaneSignalling (v_WaitForUserplaneSignalling);

}

}

}

return v_SRB;

}

## Additional Changes:

## 2.2	Change 2

| Function name | f_IsDNNForIMS |
| --- | --- |
| Reason for change | IR.88 clause 6.3.2 and NG.113 clause 7.3.2 both define the IMS well-known APN as: The Network Identifier (NI) part of the APN must be set to "IMS". The APN Operator Identifier (OI) part of the full APN must be blank as it is automatically derived and appended to the NI part by the VPMN and its value depends on the PMN 23.003 clause 9.1.2 defines the APN OI as: The APN Operator Identifier is composed of three labels. The last label (or domain) shall be "gprs". The first and second labels together shall uniquely identify the GPRS/EPS PLMN. For each operator, there is a default APN Operator Identifier (i.e. domain name). This default APN Operator Identifier is derived from the IMSI as follows: "mnc<MNC>.mcc<MCC>.gprs" |
| Summary of change | New function sets the PDU Type to IMS_DNN based on above definitions. |
| TTCN module | NG_NASTemplateFunctions.ttcn |

function f_IsDNNForIMS(octetstring p_DNNFromReq) return boolean

{

var charstring v_DNN := oct2char(p_DNNFromReq);

var Char1List_Type v_SplitCharList := {"."};

var CharStringList_Type v_FieldList := f_StringSplit(v_DNN, v_SplitCharList);

var integer v_FieldCnt := lengthof(v_FieldList);

if (v_FieldCnt == 1) {   // DNN = IMS

if (match (v_FieldList[0], "IMS")) {

return true;

}

else if (v_FieldCnt == 4) {

if ((match (v_FieldList[0], "IMS")) and (match (v_FieldList[3], "gprs"))) { // DNN = IMS.mccMCC.mncMNC.gprs

return true;

}

}

}

// if none of the above

return false;

}

function f_GetPDN_DNNTypeFromAPN(octetstring p_APN) return PDU_PDN_DNN_Type

{ // @sic R5-221467, R5-245495 sic@

var PDU_PDN_DNN_Type v_PDNType := Internet_DNN; // @sic R5s240433 sic@

if (match (p_APN, f_DomainName_Encode(pc_APN_ID_Internet))) { // @sic R5s210014 sic@

v_PDNType := Internet_DNN;

} else if (match (p_APN, f_DomainName_Encode(pc_APN_ID_IMS)) or f_IsDNNForIMS(p_APN)) { // @sic R5s210014 sic@

v_PDNType := IMS_DNN;

} else if (match (p_APN, f_DomainName_Encode(pc_APN_ID_URLLC))) {

v_PDNType := URLLC_DNN;

} else if (match (p_APN, f_DomainName_Encode(pc_APN_ID_MIOT))) {

v_PDNType := MIOT_DNN;

} else if (match (p_APN, f_DomainName_Encode(pc_APN_ID_V2X))) {

v_PDNType := V2X_DNN;

} else if (match (p_APN, f_DomainName_Encode(pc_APN_ID_Ethernet))) {

v_PDNType := Ethernet_DNN;

/*  } else if (match (p_APN, f_DomainName_Encode(pc_APN_ID_MBS))) {

v_PDNType := MBS_DNN;

} else if (match (p_APN, f_DomainName_Encode(pc_APN_ID_USS))) {

v_PDNType := UAS_USS_DNN;

} else if (match (p_APN, f_DomainName_Encode(pc_APN_ID_C2))) {

v_PDNType := UAS_C2_DNN;

*/

} else if (f_IsDNNForEmergency(p_APN)) {  // this isn't specified in table 4.8.4-1, but is a valid PDU type

v_PDNType := Emergency_PDN;

}

return v_PDNType;

}

## 2.3	Change 3

| Function name | f_NR5GC_PDUSessionEstablishment_InnerLoop |
| --- | --- |
| Reason for change | The default message contents of the PDU Session Establishment Accept message defined in 38.508-1 table 4.7.2-2 specifies that the DNN shall be: The DNN/APN ID of the entry in Table 4.8.4-1 which has been determined by the DNN IE in the UL NAS TRANSPORT message which carried the corresponding PDU SESSION ESTABLISHMENT REQUEST or by pc_APN_Default_Configuration if the DNN IE was not present |
| Summary of change | Simply encode the appropriate PICS parameter for the DNN sent in the Accept message. Same change made in f_MultipleQoSPreamble_Part2. |
| TTCN module | NG5GC_NASSteps.ttcn |

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId,

NR_SRB_COMMON_IND p_ReceivedAsp,

SRBs_AlreadyConfigured p_SRB := SRB1_2,

template (omit) SDAP_Config p_SDAP_Config,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template DNN p_ExpectedDNN := *,

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (omit) DNN p_DNN := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (present) O1_Type p_PDU_SessionId := ?,

template ProcedureTransactionIdentifier p_PTI := omit,

template (omit) PDU_SessionType p_PDU_SessionType := omit,

template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@

template NG_UE_SM_Cap p_UECap := *,

template MaxNumPacketFilters p_MaxNumPacketFilters := *,

template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?,

template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *,

template SM_PDU_DN_RequestContainer p_PDUReq := *,

template ExtdProtocolConfigOptions p_ExtdPCO := *,

template NG_PDU_SessionId p_OldPDU_SessionId := *,

template NG_Request_Type p_RequestType := ?,

template AdditionalInformation p_AdditionalInfo := *,

template (omit) GMM_GSM_Cause p_Cause := omit,

template (omit) GPRS_Timer p_RQTimer := omit,

template (omit) EAP_Message p_EAP := omit,

template (omit) GPRS_Timer3 p_BackOff := omit,

boolean p_PDUSessionAuthenticationRequired := false,

template (omit) charstring p_BuildQosFlow := omit, // set to required value if a specific QoSFlow must be built (with interworkingWithEPS parameters)

template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, // Only included when called from S1ToN1 TAU

boolean p_OmitDNN := false  // @sic R5-223987 sic@ - Only set to true if the DNN is to be omitted from Accept message

) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@

{ // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@

// Moved UT trigger up to f_NR5GC_PDUSessionEstablishment @sic R5s200751 sic@

var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); // @sic R5-195328 sic@

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) NR_RadioBearerList_Type v_SSConfig;

var template (value) QosFlow_Identification_Type v_QosFlowId;

var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList;

var SdapConfig_Type v_SdapConfig;

var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId);

var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

var template (value) SDAP_Config v_SDAP_Config;

var PDN_Index_Type v_PDN_Index;

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1;

var NG_NAS_SecurityParams_Type v_SecurityParams; // @sic R5-198184 sic@

var template (omit) DNN v_DNN;

var PDU_PDN_DNN_Type v_PDUType := None;  // @sic R5s210148 sic@

var template (omit) IMS_TestConfiguration_Type v_MediaType := omit; // @sic R5s211486 sic@

var boolean v_ExistingPDN := false; // @sic R5s210149, R5s221126 sic@

var template (omit) PDN_AddressInfo_Type v_ExtraPDN_Address := omit;

var template (omit) EthernetHeaderCompressConfig v_EthernetHeaderCompressConfig := omit;

var integer i;

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo,

p_ReceivedAsp.Signalling.Nas[0].Pdu,

p_PDU_SessionId,

p_PTI,

p_IntegrityProtMaxDataRate,

p_PDU_SessionType,

p_ExpectedSSC_Mode,

p_UECap,

p_MaxNumPacketFilters,

p_AlwaysOnPDUSessionReq,

p_PDUReq,

p_ExtdPCO,

p_OldPDU_SessionId,

p_RequestType,

p_ExpectedS_NSSAI,

p_ExpectedDNN,

p_AdditionalInfo)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

if (not isvalue(p_DNN) and not p_OmitDNN ) {  // @sic R5s210148, R5s221229 sic@

// Use the details received in the Request msg

v_PDUType := f_GetPDNTypeFromDNN(v_GSM_MobilityInfo.DNN);   // @sic R5-205939 sic@

if (v_PDUType == None) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "DNN PICS do not match received DNN");

} else {

v_DNN := cs_DNN(-, f_DomainName_Encode(f_GetDNNStringFromPICS(v_PDUType)));

}

} else { // need to overwrite DNN received in Request msg @sic R5s221229 sic@

if (isvalue(p_DNN)) {

v_DNN := p_DNN;

v_GSM_MobilityInfo.DNN := valueof(p_DNN);

}

if (p_OmitDNN) { // @sic R5-223987 sic@

v_DNN := omit;

v_GSM_MobilityInfo.DNN := omit;

}

// If using a different DNN, need to update stored PDU Type too

v_PDUType := f_GetPDNTypeFromDNN(v_GSM_MobilityInfo.DNN);

if (v_PDUType == None) { // if this DNN isn't known, just use default

v_PDUType := Internet_DNN;

}

v_GSM_MobilityInfo.PDUType := v_PDUType;

}

...

## 2.4	Change 4

| Function name | f_Check_NG_PDUSessionEstablishmentReq |
| --- | --- |
| Reason for change | Need to determine the PDN Type, including if it’s an emergency PDN, as soon as the PDU Session Establishment Request message is received |
| Summary of change | Move the check for an emergency PDN from f_NR5GC_PDUSessionEstablishment_InnerLoop to f_Check_NG_PDUSessionEstablishmentReq |
| TTCN module | NG5GC_NASSteps.ttcn |

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId,

NR_SRB_COMMON_IND p_ReceivedAsp,

SRBs_AlreadyConfigured p_SRB := SRB1_2,

template (omit) SDAP_Config p_SDAP_Config,

template S_NSSAI_Type p_ExpectedS_NSSAI := *,

template DNN p_ExpectedDNN := *,

template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B),

template (omit) DNN p_DNN := omit,

template (omit) QoS_Rules p_QoS_Rules := omit,

template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O),

template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@

template (omit) QoSFlowDescr p_QoSFlowDescr := omit,

template (present) O1_Type p_PDU_SessionId := ?,

template ProcedureTransactionIdentifier p_PTI := omit,

template (omit) PDU_SessionType p_PDU_SessionType := omit,

template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@

template NG_UE_SM_Cap p_UECap := *,

template MaxNumPacketFilters p_MaxNumPacketFilters := *,

template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?,

template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *,

template SM_PDU_DN_RequestContainer p_PDUReq := *,

template ExtdProtocolConfigOptions p_ExtdPCO := *,

template NG_PDU_SessionId p_OldPDU_SessionId := *,

template NG_Request_Type p_RequestType := ?,

template AdditionalInformation p_AdditionalInfo := *,

template (omit) GMM_GSM_Cause p_Cause := omit,

template (omit) GPRS_Timer p_RQTimer := omit,

template (omit) EAP_Message p_EAP := omit,

template (omit) GPRS_Timer3 p_BackOff := omit,

boolean p_PDUSessionAuthenticationRequired := false,

template (omit) charstring p_BuildQosFlow := omit, // set to required value if a specific QoSFlow must be built (with interworkingWithEPS parameters)

template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit, // Only included when called from S1ToN1 TAU

boolean p_OmitDNN := false  // @sic R5-223987 sic@ - Only set to true if the DNN is to be omitted from Accept message

) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@

{ // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@

// Moved UT trigger up to f_NR5GC_PDUSessionEstablishment @sic R5s200751 sic@

var GMM_MobilityInfo_Type v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); // @sic R5-195328 sic@

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) NR_RadioBearerList_Type v_SSConfig;

var template (value) QosFlow_Identification_Type v_QosFlowId;

var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList;

var SdapConfig_Type v_SdapConfig;

var PLMN_Identity v_ASN_PLMN := f_NR_CellInfo_GetPLMN (p_NR_CellId);

var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

var template (value) SDAP_Config v_SDAP_Config;

var PDN_Index_Type v_PDN_Index;

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1;

var NG_NAS_SecurityParams_Type v_SecurityParams; // @sic R5-198184 sic@

var template (omit) DNN v_DNN;

var PDU_PDN_DNN_Type v_PDUType := None;  // @sic R5s210148 sic@

var template (omit) IMS_TestConfiguration_Type v_MediaType := omit; // @sic R5s211486 sic@

var boolean v_ExistingPDN := false; // @sic R5s210149, R5s221126 sic@

var template (omit) PDN_AddressInfo_Type v_ExtraPDN_Address := omit;

var template (omit) EthernetHeaderCompressConfig v_EthernetHeaderCompressConfig := omit;

var integer i;

...

if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.pduSessionType.typeValue, '101'B)) { // @sic R5-227473 sic@

// Ethernet PDU session

if (not ispresent(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ds_TT_EthernetPortMACAddr)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Ethernet PDU Request doesn't contain MAC Address");

}

if (ispresent(p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ethernetHeaderCompressConfig)) {

v_EthernetHeaderCompressConfig := p_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request.ethernetHeaderCompressConfig;

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Ethernet PDU Request doesn't contain compression header");

}

}

if (isvalue (p_SDAP_Config)) { // @sic R5s201327 sic@ - compiler warning may be ignored due to check

v_SDAP_Config := p_SDAP_Config; //@sic R5-190720 sic@

} else {

select(v_GSM_MobilityInfo.PDUType) { // @sic R5-221467 sic@

case(IMS_DNN, Emergency_PDN) {   // @sic R5-206296, R5s210518 sic@

v_SDAP_Config := cs_SDAP_Config(-, -, {2});

v_MediaType := IPCAN_EmergencyCall; // @sic R5s211486 sic@

}

case (URLLC_DNN) {

v_SDAP_Config := cs_SDAP_Config(-, -, {9}); // @sic R5-221467 sic@

}

case else {

v_SDAP_Config := cs_SDAP_Config;

}

}

}

f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig,

v_CellGroupConfig,

v_SSConfig,

v_SDAP_ConfigurationList,

p_NR_CellId,

{v_GSM_MobilityInfo},

p_SRB,

v_SDAP_Config); // @sic R5s201532 sic@

...

function f_Check_NG_PDUSessionEstablishmentReq(out GSM_MobilityInfo_Type p_GSM_MobilityInfo,

NG_NAS_UL_Pdu_Type p_ReceivedMsgs,

template (present) O1_Type p_PDU_SessionId := ?,

template ProcedureTransactionIdentifier p_PTI := omit,

template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?,

template PDU_SessionType p_PDU_SessionType := omit,

template SSC_Mode p_SSC_Mode := *, // @sic R5s200162 sic@

template NG_UE_SM_Cap p_UECap := *, // @sic R5s200162 sic@

template MaxNumPacketFilters p_MaxNumPacketFilters := *, // @sic R5s200162 sic@

template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, // @sic R5s200106 sic@

template SM_PDU_DN_RequestContainer p_PDUReq := *, // @sic R5s200162 sic@

template ExtdProtocolConfigOptions p_ExtdPCO := *, // @sic R5s200162 sic@

template NG_PDU_SessionId p_OldPDU_SessionId  := *,

template NG_Request_Type p_RequestType  := cr_NG_Request_Type('001'B),

template S_NSSAI_Type p_S_NSSAI  := *,

template DNN p_DNN  := *,

template AdditionalInformation p_AdditionalInfo  := *,

template MA_PDUSessionInfo p_MA_PDUSessionInfo := *,      // @sic R5s201387 Baseline Moving sic@

template ReleaseAssistanceInd p_ReleaseAssistanceInd := *,

template IPHeaderCompressionConfig p_IPHeaderCompConfig := *,

template DS_TT_EthernetPortMACAddr p_DS_TT_EthernetPortMACAddr := *,

template UE_DS_TT_ResidenceTime p_UE_DS_TT_ResidenceTime := *,

template PortManagementInfoContainer p_PortManagementInfoContainer := *,

template EthernetHeaderCompressConfig p_EthernetHeaderCompConfig := *,

template PDU_Address p_SuggestedInterfaceId := *,

template ServiceLvlAAContainer p_ServiceLvlAA := *,

template RequestedMBSContainer p_RequestedMBS := *,

template PDUSessionPairId p_PduSessionPairId := *,

template RSN p_RSN := *,

template URSPruleEnforceReports p_URSPruleEnforceReports := *,

template S_NSSAI_Type p_AltS_NSSAI := *) return boolean

{

var boolean v_Result := false;

var NG_NAS_UL_Message_Type v_GSMMsg := p_ReceivedMsgs.PiggybackedPduList[0].Msg;

var template (present) ProcedureTransactionIdentifier v_PTI;

var template (present) PDU_SessionType v_SessionType;

if (ispresent(p_PTI)) {

v_PTI := p_PTI;   // compiler warning can be ignored due to previous check

} else {

v_PTI := complement('00'O, 'FF'O);

}

if (ispresent(p_PDU_SessionType)) {

v_SessionType := p_PDU_SessionType;  // compiler warning can be ignoed due to previous check

} else {

v_SessionType := cr_PDU_SessionType(('001'B, '010'B, '011'B, '101'B)); // @sic R5s190704, R5-221493 sic@

}

// Check NAS Transport msg first

v_Result := f_Check_NG_ULNASTransport (p_ReceivedMsgs.Msg,

cr_PayloadContainerType(tsc_PayloadContainerESMMsg), // @sic R5w190208 sic@

p_PDU_SessionId,

p_OldPDU_SessionId,

p_RequestType,

p_S_NSSAI,

p_DNN,

p_AdditionalInfo,

p_MA_PDUSessionInfo,   // @sic R5s201387 Baseline Moving sic@

p_ReleaseAssistanceInd,

-,                    // Jun24 @sic R5s240531 Baseline Moving sic@

p_AltS_NSSAI);

if (not v_Result) {

return v_Result;

} else if (match (v_GSMMsg, cr_NG_PDU_SESSION_ESTABLISHMENT_REQUEST (cr_NG_PDU_SessionId(omit, p_ReceivedMsgs.Msg.ul_Nas_Transport.pduSessionId.sessionId), // @sic R5-191222 sic@

v_PTI,

v_SessionType,

p_SSC_Mode,

p_UECap,

p_MaxNumPacketFilters,

p_IntegrityProtMaxDataRate,

p_AlwaysOnPDUSessionReq,

p_PDUReq,

p_ExtdPCO,

p_IPHeaderCompConfig,    // @sic R5s201387 Baseline Moving sic@

p_DS_TT_EthernetPortMACAddr,

p_UE_DS_TT_ResidenceTime,

p_PortManagementInfoContainer,

p_EthernetHeaderCompConfig,

p_SuggestedInterfaceId,

p_ServiceLvlAA,      // @sic R5s221179 Baseline Moving sic@

p_RequestedMBS,

p_PduSessionPairId,

p_RSN,

p_URSPruleEnforceReports))) {  //Jun24 @sic R5s240531 Baseline Moving sic@

v_Result := true;

} else {  // @sic R5s190944 sic@

v_Result := false;

}

p_GSM_MobilityInfo.SessionId := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionId.sessionId; // @sic R5s00564 sic@

p_GSM_MobilityInfo.PTI := v_GSMMsg.pdu_Session_Establishment_Request.procedureTransactionIdentifier;

p_GSM_MobilityInfo.SessionType := v_GSMMsg.pdu_Session_Establishment_Request.pduSessionType.typeValue;

p_GSM_MobilityInfo.EPS_Bearer := omit; // these will be set in the Accept message @sic R5-204400 sic@

p_GSM_MobilityInfo.PdnIndex := omit;   // these will be set in the Accept message @sic R5-204400 sic@

if (ispresent (p_ReceivedMsgs.Msg.ul_Nas_Transport.dnn)) {

p_GSM_MobilityInfo.DNN := p_ReceivedMsgs.Msg.ul_Nas_Transport.dnn;

} else {

p_GSM_MobilityInfo.DNN := omit;

}

if (match(p_ReceivedMsgs.Msg.ul_Nas_Transport.requestType.requestValue, '011'B) or

match(p_ReceivedMsgs.Msg.ul_Nas_Transport.requestType.requestValue, '100'B)) { // @sic R5-202550, R5s210518, R5s20883 sic@

// this is the only way to flag it's an emergency PDU

p_GSM_MobilityInfo.PDUType := Emergency_PDN;  // @sic R5-206296 sic@

} else {

p_GSM_MobilityInfo.PDUType := f_GetPDNTypeFromDNN(p_GSM_MobilityInfo.DNN);  // @sic R5-206296 sic@

}

if (isvalue(p_PDU_SessionType)) { // @sic R5-227473 sic@

if (match ('101'B, p_PDU_SessionType.typeValue)) {

// make sure the correct PDU Type is set as this is used to decide other IE values

p_GSM_MobilityInfo.PDUType := Ethernet_DNN;

}

}

if (ispresent (v_GSMMsg.pdu_Session_Establishment_Request.alwaysOnPDUSessionReq)) {

p_GSM_MobilityInfo.AlwaysOn := v_GSMMsg.pdu_Session_Establishment_Request.alwaysOnPDUSessionReq;

} else {

p_GSM_MobilityInfo.AlwaysOn := omit;

}

if (match(p_GSM_MobilityInfo.PDUType, (URLLC_DNN, Ethernet_DNN))) { // @sic R5s221179, R5-227473 sic@

if (not match(p_GSM_MobilityInfo.AlwaysOn, cr_AlwaysOnRequested)) {

v_Result := false;

}

}

if (ispresent (v_GSMMsg.pdu_Session_Establishment_Request.extdProtocolConfigurationOptions)) {

p_GSM_MobilityInfo.PCO := v_GSMMsg.pdu_Session_Establishment_Request.extdProtocolConfigurationOptions;

} else {

p_GSM_MobilityInfo.PCO := omit;

}

if (pc_PS_data_off) { // @sic R5-216155 sic@

v_Result := f_ExtdProtocolConfigOptions_CheckPSDataOff(p_GSM_MobilityInfo.PCO);

}

if (ispresent (p_ReceivedMsgs.Msg.ul_Nas_Transport.s_NSSAI)) { // @sic R5s210148 sic@

p_GSM_MobilityInfo.NSSAI := p_ReceivedMsgs.Msg.ul_Nas_Transport.s_NSSAI;

} else {

p_GSM_MobilityInfo.NSSAI := omit;

}

return v_Result;

}