3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240643

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3570 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.1.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-10-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | After step23 and step30, TTCN will clear the stored DRB and PDU sessions because of call to function f_NR5GC_MobileInfo_Init. This is fine for the devices that would set the 5GS Registration type as Initial registration at Steps 23Ca1 and 30Ca1. This behaviour is applicable for a Rel-15 device However, for a Rel-16 device, according to Note 2 in Table 6.1.1.6.3.2-3, R16 onwards UE that aply the 5GS registration type as Mobility Registration. This device will follow the path in TTCN which does not establish any DRBs. Thus, if this Rel-16 UE is IMS enabled, in postamble, it will cause an error because there wont be a DRB established, and the function flow f_NR_Postamble f_NR_UE_DeRegisterOnSwitchOff f_NR5GC_OptionalIMSDeregisterOnSwitchOff f_NR5GC_RRC_ConnectedState3N_Steps5_8 f_NR_RRCReconfigExistingDRBs f_NR_BuildRRCReconfigExistingDRBs the following is executed : if (v_NoOfDRBs == 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No DRBs to be configured"); } This needs to be adressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add condition only when R15 UE, ttcn shall clear the DRB and PDU session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

- Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

- Corrections required

2.1 Change 1

| Function name | f_TC_6_1_1_6_NR5GC_TestBody () |
| --- | --- |
| Reason for change | After step23 and step30, TTCN will clear the stored DRB and PDU sessions because of call to function f_NR5GC_MobileInfo_Init. This is fine for the devices that would set the 5GS Registration type as Initial registration at Steps 23Ca1 and 30Ca1. This behaviour is applicable for a Rel-15 device However, for a Rel-16 device, according to Note 2 in Table 6.1.1.6.3.2-3, R16 onwards UE that aply the 5GS registration type as Mobility Registration. This device will follow the path in TTCN which does not establish any DRBs. Thus, if this Rel-16 UE is IMS enabled, in postamble, it will cause an error because there wont be a DRB established, and the function flow f_NR_Postamble f_NR_UE_DeRegisterOnSwitchOff f_NR5GC_OptionalIMSDeregisterOnSwitchOff f_NR5GC_RRC_ConnectedState3N_Steps5_8 f_NR_RRCReconfigExistingDRBs f_NR_BuildRRCReconfigExistingDRBs the following is executed : if (v_NoOfDRBs == 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No DRBs to be configured"); } This needs to be adressed. |
| Summary of change | Add condition only when R15 UE, ttcn shall clear the DRB and PDU session. |
| TTCN module | \6_1_1\Idle_PLMNSelection_NR5GC.ttcn |
| MCC160 Comments | Accepted |

Before Change:

| function f_TC_6_1_1_6_NR5GC_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; timer t_HpPlmn_lower; timer t_HpPlmn_upper; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell13, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; v_CellPowerList_AtT2:={ cs_NR_CellPower(nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // SS adjusts cell levels according to row T1 f_NR_SetCellPowerList(v_CellPowerList_AtT1); // @siclog "Step 2" siclog@ // Power on the UE. f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s210729 sic@ t_HpPlmn_lower.start(f_NR_SetTimerToleranceMin(nr_Cell12, nonProtocolTimer, 120.0)); t_HpPlmn_upper.start(f_NR_SetTimerToleranceMax(nr_Cell12, nonProtocolTimer, 720.0)); // @sic R5-213673 sic@ // @siclog "Step 3 - 22a1" siclog@ // Steps 1 to 20a1 of the registration procedure described in TS 38.508-1[4] table 4.5.2.2-2 are performed on NR Cell 12 f_NR5GC_RRC_IdleState1N_Def (nr_Cell12, TESTMode_OFF, rrcConnectionRelease, Initial_NoSecurity); // @siclog "Step 23" siclog@ // Does the UE send an RRCSetupRequest on NR Cell 13 after 120 seconds, but before 720 seconds (MinimumPeriodicSearchTimer) from power on? alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell13, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { //verdict fail RRCSetupRequest received on NR Cell 13 within t_HpPlmn_lower time f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.1.1.6 Step 23"); } [] t_HpPlmn_lower.timeout { alt { [] SRB.check(receive( car_NR_SRB0_RrcPdu_IND(nr_Cell13, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_HpPlmn_upper.stop; // verdict pass RRCSetupRequest received on NR Cell 13 after t_HpPlmn_lower time but before t_HpPlmn_upper f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.6 Step 23"); } [] t_HpPlmn_upper.timeout { // verdict fail No RRCSetupRequest received f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.1.1.6 Step 23"); } } } } f_NR5GC_MobileInfo_Init(); // @sic R5s210390 sic@ // @siclog "Step 23A - 23Cb3a1" siclog@ t_HpPlmn_upper.start(f_NR_SetTimerToleranceMin(nr_Cell13, nonProtocolTimer, 420.0));// @sic R5-210628 sic@ f_NR5GC_InitialOrMobilityRegistration(nr_Cell13);// @sic R5-210628 sic@ // @siclog "Step 24 - 28a1" siclog@ // Void // @siclog "Step 29" siclog@ // SS adjusts cell levels according to row T2 f_NR_SetCellPowerList(v_CellPowerList_AtT2); // @siclog "Step 30" siclog@ // Does the UE send an RRCSetupRequest on NR Cell 11 after 420 seconds (MinimumPeriodicSearchTimer) from step 24 alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) {// verdict fail RRCSetupRequest received on NR Cell 11 within t_HpPlmn_upper time f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 30: UE sent an RRCSetupRequest on NR Cell 11 within 420s"); } [] t_HpPlmn_upper.timeout {// verdict pass RRCSetupRequest received on NR Cell 11 after t_HpPlmn_upper SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))); f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.6 Step 30"); } } f_NR5GC_MobileInfo_Init(); // @sic R5s210390 sic@ // @siclog "Step 30A - 30Cb3a1" siclog@ f_NR5GC_InitialOrMobilityRegistration(nr_Cell11);// @sic R5-210628 sic@ // @siclog "Step 31 - 35a1" siclog@ // Void } |  |
| --- | --- |

After Change:

| function f_TC_6_1_1_6_NR5GC_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; timer t_HpPlmn_lower; timer t_HpPlmn_upper; var UE_NR_Capability v_NrCapability; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell13, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; v_CellPowerList_AtT2:={ cs_NR_CellPower(nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // SS adjusts cell levels according to row T1 f_NR_SetCellPowerList(v_CellPowerList_AtT1); // @siclog "Step 2" siclog@ // Power on the UE. f_NR5GC_SwitchOnAndResetIMS(); // @sic R5s210729 sic@ t_HpPlmn_lower.start(f_NR_SetTimerToleranceMin(nr_Cell12, nonProtocolTimer, 120.0)); t_HpPlmn_upper.start(f_NR_SetTimerToleranceMax(nr_Cell12, nonProtocolTimer, 720.0)); // @sic R5-213673 sic@ // @siclog "Step 3 - 22a1" siclog@ // Steps 1 to 20a1 of the registration procedure described in TS 38.508-1[4] table 4.5.2.2-2 are performed on NR Cell 12 f_NR5GC_RRC_IdleState1N_Def (nr_Cell12, TESTMode_OFF, rrcConnectionRelease, Initial_NoSecurity); // @siclog "Step 23" siclog@ // Does the UE send an RRCSetupRequest on NR Cell 13 after 120 seconds, but before 720 seconds (MinimumPeriodicSearchTimer) from power on? alt { [] SRB.receive( car_NR_SRB0_RrcPdu_IND(nr_Cell13, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) { //verdict fail RRCSetupRequest received on NR Cell 13 within t_HpPlmn_lower time f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.1.1.6 Step 23"); } [] t_HpPlmn_lower.timeout { alt { [] SRB.check(receive( car_NR_SRB0_RrcPdu_IND(nr_Cell13, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))) { t_HpPlmn_upper.stop; // verdict pass RRCSetupRequest received on NR Cell 13 after t_HpPlmn_lower time but before t_HpPlmn_upper f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.6 Step 23"); } [] t_HpPlmn_upper.timeout { // verdict fail No RRCSetupRequest received f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "Test Case 6.1.1.6 Step 23"); } } } } v_NrCapability := f_NR_MobileInfo_NRCap_Get(); if (match (v_NrCapability.accessStratumRelease, rel15)){ f_NR5GC_MobileInfo_Init(); // @sic R5s210390 sic@ } // @siclog "Step 23A - 23Cb3a1" siclog@ t_HpPlmn_upper.start(f_NR_SetTimerToleranceMin(nr_Cell13, nonProtocolTimer, 420.0));// @sic R5-210628 sic@ f_NR5GC_InitialOrMobilityRegistration(nr_Cell13);// @sic R5-210628 sic@ // @siclog "Step 24 - 28a1" siclog@ // Void // @siclog "Step 29" siclog@ // SS adjusts cell levels according to row T2 f_NR_SetCellPowerList(v_CellPowerList_AtT2); // @siclog "Step 30" siclog@ // Does the UE send an RRCSetupRequest on NR Cell 11 after 420 seconds (MinimumPeriodicSearchTimer) from step 24 alt { [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))) {// verdict fail RRCSetupRequest received on NR Cell 11 within t_HpPlmn_upper time f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 30: UE sent an RRCSetupRequest on NR Cell 11 within 420s"); } [] t_HpPlmn_upper.timeout {// verdict pass RRCSetupRequest received on NR Cell 11 after t_HpPlmn_upper SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any)))); f_NR_PreliminaryPass (__FILE__, __LINE__, "Test Case 6.1.1.6 Step 30"); } } if (match (v_NrCapability.accessStratumRelease, rel15)){ f_NR5GC_MobileInfo_Init(); // @sic R5s210390 sic@ } // @siclog "Step 30A - 30Cb3a1" siclog@ f_NR5GC_InitialOrMobilityRegistration(nr_Cell11);// @sic R5-210628 sic@ // @siclog "Step 31 - 35a1" siclog@ // Void } |  |
| --- | --- |