3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240679

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3589 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC Test Cases 11.1.2, 11.1.5, 11.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-11-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15 is modified according to R5s240583 for TCs 11.1.2, 11.1.5, and 11.1.6 such that a completely new PDN connection is expected from UE. However, in TS 23.502, clause 4.11.2.2 for Mobility Procedure from 5GS to EPS without N26 interface it is stated that: “If the UE has remaining PDU Sessions in 5GS which it wants to transfer to EPS and maintain the same IP address/prefix, the UE performs the UE requested PDN Connectivity Procedure as specified in TS 23.401 [13] clause 5.10.2 and sets the Request Type to "handover" in Step 1 of the procedure with modification captured in clause 4.11.2.4.2. UE provides an APN and the PDU Session ID corresponding to the PDU Session it wants to transfer to EPS. The UE provides the PDU Session ID in PCO as described in clause 4.11.1.1.”. In current implementation of function, f_IP_Handling_StartPDN is always triggered, expecting a new PDN Connection, whereas the UE has already triggered a PDN Connectivity Request with type set as handover to move previously established PDU in 5GS. Hence, there is no requirement for a completely new PDN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify the implementation of R5s240582 for function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15 such that if Interwork without N26 is SUPPORTED then a new PDN Connection is not to be expected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.2.NR5GC, 11.1.5.NR5GC, 11.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact:	Kalahasti, Narendra

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_EUTRA38_MultiPDN_InitialRegistration_Step14_15 |
| --- | --- |
| Reason for change | Function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15 is modified according to R5s240583 for TCs 11.1.2, 11.1.5, and 11.1.6 such that a completely new PDN connection is expected from UE. However, in TS 23.502, clause 4.11.2.2 for Mobility Procedure from 5GS to EPS without N26 interface it is stated that: “If the UE has remaining PDU Sessions in 5GS which it wants to transfer to EPS and maintain the same IP address/prefix, the UE performs the UE requested PDN Connectivity Procedure as specified in TS 23.401 [13] clause 5.10.2 and sets the Request Type to "handover" in Step 1 of the procedure with modification captured in clause 4.11.2.4.2. UE provides an APN and the PDU Session ID corresponding to the PDU Session it wants to transfer to EPS. The UE provides the PDU Session ID in PCO as described in clause 4.11.1.1.”. In current implementation of function, f_IP_Handling_StartPDN is always triggered, expecting a new PDN Connection, whereas the UE has already triggered a PDN Connectivity Request with type set as handover to move previously established PDU in 5GS. Hence, there is no requirement for a completely new PDN. |
| Summary of change | Modify the implementation such that if Interwork without N26 is SUPPORTED then a new PDN Connection is not to be expected. |
| TTCN module | EUTRA38_NR5GC_InitialRegistration.ttcn |
| MCC160 Comment | 23.502 cl. 4.11.2.4.1 specifies: If the TAU was rejected in step 2 the IP address preservation is not provided. In this case the UE provides IMSI in the Attach Request and does not provide a Request Type "Handover" in the PDN CONNECTIVITY Request if included in the Attach Request. Therefore determined if a new PDN connection is started or not based on the Request Type in the PDN Connectivity Request message (the Request type is not specified for step 8b3, so changed this to the default value – for initial attach) |

Before Change

/* * @desc step 14 and 15 of UE Registration acc. to 36.508 cl. 4.5.2 * @param p_CellId * @param p_ForcedAttach * @param p_AdditionalUpdateType * @param p_EPS_TI * @param p_IPv4AllocationViaNasFlag * @param p_PcoFromUE * @param p_APN * @param p_T3412 (default value: cs_GprsTimer_v_deact) * @param p_T3402 (default value: omit) * @param p_T3423 (default value: omit) * @param p_EquivalentPlmnList (default value: omit) * @param p_NoOfEmergencyNumbers (default value: 0) * @param p_EMM_Cause (default value: omit) * @param p_DCN_Id (default value: omit) * @param p_ExtdPcoFromUE (default value: omit) * @param p_PduInfoFromNR (default value: omit) * @param p_UERadioCapId (default value: omit) * @param p_N1disabled (default value: false) * @return EPS_BearerIdentity * @status APPROVED (NR5GC_IRAT) */ function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15(EUTRA_CellId_Type p_CellId, EUTRA_ATTACH_Type p_ForcedAttach, template (omit) AdditionalUpdateType p_AdditionalUpdateType, ProcedureTransactionIdentifier p_EPS_TI, boolean p_IPv4AllocationViaNasFlag, template (omit) ProtocolConfigOptions p_PcoFromUE, template (omit) AccessPointName p_APN, template (value) GPRS_Timer p_T3412 := cs_GprsTimer_v_deact, template (omit) GPRS_Timer p_T3402 := omit, template (omit) GPRS_Timer p_T3423 := omit, template (omit) PLMN_List p_EquivalentPlmnList := omit, integer p_NoOfEmergencyNumbers := 0, template (omit) EMM_Cause p_EMM_Cause := omit, template (omit) DCN_ID p_DCN_Id := omit, template (omit) ExtdProtocolConfigOptions p_ExtdPcoFromUE := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit, template (omit) UERadioCapId p_UERadioCapId:= omit, boolean p_N1disabled := false) runs on EUTRA_Multi5GS_PTC return EPS_BearerIdentity { var PDN_TypeToBeUsed_Type v_PDN_TypeToBeUsed := f_EUTRA_MobileInfo_GetPdnTypeToBeUsed(); var NAS_AttDetValue_Type v_AttachType := f_GetEAttachType(p_ForcedAttach); var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(p_CellId); var template (value) MobileIdentity v_Guti := f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId (v_GutiParams.PLMN_Identity); var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(p_CellId); var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, p_ForcedAttach); var TrackingAreaCode v_TAC := f_EUTRA_CellInfo_GetTAC(p_CellId); var template (value) AccessPointName v_APN_Tx := f_EUTRA38_MultiPDN_GetAPN(p_CellId, p_APN); var template (omit) EmergNumList v_EmergNumList := omit; var template (omit) MobileIdentity v_MSId := f_GetMSId(p_ForcedAttach); var PDU_PDN_DNN_Type v_PDN_Type := f_EUTRA38_MultiPDN_GetPDNType(p_APN); // @sic R5-206296 sic@ var PDN_Index_Type v_PdnIndex := f_GetMultiPdnIndex (v_PDN_Type); // @sic R5-206296 sic@ var boolean v_IsIMS := false; var EPS_BearerIdentity v_EpsDefaultBearerId; var template (omit) ESM_Cause v_ESM_Cause := omit; var template (value) PDN_Address v_PDN_Address; var EmergencyNumList v_EmergencyNumList; var boolean v_StartPDN := true; // @sic R5s210149 sic@ var template (value) ProtocolConfigOptions v_PcoToUE; var BEARER_CONTEXT_TYPE v_BearerContextNumber := DEF_1; // @sic R5s230279 sic@ var InterworkWithoutN26 v_IwkN26 := f_EUTRA38_GetNetworkSupport(); //Anritsu TTCN CR R5s240583 MCC var integer i; if (pc_IPv6 and pc_IPv4) { select (v_PDN_TypeToBeUsed) { case (forceIPv4only) { v_ESM_Cause := cs_ESM_Cause_tv('00110010'B); } // PDN type IPv4 only allowed case (forceIPv6only) { v_ESM_Cause := cs_ESM_Cause_tv('00110011'B); } // PDN type IPv6 only allowed case (pdnTypeAsSupportedByUE) { v_ESM_Cause := omit; } } } if (p_NoOfEmergencyNumbers > 0) { v_EmergencyNumList := f_Get_EmergencyNumList (p_NoOfEmergencyNumbers, f_EUTRA_MobileInfo_GetLocalEmergencyNumberList()); f_EUTRA_MobileInfo_SetLocalEmergencyNumberList (v_EmergencyNumList); // save the list of locally usable emergency numbers v_EmergNumList := f_Build_EmergNumList(p_NoOfEmergencyNumbers, v_EmergencyNumList); } select(v_PDN_Type) { // @sic R5-206296 sic@ other options not relevant case (IMS_DNN, Emergency_PDN) { // @sic R5s230279 sic@ v_IsIMS := true; v_BearerContextNumber := DEF_2; } } v_EpsDefaultBearerId := f_EUTRA38_MultiPDN_ConfigureDRB(p_CellId, v_PDN_Type, v_IsIMS); v_PcoToUE := f_NAS_ProtocolConfigOptionsTX(f_EUTRA38_MultiPDN_GetPCOs(p_CellId, v_PDN_Type, // @sic R5s220883 sic@ v_IsIMS, v_EpsDefaultBearerId, p_ExtdPcoFromUE, p_PcoFromUE, p_N1disabled)); // @sic R5-227443 sic@ if (isvalue (p_ExtdPcoFromUE)) { // FFS - At the moment, 36.508 table 4.7.3-6 specifies ExtdPCO should be not present } if (isvalue(p_PduInfoFromNR) and v_IwkN26 == NOT_SUPPORTED) { // @sic R5s210149 sic@ //Anritsu TTCN CR R5s240583 MCC // The UE has just moved from NR. If so, data from NR should be used, rather than created anew for (i := 0; i < lengthof (p_PduInfoFromNR); i := i + 1) { if (v_PdnIndex == valueof(p_PduInfoFromNR[i].PdnIndex)) { // Data from NR should be used, rather than created anew v_StartPDN := false; // no need to change DRB mapping as this has already been done in calling function break; } } } if (v_StartPDN) { // completely new PDN connection @sic R5s210149 sic@ f_IP_Handling_StartPDN(IP, v_PdnIndex, cs_DrbInfo_EUTRA(p_CellId, (hex2int(v_EpsDefaultBearerId)-4))); } v_PDN_Address := f_EUTRA_GetPDNAddress(p_IPv4AllocationViaNasFlag, v_PdnIndex); f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams(p_CellId, v_EpsDefaultBearerId, tsc_RRC_TI_Def, p_EPS_TI, v_AttachType, v_PDN_Address, v_Guti, cs_TAIListNonConsecutive_lv(v_PLMN, {bit2oct(v_TAC)}), v_LAI, v_MSId, v_ESM_Cause, v_APN_Tx, // @sic R5-206296 sic@ v_PcoToUE, f_GetAdditionalUpdateResult(p_AdditionalUpdateType, p_ForcedAttach), p_T3412, p_T3402, p_T3423, p_EquivalentPlmnList, v_EmergNumList, -, -, v_BearerContextNumber, p_EMM_Cause, omit, p_DCN_Id, omit, -, p_UERadioCapId); // @sic R5-216280 sic@ return v_EpsDefaultBearerId; }

After Change

/* * @desc step 14 and 15 of UE Registration acc. to 36.508 cl. 4.5.2 * @param p_CellId * @param p_ForcedAttach * @param p_AdditionalUpdateType * @param p_EPS_TI * @param p_IPv4AllocationViaNasFlag * @param p_PcoFromUE * @param p_APN * @param p_T3412 (default value: cs_GprsTimer_v_deact) * @param p_T3402 (default value: omit) * @param p_T3423 (default value: omit) * @param p_EquivalentPlmnList (default value: omit) * @param p_NoOfEmergencyNumbers (default value: 0) * @param p_EMM_Cause (default value: omit) * @param p_DCN_Id (default value: omit) * @param p_ExtdPcoFromUE (default value: omit) * @param p_PduInfoFromNR (default value: omit) * @param p_UERadioCapId (default value: omit) * @param p_N1disabled (default value: false) * @return EPS_BearerIdentity * @status APPROVED (NR5GC_IRAT) */ function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15(EUTRA_CellId_Type p_CellId, EUTRA_ATTACH_Type p_ForcedAttach, template (omit) AdditionalUpdateType p_AdditionalUpdateType, ProcedureTransactionIdentifier p_EPS_TI, boolean p_IPv4AllocationViaNasFlag, template (omit) ProtocolConfigOptions p_PcoFromUE, template (omit) AccessPointName p_APN, template (value) GPRS_Timer p_T3412 := cs_GprsTimer_v_deact, template (omit) GPRS_Timer p_T3402 := omit, template (omit) GPRS_Timer p_T3423 := omit, template (omit) PLMN_List p_EquivalentPlmnList := omit, integer p_NoOfEmergencyNumbers := 0, template (omit) EMM_Cause p_EMM_Cause := omit, template (omit) DCN_ID p_DCN_Id := omit, template (omit) ExtdProtocolConfigOptions p_ExtdPcoFromUE := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit, template (omit) UERadioCapId p_UERadioCapId:= omit, boolean p_N1disabled := false) runs on EUTRA_Multi5GS_PTC return EPS_BearerIdentity { var PDN_TypeToBeUsed_Type v_PDN_TypeToBeUsed := f_EUTRA_MobileInfo_GetPdnTypeToBeUsed(); var NAS_AttDetValue_Type v_AttachType := f_GetEAttachType(p_ForcedAttach); var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(p_CellId); var template (value) MobileIdentity v_Guti := f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId (v_GutiParams.PLMN_Identity); var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(p_CellId); var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, p_ForcedAttach); var TrackingAreaCode v_TAC := f_EUTRA_CellInfo_GetTAC(p_CellId); var template (value) AccessPointName v_APN_Tx := f_EUTRA38_MultiPDN_GetAPN(p_CellId, p_APN); var template (omit) EmergNumList v_EmergNumList := omit; var template (omit) MobileIdentity v_MSId := f_GetMSId(p_ForcedAttach); var PDU_PDN_DNN_Type v_PDN_Type := f_EUTRA38_MultiPDN_GetPDNType(p_APN); // @sic R5-206296 sic@ var PDN_Index_Type v_PdnIndex := f_GetMultiPdnIndex (v_PDN_Type); // @sic R5-206296 sic@ var boolean v_IsIMS := false; var EPS_BearerIdentity v_EpsDefaultBearerId; var template (omit) ESM_Cause v_ESM_Cause := omit; var template (value) PDN_Address v_PDN_Address; var EmergencyNumList v_EmergencyNumList; var boolean v_StartPDN := true; // @sic R5s210149 sic@ var template (value) ProtocolConfigOptions v_PcoToUE; var BEARER_CONTEXT_TYPE v_BearerContextNumber := DEF_1; // @sic R5s230279 sic@ var InterworkWithoutN26 v_IwkN26 := f_EUTRA38_GetNetworkSupport(); //Anritsu TTCN CR R5s240583 MCC var integer i; if (pc_IPv6 and pc_IPv4) { select (v_PDN_TypeToBeUsed) { case (forceIPv4only) { v_ESM_Cause := cs_ESM_Cause_tv('00110010'B); } // PDN type IPv4 only allowed case (forceIPv6only) { v_ESM_Cause := cs_ESM_Cause_tv('00110011'B); } // PDN type IPv6 only allowed case (pdnTypeAsSupportedByUE) { v_ESM_Cause := omit; } } } if (p_NoOfEmergencyNumbers > 0) { v_EmergencyNumList := f_Get_EmergencyNumList (p_NoOfEmergencyNumbers, f_EUTRA_MobileInfo_GetLocalEmergencyNumberList()); f_EUTRA_MobileInfo_SetLocalEmergencyNumberList (v_EmergencyNumList); // save the list of locally usable emergency numbers v_EmergNumList := f_Build_EmergNumList(p_NoOfEmergencyNumbers, v_EmergencyNumList); } select(v_PDN_Type) { // @sic R5-206296 sic@ other options not relevant case (IMS_DNN, Emergency_PDN) { // @sic R5s230279 sic@ v_IsIMS := true; v_BearerContextNumber := DEF_2; } } v_EpsDefaultBearerId := f_EUTRA38_MultiPDN_ConfigureDRB(p_CellId, v_PDN_Type, v_IsIMS); v_PcoToUE := f_NAS_ProtocolConfigOptionsTX(f_EUTRA38_MultiPDN_GetPCOs(p_CellId, v_PDN_Type, // @sic R5s220883 sic@ v_IsIMS, v_EpsDefaultBearerId, p_ExtdPcoFromUE, p_PcoFromUE, p_N1disabled)); // @sic R5-227443 sic@ if (isvalue (p_ExtdPcoFromUE)) { // FFS - At the moment, 36.508 table 4.7.3-6 specifies ExtdPCO should be not present } if (isvalue(p_PduInfoFromNR) and v_IwkN26 == SUPPORTED) { // @sic R5s210149 sic@ //Anritsu TTCN CR R5s240583 MCC // The UE has just moved from NR. If so, data from NR should be used, rather than created anew for (i := 0; i < lengthof (p_PduInfoFromNR); i := i + 1) { if (v_PdnIndex == valueof(p_PduInfoFromNR[i].PdnIndex)) { // Data from NR should be used, rather than created anew v_StartPDN := false; // no need to change DRB mapping as this has already been done in calling function break; } } } if (v_StartPDN) { // completely new PDN connection @sic R5s210149 sic@ f_IP_Handling_StartPDN(IP, v_PdnIndex, cs_DrbInfo_EUTRA(p_CellId, (hex2int(v_EpsDefaultBearerId)-4))); } v_PDN_Address := f_EUTRA_GetPDNAddress(p_IPv4AllocationViaNasFlag, v_PdnIndex); f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams(p_CellId, v_EpsDefaultBearerId, tsc_RRC_TI_Def, p_EPS_TI, v_AttachType, v_PDN_Address, v_Guti, cs_TAIListNonConsecutive_lv(v_PLMN, {bit2oct(v_TAC)}), v_LAI, v_MSId, v_ESM_Cause, v_APN_Tx, // @sic R5-206296 sic@ v_PcoToUE, f_GetAdditionalUpdateResult(p_AdditionalUpdateType, p_ForcedAttach), p_T3412, p_T3402, p_T3423, p_EquivalentPlmnList, v_EmergNumList, -, -, v_BearerContextNumber, p_EMM_Cause, omit, p_DCN_Id, omit, -, p_UERadioCapId); // @sic R5-216280 sic@ return v_EpsDefaultBearerId; }

#### MCC160 Implementation

function f_TAUReject_AttachWithoutN26 (EUTRA_CellId_Type p_CellId) runs on EUTRA_Multi5GS_PTC return EUTRA_NR_PduSessionInfoList_Type

{

var SRB_COMMON_IND v_ReceivedAsp;

var NAS_MSG_Indication_Type v_NasInd;

var template (present) NAS_MSG_Indication_Type v_NAS_Expected;

var EUTRA_NR_CoOrd_GUTI_Type v_CoOrd_Guti;

var template (value) MobileIdentity v_MappedGUTI;

var template (value) MobileIdentity v_MobileId;

var GutiParameters_Type v_GutiParams;

var template AdditionalUpdateType v_AdditionalUpdateType;

var NAS_PlmnId v_PLMN;

var TrackingAreaCode v_Tac;

var template TrackingAreaId v_TAI:= omit;

var template (present) NAS_KsiValue v_KsiValue;

var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList;

var template (omit) integer v_SessionId;

var NAS_AttDetValue_Type v_AttachType := f_GetEAttachType(NORMAL);

var B3_Type v_PDNType := f_GetPdnType();

var B3_Type v_RequestType;

v_CoOrd_Guti := f_EUTRA_NR_WaitForCoOrd_GutiParams(NR);

v_MappedGUTI := f_EUTRA38_NR_Convert5G_CoOrdToEPS_GUTI (v_CoOrd_Guti, omit);

v_PduSessionInfoList := f_EUTRA38_WaitForPDUSessionInfoList(p_CellId); // @sic R5s210149 sic@

// Tell IMS PTC the RAT has changed

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_EUTRA_IMS_IpcanInfo()));

v_AdditionalUpdateType := f_GetAdditionalUpdateType (NORMAL);

v_KsiValue := f_EUTRA_SecurityKSIasme_Get();

v_GutiParams:= f_EUTRA_CellInfo_GetGuti(p_CellId);

v_PLMN := f_Asn2Nas_PlmnId(v_GutiParams.PLMN_Identity);

v_Tac := f_EUTRA_CellInfo_GetTAC(p_CellId);

v_TAI := cr_TAI(v_PLMN, bit2oct(v_Tac));

v_MobileId := f_GutiParameters2MobileIdentity(omit, v_GutiParams); // @sic R5s211706 sic@ // @sic R5-212453, R5-223431, R5-225449 sic@

// @siclog "Step 7 - 8" siclog@

v_NAS_Expected := (cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cr_TAU_Request_N1ToS1(f_GetEPSTAUType(NORMAL),

v_KsiValue,

cr_DRXparameter_Any ('5C'O) ifpresent,

v_AdditionalUpdateType,

v_MappedGUTI,

v_TAI,

tsc_EpsUpdate_Active,

-, -,

omit,

cr_UeRadioCap ifpresent)),

cr_NAS_IndicationWithPiggybacking(tsc_SHT_IntegrityProtected,

cdr_ATTACH_REQUEST_N1ToS1(v_AttachType,

v_AdditionalUpdateType,

v_KsiValue,

v_MobileId,

v_TAI),

cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover,

v_PDNType)));

// @siclog "Step 8a1/8b1" siclog@

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(p_CellId,

tsc_RRC_TI_Def,

?,

v_NAS_Expected);

if (ischosen (v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST)) {

// @siclog "Step 8b2" siclog@

SRB.send(cas_SRB1_NasPdu_REQ(p_CellId,

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_NoSecurityProtection,

cs_508_TRACKING_AREA_UPDATE_REJECT(tsc_EmmCause_UEIdentityNotDerived))));

// @siclog "Step 8b3" siclog@

SRB.receive(car_SRB1_NasPdu_IND(p_CellId,

cr_NAS_IndicationWithPiggybacking((tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected),

cdr_ATTACH_REQUEST_IMSIonly(v_AttachType,

v_AdditionalUpdateType,

v_TAI),

cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_InitialAttach,

v_PDNType))))

-> value v_ReceivedAsp;

v_NasInd := v_ReceivedAsp.Signalling.Nas[0];

}

v_RequestType := v_NasInd.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.requestType.typeValue;

// @siclog "Step 9 - 20" siclog@

v_SessionId := f_ProtocolConfigOptions_GetSessionId(v_NasInd.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions);

if (not isvalue (v_SessionId)) {

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Id not present in PCO");

}

f_EUTRA38_MultiPDN_InitialRegistration_Step5_16 (p_CellId, TESTMode_OFF, v_NasInd, -, v_PduSessionInfoList, true, match(v_RequestType, tsc_PdnRequest_Handover)); // @sic R5s210149, R5s220177 sic@

return v_PduSessionInfoList; // @sic R5s230015 sic@

}

function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15(EUTRA_CellId_Type p_CellId,

EUTRA_ATTACH_Type p_ForcedAttach,

template (omit) AdditionalUpdateType p_AdditionalUpdateType,

ProcedureTransactionIdentifier p_EPS_TI,

boolean p_IPv4AllocationViaNasFlag,

template (omit) ProtocolConfigOptions p_PcoFromUE,

template (omit) AccessPointName p_APN,

template (value) GPRS_Timer p_T3412 := cs_GprsTimer_v_deact,

template (omit) GPRS_Timer p_T3402 := omit,

template (omit) GPRS_Timer p_T3423 := omit,

template (omit) PLMN_List p_EquivalentPlmnList := omit,

integer p_NoOfEmergencyNumbers := 0,

template (omit) EMM_Cause p_EMM_Cause := omit,

template (omit) DCN_ID p_DCN_Id := omit,

template (omit) ExtdProtocolConfigOptions p_ExtdPcoFromUE := omit,

template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromNR := omit,

template (omit) UERadioCapId p_UERadioCapId:= omit,

boolean p_N1disabled := false,

boolean p_PreserveIPaddress := true) runs on EUTRA_Multi5GS_PTC return EPS_BearerIdentity

{

var PDN_TypeToBeUsed_Type v_PDN_TypeToBeUsed := f_EUTRA_MobileInfo_GetPdnTypeToBeUsed();

var NAS_AttDetValue_Type v_AttachType := f_GetEAttachType(p_ForcedAttach);

var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(p_CellId);

var template (value) MobileIdentity v_Guti := f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams);

var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId (v_GutiParams.PLMN_Identity);

var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(p_CellId);

var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, p_ForcedAttach);

var TrackingAreaCode v_TAC := f_EUTRA_CellInfo_GetTAC(p_CellId);

var template (value) AccessPointName v_APN_Tx := f_EUTRA38_MultiPDN_GetAPN(p_CellId, p_APN);

var template (omit) EmergNumList v_EmergNumList := omit;

var template (omit) MobileIdentity v_MSId := f_GetMSId(p_ForcedAttach);

var PDU_PDN_DNN_Type v_PDN_Type := f_EUTRA38_MultiPDN_GetPDNType(p_APN); // @sic R5-206296 sic@

var PDN_Index_Type v_PdnIndex := f_GetMultiPdnIndex (v_PDN_Type);  // @sic R5-206296 sic@

var boolean v_IsIMS := false;

var EPS_BearerIdentity v_EpsDefaultBearerId;

var template (omit) ESM_Cause v_ESM_Cause := omit;

var template (value) PDN_Address v_PDN_Address;

var EmergencyNumList v_EmergencyNumList;

var boolean v_StartPDN := true; // @sic R5s210149 sic@

var template (value) ProtocolConfigOptions v_PcoToUE;

var BEARER_CONTEXT_TYPE v_BearerContextNumber := DEF_1;  // @sic R5s230279 sic@

var InterworkWithoutN26 v_IwkN26 := f_EUTRA38_GetNetworkSupport();

var integer i;

if (pc_IPv6 and pc_IPv4) {

select (v_PDN_TypeToBeUsed) {

case (forceIPv4only)          { v_ESM_Cause := cs_ESM_Cause_tv('00110010'B); }     // PDN type IPv4 only allowed

case (forceIPv6only)          { v_ESM_Cause := cs_ESM_Cause_tv('00110011'B); }     // PDN type IPv6 only allowed

case (pdnTypeAsSupportedByUE) { v_ESM_Cause := omit; }

}

}

if (p_NoOfEmergencyNumbers > 0) {

v_EmergencyNumList := f_Get_EmergencyNumList (p_NoOfEmergencyNumbers, f_EUTRA_MobileInfo_GetLocalEmergencyNumberList());

f_EUTRA_MobileInfo_SetLocalEmergencyNumberList (v_EmergencyNumList); // save the list of locally usable emergency numbers

v_EmergNumList := f_Build_EmergNumList(p_NoOfEmergencyNumbers, v_EmergencyNumList);

}

select(v_PDN_Type) { // @sic R5-206296 sic@ other options not relevant

case (IMS_DNN, Emergency_PDN) {  // @sic R5s230279 sic@

v_IsIMS := true;

v_BearerContextNumber := DEF_2;

}

}

v_EpsDefaultBearerId := f_EUTRA38_MultiPDN_ConfigureDRB(p_CellId,

v_PDN_Type,

v_IsIMS);

v_PcoToUE := f_NAS_ProtocolConfigOptionsTX(f_EUTRA38_MultiPDN_GetPCOs(p_CellId,

v_PDN_Type, // @sic R5s220883 sic@

v_IsIMS,

v_EpsDefaultBearerId,

p_ExtdPcoFromUE,

p_PcoFromUE,

p_N1disabled));  // @sic R5-227443 sic@

if (isvalue (p_ExtdPcoFromUE)) {

// FFS - At the moment, 36.508 table 4.7.3-6 specifies ExtdPCO should be not present

}

if (isvalue(p_PduInfoFromNR) andand p_PreserveIPaddress) { // @sic R5s210149, R5s240583 sic@

// The UE has just moved from NR. If so, data from NR should be used, rather than created anew

for (i := 0; i < lengthof (p_PduInfoFromNR); i := i + 1) {

if (v_PdnIndex == valueof(p_PduInfoFromNR[i].PdnIndex)) {

// Data from NR should be used, rather than created anew

v_StartPDN := false; // no need to change DRB mapping as this has already been done in calling function

break;

}

}

}

if (v_StartPDN) { // completely new PDN connection @sic R5s210149 sic@

f_IP_Handling_StartPDN(IP, v_PdnIndex, cs_DrbInfo_EUTRA(p_CellId, (hex2int(v_EpsDefaultBearerId)-4)));

}

v_PDN_Address := f_EUTRA_GetPDNAddress(p_IPv4AllocationViaNasFlag, v_PdnIndex);

f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams(p_CellId,

v_EpsDefaultBearerId,

tsc_RRC_TI_Def,

p_EPS_TI,

v_AttachType,

v_PDN_Address,

v_Guti,

cs_TAIListNonConsecutive_lv(v_PLMN, {bit2oct(v_TAC)}),

v_LAI,

v_MSId,

v_ESM_Cause,

v_APN_Tx,  // @sic R5-206296 sic@

v_PcoToUE,

f_GetAdditionalUpdateResult(p_AdditionalUpdateType, p_ForcedAttach),

p_T3412,

p_T3402,

p_T3423,

p_EquivalentPlmnList,

v_EmergNumList,

-,

-,

v_BearerContextNumber,

p_EMM_Cause,

omit,

p_DCN_Id,

omit,

-,

p_UERadioCapId); // @sic R5-216280 sic@

return v_EpsDefaultBearerId;

}

function f_EUTRA38_MultiPDN_InitialRegistration_Step5_16(EUTRA_CellId_Type p_CellId,

NR_TESTMODE_STATE_Type p_State,

NAS_MSG_Indication_Type p_NAS_Ind,

template (omit) PLMN_List p_EquivalentPlmnList := omit,

template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduSessionInfoList := omit,

boolean p_FallbackSignalling := false,

boolean p_PreserveIPaddress := true

) runs on EUTRA_Multi5GS_PTC

{

var template (omit) AdditionalUpdateType v_AdditionalUpdateType;

var NAS_UL_Message_Type v_PdnConnectivityRequest;

var ProcedureTransactionIdentifier v_EPS_TI;

var template (omit) ProtocolConfigOptions v_PcoFromUE := omit;  // may be omit

var template (omit) ExtdProtocolConfigOptions v_ExtdPcoFromUE := omit;

var EPS_BearerIdentity v_EpsBearerId;

var boolean v_IPv4AllocationViaNasFlag;

var APNandPCOs_Type v_APNandPCOs;

var NasCount_Type v_NasCountUL;

var NR_Configuration_Type v_NR_ConfigType := f_EUTRA38_NR_Config_Get();

var DelayForUserPlaneSignalling_Type v_WaitForUserplaneSignalling := noDelay; // Default: no IP signalling possible

var template (omit) UERadioCapId v_UERadioCapIdToSend := omit; // @sic R5s220478 sic@

if (not ispresent (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueAddSecurityCap)){

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueAddSecurityCap should be present");

}

select (v_NR_ConfigType) {

case (EN_DC) {

if (not match (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_EUTRA_UENetworkCap(omit))){

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for ENDC");

}

}

case else { // other options FFS

if (not match (p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.ueNetworkCapability, cdr_NR5GC_UENetworkCap(omit))){

f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "ueNetworkCapability does not indicate support for EUTRA38");

}

}

}

v_AdditionalUpdateType := p_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.addUpdateType;

v_PdnConnectivityRequest := p_NAS_Ind.Pdu.PiggybackedPduList[0].Msg;

v_EPS_TI := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier;

v_PcoFromUE := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions;

v_ExtdPcoFromUE := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.extdProtocolConfigurationOptions;

// Steps 5-10b6

v_NasCountUL := f_EUTRA_MultiPDN_InitialRegistration_Step5_8 (p_CellId, p_NAS_Ind);  // @sic R5-216280, R5-222047 sic@

v_APNandPCOs := f_EUTRA38_MultiPDN_InitialRegistration_Step9_13 (p_CellId, p_State, p_NAS_Ind, v_NasCountUL);

if (ispresent(v_APNandPCOs.protocolConfigurationOptions)) {

v_PcoFromUE := v_APNandPCOs.protocolConfigurationOptions;

} else if (ispresent(v_APNandPCOs.extdProtocolConfigurationOptions)) {

v_ExtdPcoFromUE := v_APNandPCOs.extdProtocolConfigurationOptions;

}

if (isvalue(v_PcoFromUE)) {

v_IPv4AllocationViaNasFlag := f_CheckPCOforIPallocationViaNas(v_PcoFromUE);

} else {

v_IPv4AllocationViaNasFlag := f_CheckExtdPCOforIPallocationViaNas(v_ExtdPcoFromUE);

}

if (pc_EPC_RACS) { // @sic R5-216280 sic@

v_UERadioCapIdToSend := f_EUTRA_MobileInfo_GetRACS_IdToSend(); // @sic R5s220478 sic@

}

v_EpsBearerId := f_EUTRA38_MultiPDN_InitialRegistration_Step14_15(p_CellId,

NORMAL,

v_AdditionalUpdateType,

v_EPS_TI,

v_IPv4AllocationViaNasFlag,

v_PcoFromUE,

v_APNandPCOs.accessPointName,

cs_GprsTimer_v_deact,

omit,

omit,

p_EquivalentPlmnList,

0,

omit,

-,

v_ExtdPcoFromUE,

p_PduSessionInfoList,  // @sic R5s210149 sic@

v_UERadioCapIdToSend,  // @sic R5-216280 sic@

-, p_PreserveIPaddress);

// Step 10b7

if (pc_noOf_PDNsSameConnection > 0 or p_FallbackSignalling) {  // @sic R5-206296, R5s220177 sic@

/* In case of another PDN connectivity request this may be sent immediately by the UE

=> we cannot wait after the ATTACH_COMPLETE for any trigger from the IMS_PTC but need to do this when all PDNs are configured */

v_WaitForUserplaneSignalling := noDelay;

} else {

select (f_EUTRA38_MultiPDN_GetPDNType(v_APNandPCOs.accessPointName)) { // @sic R5-206296 sic@

case (IMS_DNN) {

v_WaitForUserplaneSignalling := waitForIMS;

}

case (Internet_DNN) {

v_WaitForUserplaneSignalling := dontWaitForIMS;

}

}

}

f_EUTRA_NAS_AttachComplete(p_CellId, v_EpsBearerId, v_WaitForUserplaneSignalling);

}