3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240550

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3450 | rev | 1 | Current version: | 17.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC test case 7.1.1.3.2b |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-05-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the case when pc_configuredUL_GrantType1 is TRUE, SRB1 should have the ConfiguredGrantType1Allowed set to TRUE as per Prose CR R5-243088. Additionally, there are other changes needed: According to TS 38.321 sec 5.8.2: After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider that the uplink grant recurs associated with each symbol for which: [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = (timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), for all N >= 0. To satisfy the above condition, it becomes necessary, for the case when pc_configuredUL_GrantType1 is TRUE, to transmit the DL Data at SFN 1022/2/0 so that with timeDomainOffset = 9 as per Table 7.1.1.3.2b.3.3-3, the first SFN after the wrap around would fulfil the condition above. Hence, TTCN changes are needed to ensure this. Further, a RRC signalling procedure needs to be added to release the configured grant type 1 after the TestBody is completed so that the postamble is completed using the default OnSR UL grant procedure. Also, for the case of ENDC, some of the TTCN changes w.r.t configured grant period and configured grant setup are not in alignment with the NR5GC version which needs to be corrected. Secondly, there is no necessity for the UE to perform a RACH procedure on the NR Cell when the only purpose of the RRC Reconfiguration procedure is to setup the configured grant. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set the ConfiguredGrantType1Allowed = TRUE for SRB 1. Introduced TTCN changes to ensure that, for the case when pc_configuredUL_GrantType1 is TRUE, DL Data is transmitted at 1022/2/0 and also consequently changed the watchdog timeout value to 11.0s to ensure all RLC SDUs are looped back. Further, introduced a RRC signalling procedure to ensure the release of the configured grant type 1 Corrected the ENDC version to ensure the TTCN is aligned with the NR5GC version as well set the IEs reconfigurationWithSync and rlf_TimersAndConstants to omit. The local end config is also modified to omit the Common IE of BWP which would have configured the RACH parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case when pc_configuredUL_GrantType1 is TRUE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.2b.ENDC, 7.1.1.3.2b.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	4

2.3	Change 3	5

2.4	Change 4	10

2.5	Change 5	12

2.6	Change 6	14

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:  Bhise Parikshit
parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Template name | cs_38508_RLC_BearerConfig_SRB |
| --- | --- |
| Reason for change | For the case when pc_configuredUL_GrantType1 is TRUE, SRB1 should have the ConfiguredGrantType1Allowed set to TRUE as per Prose CR R5-243088 |
| Summary of change | Set the ConfiguredGrantType1Allowed = TRUE for SRB 1. |
| TTCN module | NR_RRC_Templates.ttcn |

Before Change:

template (value) RLC_BearerConfig cs_38508_RLC_BearerConfig_SRB(SRB_Identity p_SRB_Id, template (omit) RLC_BearerConfig.reestablishRLC p_ReestablishRLC := omit ) := /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ /* @sic R5-192737: LogicalChannelConfig for SRBs; p_RLC_Config, p_MAC_LogicalChannelConfig, p_ReestablishRLC removed sic@ */ cs_NR_BearerConfigDef(f_NR_LogicalChannelId_SRB(p_SRB_Id), cs_NR_ServedRadioBearerSRB(p_SRB_Id), omit, cs_38508_LogicalChannelConfig_SRB(p_SRB_Id), p_ReestablishRLC);

After Change:

template (value) RLC_BearerConfig cs_38508_RLC_BearerConfig_SRB(SRB_Identity p_SRB_Id, template (omit) RLC_BearerConfig.reestablishRLC p_ReestablishRLC := omit, template (omit) NR_ConfiguredGrantType1Allowed p_NR_ConfiguredGrantType1Allowed := omit ) := /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ /* @sic R5-192737: LogicalChannelConfig for SRBs; p_RLC_Config, p_MAC_LogicalChannelConfig, p_ReestablishRLC removed sic@ */ cs_NR_BearerConfigDef(f_NR_LogicalChannelId_SRB(p_SRB_Id), cs_NR_ServedRadioBearerSRB(p_SRB_Id), omit, cs_38508_LogicalChannelConfig_SRB(p_SRB_Id, p_NR_ConfiguredGrantType1Allowed), p_ReestablishRLC); //WA#

## Change 2

| Template name | cs_38508_LogicalChannelConfig_SRB |
| --- | --- |
| Reason for change | For the case when pc_configuredUL_GrantType1 is TRUE, SRB1 should have the ConfiguredGrantType1Allowed set to TRUE as per Prose CR R5-243088 |
| Summary of change | Set the ConfiguredGrantType1Allowed = TRUE for SRB 1. |
| TTCN module | NR_RRC_Templates.ttcn |

Before Change:

template (value) LogicalChannelConfig cs_38508_LogicalChannelConfig_SRB(SRB_Identity p_SRBId) := //As per 38.508-1 Table 4.6.3-66: LogicalChannelConfig /* @sic R5-192742 sic@ */ /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ cs_NR_LogicalChannelConfigDef(-, -, f_NR_LogicalChannelPriority_SRB(p_SRBId));

After Change:

template (value) LogicalChannelConfig cs_38508_LogicalChannelConfig_SRB(SRB_Identity p_SRBId, template (omit) NR_ConfiguredGrantType1Allowed p_NR_ConfiguredGrantType1Allowed := omit) := //As per 38.508-1 Table 4.6.3-66: LogicalChannelConfig /* @sic R5-192742 sic@ */ /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ cs_NR_LogicalChannelConfigDef(-, -, f_NR_LogicalChannelPriority_SRB(p_SRBId),-,-,-,-,-,-,-,p_NR_ConfiguredGrantType1Allowed); //WA#

## Change 3

| Template name | f_TC_7_1_1_3_2b_NR5GC |
| --- | --- |
| Reason for change | For the case when pc_configuredUL_GrantType1 is TRUE, SRB1 should have the ConfiguredGrantType1Allowed set to TRUE as per Prose CR R5-243088 According to TS 38.321 sec 5.8.2: After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider that the uplink grant recurs associated with each symbol for which: [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = (timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), for all N >= 0. To satisfy the above condition, it becomes necessary, for the case when pc_configuredUL_GrantType1 is TRUE, to transmit the DL Data at SFN 1022/2/0 so that with timeDomainOffset = 9 as per Table 7.1.1.3.2b.3.3-3, the first SFN after the wrap around would fulfil the condition above. Hence, TTCN changes are needed to ensure this. Further, a RRC signalling procedure needs to be added to release the configured grant type 1 after the TestBody is completed so that the postamble is completed using the default OnSR UL grant procedure |
| Summary of change | Set the ConfiguredGrantType1Allowed = TRUE for SRB 1. Introduced TTCN changes to ensure that, for the case when pc_configuredUL_GrantType1 is TRUE, DL Data is transmitted at 1022/2/0 and also consequently changed the watchdog timeout value to 11.0s to ensure all RLC SDUs are looped back. Further, introduced a RRC signalling procedure to ensure the release of the configured grant type 1 |
| TTCN module | MAC_NR5GC.ttcn |

Before Change:

function f_TC_7_1_1_3_2b_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling with Mapping restrictions <<SKIPPED CODE>> //@sic steps 9-10 sic@ if (pc_configuredUL_GrantType1) { //@sic R5s213458 R5s230442 sic@ v_RRC_MsgRcvd := false; v_DRB_Rcvd := false; v_RLC_BearerConfigList[0] := cs_38508_RLC_BearerConfig_DRB(v_DRBId1, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps8,6,ms100,-,-,-,-,-,-,true_)) ; v_RLC_BearerConfigList[1] := cs_38508_RLC_BearerConfig_DRB(v_DRBId2, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps16,7,ms100,-,-,-,-,-,-,-)) ; v_RLC_BearerConfigList[2] := cs_38508_RLC_BearerConfig_DRB(v_DRBId3, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps32,8,ms100,-,-,-,-,-,-, true_)) ; v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1, f_NR_GetServingCellConfig(nr_Cell1)); //@sic R5-202561 sic@ v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); v_LocationAndBandwidth := v_NR_UplinkBWP_List[0].Common.R15.genericParameters.locationAndBandwidth; v_BwpSize := fl_NR_RIV_GetLength(valueof(v_LocationAndBandwidth)); v_RIV := fl_NR_RIV_Calculate(v_BwpSize, 23, 0); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_DMRS_UL_AdditionalPosition := pos1;} else { v_DMRS_UL_AdditionalPosition := pos0;} select (v_SubcarrierSpacing) {//@sic R5-232872 R5s230442 sic@ case(kHz15) { v_ConfigGrant_Period := sym40x14} case(kHz30) { v_ConfigGrant_Period := sym80x14} case(kHz60) { v_ConfigGrant_Period := sym160x14} case(kHz120) { v_ConfigGrant_Period := sym320x14} case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig.setup := valueof (cs_ConfiguredGrantConfig_Def(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), v_ConfigGrant_Period, cs_RRC_ConfiguredUplinkGrant(int2bit (v_RIV,18),16,9 )));//@sic R5-232872 R5s230442 sic@ //@sic R5s230442 sic@ v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_7116x(4, 27); v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeB := v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeA; f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP_List[0], tsc_NR_BWP_Id); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.cs_RNTI := { setup := tsc_RNTI_Value_CS_RNTI}; v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.configuredGrantConfig := v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig; //@sic R5s230442 sic@ v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_7116x(4, 27); v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeB := v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeA; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , v_RLC_BearerConfigList,//@sic R5s230442 sic@ omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future f_NR_ULGrantConfiguration_Start(nr_Cell1); //@sic R5s210863 sic@ f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,cs_TimingInfo_NR(v_Timing),false); f_NR_SS_ServingCellConfig(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_NR_UplinkBWP_List)); t_Watchdog.start; alt { [not v_RRC_MsgRcvd] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) { v_RRC_MsgRcvd := true; if (v_DRB_Rcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [not v_DRB_Rcvd] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, v_DRBId2)) { v_DRB_Rcvd := true; if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [] t_Watchdog.timeout { if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE is not sending back ReconfigurationComplete ");} } }; //@sic steps 11-14 sic@ //@sic R5s220045 R5s230442 R5-241506 sic@ f_TC_7_1_1_3_2b_NR_TestBody_Loop(v_DRBId1,v_DRBId2,v_DRBId3, 13,1,50,40,0,4160,0,16000,cs_NR_DciCommon_TimeDomainResourceAssignment('1'B)); //T1 =0 to C-RNTI grant. }; f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

After Change:

function f_TC_7_1_1_3_2b_NR5GC() runs on NR5GC_PTC { // Logical channel prioritization handling with Mapping restrictions <<SKIPPED CODE>> //@sic steps 9-10 sic@ if (pc_configuredUL_GrantType1) { //@sic R5s213458 R5s230442 sic@ v_RRC_MsgRcvd := false; v_DRB_Rcvd := false; v_RLC_BearerConfigList[0]:= cs_38508_RLC_BearerConfig_SRB(tsc_SRB1, -,true_); //WA# v_RLC_BearerConfigList[1]:= cs_38508_RLC_BearerConfig_DRB(v_DRBId1, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps8,6,ms100,-,-,-,-,-,-,true_)) ; v_RLC_BearerConfigList[2]:= cs_38508_RLC_BearerConfig_DRB(v_DRBId2, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps16,7,ms100,-,-,-,-,-,-,-)) ; v_RLC_BearerConfigList[3]:= cs_38508_RLC_BearerConfig_DRB(v_DRBId3, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps32,8,ms100,-,-,-,-,-,-, true_)) ; v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1, f_NR_GetServingCellConfig(nr_Cell1)); //@sic R5-202561 sic@ v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); v_LocationAndBandwidth := v_NR_UplinkBWP_List[0].Common.R15.genericParameters.locationAndBandwidth; v_BwpSize := fl_NR_RIV_GetLength(valueof(v_LocationAndBandwidth)); v_RIV := fl_NR_RIV_Calculate(v_BwpSize, 23, 0); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_DMRS_UL_AdditionalPosition := pos1;} else { v_DMRS_UL_AdditionalPosition := pos0;} select (v_SubcarrierSpacing) {//@sic R5-232872 R5s230442 sic@ case(kHz15) { v_ConfigGrant_Period := sym40x14} case(kHz30) { v_ConfigGrant_Period := sym80x14} case(kHz60) { v_ConfigGrant_Period := sym160x14} case(kHz120) { v_ConfigGrant_Period := sym320x14} case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig.setup := valueof (cs_ConfiguredGrantConfig_Def(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), v_ConfigGrant_Period, cs_RRC_ConfiguredUplinkGrant(int2bit (v_RIV,18),16,9 )));//@sic R5-232872 R5s230442 sic@ //@sic R5s230442 sic@ v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_7116x(4, 27); v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeB := v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeA; f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP_List[0], tsc_NR_BWP_Id); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.cs_RNTI := { setup := tsc_RNTI_Value_CS_RNTI}; v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.configuredGrantConfig := v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig; //@sic R5s230442 sic@ v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_7116x(4, 27); v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeB := v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeA; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , v_RLC_BearerConfigList,//@sic R5s230442 sic@ omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig //SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future f_NR_ULGrantConfiguration_Start(nr_Cell1); //@sic R5s210863 sic@ f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext,-,-,cs_TimingInfo_NR(v_Timing),false); f_NR_SS_ServingCellConfig(nr_Cell1, cs_TimingInfo_NR(v_Timing), cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, cs_TimingInfo_NR(v_Timing), v_NR_UplinkBWP_List)); t_Watchdog.start; alt { [not v_RRC_MsgRcvd] SRB.receive(car_NR_SRB_RrcPdu_IND(nr_Cell1, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)) { v_RRC_MsgRcvd := true; if (v_DRB_Rcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [not v_DRB_Rcvd] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, v_DRBId2)) { v_DRB_Rcvd := true; if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [] t_Watchdog.timeout { if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE is not sending back ReconfigurationComplete ");} } }; //@sic steps 11-14 sic@ //@sic R5s220045 R5s230442 R5-241506 sic@ WA# f_TC_7_1_1_3_2b_NR_TestBody_Loop(v_DRBId1,v_DRBId2,v_DRBId3, 13,1,50,40,0,4160,0,16000,cs_NR_DciCommon_TimeDomainResourceAssignment('1'B), true); //T1 =0 to C-RNTI grant. v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig := {release := NULL}; f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP_List[0], tsc_NR_BWP_Id); v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.configuredGrantConfig := {release := NULL}; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG , omit, omit, omit, omit, v_SpCellConfig ); v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_CellGroupConfig))); f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config f_NR_SendRRCReconfiguration (nr_Cell1, -,-, v_V1530Ext); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); }; f_NR_TestBody_Set(false); //Postamble f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config //@sic R5s230130 sic@ //f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); };

## Change 4

| Template name | f_TC_7_1_1_3_2b_NR_TestBody_Loop |
| --- | --- |
| Reason for change | According to TS 38.321 sec 5.8.2: After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider that the uplink grant recurs associated with each symbol for which: [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = (timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), for all N >= 0. To satisfy the above condition, it becomes necessary, for the case when pc_configuredUL_GrantType1 is TRUE, to transmit the DL Data at SFN 1022/2/0 so that with timeDomainOffset = 9 as per Table 7.1.1.3.2b.3.3-3, the first SFN after the wrap around would fulfil the condition above. Hence, TTCN changes are needed to ensure this. |
| Summary of change | Introduced TTCN changes to ensure that, for the case when pc_configuredUL_GrantType1 is TRUE, DL Data is transmitted at 1022/2/0 and consequently changed the watchdog timeout value to 11.0s to ensure all RLC SDUs are looped back. |
| TTCN module | MAC_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_3_2b_NR_TestBody_Loop(DRB_Identity p_NR_DRB_Id1, DRB_Identity p_NR_DRB_Id2, DRB_Identity p_NR_DRB_Id3, integer p_N1, //Number of SDUs for each run integer p_N2, integer p_N3, integer p_T1,// Time period and total duration integer p_T2, integer p_D1, //Amount of data to compare with (octets) integer p_D2, integer p_D3, template (omit) NR_DciCommon_TimeDomainResourceAssignment_Type p_TimeDomainResourceAssignment := omit) runs on NR_BASE_PTC { <<SKIPPED CODE>> //@siclog "Step 1" siclog@ // SS sends: // N1 320-octet RLC SDUs on DRB1 // N2 320-octet RLC SDUs on DRB2 // N3 320-octet RLC SDUs on DRB3 v_DRB_DataPerSlotList1:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N1); v_DRB_DataPerSlotList2:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N2); v_DRB_DataPerSlotList3:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N3); v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future //Exceptionally the time is extended to 300 as in loop 4, 179 PDCP SDU's each of 317 bytes get scheduled; f_TC_7_1_1_3_2_Step1(v_DRB_DataPerSlotList1, v_DRB_DataPerSlotList2, v_DRB_DataPerSlotList3, p_NR_DRB_Id1, p_NR_DRB_Id2, p_NR_DRB_Id3, v_Timing); <<SKIPPED CODE>> //@siclog "Step 3" siclog@ // RLC SDUs back to SS/check total number of octets of UL Rx f_TC_7_1_1_3_2_Step3(p_D1, p_D2, p_D3,p_NR_DRB_Id1,p_NR_DRB_Id2,p_NR_DRB_Id3); f_NR_HarqErrorHandling_Set(ignore); // Stop 'Harq Errors not tolerant' f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config <<SKIPPED CODE>>

After Change:

function f_TC_7_1_1_3_2b_NR_TestBody_Loop(DRB_Identity p_NR_DRB_Id1, DRB_Identity p_NR_DRB_Id2, DRB_Identity p_NR_DRB_Id3, integer p_N1, //Number of SDUs for each run integer p_N2, integer p_N3, integer p_T1,// Time period and total duration integer p_T2, integer p_D1, //Amount of data to compare with (octets) integer p_D2, integer p_D3, template (omit) NR_DciCommon_TimeDomainResourceAssignment_Type p_TimeDomainResourceAssignment := omit, boolean p_CGPUSCH := false) runs on NR_BASE_PTC { <<SKIPPED CODE>> //@siclog "Step 1" siclog@ // SS sends: // N1 320-octet RLC SDUs on DRB1 // N2 320-octet RLC SDUs on DRB2 // N3 320-octet RLC SDUs on DRB3 v_DRB_DataPerSlotList1:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N1); v_DRB_DataPerSlotList2:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N2); v_DRB_DataPerSlotList3:= f_NR_BuildPDCPSDUsToBeSentInConsecutiveFrames(v_NR_PDCP_SDU, p_N3); //WA# if (p_CGPUSCH) { v_Timing := f_NR_GetCurrentTiming(nr_Cell1); v_Timing.SFN.Number := 1022; v_Timing.Subframe.Number := 2; v_Timing.Slot.SlotOffset.Numerology1 := 0; v_Timing.Symbol := valueof (cs_SymbolTimingInfo_FirstSlot); } else { v_Timing := f_NR_GetNextSendOccasion_DL(nr_Cell1, 300); //300 ms in future } //Exceptionally the time is extended to 300 as in loop 4, 179 PDCP SDU's each of 317 bytes get scheduled; f_TC_7_1_1_3_2_Step1(v_DRB_DataPerSlotList1, v_DRB_DataPerSlotList2, v_DRB_DataPerSlotList3, p_NR_DRB_Id1, p_NR_DRB_Id2, p_NR_DRB_Id3, v_Timing); <<SKIPPED CODE>> //@siclog "Step 3" siclog@ // RLC SDUs back to SS/check total number of octets of UL Rx //WA# if (p_CGPUSCH){ f_TC_7_1_1_3_2_Step3(p_D1, p_D2, p_D3,p_NR_DRB_Id1,p_NR_DRB_Id2,p_NR_DRB_Id3, 11.0); } else { f_TC_7_1_1_3_2_Step3(p_D1, p_D2, p_D3,p_NR_DRB_Id1,p_NR_DRB_Id2,p_NR_DRB_Id3); } <<SKIPPED CODE>>

## Change 5

| Template name | f_TC_7_1_1_3_2_Step3 |
| --- | --- |
| Reason for change | According to TS 38.321 sec 5.8.2: After an uplink grant is configured for a configured grant Type 1, the MAC entity shall consider that the uplink grant recurs associated with each symbol for which: [(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = (timeDomainOffset × numberOfSymbolsPerSlot + S + N × periodicity) modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot), for all N >= 0. To satisfy the above condition, it becomes necessary, for the case when pc_configuredUL_GrantType1 is TRUE, to transmit the DL Data at SFN 1022/2/0 so that with timeDomainOffset = 9 as per Table 7.1.1.3.2b.3.3-3, the first SFN after the wrap around would fulfil the condition above. Hence, TTCN changes are needed to ensure this. |
| Summary of change | Introduced TTCN changes to ensure that, for the case when pc_configuredUL_GrantType1 is TRUE, DL Data is transmitted at 1022/2/0 and also consequently changed the watchdog timeout value to 11.0s to ensure all RLC SDUs are looped back. |
| TTCN module | MAC_TC_Common_NR.ttcn |

Before Change:

function f_TC_7_1_1_3_2_Step3(integer p_DataOctetsDRB1, integer p_DataOctetsDRB2, integer p_DataOctetsDRB3, DRB_Identity p_NR_DRB_Id1, DRB_Identity p_NR_DRB_Id2, DRB_Identity p_NR_DRB_Id3) runs on NR_BASE_PTC { <<SKIPPED CODE>> timer t_Watchdog := 5.0; // Local wait Timer t_Watchdog.start; alt { [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, (p_NR_DRB_Id1, p_NR_DRB_Id2, p_NR_DRB_Id3))) -> value v_DRB_COMMON_IND { v_PDCP_SDU_List := v_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.PdcpSdu; v_DrbId := v_DRB_COMMON_IND.Common.RoutingInfo.RadioBearerId.Drb; select (v_DrbId) { case (p_NR_DRB_Id1) { k := k + lengthof(v_PDCP_SDU_List); } case (p_NR_DRB_Id2) { l := l + lengthof(v_PDCP_SDU_List); } case (p_NR_DRB_Id3) { m := m + lengthof(v_PDCP_SDU_List); } } repeat; } [] t_Watchdog.timeout { if ((0.9 * int2float(p_DataOctetsDRB1) > int2float(k * 320)) or (int2float(k * 320) > int2float(p_DataOctetsDRB1) * 1.1) or (0.9 * int2float(p_DataOctetsDRB2) > int2float(l * 320)) or (int2float(l * 320) > int2float(p_DataOctetsDRB2) * 1.1) or (0.9 * int2float(p_DataOctetsDRB3) > int2float(m * 320)) or (int2float(m * 320) > int2float(p_DataOctetsDRB3) * 1.1)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3: UE is not sending back all RLC SDUs "); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3: Loop back SDUs received"); } } } }

After Change:

function f_TC_7_1_1_3_2_Step3(integer p_DataOctetsDRB1, integer p_DataOctetsDRB2, integer p_DataOctetsDRB3, DRB_Identity p_NR_DRB_Id1, DRB_Identity p_NR_DRB_Id2, DRB_Identity p_NR_DRB_Id3, float p_Watchdog := 5.0 ) runs on NR_BASE_PTC { <<SKIPPED CODE>> timer t_Wait; // WA# Local wait Timer t_Wait.start(p_Watchdog); alt { [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, (p_NR_DRB_Id1, p_NR_DRB_Id2, p_NR_DRB_Id3))) -> value v_DRB_COMMON_IND { v_PDCP_SDU_List := v_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.PdcpSdu; v_DrbId := v_DRB_COMMON_IND.Common.RoutingInfo.RadioBearerId.Drb; select (v_DrbId) { case (p_NR_DRB_Id1) { k := k + lengthof(v_PDCP_SDU_List); } case (p_NR_DRB_Id2) { l := l + lengthof(v_PDCP_SDU_List); } case (p_NR_DRB_Id3) { m := m + lengthof(v_PDCP_SDU_List); } } repeat; } [] t_Wait.timeout //WA# { if ((0.9 * int2float(p_DataOctetsDRB1) > int2float(k * 320)) or (int2float(k * 320) > int2float(p_DataOctetsDRB1) * 1.1) or (0.9 * int2float(p_DataOctetsDRB2) > int2float(l * 320)) or (int2float(l * 320) > int2float(p_DataOctetsDRB2) * 1.1) or (0.9 * int2float(p_DataOctetsDRB3) > int2float(m * 320)) or (int2float(m * 320) > int2float(p_DataOctetsDRB3) * 1.1)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3: UE is not sending back all RLC SDUs "); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3: Loop back SDUs received"); } } } }

## Change 6

| Template name | f_TC_7_1_1_3_2b_ENDC_NR |
| --- | --- |
| Reason for change | For the case when pc_configuredUL_GrantType1 is TRUE, SRB1 should have the ConfiguredGrantType1Allowed set to TRUE as per Prose CR R5-243088 Also, for the case of ENDC, some of the TTCN changes w.r.t configured grant period and configured grant setup are not in alignment with the NR5GC version which needs to be corrected. Secondly, there is no necessity for the UE to perform a RACH procedure on the NR Cell when the only purpose of the RRC Reconfiguration procedure is to setup the configured grant. |
| Summary of change | Corrected the ENDC version to ensure the TTCN is aligned with the NR5GC version as well set the IEs reconfigurationWithSync and rlf_TimersAndConstants to omit. The local end config is also modified to omit the Common IE of BWP which would have configured the RACH parameters. |
| TTCN module | MAC_ENDC_NR.ttcn |

Before Change:

function f_TC_7_1_1_3_2b_ENDC_NR() runs on ENDC_NR_PTC { <<SKIPPED CODE>> //@sic steps 9-10 sic@ if (pc_configuredUL_GrantType1) { //@sic R5s213458 sic@ v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); if (ispresent(v_NR_UplinkBWP_List[0].Common.R15.genericParameters.locationAndBandwidth)) { v_LocationAndBandwidth := v_NR_UplinkBWP_List[0].Common.R15.genericParameters.locationAndBandwidth; //compiler warning may be ignored due to check v_BwpSize := fl_NR_RIV_GetLength(valueof(v_LocationAndBandwidth)); } else { FatalError(__FILE__, __LINE__, "Invalid locationAndBandwidth"); } v_RIV := fl_NR_RIV_Calculate(v_BwpSize, 23, 0); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_DMRS_UL_AdditionalPosition := pos1;} else { v_DMRS_UL_AdditionalPosition := pos0;} select (v_SubcarrierSpacing) { case(kHz15) { v_ConfigGrant_Period := sym20x14} case(kHz30) { v_ConfigGrant_Period := sym20x14} case(kHz60) { v_ConfigGrant_Period := sym80x14} case(kHz120) { v_ConfigGrant_Period := sym160x14} case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig.setup := valueof (cs_ConfiguredGrantConfig_Def(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), v_ConfigGrant_Period, cs_RRC_ConfiguredUplinkGrant(int2bit (v_RIV,18),16))); f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP_List[0], tsc_NR_BWP_Id); v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.cs_RNTI := { setup := tsc_RNTI_Value_CS_RNTI}; v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.configuredGrantConfig := v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG , //@sic R5s220053 sic@ omit, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig //SpCellConfig ); f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); // @sic R5s200549 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell1); //@sic R5s210863 sic@ f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig,-,-,-,reconfigurationWithSync_CheckRACH); //@sic R5s190541 sic@ t_Watchdog.start; alt { [not v_RRC_MsgRcvd] EUTRA.receive(cmr_EUTRA_NR_OctetData( "Complete")) -> value v_ReceivedMsg { f_NR_CheckReceived_RRCReconfigurationComplete(v_ReceivedMsg); v_RRC_MsgRcvd := true; if (v_DRB_Rcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [not v_DRB_Rcvd] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, v_DRBId2)) { v_DRB_Rcvd := true; if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [] t_Watchdog.timeout { if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE is not sending back ReconfigurationComplete ");} } }; //@sic steps 11-14 sic@ //@sic R5s220053 sic@ f_TC_7_1_1_3_2b_NR_TestBody_Loop(v_DRBId1,v_DRBId2,v_DRBId3, 13,1,50,20,360,4160,0,16000,cs_NR_DciCommon_TimeDomainResourceAssignment('1'B)); //T1 =0 to C-RNTI grant. }; f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }

After Change:

function f_TC_7_1_1_3_2b_ENDC_NR() runs on ENDC_NR_PTC { <<SKIPPED CODE>> //@sic steps 9-10 sic@ if (pc_configuredUL_GrantType1) { //@sic R5s213458 sic@ v_RLC_BearerConfigList[0]:= cs_38508_RLC_BearerConfig_DRB(v_DRBId1, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps8,6,ms100,-,-,-,-,-,-,true_)) ; v_RLC_BearerConfigList[1]:= cs_38508_RLC_BearerConfig_DRB(v_DRBId2, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps16,7,ms100,-,-,-,-,-,-,-)) ; v_RLC_BearerConfigList[2]:= cs_38508_RLC_BearerConfig_DRB(v_DRBId3, cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)), cs_NR_LogicalChannelConfigDef(-, kBps32,8,ms100,-,-,-,-,-,-, true_)) ; v_NR_UplinkBWP_List := f_NR_CellInfo_GetUplinkBWP_List(nr_Cell1); v_SpCellConfig := f_NR_GetSpCellConfigDef(nr_Cell1); v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(nr_Cell1); if (ispresent(v_NR_UplinkBWP_List[0].Common.R15.genericParameters.locationAndBandwidth)) { v_LocationAndBandwidth := v_NR_UplinkBWP_List[0].Common.R15.genericParameters.locationAndBandwidth; //compiler warning may be ignored due to check v_BwpSize := fl_NR_RIV_GetLength(valueof(v_LocationAndBandwidth)); } else { FatalError(__FILE__, __LINE__, "Invalid locationAndBandwidth"); } v_RIV := fl_NR_RIV_Calculate(v_BwpSize, 23, 0); if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) {v_DMRS_UL_AdditionalPosition := pos1;} else { v_DMRS_UL_AdditionalPosition := pos0;} select (v_SubcarrierSpacing) { //WA# case(kHz15) { v_ConfigGrant_Period := sym40x14} case(kHz30) { v_ConfigGrant_Period := sym80x14} case(kHz60) { v_ConfigGrant_Period := sym160x14} case(kHz120) { v_ConfigGrant_Period := sym320x14} case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); } } v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig.setup := valueof (cs_ConfiguredGrantConfig_Def(cs_38508_DMRS_UplinkConfig(v_DMRS_UL_AdditionalPosition), v_ConfigGrant_Period, cs_RRC_ConfiguredUplinkGrant(int2bit (v_RIV,18),16,9))); //WA# //WA# v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_7116x(4, 27); v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeB := v_NR_UplinkBWP_List[0].Dedicated.R15.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeA; f_NR_CellInfo_SetUplinkBWP(nr_Cell1, v_NR_UplinkBWP_List[0], tsc_NR_BWP_Id); v_NR_UplinkBWP_List[0].Common := omit; //WA# v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(nr_Cell1); v_PhysicalCellGroupConfig.cs_RNTI := { setup := tsc_RNTI_Value_CS_RNTI}; v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.configuredGrantConfig := v_NR_UplinkBWP_List[0].Dedicated.R15.configuredGrantConfig; //WA# v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config.setup.pusch_TimeDomainAllocationList.setup := cs_NR_PUSCH_TimeDomainResourceAllocationList_7116x(4, 27); v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeB := v_SpCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config.setup.dmrs_UplinkForPUSCH_MappingTypeA; v_SpCellConfig.reconfigurationWithSync := omit; v_SpCellConfig.rlf_TimersAndConstants := omit; v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG , //@sic R5s220053 sic@ v_RLC_BearerConfigList, omit, omit, v_PhysicalCellGroupConfig, v_SpCellConfig //SpCellConfig ); f_NR_SS_ServingCellConfig(nr_Cell1, -, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, omit, v_PhysicalCellGroupConfig))); f_NR_SS_CommonCellConfig(nr_Cell1, cads_NR_UplinkBWP_ConfigCommon_REQ(nr_Cell1, -, v_NR_UplinkBWP_List)); // @sic R5s200549 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell1); //@sic R5s210863 sic@ f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig); //WA# t_Watchdog.start; alt { [not v_RRC_MsgRcvd] EUTRA.receive(cmr_EUTRA_NR_OctetData( "Complete")) -> value v_ReceivedMsg { f_NR_CheckReceived_RRCReconfigurationComplete(v_ReceivedMsg); v_RRC_MsgRcvd := true; if (v_DRB_Rcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [not v_DRB_Rcvd] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, v_DRBId2)) { v_DRB_Rcvd := true; if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {repeat;} } [] t_Watchdog.timeout { if (v_RRC_MsgRcvd) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");} else {f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE is not sending back ReconfigurationComplete ");} } }; //@sic steps 11-14 sic@ //@sic R5s220053 sic@ WA# f_TC_7_1_1_3_2b_NR_TestBody_Loop(v_DRBId1,v_DRBId2,v_DRBId3, 13,1,50,40,0,4160,0,16000,cs_NR_DciCommon_TimeDomainResourceAssignment('1'B),true); //T1 =0 to C-RNTI grant. }; f_NR_TestBody_Set(false); // Send Trigger to EUTRA indicating Finish of Test Body f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait from coordination AFTER the connection is released in E-UTRA f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); //Postamble f_NR_ReleaseAllCells(); }