3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240575

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3534 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_TC_8_1_1_4_x_NR5GC_TestBody |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, LTE_NR_DC_CA_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current implementation of the function f_TC_8_1_1_4_x_NR5GC_TestBody, - At step 3 , when sending RRCResume message , mac bearer routing is not considered - At step 4 , when receiving RRCResumeComplete , mac bearer routing is not considered - After receiving the resume complete , Reconfigure SS to DCI_1_1 needs to be done with the CNF expected so that , the configuration is for sure applied before sending the IP Data on Step 5 For the above points, we need to import some of the modules in the file RRC_Resume_CA_NR5GC.ttcn |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Mac Bearer Routing is added while sending RRCResume Message - Mac Bearer Routing is added while receiving RRCResumeComplete message - Reconfigured SS to DCI_1_1 is done with CNF Expected , so that TTCN can send IP packet once the DCI_1_1 reconfiguration is completed Extra Modules are imported in the file RRC_Resume_CA_NR5GC.ttcn which are required for the above changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.4.4.NR5GC, 8.1.1.4.5.NR5GC and 8.1.1.4.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change to the Function f_TC_8_1_1_4_x_NR5GC_TestBody	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change to the Function f_TC_8_1_1_4_x_NR5GC_TestBody

| Function name | f_TC_8_1_1_4_x_NR5GC_TestBody |
| --- | --- |
| Reason for change | In the current implementation of the function f_TC_8_1_1_4_x_NR5GC_TestBody, - At step 3 , when sending RRCResume message , mac bearer routing is not considered - At step 4 , when receiving RRCResumeComplete , mac bearer routing is not considered - After receiving the resume complete , Reconfigure SS to DCI_1_1 needs to be done with the CNF expected so that , the configuration is for sure applied before sending the IP Data on Step 5 For the above points, we need to import some of the modules in the file RRC_Resume_CA_NR5GC.ttcn |
| Summary of change | - Mac Bearer Routing is added while sending RRCResume Message - Mac Bearer Routing is added while receiving RRCResumeComplete message - Reconfigured SS to DCI_1_1 is done with CNF Expected , so that TTCN can send IP packet once the DCI_1_1 reconfiguration is completed Extra Modules are imported in the file RRC_Resume_CA_NR5GC.ttcn which are required for the above changes |
| TTCN module | RRC_Resume_CA_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_1_4_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template (value) CellGroupConfig v_SCGConfigCA := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_NR_SCellIndex_1, -, -, -, -, activated); var template (value) CellGroupConfig v_CellGroupConfig; var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfo; //Prepare variables //@sic R5s240525 sic@ v_CellGroupConfig := f_NR_GetSCGConfigResume(p_PCell); // condition RESUME v_CellGroupConfig.sCellToAddModList := v_SCGConfigCA.sCellToAddModList; // condition SCell_Add //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(p_PCell, RRC_INACTIVE); //@siclog "Step 2" siclog@ //The UE transmits an RRCResumeRequest message. f_NR_RRC_ResumeRequest_Def(p_PCell); //Configure SS with CA v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); f_NR_ConfigPCell_AddRelSCells (p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_CellGroupConfig, cs_TimingInfo_NR(v_TimingInfo2)); //@siclog "Step 3" siclog@ //The SS transmits an RRCResume message f_NR_SendRRCResume_Def(p_PCell, cs_TimingInfo_NR(v_TimingInfo), -, v_CellGroupConfig); //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCResumeComplete message? f_NR_RRCResumeComplete_Def(p_PCell, cr_38508_RRCResumeComplete); //@sic R5s240461 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //@siclog "Step 5" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB1 and SCell? f_NR5GC_CheckDataPath_CA(p_PCell, p_SCell, "Step 5", false ); //@sic R5s240525 sic@ //@siclog "Steps 5-6" siclog@ //@sic R5-244450 sic@ //The SS transmits an RRCReconfigurationmessage containing an sCellToReleaseList with SCell NR Cell 3. //The UE transmit an RRCReconfigurationComplete message? f_NR5GC_Rel_1SCell(p_PCell, p_SCell); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); }

After Change:

import from NR_ASP_Templates_SRB all; //WA#WI=1289070 import from NR_ASP_Templates_CommonPart all; //WA#WI=1289070 import from NR_DCI_Configuration all; //WA#WI=1289070 function f_TC_8_1_1_4_x_NR5GC_TestBody(NR_CellId_Type p_PCell, NR_CellId_Type p_SCell) runs on NR5GC_PTC { var template (value) CellGroupConfig v_SCGConfigCA := f_NR_GetSCGConfig_SCellAdd(p_SCell, tsc_NR_SCellIndex_1, -, -, -, -, activated); var template (value) CellGroupConfig v_CellGroupConfig; var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfo; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_PCell); //WA#WI=1289070 //Prepare variables //@sic R5s240525 sic@ v_CellGroupConfig := f_NR_GetSCGConfigResume(p_PCell); // condition RESUME v_CellGroupConfig.sCellToAddModList := v_SCGConfigCA.sCellToAddModList; // condition SCell_Add //@siclog "Step 1" siclog@ //The SS transmits a Paging message including a matched identity (correct fullI-RNTI). f_NR_Paging_Def(p_PCell, RRC_INACTIVE); //@siclog "Step 2" siclog@ //The UE transmits an RRCResumeRequest message. f_NR_RRC_ResumeRequest_Def(p_PCell); //Configure SS with CA v_TimingInfo := f_NR_GetNextSendOccasion_DL(p_PCell); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo, 5); f_NR_ConfigPCell_AddRelSCells (p_PCell, {p_SCell}, tsc_PCellIndex_0, cs_TimingInfo_NR(v_TimingInfo2), f_NR_CellInfo_GetRNTI(p_PCell)); f_NR_ConfigSCell(p_SCell,v_CellGroupConfig, cs_TimingInfo_NR(v_TimingInfo2)); //@siclog "Step 3" siclog@ //The SS transmits an RRCResume message f_NR_SendRRCResume_Def(p_PCell, cs_TimingInfo_NR(v_TimingInfo), -, v_CellGroupConfig,-,-,-,crs_MacBearerRouting(p_PCell)); //WA#WI=1289070 //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCResumeComplete message? //f_NR_RRCResumeComplete_Def(p_PCell, cr_38508_RRCResumeComplete); //@sic R5s240461 sic@ SRB.receive(car_NR_SRB1_RrcPdu_IND(p_PCell, cr_38508_RRCResumeComplete,-,crs_MacBearerRouting(p_PCell))); //WA#WI=1289070 f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //f_NR_SS_ReconfigSStoUSS_DCI1_1(nr_Cell1); //WA#WI=1289070 f_NR_SS_CommonCellConfig(p_PCell, cas_NR_SearchSpaceDlDciAssignment_REQ(p_PCell, cs_TimingInfo_Now, f_NR_SearchSpaceDlDciAssignment_USS(v_NR_PhysicalParameters))); //WA#WI=1289070 //@siclog "Step 5" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB1 and SCell? f_NR5GC_CheckDataPath_CA(p_PCell, p_SCell, "Step 5", false ); //@sic R5s240525 sic@ //@siclog "Steps 5-6" siclog@ //@sic R5-244450 sic@ //The SS transmits an RRCReconfigurationmessage containing an sCellToReleaseList with SCell NR Cell 3. //The UE transmit an RRCReconfigurationComplete message? f_NR5GC_Rel_1SCell(p_PCell, p_SCell); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); }