3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240579

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4856 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT NTN testcase 22.3.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Default value of ul-SyncValidityDuration-r17 IE in SIB31-NB is s240 and timer T317 is started with this value up on reception of SIB31-NB. When T317 expires, RRC shall inform lower layers that the UL synchronisation is lost and uplink transmissions are not performed till UL synchronization is restored. Since TC 22.3.1.6 involves verifying different transport block sizes, the test duration exceeds 4 minutes (s240) and therefore it is proposed to increase this to s900 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced new function f_GetTestcaseAttrib_NbiotNtnMaxUlSync to determine if the test case requires max uplink sync valid duration and updated function f_NBIOT_CellInfo_InitSIBs to assign v_UL_SyncValidityDuration_r17 based on this attribute |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for test case 22.3.1.6	2

2.1	Change to f_NBIOT_CellInfo_InitSIBs	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for test case 22.3.1.6

## Change to f_NBIOT_CellInfo_InitSIBs

| Function name | f_NBIOT_CellInfo_InitSIBs |
| --- | --- |
| Reason for change | Default value of ul-SyncValidityDuration-r17 IE in SIB31-NB is s240 and timer T317 is started with this value up on reception of SIB31-NB. When T317 expires, RRC shall inform lower layers that the UL synchronisation is lost and uplink transmissions are not performed till UL synchronization is restored. Since TC 22.3.1.6 involves verifying different transport block sizes, the test duration exceeds 4 minutes (s240) and therefore it is proposed to increase this to s900 |
| Summary of change | Introduced new function f_GetTestcaseAttrib_NbiotNtnMaxUlSync to determine if the test case requires max uplink sync valid duration and updated function f_NBIOT_CellInfo_InitSIBs to assign v_UL_SyncValidityDuration_r17 based on this attribute |
| TTCN module | NBIOT_CellInfo |
| MCC160 Comment |  |

Before Change:

function f_NBIOT_CellInfo_InitSIBs(BlockTypeNumbersList_Type p_ListOfBlockTypeNums, NBIOT_CellId_Type p_CellId, integer p_FrequencyBand, NB_OperationMode_Type p_OperationMode, EUTRA_NAS_NumOfPLMN_Type p_CellsOnPLMN) runs on NBIOT_PTC return template (value) NB_SI_List_Type { var template (value) NB_SI_List_Type v_SI_List; var template (value) SystemInformation_NB_r13_IEs v_SIB; var integer v_Index; var integer v_IndexBckp := -1; var NB_CarrierFreq_Type v_CarrierFreq := fl_NBIOT_Get_CarrierFreq(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN); // @sic R5-185113 sic@ var NTN_Satellite_Type v_SatelliteType := f_NBIOT_MobileInfo_GetNTNSatelliteType(); for (v_Index:=0; v_Index<lengthof(p_ListOfBlockTypeNums); v_Index:= v_Index + 1) { select (p_ListOfBlockTypeNums[v_Index]) { // blocktype to initialise case (2) { // BlockType2 v_SIB := cs_SI_NB_SIB2(f_NBIOT_InitSystemInformation2(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN)); if(v_SatelliteType == ntn_GSO){v_SIB.sib_TypeAndInfo_r13[0].sib2_r13.radioResourceConfigCommon_r13.ntn_ConfigCommon_r17 := cs_508_NTN_ConfigCommon_r17;} else if(v_SatelliteType == ntn_NGSO){v_SIB.sib_TypeAndInfo_r13[0].sib2_r13.radioResourceConfigCommon_r13.ntn_ConfigCommon_r17 := cs_508_NTN_ConfigCommon_r17_NGSO;} //@sic R5w240103 sic@ } case (3) { // BlockType3 v_SIB := cs_SI_NB_SIB3(cs_508_SystemInformationBlockType3_NB_Def); } case (4) { // BlockType4 v_SIB := cs_SI_NB_SIB4(cs_508_SystemInformationBlockType4_NB_Def); } case (5) { // BlockType5 v_SIB := cs_SI_NB_SIB5(f_NBIOT_InitSystemInformation5(p_CellId, p_FrequencyBand, p_OperationMode)); } case (14) { // BlockType14 v_SIB := cs_SI_NB_SIB14(cs_SystemInformationBlockType14_NB_Def); } case (20) { // BlockType20 v_SIB := cs_SI_NB_SIB20(cs_508_SystemInformationBlockType20_NB_Def); } case (22) { // BlockType22 v_SIB := cs_SI_NB_SIB22(cs_508_SystemInformationBlockType22_NB_Def(cs_DL_CarrierConfigCommon_NB_r14(v_CarrierFreq.DL.carrierFreq_r13, v_CarrierFreq.DL.carrierFreqOffset_r13), v_CarrierFreq.UL)); } case (23) { // BlockType23 v_SIB := cs_SI_NB_SIB23(cs_508_SystemInformationBlockType23_NB_Def); } case (31) { // BlockType31 if (p_ListOfBlockTypeNums == {2,3,31}) //a special order due to SIB2 and SIB31 being concatenated) { v_IndexBckp := v_Index; v_Index := 0; v_SIB := cs_SI_NB_SIB2_and_SIB31(f_NBIOT_InitSystemInformation2(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN),f_NBIOT_InitSIB31().sib_TypeAndInfo_r13[0].sib31_v1700); // @sic R5-235468 sic@ } else { v_SIB := f_NBIOT_InitSIB31(); } }

After Change:

function f_NBIOT_CellInfo_InitSIBs(BlockTypeNumbersList_Type p_ListOfBlockTypeNums, NBIOT_CellId_Type p_CellId, integer p_FrequencyBand, NB_OperationMode_Type p_OperationMode, EUTRA_NAS_NumOfPLMN_Type p_CellsOnPLMN) runs on NBIOT_PTC return template (value) NB_SI_List_Type { var template (value) NB_SI_List_Type v_SI_List; var template (value) SystemInformation_NB_r13_IEs v_SIB; var integer v_Index; var integer v_IndexBckp := -1; var NB_CarrierFreq_Type v_CarrierFreq := fl_NBIOT_Get_CarrierFreq(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN); // @sic R5-185113 sic@ var NTN_Satellite_Type v_SatelliteType := f_NBIOT_MobileInfo_GetNTNSatelliteType(); var template (omit) SystemInformationBlockType31_NB_r17.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 v_UL_SyncValidityDuration_r17 :=omit; for (v_Index:=0; v_Index<lengthof(p_ListOfBlockTypeNums); v_Index:= v_Index + 1) { select (p_ListOfBlockTypeNums[v_Index]) { // blocktype to initialise case (2) { // BlockType2 v_SIB := cs_SI_NB_SIB2(f_NBIOT_InitSystemInformation2(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN)); if(v_SatelliteType == ntn_GSO){v_SIB.sib_TypeAndInfo_r13[0].sib2_r13.radioResourceConfigCommon_r13.ntn_ConfigCommon_r17 := cs_508_NTN_ConfigCommon_r17;} else if(v_SatelliteType == ntn_NGSO){v_SIB.sib_TypeAndInfo_r13[0].sib2_r13.radioResourceConfigCommon_r13.ntn_ConfigCommon_r17 := cs_508_NTN_ConfigCommon_r17_NGSO;} //@sic R5w240103 sic@ } case (3) { // BlockType3 v_SIB := cs_SI_NB_SIB3(cs_508_SystemInformationBlockType3_NB_Def); } case (4) { // BlockType4 v_SIB := cs_SI_NB_SIB4(cs_508_SystemInformationBlockType4_NB_Def); } case (5) { // BlockType5 v_SIB := cs_SI_NB_SIB5(f_NBIOT_InitSystemInformation5(p_CellId, p_FrequencyBand, p_OperationMode)); } case (14) { // BlockType14 v_SIB := cs_SI_NB_SIB14(cs_SystemInformationBlockType14_NB_Def); } case (20) { // BlockType20 v_SIB := cs_SI_NB_SIB20(cs_508_SystemInformationBlockType20_NB_Def); } case (22) { // BlockType22 v_SIB := cs_SI_NB_SIB22(cs_508_SystemInformationBlockType22_NB_Def(cs_DL_CarrierConfigCommon_NB_r14(v_CarrierFreq.DL.carrierFreq_r13, v_CarrierFreq.DL.carrierFreqOffset_r13), v_CarrierFreq.UL)); } case (23) { // BlockType23 v_SIB := cs_SI_NB_SIB23(cs_508_SystemInformationBlockType23_NB_Def); } case (31) { // BlockType31 if (f_GetTestcaseAttrib_NbiotNtnMaxUlSync(testcasename())) // v_UL_SyncValidityDuration added {v_UL_SyncValidityDuration_r17 :=s900} if (p_ListOfBlockTypeNums == {2,3,31}) //a special order due to SIB2 and SIB31 being concatenated) { v_IndexBckp := v_Index; v_Index := 0; v_SIB := cs_SI_NB_SIB2_and_SIB31(f_NBIOT_InitSystemInformation2(p_CellId, p_FrequencyBand, p_OperationMode, p_CellsOnPLMN),f_NBIOT_InitSIB31(v_UL_SyncValidityDuration_r17).sib_TypeAndInfo_r13[0].sib31_v1700); // @sic R5-235468 sic@ // v_UL_SyncValidityDuration added } else { v_SIB := f_NBIOT_InitSIB31(v_UL_SyncValidityDuration_r17); // v_UL_SyncValidityDuration added } }

New Function:

function f_GetTestcaseAttrib_NbiotNtnMaxUlSync(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_22_3_1_6") { return true; } } return false; }