3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240580

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3538 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 6.3.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eCPSOR_CON-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation of function fl_TC_6_3_2_2_TestBody : - v_SOR_CMCIRule := cs_SOR_CMCIRule (v_TsorcmTimer, v_CriterionType, omit); is initialized. However, in the template definition of cs_SOR_CMCIRule, the IEL uses lengthof(p_CriterionValue), and p_CriterionValue is set s OMIT. This causes a runtime error. This needs to be fixed. - v_SOR_Data is initialized, and one of the arguments passed is cs_SOR_CMCI({v_SOR_CMCIRule}). This results in the IEL within template cs_SOR_CMCI being set to 1, as lengthof (p_CMCIList) will result in returning number of elements in the list, not the length of the element itself. This also needs to be corrected. In the TTCN implementation of function f_NG_DecodeAndMatchSORTransportContainer, instead of matching the Ie Header within SORContainer, the whole SORContainer is being matched. However as per the Prose, only the header should be checked. This also needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the definition of templates cs_SOR_CMCIRule and cs_SOR_CMCI. Also updated the implementation of function f_NG_DecodeAndMatchSORTransportContainer Please see below for detailed change description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the tetstcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	8

2.3	Change 3	9

2.4	Change 4	10

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_6_3_2_2_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation of function fl_TC_6_3_2_2_TestBody : - v_SOR_CMCIRule := cs_SOR_CMCIRule (v_TsorcmTimer, v_CriterionType, omit); is initialized. However, in the template definition of cs_SOR_CMCIRule, the IEL uses lengthof(p_CriterionValue), and p_CriterionValue is set s OMIT. This causes a runtime error. This needs to be fixed. - v_SOR_Data is initialized, and one of the arguments passed is cs_SOR_CMCI({v_SOR_CMCIRule}). This results in the IEL within template cs_SOR_CMCI being set to 1, as lengthof (p_CMCIList) will result in returning number of elements in the list, not the length of the element itself. This also needs to be corrected. |
| Summary of change | - Declared a variable v_Length and set it to correct value for template cs_SOR_CMCIRule - Updated the definition of template cs_SOR_CMCIRule to accept another parameter for IEL. Passed the variable v_Length to this template - Updated the definition of template cs_SOR_CMCI to accept another parameter for IEL. Passed the total length of the SOR_CMCI to this template. |
| TTCN module | SOR_CMCI_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> var O2_Type v_Counter :='0001'O; //var O1_Type v_Octet_O :='03'O; // 24.501 Table 9.11.3.51.2A Octet O (Bits 8-4 are spare, SSSI = '0'B, SSCMI = '1'B, SI = '1'B) var template (value) SOR_Header v_Header := cs_NG_SOR_HeaderType0('1'B,'1'B,'1'B,'0'B,'1'B); //@sic R5s240259, R5s240531 sic@ var octetstring v_CriterionType:= '05'O; //@sic R5s240259 sic@ var octetstring v_TsorcmTimer:= '21'O; //@sic R5s240259 sic@ var O1_Type v_Length_PlmnID_AccessTechId := int2oct(lengthof(v_PlmnID & v_AccessTechId),1); //@sic R5s240259 sic@ var template (value) SOR_DataType0WithAddInfo v_SOR_Data; var template (value) SOR_CMCIRule v_SOR_CMCIRule; timer t_Tmax; timer t_Tsor_cm := 60.0; timer t_T1 := 10.0; //@sic R5-242145 sic@ //@siclog "Step 22 -34" siclog@ //FFS : Prose CR needed // Steps 1 to step 13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.15.2.2-1 take place. f_UT_RequestIMSCall(UT); f_NR_RRC_SetupRequest_Def(nr_Cell12, mo_VoiceCall); //@sic R5s240259 sic@ v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, //@sic R5s240259 sic@ cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell13)), '0001'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); //@sic R5s240259 sic@ v_GSM_MobilityInfo:= f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); //Now wait for trigger from IMS PTC to add voice f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Add Voice Bearer"); // Now Modify the PDU Session to add voice QoS f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell12, valueof(v_GSM_MobilityInfo)); //@sic R5s240259 sic@ if (not pc_Preconditions) { f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Voice Bearer Added")); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Voice Call Established"); //@siclog "Step 35" siclog@ /*The SS transmits an DLInformationTransfer message containing steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception*/ v_SOR_CMCIRule := cs_SOR_CMCIRule (v_TsorcmTimer, v_CriterionType, omit); v_SOR_Data := cs_SOR_DataWithAddInfo(v_Length_PlmnID_AccessTechId, {cs_SOR_PLMN_AccessTech(v_PlmnID, v_AccessTechId)}, '0'B, '0'B, '1'B, '1'B, // octet O = '03' cs_SOR_CMCI({v_SOR_CMCIRule})); v_Data := bit2oct(encvalue(v_SOR_Data)); // @sic R5s240531 sic@ //v_Data := v_Length_PlmnID_AccessTechId & v_PlmnID & v_AccessTechId & v_Octet_O & v_Length_SOR_CMCI_contents & v_Length_SOR_CMCI_rule_contents & v_TsorcmTimer & v_CriterionType; //@sic R5s240259 sic@ v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_Counter, v_Data, v_NAS_SecurityParams.KDF); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> var O2_Type v_Counter :='0001'O; //var O1_Type v_Octet_O :='03'O; // 24.501 Table 9.11.3.51.2A Octet O (Bits 8-4 are spare, SSSI = '0'B, SSCMI = '1'B, SI = '1'B) var template (value) SOR_Header v_Header := cs_NG_SOR_HeaderType0('1'B,'1'B,'1'B,'0'B,'1'B); //@sic R5s240259, R5s240531 sic@ var octetstring v_CriterionType:= '05'O; //@sic R5s240259 sic@ var octetstring v_Length; //WA#WI1335182 WA#6_3_2_2 var octetstring v_TsorcmTimer:= '21'O; //@sic R5s240259 sic@ var O1_Type v_Length_PlmnID_AccessTechId := int2oct(lengthof(v_PlmnID & v_AccessTechId),1); //@sic R5s240259 sic@ var template (value) SOR_DataType0WithAddInfo v_SOR_Data; var template (value) SOR_CMCIRule v_SOR_CMCIRule; timer t_Tmax; timer t_Tsor_cm := 60.0; timer t_T1 := 10.0; //@sic R5-242145 sic@ //@siclog "Step 22 -34" siclog@ //FFS : Prose CR needed // Steps 1 to step 13 from test procedure for IMS MO speech call establishment as described in TS 38.508-1 [4] Table 4.9.15.2.2-1 take place. f_UT_RequestIMSCall(UT); f_NR_RRC_SetupRequest_Def(nr_Cell12, mo_VoiceCall); //@sic R5s240259 sic@ v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, //@sic R5s240259 sic@ cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected ); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell13)), '0001'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); //@sic R5s240259 sic@ v_GSM_MobilityInfo:= f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); //Now wait for trigger from IMS PTC to add voice f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Add Voice Bearer"); // Now Modify the PDU Session to add voice QoS f_NR5GC_ModifyPDUSession_AddVoice(nr_Cell12, valueof(v_GSM_MobilityInfo)); //@sic R5s240259 sic@ if (not pc_Preconditions) { f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IMS_IPCAN_Trigger("Voice Bearer Added")); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Voice Call Established"); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "Voice Call Established"); //@siclog "Step 35" siclog@ /*The SS transmits an DLInformationTransfer message containing steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception*/ v_Length := int2oct((lengthof(v_CriterionType) + lengthof(v_TsorcmTimer)),2); //WA#WI1335182 WA#6_3_2_2 v_SOR_CMCIRule := cs_SOR_CMCIRule (v_TsorcmTimer, v_CriterionType, omit, v_Length); v_SOR_Data := cs_SOR_DataWithAddInfo(v_Length_PlmnID_AccessTechId, {cs_SOR_PLMN_AccessTech(v_PlmnID, v_AccessTechId)}, '0'B, '0'B, '1'B, '1'B, // octet O = '03' cs_SOR_CMCI({v_SOR_CMCIRule}, int2oct( (lengthof(v_CriterionType) + lengthof(v_TsorcmTimer) + 2),2))); v_Data := bit2oct(encvalue(v_SOR_Data)); // @sic R5s240531 sic@ //v_Data := v_Length_PlmnID_AccessTechId & v_PlmnID & v_AccessTechId & v_Octet_O & v_Length_SOR_CMCI_contents & v_Length_SOR_CMCI_rule_contents & v_TsorcmTimer & v_CriterionType; //@sic R5s240259 sic@ v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(v_Header)), v_NAS_SecurityParams.KAUSF, v_Counter, v_Data, v_NAS_SecurityParams.KDF); <<SKIPPED CODE>>

## Change 2

| Function name | cs_SOR_CMCIRule |
| --- | --- |
| Reason for change | In the current TTCN implementation of function fl_TC_6_3_2_2_TestBody : v_SOR_CMCIRule := cs_SOR_CMCIRule (v_TsorcmTimer, v_CriterionType, omit); is initialized. However, in the template definition of cs_SOR_CMCIRule, the IEL uses lengthof(p_CriterionValue), and p_CriterionValue is set s OMIT. This causes a runtime error. This needs to be fixed. |
| Summary of change | Updated the definition of template cs_SOR_CMCIRule to accept another parameter for IEL. Passed the variable v_Length to this template |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) SOR_CMCIRule cs_SOR_CMCIRule (O1_Type p_TsorcmTimer, O1_Type p_CriterionType, template (omit) octetstring p_CriterionValue):= { /* @status APPROVED (NR5GC) */ /* 24.501 cl. 9.11.3.51 */ iel := int2oct(lengthof (p_CriterionValue) + 2, 2), tSORcmTimer := p_TsorcmTimer, criterionType := p_CriterionType, criterionValue := p_CriterionValue };

After Change:

template (value) SOR_CMCIRule cs_SOR_CMCIRule (O1_Type p_TsorcmTimer, O1_Type p_CriterionType, template (omit) octetstring p_CriterionValue, octetstring p_len := '00'O):= //WA#WI1335182 WA#6_3_2_2 { /* @status APPROVED (NR5GC) */ /* 24.501 cl. 9.11.3.51 */ //WA#WI1335182 WA#6_3_2_2 COMMENTED OUT iel := int2oct(lengthof (p_CriterionValue) + 2, 2), iel := int2oct(lengthof (p_len), 2), tSORcmTimer := p_TsorcmTimer, criterionType := p_CriterionType, criterionValue := p_CriterionValue };

## Change 3

| Function name | cs_SOR_CMCI |
| --- | --- |
| Reason for change | In the current TTCN implementation of function fl_TC_6_3_2_2_TestBody : v_SOR_Data is initialized, and one of the arguments passed is cs_SOR_CMCI({v_SOR_CMCIRule}). This results in the IEL within template cs_SOR_CMCI being set to 1, as lengthof (p_CMCIList) will result in returning number of elements in the list, not the length of the element itself. This also needs to be corrected. |
| Summary of change | Updated the definition of template cs_SOR_CMCI to accept another parameter for IEL. Passed the total length of the SOR_CMCI to this template. |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

template (value) SOR_CMCI cs_SOR_CMCI (template (value) SOR_CMCIRuleList p_CMCIList) := { /* 24.501 cl. 9.11.3.51 */ /* @status APPROVED (NR5GC) */ iel := int2oct(lengthof (p_CMCIList), 2), list := p_CMCIList };

After Change:

template (value) SOR_CMCI cs_SOR_CMCI (template (value) SOR_CMCIRuleList p_CMCIList, octetstring p_len := '00'O) := { /* 24.501 cl. 9.11.3.51 */ /* @status APPROVED (NR5GC) */ //WA#WI1335182 WA#6_3_2_2 COMMENTED OUT iel := int2oct(lengthof (p_CMCIList), 2), iel := p_len, list := p_CMCIList };

## Change 4

| Function name | fl_TC_6_3_2_2_TestBody |
| --- | --- |
| Reason for change | In the TTCN implementation of function f_NG_DecodeAndMatchSORTransportContainer, instead of matching the Ie Header within SORContainer, the whole SORContainer is being matched. However as per the Prose, only the header should be checked. This also needs to be addressed. |
| Summary of change | Updated the function impelmentation to check for IE Header. Please see below. |
| TTCN module | NR5GC_CommonFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NG_DecodeAndMatchSORTransportContainer (NR_SRB_COMMON_IND p_ReceivedAsp, template SOR_Header p_SORContainerRX := cr_NG_SOR_Header (?, ?, ?, ?)) runs on NR_BASE_PTC { var SORTransparentContainer v_SORContainerRX; var octetstring v_SORPayloadRxBytes; var bitstring v_SORPayloadRx; v_SORPayloadRxBytes := p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.payload.payload; v_SORPayloadRx := oct2bit(v_SORPayloadRxBytes); if (decvalue(v_SORPayloadRx, v_SORContainerRX) != 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "SOR Container Failed"); } // If SOR Header template to be tested if (ispresent(p_SORContainerRX)) { if (not match (v_SORContainerRX, cr_NG_SORTransparentContainer (p_SORContainerRX))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "SOR Header Failed"); } } }

After Change:

function f_NG_DecodeAndMatchSORTransportContainer (NR_SRB_COMMON_IND p_ReceivedAsp, template SOR_Header p_SORContainerRX := cr_NG_SOR_Header (?, ?, ?, ?)) runs on NR_BASE_PTC { var SORTransparentContainer v_SORContainerRX; var octetstring v_SORPayloadRxBytes; var bitstring v_SORPayloadRx; var template SORTransparentContainer v_SORTransparentContainer := cr_NG_SORTransparentContainer (p_SORContainerRX); //WA#WI1335182 WA#6_3_2_2 v_SORPayloadRxBytes := p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.payload.payload; v_SORPayloadRx := oct2bit(v_SORPayloadRxBytes); if (decvalue(v_SORPayloadRx, v_SORContainerRX) != 0) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "SOR Container Failed"); } // If SOR Header template to be tested if (ispresent(p_SORContainerRX)) { //WA#WI1335182 WA#6_3_2_2 if COMMENTED OUT (not match (v_SORContainerRX, cr_NG_SORTransparentContainer (p_SORContainerRX))) { if (not match (v_SORContainerRX.header, v_SORTransparentContainer.header)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "SOR Header Failed"); } } }