3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240581

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4857 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_NBIOT_508Check_CP_ResponseToPagingForMTAccess |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | R5-245649 introduced Step 6 and this is currently part of function f_NBIOT_508Check_CP_ResponseToPagingForMTAccess. Some test cases uses the function f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_5 directly and therefore Step 6 does not get applied. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed Step 6 from f_NBIOT_508Check_CP_ResponseToPagingForMTAccess and moved into f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.4.1 and 22.4.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for test case 22.3.1.6	2

2.1	Change to f_NBIOT_CellInfo_InitSIBs	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for test case 22.4.1 and 22.4.24

## Change 1

| Function name | f_NBIOT_508Check_CP_ResponseToPagingForMTAccess |
| --- | --- |
| Reason for change | R5-245649 introduced Step 6 and this is currently part of function f_NBIOT_508Check_CP_ResponseToPagingForMTAccess. Some test cases uses the function f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_5 directly and therefore Step 6 does not get applied. |
| Summary of change | removed Step 6 |
| TTCN module | NBIOT_CommonProcedures |
| MCC160 Comment |  |

Before Change:

function f_NBIOT_508Check_CP_ResponseToPagingForMTAccess(NBIOT_CellId_Type p_CellId, template (omit) PagingRecordList_NB_r13 p_PagingRecordList := omit) runs on NBIOT_PTC {/*@sic R5-245649 sic@*/ // Step 1 f_NBIOT_UE_Page(p_CellId, p_PagingRecordList); f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_5 (p_CellId); // Step 6 //For NTN access, SS waits for timer 2* TNTN,offset expiry (Note 1) f_Delay(2.0*f_NBIOT_NTN_Get_T_NTNoffset_InSeconds(p_CellId)); //for non-NTN access the delay will result in 0 }

After Change:

function f_NBIOT_508Check_CP_ResponseToPagingForMTAccess(NBIOT_CellId_Type p_CellId, template (omit) PagingRecordList_NB_r13 p_PagingRecordList := omit) runs on NBIOT_PTC {/*@sic R5-245649 sic@*/ // Step 1 f_NBIOT_UE_Page(p_CellId, p_PagingRecordList); f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_5 (p_CellId); // Step 6 // moved into f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_5 //For NTN access, SS waits for timer 2* TNTN,offset expiry (Note 1) // f_Delay(2.0*f_NBIOT_NTN_Get_T_NTNoffset_InSeconds(p_CellId)); //for non-NTN access the delay will result in 0 }

## Change 2

| Function name | f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_5 |
| --- | --- |
| Reason for change | See change 2.1 |
| Summary of change | Moved Step 6 into f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_5 |
| TTCN module | NBIOT_CellInfo |
| MCC160 Comment |  |

Before Change:

function f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_5(NBIOT_CellId_Type p_CellId) runs on NBIOT_PTC { var ExtdProtocolConfigOptions v_PcoToUE; var template (omit) ExtdProtocolConfigOptions v_PcoFromUE := omit; var template (omit) HeaderCompressConfig v_HeaderCompressConfigTX := omit; var template (omit) NAS_UL_Message_Type v_ReturnedValue; var NAS_UL_Message_Type v_PDNConnectivityReq_Rxd; var AccessPointName v_APN; var integer v_PDNIndex := 0; var template (value) PDN_Address v_PDN_Address; if (pc_HCCPCIoT and pc_IP_PDN){ v_HeaderCompressConfigTX := crs_HeaderCompressConfigZeros; } v_ReturnedValue := f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_4 (p_CellId); // @sic R5-169027 sic@ if (isvalue (v_ReturnedValue)) { // @sic R5-169027 sic@ v_PDNConnectivityReq_Rxd := valueof(v_ReturnedValue); v_PDNIndex := f_NBIOT_MobileInfo_GetNoOfPDNsConfigured(); f_NBIOT_MobileInfo_SetAssignedPdnAndPTI (v_PDNIndex, v_PDNConnectivityReq_Rxd.pDN_CONNECTIVITY_REQUEST.pdnType.typeValue, v_PDNConnectivityReq_Rxd.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier); v_PcoFromUE := v_PDNConnectivityReq_Rxd.pDN_CONNECTIVITY_REQUEST.extdProtocolConfigurationOptions; v_PcoToUE := valueof(f_GetDefaultExtdProtocolConfigOptions(v_PcoFromUE)); f_NBIOT_MobileInfo_SetExtPCO(v_PDNIndex, v_PcoFromUE); if (ispresent (v_PDNConnectivityReq_Rxd.pDN_CONNECTIVITY_REQUEST.accessPointName)) { v_APN := v_PDNConnectivityReq_Rxd.pDN_CONNECTIVITY_REQUEST.accessPointName; v_APN.iei := omit; } else { v_APN := valueof(cs_AccessPointName(px_AccessPointName)); } f_NBIOT_MobileInfo_SetAPN (v_PDNIndex, v_APN); // NOTE: APN is not omit anymore v_PDN_Address := f_CIOT_GetPDNAddress(v_PDNConnectivityReq_Rxd.pDN_CONNECTIVITY_REQUEST.pdnType.typeValue, f_CheckExtdPCOforIPallocationViaNas(v_PcoFromUE)); // @sic R5s170597 sic@ // Step 4a1b3a3 SRB.send(cas_NB_SRB_NasPdu_REQ(p_CellId, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_CIOT_ActivateDefEpsBearerContextRequest(tsc_EpsDefaultBearerId, f_NBIOT_MobileInfo_GetPTI(0), v_APN, v_PDN_Address, cs_ExtdPCO (int2oct(oct2int(v_PcoToUE.iel), 2), v_PcoToUE.pco), -, -, -, v_HeaderCompressConfigTX, cs_ControlPlaneInd, cs_ServingPLMNRateControl(int2oct (4, 2)))))); // Step 4a1b3a4 SRB.receive(car_NB_SRB_NasPdu_IND(p_CellId, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cdr_CIOT_ActivateDefEpsBearerContextAccept(tsc_EpsDefaultBearerId)))); if (pc_IP_PDN) { f_NBIOT_DelayForUserPlaneSignalling_CP(p_CellId); } } // Step 5 SRB.send(cas_NB_SRB_NasPdu_REQ(p_CellId, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_SERVICE_ACCEPT))); //The relevant code has been moved to f_NBIOT_508Check_CP_ResponseToPagingForMTAccess /*@sic R5s240230, R5-245649 sic@ */ }

After Change:

function f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_5(NBIOT_CellId_Type p_CellId) runs on NBIOT_PTC { var ExtdProtocolConfigOptions v_PcoToUE; var template (omit) ExtdProtocolConfigOptions v_PcoFromUE := omit; var template (omit) HeaderCompressConfig v_HeaderCompressConfigTX := omit; var template (omit) NAS_UL_Message_Type v_ReturnedValue; var NAS_UL_Message_Type v_PDNConnectivityReq_Rxd; var AccessPointName v_APN; var integer v_PDNIndex := 0; var template (value) PDN_Address v_PDN_Address; if (pc_HCCPCIoT and pc_IP_PDN){ v_HeaderCompressConfigTX := crs_HeaderCompressConfigZeros; } v_ReturnedValue := f_NBIOT_508Check_CP_ResponseToPagingForMTAccess_Step2_4 (p_CellId); // @sic R5-169027 sic@ if (isvalue (v_ReturnedValue)) { // @sic R5-169027 sic@ v_PDNConnectivityReq_Rxd := valueof(v_ReturnedValue); v_PDNIndex := f_NBIOT_MobileInfo_GetNoOfPDNsConfigured(); f_NBIOT_MobileInfo_SetAssignedPdnAndPTI (v_PDNIndex, v_PDNConnectivityReq_Rxd.pDN_CONNECTIVITY_REQUEST.pdnType.typeValue, v_PDNConnectivityReq_Rxd.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier); v_PcoFromUE := v_PDNConnectivityReq_Rxd.pDN_CONNECTIVITY_REQUEST.extdProtocolConfigurationOptions; v_PcoToUE := valueof(f_GetDefaultExtdProtocolConfigOptions(v_PcoFromUE)); f_NBIOT_MobileInfo_SetExtPCO(v_PDNIndex, v_PcoFromUE); if (ispresent (v_PDNConnectivityReq_Rxd.pDN_CONNECTIVITY_REQUEST.accessPointName)) { v_APN := v_PDNConnectivityReq_Rxd.pDN_CONNECTIVITY_REQUEST.accessPointName; v_APN.iei := omit; } else { v_APN := valueof(cs_AccessPointName(px_AccessPointName)); } f_NBIOT_MobileInfo_SetAPN (v_PDNIndex, v_APN); // NOTE: APN is not omit anymore v_PDN_Address := f_CIOT_GetPDNAddress(v_PDNConnectivityReq_Rxd.pDN_CONNECTIVITY_REQUEST.pdnType.typeValue, f_CheckExtdPCOforIPallocationViaNas(v_PcoFromUE)); // @sic R5s170597 sic@ // Step 4a1b3a3 SRB.send(cas_NB_SRB_NasPdu_REQ(p_CellId, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_CIOT_ActivateDefEpsBearerContextRequest(tsc_EpsDefaultBearerId, f_NBIOT_MobileInfo_GetPTI(0), v_APN, v_PDN_Address, cs_ExtdPCO (int2oct(oct2int(v_PcoToUE.iel), 2), v_PcoToUE.pco), -, -, -, v_HeaderCompressConfigTX, cs_ControlPlaneInd, cs_ServingPLMNRateControl(int2oct (4, 2)))))); // Step 4a1b3a4 SRB.receive(car_NB_SRB_NasPdu_IND(p_CellId, tsc_SRB1bis, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cdr_CIOT_ActivateDefEpsBearerContextAccept(tsc_EpsDefaultBearerId)))); if (pc_IP_PDN) { f_NBIOT_DelayForUserPlaneSignalling_CP(p_CellId); } } // Step 5 SRB.send(cas_NB_SRB_NasPdu_REQ(p_CellId, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_SERVICE_ACCEPT))); //The relevant code has been moved to f_NBIOT_508Check_CP_ResponseToPagingForMTAccess /*@sic R5s240230, R5-245649 sic@ */ //put it back here as this function is called directly (22.4.1, 22.4.24) f_Delay(2.0*f_NBIOT_NTN_Get_T_NTNoffset_InSeconds(p_CellId)); //for non-NTN access the delay will result in 0 }