3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240587

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3544 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_Check_Rf_Parameters_NRBand_PICS_Supp |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current implementation of the function fl_Check_Rf_Parameters_NRBand_PICS_Supp, maxNumberCSI_RS_BFD and maxNumberCSI_RS_SSB_CBD are checked without considering pc_maxNumberCSI_RS_BFD and pc_maxNumberCSI_RS_SSB_CBD respectively . This is not in line with R5-245527. It is proposed to check for maxNumberCSI_RS_BFD and maxNumberCSI_RS_SSB_CBD when capabilities pc_maxNumberCSI_RS_BFD and pc_maxNumberCSI_RS_SSB_CBD are set respectively. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Check for maxNumberCSI_RS_BFD and maxNumberCSI_RS_SSB_CBD when capabilities pc_maxNumberCSI_RS_BFD and pc_maxNumberCSI_RS_SSB_CBD are set respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.1.1.ENDC, 8.1.5.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Function fl_Check_Rf_Parameters_NRBand_PICS_Supp	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D24wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Function fl_Check_Rf_Parameters_NRBand_PICS_Supp

| Function name | fl_Check_Rf_Parameters_NRBand_PICS_Supp |
| --- | --- |
| Reason for change | In the current implementation of the function fl_Check_Rf_Parameters_NRBand_PICS_Supp, maxNumberCSI_RS_BFD and maxNumberCSI_RS_SSB_CBD are checked without considering pc_maxNumberCSI_RS_BFD and pc_maxNumberCSI_RS_SSB_CBD respectively . This is not in line with R5-245527. It is proposed to check for maxNumberCSI_RS_BFD and maxNumberCSI_RS_SSB_CBD when capabilities pc_maxNumberCSI_RS_BFD and pc_maxNumberCSI_RS_SSB_CBD are set respectively. |
| Summary of change | Check for maxNumberCSI_RS_BFD and maxNumberCSI_RS_SSB_CBD when capabilities pc_maxNumberCSI_RS_BFD and pc_maxNumberCSI_RS_SSB_CBD are set respectively. |
| TTCN module | Common\NR\NR_CapabilityFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_Check_Rf_Parameters_NRBand_PICS_Supp (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { …… v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters) and isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0])) { // rf_Parameters and rf_Parameters.supportedBandListNR[x] are mandatory, so need to check first for (v_Index := 0; v_Index < lengthof(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR); v_Index:=v_Index+1) { fl_CheckNRBandCapability(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].bandNR); // checks bandNR against capability PICS if (fl_Check_maxNumberCSI_RS_BFD(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index])) { // maxNumberCSI_RS_BFD - @sic R5-245527 sic@ v_Found_maxNumberCSI_RS_BFD := true; } if (fl_Check_maxNumberCSI_RS_SSB_CBD(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index])) { // maxNumberCSI_RS_SSB_CBD - @sic R5-245527 sic@ v_Found_maxNumberCSI_RS_SSB_CBD := true; } if (v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].multipleTCI != supported) { // @sic R5-211500 R5-211501 sic@ f_SetVerdict(fail,__FILE__, __LINE__, "Acc. to TS38.306 cl. 4.2.7.2 field multipleTCI shall be set to supported."); } ………….. }

After Change:

function fl_Check_Rf_Parameters_NRBand_PICS_Supp (UE_NR_Capability p_NRCapability, NR_Configuration_Type p_Configuration_Type) { //@sic R5-194844 R5-201141 R5-201123 R5-225384 R5s221147 R5-227571 R5-227572 R5-227573 R5-231410 R5-232202 R5-233295 R5-235311 R5-236307 R5-245527 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var integer v_Index; var FreqBandIndicatorNR v_Band; var boolean v_Found_maxNumberCSI_RS_BFD := false; // pc_maxNumberCSI_RS_BFD var boolean v_Found_maxNumberCSI_RS_SSB_CBD := false; // pc_maxNumberCSI_RS_SSB_CBD var boolean v_Found_bwp_WithoutRestriction := false; var boolean v_Found_condHandover_r16 := false; var boolean v_Found_condHandoverFailure_r16 := false; var boolean v_Found_condHandoverTwoTriggerEvents_r16 := false; var boolean v_Found_condPSCellChange_r16 := false; var boolean v_Found_sps_r16 := false; // @sic R5-236307 sic@ - pc_multi_sps_r16 var boolean v_Found_txDiversity_r16 := false; // pc_txDiversity_r16 - only specified in NR5GC so far in FR1 var boolean v_Found_ul_Semi_StaticChAccess_r16 := false; // pc_ul_Semi_StaticChAccess_r16 var boolean v_Found_rssi_ChannelOccupancyReporting_r16 := false; // pc_rssi_ChannelOccupancyReporting_r16 var boolean v_Found_cg_SDT_r17 := false; // pc_cg_SDT_r17 - only present in NR5GC var boolean v_Found_eventA4BasedCondHandover_r17 := false; // pc_eventA4BasedCondHandover_r17 var boolean v_HalfDuplexFDD_TypeA_RedCap_r17 := false; // pc_halfDuplexFDD_TypeA_RedCap_r17 - only present in RedCap FDD FR1 var boolean v_Found_maxNumberPUSCH_TypeA_Repetition_r17 := false; // pc_maxNumberPUSCH_TypeA_Repetition_r17 var boolean v_Found_puschTypeA_RepetitionsAvailSlot_r17 := false; // pc_puschTypeA_RepetitionsAvailSlot_r17 var boolean v_Found_tb_ProcessingMultiSlotPUSCH_r17 := false; // pc_tb_ProcessingMultiSlotPUSCH_r17 var boolean v_Found_tb_ProcessingRepMultiSlotPUSCH_r17 := false; // pc_tb_ProcessingRepMultiSlotPUSCH_r17 var boolean v_Found_pusch_RepetitionMsg3_r17 := false; // pc_pusch_RepetitionMsg3_r17 var boolean v_Found_mt_CG_SDT_r18 := false; // pc_mt_CG_SDT_r18 var boolean v_Found_cg_SDT_PeriodicityExt_r18 := false; // pc_cg_SDT_PeriodicityExt_r18 var boolean v_Found_condHandoverWithCandSCG_change_r18 := false; // pc_condHandoverWithCandSCG_change_r18 var boolean v_Found_scellDormancyOutsideActiveTime_r16 := false; var boolean v_Found_interFreqDAPS_r16 := false; var boolean v_BWP_SameNumerology_Set := (pc_bwp_SameNumerology_upto2_FR1_FDD or pc_bwp_SameNumerology_upto2_FR1_TDD or pc_bwp_SameNumerology_upto2_FR2 or pc_bwp_SameNumerology_upto4_FR1_FDD or pc_bwp_SameNumerology_upto4_FR1_TDD or pc_bwp_SameNumerology_upto4_FR2); //@sic R5-230775 sic@ v_Received_NRCapabilityMsg := p_NRCapability; if (isvalue(v_Received_NRCapabilityMsg.rf_Parameters) and isvalue(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[0])) { // rf_Parameters and rf_Parameters.supportedBandListNR[x] are mandatory, so need to check first for (v_Index := 0; v_Index < lengthof(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR); v_Index:=v_Index+1) { fl_CheckNRBandCapability(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index].bandNR); // checks bandNR against capability PICS if(pc_maxNumberCSI_RS_BFD) //WA#WI=1336145 { if (fl_Check_maxNumberCSI_RS_BFD(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index])) { // maxNumberCSI_RS_BFD - @sic R5-245527 sic@ v_Found_maxNumberCSI_RS_BFD := true; } } if(pc_maxNumberCSI_RS_SSB_CBD) //WA#WI=1336145 { if (fl_Check_maxNumberCSI_RS_SSB_CBD(v_Received_NRCapabilityMsg.rf_Parameters.supportedBandListNR[v_Index])) { // maxNumberCSI_RS_SSB_CBD - @sic R5-245527 sic@ v_Found_maxNumberCSI_RS_SSB_CBD := true; } } ………….. }