3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240599

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3552 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC eCall over IMS testcase 11.5.12 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu, ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2024-09-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To verify NR5GC eCall over IMS testcase 11.5.12 using IWD 24wk37 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC eCall over IMS testcase 11.5.12 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required:	3

2.1	Correction to function TC_11_5_12_NR5GC	3

2.2	New function f_TC_11_5_12_NR5GC_IMS1	4

2.3	Correction to function f_TC_11_5_12_NR5GC_IMS2	5

2.4	Correction to function f_EUTRA_508_IMSeCallOnly_Steps3_25	5

2.5	Correction to function f_TC_11_5_12_NR5GC	7

2.6	Correction to function fl_TC_11_5_12_Body	9

2.7	Correction to function f_TC_11_5_12_NR5GC_EUTRA	10

2.8	Correction to function fl_TC_11_5_12_NR5GC_EUTRA_TestBody	11

3	Branches executed	12

4	Execution Log Files	13

4.1	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies.	13

5	References	13

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC eCall over IMS testcase 11.5.12 which is part of the 	iwd-TTCN3-B2024-06_D24wk37.zip test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact: 	Narendra Kalahasti

Narendra.kalahasti@anritsu.com

Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

## Verification Test Summary

Test Case:	11.5.12.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D24wk37

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

Rohde & Schwarz 5G Protocol Conformance Test platform

UE used:	Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies

Verification Status:	PASS

# Corrections required:

## Correction to function TC_11_5_12_NR5GC

| Function name | All the messages of this test case will be send amd received in LTE RAT, initial IMS registration, if it happens, will occur in LTE. Being a NR5GC test case, IMS default handler will expect IMS messages over NR and not LTE. |
| --- | --- |
| Reason for change | Created a new IMS1 PTC to specify that IMS messages will be received over LTE. |
| Summary of change | All the messages of this test case will be send amd received in LTE RAT, initial IMS registration, if it happens, will occur in LTE. Being a NR5GC test case, IMS default handler will expect IMS messages over NR and not LTE. |
| TTCN module | ECall_IRAT_NR5GC_EUTRA |

Before Change:

testcase TC_11_5_12_NR5GC() runs on MTC_NR5GC_IRAT system SYSTEM_NR5GC_IRAT { // @purpose // eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported on 5GS / eCall over EPS var NR5GC_PTC v_NR5GC := null; var EUTRA_Multi5GS_PTC v_EUTRA_Multi5GS := null; var UTRAN_NR5GC_PTC v_UTRAN_NR5GC := null; var GERAN_NR5GC_PTC v_GERAN_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; var WLAN_PTC v_WLAN := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_EUTRA_Multi5GS := EUTRA_Multi5GS_PTC.create("EUTRA_Multi5GS") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC_IRAT(system, v_NR5GC, v_EUTRA_Multi5GS, v_UTRAN_NR5GC, v_GERAN_NR5GC, v_IMS1, v_IMS2, v_WLAN); v_NR5GC.start(f_TC_11_5_12_NR5GC()); v_EUTRA_Multi5GS.start(f_TC_11_5_12_NR5GC_EUTRA()); v_IMS2.start(f_TC_11_5_12_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC_IRAT(t_GuardTimer); }

After Change:

testcase TC_11_5_12_NR5GC() runs on MTC_NR5GC_IRAT system SYSTEM_NR5GC_IRAT { // @purpose // eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported on 5GS / eCall over EPS var NR5GC_PTC v_NR5GC := null; var EUTRA_Multi5GS_PTC v_EUTRA_Multi5GS := null; var UTRAN_NR5GC_PTC v_UTRAN_NR5GC := null; var GERAN_NR5GC_PTC v_GERAN_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; var WLAN_PTC v_WLAN := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_EUTRA_Multi5GS := EUTRA_Multi5GS_PTC.create("EUTRA_Multi5GS") alive; v_IMS1 := IMS_PTC.create("IMS1") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC_IRAT(system, v_NR5GC, v_EUTRA_Multi5GS, v_UTRAN_NR5GC, v_GERAN_NR5GC, v_IMS1, v_IMS2, v_WLAN); v_NR5GC.start(f_TC_11_5_12_NR5GC()); v_EUTRA_Multi5GS.start(f_TC_11_5_12_NR5GC_EUTRA()); v_IMS1.start(f_TC_11_5_12_NR5GC_IMS1()); v_IMS2.start(f_TC_11_5_12_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC_IRAT(t_GuardTimer); }

## New function f_TC_11_5_12_NR5GC_IMS1

| Function name | f_TC_11_5_12_NR5GC_IMS1 |
| --- | --- |
| Reason for change | All the messages of this test case will be send amd received in LTE RAT, initial IMS registration, if it happens, will occur in LTE. Being a NR5GC test case, IMS default handler will expect IMS messages over NR and not LTE. |
| Summary of change | Created a new IMS1 PTC to specify that IMS messages will be received over LTE. |
| TTCN module | ECall_IRAT_NR5GC_IMS |

Before Change:

function f_TC_11_5_12_NR5GC_IMS1() runs on IMS_PTC { f_IMS_PTC_Init(PDN_1); f_IMS_Registration(OtherIPCAN_E); //Coord message when call is hanged f_IMS_IpcanReleaseWithOptionalDeregistrationOtherIPCAN(); } // End of f_TC_11_5_12_IRAT_IMS1()

## Correction to function f_TC_11_5_12_NR5GC_IMS2

| Function name | f_TC_11_5_12_NR5GC_IMS2 |
| --- | --- |
| Reason for change | - As pointed in Correction 2.1, IMS messages will be expected in LTE RAT, this needs to be set in f_IMS_EmergencyRegistration for a proper handling. - Improvement: This change can improve visibility of coordination messages in ttcn3 code. (Change paired with correction 2.8.4) |
| Summary of change | - Used OtherIPCAN_E as IMS messages are expected in LTE. - Added reasoning to IPCAN coord message. |
| TTCN module | ECall_IRAT_NR5GC_IMS |

Before Change:

function f_TC_11_5_12_NR5GC_IMS2() runs on IMS_PTC { var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_2); f_IMS_EmergencyRegistration(); f_ECall_CallSetup_AnnexC47(AUTOMATIC); //f_ECall_MSDUpdate_AnnexC47(); //Prose CR needed ? // @siclog "Step 24" siclog@ // Release eCall over IMS using the generic procedure described in TS 34.229-1 subclause C.33 v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); }

After Change:

function f_TC_11_5_12_NR5GC_IMS2() runs on IMS_PTC { var SipUrl v_ContactUrlUE; f_IMS_PTC_Init(PDN_2); f_IMS_EmergencyRegistration(-, -, OtherIPCAN_E); f_ECall_CallSetup_AnnexC47(AUTOMATIC); // @siclog "Step 24" siclog@ // Release eCall over IMS using the generic procedure described in TS 34.229-1 subclause C.33 v_ContactUrlUE := f_IMS_PTC_ImsInfo_GetContactUrlUE(dialog); f_IMS_CallReleaseMT(v_ContactUrlUE); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN, cms_IMS_IPCAN_Trigger("IMS Call Release Msgs Sent")); }

## Correction to function f_EUTRA_508_IMSeCallOnly_Steps3_25

| Function name | f_EUTRA_508_IMSeCallOnly_Steps3_25 |
| --- | --- |
| Reason for change | v_EPS_TI (EPS transaction Identifier) has a hardcoded value of '01'O, this can cause issues in case a different value is used. This can be corrected checking EPSTI in PDN Connectivity request for later usage. |
| Summary of change | Assigning v_EPS_TI from value included in PDN Connectivity Request. |
| TTCN module | ECall_IRAT_NR5GC_IMS |

Before Change:

function f_EUTRA_508_IMSeCallOnly_Steps3_25(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC { var NAS_MSG_Indication_Type v_NAS_Ind; var ProcedureTransactionIdentifier v_EPS_TI := '01'O; var APNandPCOs_Type v_APNandPCOs; var template (omit) ProtocolConfigOptions v_PcoFromUE := omit; var template (omit) ProtocolConfigOptions v_PcoToUE := omit; var PDN_Index_Type v_PdnIndex := f_EUTRA_InitialRegistrationPdnIndex(firstPDN); f_EUTRA_MobileInfo_SetNetworkFeatureSupport(f_GetDefaultValueNetworkFeatureSupport()); // Step 4 SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_emergency_Access)))); // @sic R5-233152 sic@ v_NAS_Ind := f_EUTRA_InitialRegistration_Step1_4 (p_CellId, PREAMBLE, NORMAL, DEFAULT); // Steps 5 - 16 v_APNandPCOs := f_EUTRA_InitialRegistration_Step5_13 (p_CellId, STATE2_IDLEUPDATE, v_NAS_Ind); v_PcoFromUE := v_NAS_Ind.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions; if (isvalue(v_APNandPCOs.protocolConfigurationOptions)) { v_PcoFromUE := v_APNandPCOs.protocolConfigurationOptions; } v_PcoToUE := f_GetDefaultProtocolConfigOptions (v_PcoFromUE, v_PdnIndex); // Step 17 // The SS transmits an ATTACH ACCEPT message with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message and SS assigns value of 186 minutes for the timer T3412. f_EUTRA_InitialRegistration_Step14_15(p_CellId, NORMAL, // @sic R5-233150 sic@ v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.addUpdateType, v_EPS_TI, f_CheckPCOforIPallocationViaNas (v_PcoFromUE), v_PcoToUE, v_APNandPCOs.accessPointName); // Step 18 // Step 16 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. f_EUTRA_NAS_AttachComplete(p_CellId, f_EUTRA_InitialRegistrationEpsBearerId(firstPDN), noDelay); // @sic R5-233152 sic@ f_EUTRA_508_EstablishIMSEmergencyCall_Steps9_15(p_CellId); }

After Change:

function f_EUTRA_508_IMSeCallOnly_Steps3_25(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC { var NAS_MSG_Indication_Type v_NAS_Ind; var NAS_UL_Message_Type v_PdnConnectivityRequest; var ProcedureTransactionIdentifier v_EPS_TI; var template (omit) ProtocolConfigOptions v_PcoFromUE := omit; var template (omit) ProtocolConfigOptions v_PcoToUE := omit; var PDN_Index_Type v_PdnIndex := f_EUTRA_InitialRegistrationPdnIndex(firstPDN); f_EUTRA_MobileInfo_SetNetworkFeatureSupport(f_GetDefaultValueNetworkFeatureSupport()); // Step 4 SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_emergency_Access)))); // @sic R5-233152 sic@ v_NAS_Ind := f_EUTRA_InitialRegistration_Step1_4 (p_CellId, PREAMBLE, NORMAL, DEFAULT); v_PdnConnectivityRequest := v_NAS_Ind.Pdu.PiggybackedPduList[0].Msg; v_EPS_TI := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier; // Steps 5 - 16 v_APNandPCOs := f_EUTRA_InitialRegistration_Step5_13 (p_CellId, STATE2_IDLEUPDATE, v_NAS_Ind); v_PcoFromUE := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions; if (isvalue(v_APNandPCOs.protocolConfigurationOptions)) { v_PcoFromUE := v_APNandPCOs.protocolConfigurationOptions; } v_PcoToUE := f_GetDefaultProtocolConfigOptions (v_PcoFromUE, v_PdnIndex); // Step 17 // The SS transmits an ATTACH ACCEPT message with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message and SS assigns value of 186 minutes for the timer T3412. f_EUTRA_InitialRegistration_Step14_15(p_CellId, NORMAL, // @sic R5-233150 sic@ v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.addUpdateType, v_EPS_TI, f_CheckPCOforIPallocationViaNas (v_PcoFromUE), v_PcoToUE, v_APNandPCOs.accessPointName); // Step 18 // Step 16 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. f_EUTRA_NAS_AttachComplete(p_CellId, f_EUTRA_InitialRegistrationEpsBearerId(firstPDN), noDelay); // @sic R5-233152 sic@ f_EUTRA_508_EstablishIMSEmergencyCall_Steps9_15(p_CellId); }

## Correction to function f_TC_11_5_12_NR5GC

| Function name | f_TC_11_5_12_NR5GC |
| --- | --- |
| Reason for change | Although being a NR test case, connection and messages are expected to be received in LTE. To simplify changes UE configuration and initialisation (as well as corresponding MMIs) are moved to LTE PTC. |
| Summary of change | Moved code to function f_TC_11_5_12_NR5GC_EUTRA, please check Correction 2.7.2. |
| TTCN module | ECall_IRAT_NR5GC_NR |

Before Change:

function f_TC_11_5_12_NR5GC() runs on NR5GC_PTC { /* eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported on 5GS / eCall over EPS */ f_NR5GC_IRAT_Init(NR_6); f_NR_CellInfo_SetSIB1_eCallOverIMS_Support(nr_Cell1, omit); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); <<MOVED CODE>> //The eCall capable UE is equipped with eCall only enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-24. f_UT_USIM_Insert(UT, "USIM configuration 24 specified in table 6.4.1-24 TS 38.508-1"); <<MOVED CODE>> // Preamble: The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0A); f_NR_TestBody_Set(true); fl_TC_11_5_12_Body(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_DEREGISTERED); } //End of function f_TC_11_5_12_NR5GC()

After Change:

function f_TC_11_5_12_NR5GC() runs on NR5GC_PTC { /* eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported on 5GS / eCall over EPS */ f_NR5GC_IRAT_Init(NR_6); f_NR_CellInfo_SetSIB1_eCallOverIMS_Support(nr_Cell1, omit); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); // Preamble: The UE is in test state 0-A (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0A); f_NR_TestBody_Set(true); fl_TC_11_5_12_Body(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_DEREGISTERED); } //End of function f_TC_11_5_12_NR5GC()

## Correction to function fl_TC_11_5_12_Body

| Function name | fl_TC_11_5_12_Body |
| --- | --- |
| Reason for change | - Although being a NR test case, connection and messages are expected to be received in LTE. To simplify changes UE configuration and initialisation (as well as corresponding MMIs) are moved to LTE PTC. - A reference power needs to be set, in addition simplified how power is set on NR. |
| Summary of change | - Moved code to function f_TC_11_5_12_NR5GC_EUTRA, please check Correction 2.8.3. - Corrected initial power settins on NR at T0. |
| TTCN module | ECall_IRAT_NR5GC_NR |

Before Change:

function fl_TC_11_5_12_Body()runs on NR_BASE_PTC { // - NR Cell 1 as "Serving cell" //@sic R5-235074 sic@ if (f_NR_CellInfo_GetIsFR1(nr_Cell1)) { f_NR_SetCellPower(nr_Cell1, -85, tsc_NR_ServingCellSSS_EPRE_FR2); } else { f_NR_SetCellPower(nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); } <<MOVED CODE>> // @siclog "Step 1" siclog@ // The UE is Switched ON f_UT_SwitchOnUE(UT); // @siclog "Step 2" siclog@ // Wait for 60s for the UE to enter 5GMM-DEREGISTERED.eCALL-INACTIVE state f_Delay(60.0); // @siclog "Step 3" siclog@ // An eCall to URI for test service is initiated. (Note 1) // Note 1: The request to originate a manual eCall may be performed by MMI or AT command. f_UT_InitiateECall(UT, AUTOMATIC); <<MOVED CODE>> f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); } //End of fl_TC_11_5_12_Body()

After Change:

function fl_TC_11_5_12_Body()runs on NR_BASE_PTC { var NR_AbsoluteCellPower_Type v_NR_ServingCellSSS_EPRE_FR1 := -85; // - NR Cell 1 as "Serving cell" f_NR_CellInfo_InitMaxReferencePower(nr_Cell1, v_NR_ServingCellSSS_EPRE_FR1,tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower(nr_Cell1, v_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); } //End of fl_TC_11_5_12_Body()

## Correction to function f_TC_11_5_12_NR5GC_EUTRA

| Function name | f_TC_11_5_12_NR5GC_EUTRA |
| --- | --- |
| Reason for change | - RTP loopback needs to be enabled for the eCall established during this test case. - Although being a NR test case, connection and messages are expected to be received in LTE. To simplify changes UE configuration and initialisation (as well as corresponding MMIs) are moved to LTE PTC. |
| Summary of change | - Added HANDLE_UM_DATA as input parameter to activate RtpRtcpLoopback. - Moved code to function f_TC_11_5_12_NR5GC_EUTRA, please check Correction 2.5. |
| TTCN module | ECall_IRAT_NR5GC_EUTRA |

Before Change:

function f_TC_11_5_12_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { /* eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported on 5GS / eCall over EPS */ f_EUTRA38_IRAT_Init(c31); // E-UTRA Cell 1 as defined in TS 36.508 [7] Table 4.4.2-2. System information combination 31 as defined in TS 36.508 [7], sub-clause 4.4.3.1.1. // Create cell and send out system information f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_11_5_12_NR5GC_EUTRA_TestBody(); f_EUTRA_TestBody_Set(false); // tell the NR to release the resources and tke down the Eutra cell now f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); f_EUTRA_ReleaseAllCells(); } //End of f_TC_11_5_12_NR5GC_EUTRA()

After Change:

function f_TC_11_5_12_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { /* eCall Only mode / 5GS supports IMS voice over PS session / 5GS supports emergency service / eCall over IMS is not supported on 5GS / eCall over EPS */ f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA); // E-UTRA Cell 1 as defined in TS 36.508 [7] Table 4.4.2-2. System information combination 31 as defined in TS 36.508 [7], sub-clause 4.4.3.1.1. // Create cell and send out system information f_EUTRA_CellConfig_Def(eutra_Cell1); //The eCall capable UE is equipped with eCall only enabled USIM configured as per TS 38.508-1 [4] Table 6.4.1-24. f_UT_USIM_Insert(UT, "configuration 24 specified in table 6.4.1-24 TS 38.508-1"); f_EUTRA_TestBody_Set(true); fl_TC_11_5_12_NR5GC_EUTRA_TestBody(); f_EUTRA_TestBody_Set(false); // tell the NR to release the resources and tke down the Eutra cell now f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); f_EUTRA_ReleaseAllCells(); } //End of f_TC_11_5_12_NR5GC_EUTRA()

## Correction to function fl_TC_11_5_12_NR5GC_EUTRA_TestBody

| Function name | fl_TC_11_5_12_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | - f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); seems like an unrelated comment not adding any information. - Correction and simplification on initial power settings for LTE with f_EUTRA38_SetCellPower usage. - Although being a NR test case, connection and messages are expected to be received in LTE. To simplify changes UE configuration and initialisation (as well as corresponding MMIs) are moved to LTE PTC. - f_EUTRA_508_IMSeCallOnly_Steps3_25 can be used in this test case and other 11.5.x.NR5GC to avoid code duplication. - Improvement: This change can improve visibility of coordination messages in ttcn3 code. (Change paired with correction 2.3.2) |
| Summary of change | - Removed not related comment. - Corrected initial power settings on LTE at T0. - Moved code to function f_TC_11_5_12_NR5GC_EUTRA_TestBody, please check Correction 2.6.1. - Used common function to place eCall over LTE. - Added reasoning to IPCAN coord message. |
| TTCN module | ECall_IRAT_NR5GC_EUTRA |

Before Change:

function fl_TC_11_5_12_NR5GC_EUTRA_TestBody()runs on EUTRA_5GS_PTC { var NAS_MSG_Indication_Type v_NAS_Ind; var integer v_T1_RS_EPRE_Cell1_FR1 := -93; var integer v_T1_RS_EPRE_Cell1_FR2 := -96; var template (value) CellPowerList_Type v_CellPowerList_FR1; var template (value) CellPowerList_Type v_CellPowerList_FR2; v_CellPowerList_FR1 := { cs_CellPower(eutra_Cell1, v_T1_RS_EPRE_Cell1_FR1) }; v_CellPowerList_FR2 := { cs_CellPower(eutra_Cell1, v_T1_RS_EPRE_Cell1_FR2) }; //Eutra Cell set to -93 dBm according to post-preamble condition //@sic R5-235074 sic@ if (f_EUTRA38_NR_IsFR1_Get()) { f_EUTRA_SetCellPowerList(v_CellPowerList_FR1); } else { f_EUTRA_SetCellPowerList(v_CellPowerList_FR2); } // @siclog "Steps 4 - 23" siclog@ f_EUTRA_MobileInfo_SetNetworkFeatureSupport(f_GetDefaultValueNetworkFeatureSupport()); v_NAS_Ind := f_EUTRA_InitialRegistration_Step1_4 (eutra_Cell1, PREAMBLE, Combined); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); f_EUTRA_InitialRegistration_Step5_16c1 (eutra_Cell1, STATE2_IDLEUPDATE, Combined, v_NAS_Ind); f_EUTRA_508_EstablishIMSEmergencyCall_Steps9_15(eutra_Cell1); //IMS call has been established, wait for trigger from IMS PTC to release rrc connection f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // @siclog "Step 25" siclog@ //Now release the rrcConnection as the IMS PTC would be releasing the eCall f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, '7'H); // Emergency default bearer hardcoded to 6 and dedicated bearer 7 f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); } // End of fl_TC_11_5_12_NR5GC_EUTRA_TestBody()

After Change:

function fl_TC_11_5_12_NR5GC_EUTRA_TestBody()runs on EUTRA_5GS_PTC { var NAS_MSG_Indication_Type v_NAS_Ind; var AbsoluteCellPower_Type v_AbsoluteCellPowerLevel_FR1 := -93; var AbsoluteCellPower_Type v_AbsoluteCellPowerLevel_FR2 := tsc_ServingCellRS_EPRE_FR2; // First switch cell back on // First switch cell back on f_EUTRA38_SetCellPower(eutra_Cell1, v_AbsoluteCellPowerLevel_FR1, v_AbsoluteCellPowerLevel_FR2); // @siclog "Step 1" siclog@ f_EUTRA_SwitchOnUEandStartIP(eutra_Cell1); // @siclog "Step 2" siclog@ // Wait for 60s for the UE to enter 5GMM-DEREGISTERED.eCALL-INACTIVE state f_Delay(60.0); // @siclog "Step 3" siclog@ // Note 1: The request to originate an automatic eCall may be performed by MMI or AT command. f_UT_InitiateECall(UT, AUTOMATIC); // @siclog "Steps 4" siclog@ // @siclog "Steps 5-23" siclog@ f_EUTRA_508_IMSeCallOnly_Steps3_25(eutra_Cell1); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 4"); //IMS call has been established, wait for trigger from IMS PTC to release rrc connection f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2], "IMS Call Release Msgs Sent"); // @siclog "Step 25" siclog@ //Now release the rrcConnection as the IMS PTC would be releasing the eCall f_EUTRA_DeactivateEPS_BearerContext (eutra_Cell1, '7'H); // Emergency default bearer hardcoded to 6 and dedicated bearer 7 f_EUTRA_RRC_ConnectionRelease(eutra_Cell1); } // End of fl_TC_11_5_12_NR5GC_EUTRA_TestBody()

# Branches executed

The test case was executed on NR5GC band n41 with and using 128-EIA1 Integrity and 128-EEA1 ciphering algorithms.

# Execution Log Files

## Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies.

Snapdragon Auto 5G Modem-RF Gen 2 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR  and Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/ TC_11_5_12_LOG.xml
RnS/ TC_11_5_12_LOG.log

- PICS/PIXIT parameter file:

Anritsu/ TC_11_5_12_PIXIT.txt
RnS/ TC_11_5_12_LOG.log

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s240597: Supporting information for addition of NR5GC eCall over IMS testcase 11.5.12 in FR1 |
| --- | --- |