3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240604

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4850 | rev | 1 | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for LTE EIEI test case 11.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2024-08-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current implementation of eCall 11.3.1 test case presents two issues that needs to be addressed and corrected: - v_EPS_TI (EPS transaction Identifier) has a hardcoded value of '01'O, this can cause issues in case a different value is used. This can be corrected checking EPSTI in PDN Connectivity request for later usage. - This test case presents a particular scenario where emergency calls and normal IMS calls are established, that can cause clashes as different bearers are activated with same Ids, Emergeny Default bearer and IMS dedicated bearer in this case. This can easily solved using a different bearer Id while establishing the IMS Calll. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Assigning v_EPS_TI from value included in PDN Connectivity Request. - Usage of a different Bearer ID for IMS call establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to function fl_TC_11_3_1_Body	2

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2022-09_D24wk24 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Correction to function fl_TC_11_3_1_Body

| Function name | fl_TC_11_3_1_Body |
| --- | --- |
| Reason for change | - 1 v_EPS_TI (EPS transaction Identifier) has a hardcoded value of '01'O, this can cause issues in case a different value is used. This can be corrected checking EPSTI in PDN Connectivity request for later usage. - This test case presents a particular scenario where emergency calls and normal IMS calls are established, that can cause clashes as different bearers are activated with same Ids, Emergeny Default bearer and IMS dedicated bearer in this case. This can easily solved using a different bearer Id while establishing the IMS Calll. |
| Summary of change | 1. Assigning v_EPS_TI from value included in PDN Connectivity Request. 2. Usage of a different Bearer ID for IMS call establishment. |
| TTCN module | IMS_ECall.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_11_3_1_Body()runs on EUTRA_PTC { var GutiParameters_Type v_Guti1Params := f_EUTRA_CellInfo_GetGuti(eutra_CellA); var EPS_BearerIdentity v_EpsDefBearerId2 := tsc_EpsDefaultBearerId2; var ProcedureTransactionIdentifier v_EPS_TI := '01'O; var APNandPCOs_Type v_APNandPCOs; var template (omit) ProtocolConfigOptions v_PcoFromUE := omit; var template (omit) ProtocolConfigOptions v_PcoToUE := omit; var PDN_Index_Type v_PdnIndex := f_EUTRA_InitialRegistrationPdnIndex(firstPDN); var template (value) GPRS_Timer v_T3412Value := cs_GprsTimer_v('010'B, '11111'B); //T3412 value = 186 (6 x 31) mins var template (value) GPRS_Timer v_T3412_TV := cs_GprsTimer_tv('5A'O, '010'B, '11111'B); //T3412 value = 186 (6 x 31) mins var NAS_AttDetValue_Type v_DetachType; var NAS_MSG_Indication_Type v_NasInd; var boolean v_T3444Min_elapsed := false; var boolean v_T3412Min_elapsed := false; timer t_EcallDeregisteredInactive := 120.0; timer t_T3412Min := f_EUTRA_SetTimerToleranceMin(eutra_CellA, nasTimer, 11160.0); // 186 minutes timer t_T3412Max := f_EUTRA_SetTimerToleranceMax(eutra_CellA, nasTimer, 11160.0); // 186 minutes timer t_T3444Max := f_EUTRA_SetTimerToleranceMax(eutra_CellA, nasTimer, 43200.0); // 12 hours timer t_T3444Min := f_EUTRA_SetTimerToleranceMin(eutra_CellA, nasTimer, 43200.0); // 12 hours f_EUTRA_MobileInfo_SetNetworkFeatureSupport(cs_EPS_NwkFtSup_IMSVoice_EMCBS); // @sic R5s180648 sic@ emergency bearer services in S1 mode and IMS voice over PS session in S1 mode are supported //@siclog "Step 1" siclog@ // The UE is switched on. f_EUTRA_SwitchOnUEandStartIP(eutra_CellA); //@siclog "Step 2" siclog@ // Check: Does UE send an RRCConnectionRequest on Cell A within 120 seconds? t_EcallDeregisteredInactive.start; alt { [] t_EcallDeregisteredInactive.timeout {} // Continue the test [] SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_Any)))) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2 - Unexpected RRCConnectionRequest"); } } //@siclog "Step 3" siclog@ // A manual eCall is initiated. f_UT_InitiateECall(UT, MANUAL); //@siclog "Step 3A" siclog@ SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_emergency_Access)))); // @sic R5-231901 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 3A"); //@siclog "Step 3B-4" siclog@ // Steps 4-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. // Now establishment cause has been checked, stepw 5-13 are as per default v_NasInd := f_EUTRA_InitialRegistration_Step1_4 (eutra_CellA, PREAMBLE, NORMAL); // @sic R5-231901 sic@ //@siclog "Step 5-13" siclog@ v_APNandPCOs := f_EUTRA_InitialRegistration_Step5_13(eutra_CellA, STATE2_IDLEUPDATE, v_NasInd); v_PcoFromUE := v_NasInd.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions; if (isvalue(v_APNandPCOs.protocolConfigurationOptions)) { v_PcoFromUE := v_APNandPCOs.protocolConfigurationOptions; } v_PcoToUE := f_GetDefaultProtocolConfigOptions (v_PcoFromUE, v_PdnIndex); //@siclog "Step 14-15" siclog@ // The SS transmits an ATTACH ACCEPT message with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message and SS assigns value of 186 minutes for the timer T3412. f_EUTRA_InitialRegistration_Step14_15(eutra_CellA, NORMAL, // @sic R5-233150 sic@ v_NasInd.Pdu.Msg.aTTACH_REQUEST.addUpdateType, v_EPS_TI, f_CheckPCOforIPallocationViaNas (v_PcoFromUE), v_PcoToUE, v_APNandPCOs.accessPointName, v_T3412Value);// 186 minutes //@siclog "Step 16" siclog@ // Step 16 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. f_EUTRA_NAS_AttachComplete(eutra_CellA, f_EUTRA_InitialRegistrationEpsBearerId(firstPDN), noDelay); // @sic R5-231901 sic@ //@siclog "Step 17-18b1 Void" siclog@ //@siclog "Step 19" siclog@ // Check: Does UE transmit a PDN connectivity REQUEST message to request an additional PDN, with 'Request type' set to 'emergency ('0100'B)'? v_NasInd := f_EUTRA_508_EstablishIMSEmergencyCall_Step9(eutra_CellA); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 19"); //@siclog "Step 20-25" siclog@ // The UE performs Steps 20-25 of the generic test procedure for eCall over IMS establishment in EUTRA: eCall only mode described in TS 36.508 subclause 4.5A.27.3. f_EUTRA_508_EstablishIMSEmergencyCall_Steps10_15(eutra_CellA, v_EpsDefBearerId2, v_NasInd); //@siclog "Step 26-30" siclog@ // Release eCall over IMS using the generic procedure described in TS 34.229-1 [35] subclause C.33 //IMS call has been released. @sic R5-223446 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // The SS deactivates the EPS bearer. [This is the rest of C.33] f_EUTRA_DeactivateEPS_BearerContext (eutra_CellA, tsc_EpsDedicatedBearerId2); //@siclog "Step 31" siclog@ // The SS releases the RRC connection. f_EUTRA_RRC_ConnectionRelease(eutra_CellA); // start after the end of the eCall t_T3444Max.start; t_T3444Min.start; //@siclog "Step 32" siclog@ // The SS waits 1 minute. f_Delay(60.0); //@siclog "Step 33-56" siclog@ // Steps 1 to 24 of the generic test procedure for IMS MT speech call specified in TS 36.508 subclause 4.5A.7.3 // from function f_EUTRA_GenericIMS_MT_SpeechCall(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC f_EUTRA_RbEst_Def(eutra_CellA); //Steps 1-8 // Send trigger to IMS PTC f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); // Steps 9 to 12 are executed in IMS PTC //Wait Trigger from IMS PTC: steps 1-4 have been done f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_ActivateDedicatedEpsBearer_Speech(eutra_CellA, -, -, -, true); // @sic R5s230367 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 56"); //@siclog "Step 57-60" siclog@ // Generic test procedure for MT release of IMS call as described in annex C.33 of TS 34.229-1 [35] takes place. f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //Wait Trigger from IMS PTC: MT release has happened f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); // The SS deactivates the EPS bearer. [This is the rest of C.33] f_EUTRA_DeactivateEPS_BearerContext (eutra_CellA, tsc_EpsDedicatedBearerId); // @sic R5s230367 sic@ <<SKIPPED CODE>>

After Change:

function fl_TC_11_3_1_Body()runs on EUTRA_PTC { var GutiParameters_Type v_Guti1Params := f_EUTRA_CellInfo_GetGuti(eutra_CellA); var EPS_BearerIdentity v_EpsDefBearerId2 := tsc_EpsDefaultBearerId2; var NAS_UL_Message_Type v_PdnConnectivityRequest; var ProcedureTransactionIdentifier v_EPS_TI; var APNandPCOs_Type v_APNandPCOs; var template (omit) ProtocolConfigOptions v_PcoFromUE := omit; var template (omit) ProtocolConfigOptions v_PcoToUE := omit; var PDN_Index_Type v_PdnIndex := f_EUTRA_InitialRegistrationPdnIndex(firstPDN); var template (value) GPRS_Timer v_T3412Value := cs_GprsTimer_v('010'B, '11111'B); //T3412 value = 186 (6 x 31) mins var template (value) GPRS_Timer v_T3412_TV := cs_GprsTimer_tv('5A'O, '010'B, '11111'B); //T3412 value = 186 (6 x 31) mins var NAS_AttDetValue_Type v_DetachType; var NAS_MSG_Indication_Type v_NasInd; var boolean v_T3444Min_elapsed := false; var boolean v_T3412Min_elapsed := false; timer t_EcallDeregisteredInactive := 120.0; timer t_T3412Min := f_EUTRA_SetTimerToleranceMin(eutra_CellA, nasTimer, 11160.0); // 186 minutes timer t_T3412Max := f_EUTRA_SetTimerToleranceMax(eutra_CellA, nasTimer, 11160.0); // 186 minutes timer t_T3444Max := f_EUTRA_SetTimerToleranceMax(eutra_CellA, nasTimer, 43200.0); // 12 hours timer t_T3444Min := f_EUTRA_SetTimerToleranceMin(eutra_CellA, nasTimer, 43200.0); // 12 hours f_EUTRA_MobileInfo_SetNetworkFeatureSupport(cs_EPS_NwkFtSup_IMSVoice_EMCBS); // @sic R5s180648 sic@ emergency bearer services in S1 mode and IMS voice over PS session in S1 mode are supported //@siclog "Step 1" siclog@ // The UE is switched on. f_EUTRA_SwitchOnUEandStartIP(eutra_CellA); //@siclog "Step 2" siclog@ // Check: Does UE send an RRCConnectionRequest on Cell A within 120 seconds? t_EcallDeregisteredInactive.start; alt { [] t_EcallDeregisteredInactive.timeout {} // Continue the test [] SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_Any)))) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2 - Unexpected RRCConnectionRequest"); } } //@siclog "Step 3" siclog@ // A manual eCall is initiated. f_UT_InitiateECall(UT, MANUAL); //@siclog "Step 3A" siclog@ SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_emergency_Access)))); // @sic R5-231901 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 3A"); //@siclog "Step 3B-4" siclog@ // Steps 4-13 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. // Now establishment cause has been checked, stepw 5-13 are as per default v_NasInd := f_EUTRA_InitialRegistration_Step1_4 (eutra_CellA, PREAMBLE, NORMAL); // @sic R5-231901 sic@ v_PdnConnectivityRequest := v_NasInd.Pdu.PiggybackedPduList[0].Msg; v_EPS_TI := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier; //@siclog "Step 5-13" siclog@ v_APNandPCOs := f_EUTRA_InitialRegistration_Step5_13(eutra_CellA, STATE2_IDLEUPDATE, v_NasInd); v_PcoFromUE := v_PdnConnectivityRequest.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions; if (isvalue(v_APNandPCOs.protocolConfigurationOptions)) { v_PcoFromUE := v_APNandPCOs.protocolConfigurationOptions; } v_PcoToUE := f_GetDefaultProtocolConfigOptions (v_PcoFromUE, v_PdnIndex); //@siclog "Step 14-15" siclog@ // The SS transmits an ATTACH ACCEPT message with the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message and SS assigns value of 186 minutes for the timer T3412. f_EUTRA_InitialRegistration_Step14_15(eutra_CellA, NORMAL, // @sic R5-233150 sic@ v_NasInd.Pdu.Msg.aTTACH_REQUEST.addUpdateType, v_EPS_TI, f_CheckPCOforIPallocationViaNas (v_PcoFromUE), v_PcoToUE, v_APNandPCOs.accessPointName, v_T3412Value);// 186 minutes //@siclog "Step 16" siclog@ // Step 16 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. f_EUTRA_NAS_AttachComplete(eutra_CellA, f_EUTRA_InitialRegistrationEpsBearerId(firstPDN), noDelay); // @sic R5-231901 sic@ //@siclog "Step 17-18b1 Void" siclog@ //@siclog "Step 19" siclog@ // Check: Does UE transmit a PDN connectivity REQUEST message to request an additional PDN, with 'Request type' set to 'emergency ('0100'B)'? v_NasInd := f_EUTRA_508_EstablishIMSEmergencyCall_Step9(eutra_CellA); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 19"); //@siclog "Step 20-25" siclog@ // The UE performs Steps 20-25 of the generic test procedure for eCall over IMS establishment in EUTRA: eCall only mode described in TS 36.508 subclause 4.5A.27.3. f_EUTRA_508_EstablishIMSEmergencyCall_Steps10_15(eutra_CellA, v_EpsDefBearerId2, v_NasInd); //@siclog "Step 26-30" siclog@ // Release eCall over IMS using the generic procedure described in TS 34.229-1 [35] subclause C.33 //IMS call has been released. @sic R5-223446 sic@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // The SS deactivates the EPS bearer. [This is the rest of C.33] f_EUTRA_DeactivateEPS_BearerContext (eutra_CellA, tsc_EpsDedicatedBearerId2); //@siclog "Step 31" siclog@ // The SS releases the RRC connection. f_EUTRA_RRC_ConnectionRelease(eutra_CellA); // start after the end of the eCall t_T3444Max.start; t_T3444Min.start; //@siclog "Step 32" siclog@ // The SS waits 1 minute. f_Delay(60.0); //@siclog "Step 33-56" siclog@ // Steps 1 to 24 of the generic test procedure for IMS MT speech call specified in TS 36.508 subclause 4.5A.7.3 // from function f_EUTRA_GenericIMS_MT_SpeechCall(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC f_EUTRA_RbEst_Def(eutra_CellA); //Steps 1-8 // Send trigger to IMS PTC f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); // Steps 9 to 12 are executed in IMS PTC //Wait Trigger from IMS PTC: steps 1-4 have been done f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_ActivateDedicatedEpsBearer_Speech(eutra_CellA, -, tsc_EpsDedicatedBearerId3, -, true); // @sic R5s230367 sic@ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 56"); //@siclog "Step 57-60" siclog@ // Generic test procedure for MT release of IMS call as described in annex C.33 of TS 34.229-1 [35] takes place. f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //Wait Trigger from IMS PTC: MT release has happened f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); // The SS deactivates the EPS bearer. [This is the rest of C.33] f_EUTRA_DeactivateEPS_BearerContext (eutra_CellA, tsc_EpsDedicatedBearerId3); // @sic R5s230367 sic@ <<SKIPPED CODE>>